| U.S. Radiocommunications Sector Fact Sheet |  |
| --- | --- |
| Working Party: ITU-R WP 5B | Document No: USWP5B25-FD-24 |
| Ref: Annex 14 to document 5B/712-E Annex 09 to document 5B/93 | Date: 24 September 2020 |
| Document Title: Update to the working document towards a preliminary draft new Report ITU-R M.[FOD_EESS_SHARE] |  |
| Author: Daniel Bishop NASA Ryan McDonough NASA Joshua Whitlinger NASA (ASTS) | Phone/ Email : 216-433-5220 daniel.w.bishop@nasa.gov 216-433-2862 ryan.s.mcdonough@nasa.gov 216-433-6288 joshua.m.whitlinger@nasa.gov |
| Purpose/Objective: To update the working document towards a draft new Report ITU-R M.[FOD_EESS_SHARE] document. This work will continue previous work on coexistence between Foreign Object Debris (FOD) detection systems operating in the 92-100 GHz band with EESS (passive) service in the 86-92 GHz and the EESS (active) service in the 94-94.1 GHz for radiofrequency interference over Japan. |  |
| Abstract: This study provides will provide analyses of possible radiofrequency interference between the FOD detection system placed along runways in the 92-100 GHz band with EESS (passive) in the adjacent band and EESS (active) in the 94-94.1 GHz band over Japan. This document contains two dynamic analyses of potential in-band interference to a spaceborne cloud profile radar in EESS (active) and OOB interference to a spaceborne radiometer in EESS (passive) in the 86-92 GHz band from FOD detection systems in the Radiolocation Service to be included in the Report. |  |
| Fact Sheet Prepared by: Joshua Whitlinger NASA (ASTS) |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 5B/TEMP/21-E | Annex 9 to Document 5B/93-E |
|  | 18 August 2020 |
|  | English only |
| Annex 9 to Working Party 5B Chairman’s Report |  |
| WORKING DOCUMENT TOWARDS A PRELIMINARY DRAFT NEW REPORT ITU-R M.[FOD_EESS_SHARE] |  |
| Sharing and compatibility studies between earth exploration satellite service sensors and foreign object debris detection system in the frequency range 92-100 GHz |  |

# 1	Introduction

The range resolution of a radar is dependent on the pulse repetition frequency or the bandwidth of radar signals. To achieve a range resolution of less than a few cm, radar bandwidths in the order of GHz are required. Since the band 92-100 GHz is allocated for radiolocation services on a primary basis, the bandwidth of 8 GHz can provide the high range resolution around 2 cm. The study on foreign object debris (FOD) detection systems to utilize such broad bandwidth for airport safety operation is currently conducted by WP 5B and the technical and operational characteristics will be provided by Report ITU-R M.[FOD 92-100 GHZ]. However, FOD detection systems should be deployed to protect a number of incumbent services operating in the band 92-100 GHz and the adjacent bands.

The 94.0-94.1 GHz band is allocated to the Earth exploration satellite service (EESS) (active) on a primary basis for spaceborne active sensors, such as cloud profile radars (CPRs). Herein are presented technical characteristics of two typical EESS (active) cloud profile radar systems operating in the frequency band 94.0-94.1 GHz and technical characteristics for typical FOD detection systems proposed for the frequency range 92-100 GHz. Performance criteria and interference criteria for active spaceborne cloud profile radar operating in the 94.0-94.1 GHz frequency band are provided. Preliminary calculations are made of the attenuation required of the FOD detection systems in-band emission levels in order for those emissions to be below the EESS (active) interference protection criteria level of the most susceptible CPR.

The 86-92 GHz band is allocated to EESS (passive) on a primary basis for EESS (passive) spaceborne passive sensors, such as radiometers. Herein are presented technical characteristics of one typical EESS (passive) radiometer system operating in the frequency band 86-92 GHz and technical characteristics for typical FOD detection systems proposed for the frequency range 92-100 GHz. Performance criteria and interference criteria for passive spaceborne radiometers operating in the 86-92 GHz frequency band are provided. Preliminary calculations are made of the attenuation required of the FOD detection systems out-of-band (OOB) emission levels in order for those emissions to be below the EESS (passive) interference protection criteria level of the most susceptible radiometer.

# 2	Foreign object debris detection system characteristics

Information on 94 GHz band FOD detection systems is provided in section 2 of the Working document toward a preliminary draft new Report ITU-R M.[FOD 92-100 GHz], “Technical and operational characteristics of the FOD detection system operating in the frequency range 92-100 GHz”. The characteristics of the FOD detection systems are summarized in the Table 1 and the spectrum mask and antenna radiation pattern are shown in Figures 2 and 3. The measured data at 92 GHz, 94 GHz, 96 GHz, 98 GHz and 100 GHz is provided by the following table.

Table 1

Technical and operational characteristics of foreign object debris detection system 
operating in the frequency range 92-100 GHz

| Parameters | Units | Values |
| --- | --- | --- |
| Frequency range | GHz | 92….100 |
| Channel plan |  | See Figure 1 |
| Chanele bandwidth | GHz | 0.58…..7.98 |
| Output power | mW | 200 |
| Spectrum envelope | - | See Figure 2 |
| Sweep frequency | kHz | 1.250 |
| Antenna type | - | Cassegrain |
| Antenna gain | dBi | 42 |
| Antenna pattern | - | See Figure 3 Recommendation ITU-R M.1851 |
| Antenna height | m | 6....8 |
| Full width at half maximum | degrees | Elevation: 1.0, Azimuth: 1.0 |
| Antenna rotation speed | rpm | 15 |
| Detection distance | m | 200 |
| Antenna elevation angle | degrees | −1.8 |
| Radiated rotation angle | degrees | ±60 |
| Radar cross section | dB/m2 | -20 |
| Range resolution | cm | 3…..50 |
| Emission bandwidth (–3 dB) | MHz | 1 (see Figure 2 (b)) |
| Emission bandwidth (–20 dB) | MHz | 3.5 (see Figure 2 (b)) |
| Adjacent channel leakage ratio | dBc | < −70 |
| Receiver noise figure | dB | 10 |
| Maximum number of FOD radars per 4-km runway |  | 48 (see Figure 5) |
| I/N protection criteria | dB | [−6 or −10] |

FIGURE 1

Channel plan for foreign object debris detection system operating in the frequency range 92-100 GHz

Figure 2

Measure spectrum of foreign object debris detection system

(a) Spectrum envelope of FWCW signal in the frequency band 92-97 GHz

(b) Adjacent channel leakage ratio at an offset frequency of 10 MHz

Figure 3

Antenna radiation pattern of 42 dBi at 96-GHz

# 3	Technical characteristics of incumbent services

## 3.1	Earth exploration satellite service (active) spaceborne cloud profile radars

Technical characteristics of CPRs in the frequency band of 94.0-94.1 GHz are provided in “Recommendation ITU-R RS.2105 – Typical technical and operational characteristics of Earth exploration-satellite service (active) systems using allocations between 432 MHz and 238 GHz”. The technical characteristics for two CPR systems CPR-L1 and CPR-L2 are presented below in Table 2.

Table 2

Characteristics of Earth exploration satellite service (active) missions in the frequency band 94-94.1 GHz

| Parameter | Units | CPR-L1 | CPR-L2 |
| --- | --- | --- | --- |
| Sensor type |  | Cloud Profiling Radar | Cloud Profiling Radar |
| Type of orbit |  | Sun synchronous orbit | Sun synchronous orbit |
| Altitude | km | 705 | 393 |
| Inclination | degrees | 98.2 | 97 |
| Ascending Node LST |  | 13:30 | 10:30 |
| Repeat period, days |  | 16 | 25 |
| Antenna type |  | Parabolic reflector to Offset cassegrain antenna | Parabolic reflector |
| Antenna diameter |  | 1.85-2.5 m | 2.5 m |
| Antenna (transmit & receive) peak gain | dBi | 63.1-65.2 | 65.2 |
| Polarization |  | linear | LHC, RHC |
| Incidence angle at Earth | degrees | 0 | 0 |
| Azimuth scan rate | rpm | 0 | 0 |
| Antenna beam look angle | degrees | 0 | 0 |
| Antenna beam azimuth angle | degrees | 0 | 0 |
| Antenna elevation. beamwidth | degrees | 0.12 | 0.095 |
| Antenna azimuth. beamwidth | degrees | 0.12 | 0.095 |
| Beamwidth | degrees | 0.095-0.108 | 0.095 |
| RF center frequency | MHz | 94.050 | 94.050 |
| RF bandwidth | MHz | 0.36 | 7 |
| Transmit peak power | W | 1 000 | 1 430 |
| Transmit Average power | W | 21.31 | 28.8 |
| Pulsewidth, | μsec | 3.33 | 3.3 |
| Pulse Repetition Frequency (PRF), | Hz | 4 300 | 6 100-7 500 |
| Chirp rate | MHz/μsec | N/A | 2.1 |
| Transmit duty cycle | % | 1.33 | 2.01 |
| Minimum sensitivity | dBZ | –30 to –35 | –30 to –35 |
| Horizontal resolution |  | 0.7-1.9 km | 800 m |
| Vertical resolution |  | 250-500 m | 500 m |
| Doppler range |  | ±10 m/s | –10 ~ +10 m/s |
| Doppler accuracy |  | 1 m/s | 1 m/s |
| System noise figure | dB | 7 | 7 |

## 3.2	Earth exploration satellite service (passive) spaceborne radiometers

Technical characteristics of spaceborne radiometers in the frequency band of 86-92 GHz are provided in “Recommendation ITU-R RS.1861 – “Typical technical and operational characteristics of Earth exploration-satellite service (passive) systems using allocations between 1.4 and 275 GHz”. The technical characteristics for one spaceborne radiometer system L8 is presented below in Table 3.

Table 3

Characteristics of Earth exploration satellite service (passive) missions in the frequency band 86-92 GHz

| Parameter | Units | L8 |
| --- | --- | --- |
| Sensor type |  | Conical scan radiometer |
| Type of orbit |  | SSO |
| Altitude | km | 700 |
| Inclination | degrees | 98.2 |
| Repeat period |  | 16 |
| Antenna type |  | Parabolic reflector antenna |
| Antenna diameter |  | 2.0 m |
| Antenna (receive) peak gain | dBi | 62.4 |
| Polarization |  | Linear H,V |
| Incidence angle at Earth | degrees | 55 |
| Azimuth scan rate | rpm | 40 |
| Antenna beam look angle | degrees | 47.5 |
| Antenna beam azimuth angle | degrees | 0 to 360 |
| Antenna elevevation beamwidth | degrees | 0.15 |
| Antenna azimuth beamwidth | degrees | 0.15 |
| Beamwidth | degrees | 0.15 |
| RF centre frequency | GHz | 89.0 |
| RF bandwidth | MHz | 3000 |
| Horizontal resolution |  | 2.9 km |
| Vertical resolution |  | 5.1 km |

# 4	Performance criteria of Earth exploration satellite service (active) and Earth exploration satellite service (passive)

Performance requirements for spaceborne active sensors in the EESS (active) are provided by Recommendation ITU-R RS.1166-4 “Performance and interference criteria for active spaceborne sensors”. The performance criteria for the EESS (active) cloud profile radars specifies measurements of a minimum reflectivity of –30 dBz 10%, I/N no greater than –10 dB and random data availability criteria of at least 99.8%.

Performance requirements for spaceborne passive sensors in the EESS (passive) are provided by Recommendation ITU-R RS.2017 – “Performance and interference criteria for satellite passive remote sensing”. The performance criteria for the EESS (passive) radiometers specifies a temperature differential, Te of 0.05 K and data availability criteria of at least 99.99%.

# 5	Interference criteria

## 5.1	Earth exploration satellite service (active) and Earth exploration satellite service (passive)

Interference requirements for spaceborne active sensors in the EESS (active) are given in preliminary draft revision of Recommendation ITU-R RS.1166-4 “Performance and interference criteria for active spaceborne sensors” (Annex 2 of 7C/344). The interference threshold criteria for the EESS (active) CPR in the 94.0-94.1 GHz frequency band is –155 dBW over 300 kHz. The availability criteria is 99%.

[Chairman’s note: The reference to Doc. 7C/344 will need to have been removed in the final document.]

Interference requirements for spaceborne passive sensors in the EESS (passive) are given in Recommendation ITU-R RS.2017 – “Performance and interference criteria for satellite passive remote sensing”. The interference threshold criteria for the EESS (passive) radiometer in the 86-92 GHz frequency band is –169 dBW with a percentage of area or time permissible interference level may be exceeded of 0.01%.

## 5.2	foreign object debris detection system

# 6	Deployment scenarios of foreign object debris detection system

Figure 4 shows the location of major airports in Japan where FOD detection systems may be implemented in the future. The number of FOD implemented in runway is estimated by using Figure 5. The important parameter of the deployment scenario is the separation distance of FOD radars which is 200 m. If the runway length is 4,000 m, the number of FOD radars becomes 48, as shown in Figure 5. FOD radars are deployed at each side of runway. Table 4 summarizes the estimated number of FOD radars which will be implemented in major airports. Those number of FOS radars should be taken into account when interference from FOD radars is studied

Figure 4

Foreign object debris detection systems planned to operate in the airports

Figure 5

Basic concept of implementation of foreign object debris radars in 4-km runway

TABLE 4

Estimated number of foreign object debris radars in each airport

| Airport | No. of runway | Runway length (m) | Estimated number of FOD radars |
| --- | --- | --- | --- |
| CTS1 | 2 | 3 000, 3 000 | 68 |
| FUK2 | 1 | 2 800 | 17 |
| HND3 | 4 | 2 500, 2 500, 3 000, 3 360 | 120 |
| ITM4 | 2 | 1 828, 3 000 | 52 |
| KIX5 | 2 | 3 500, 4 000 | 84 |
| KOJ6 | 1 | 3 000 | 32 |
| NGO7 | 1 | 3 500 | 36 |
| NRT8 | 2 | 2 500, 4 000 | 74 |
| OKA9 | 1 | 3 000 | 32 |
| 1 New Chitose Airport, 2 Fukuoka Airport, 3 Tokyo International Airport (Hneda), 4 Osaka International Airport - Itami Airport, 5 Kansai International Airport, 6 Kagoshima Airport, 7 Chubu Centrair International Airport, 8 Narita International Airport, 9 Naha Airport |  |  |  |

# 7	Sharing and compatibility studies

## 7.1	In-Band emissions of foreign object debris detection systems into Earth exploration satellite service (active)

FOD detection systems are proposed to operate in the 92-100 GHz frequency band. The 94.0-94.1 GHz band which is allocated to the EESS (active) CPR would be in-band with the FOD detection band 92-100 GHz. The maximum in-band emissions occur when the FOD FMCW signal spectrum is positioned over the spectrum of the EESS (active) band.

## 7.2	Compatibility studies for Earth exploration-satellite service (active)

The study results are included in Annex 1. Section A1 contains the static analysis and section A2 contains the dynamic analysis.

## 7.3	Out-of-Band emissions of foreign object debris detection systems into Earth exploration satellite service (passive)

FOD detection systems are proposed to operate in the 92-100 GHz frequency band. The 86-92 GHz band which is allocated to the EESS (passive) spaceborne radiometer would be OOB with respect to the FOD detection band 92-100 GHz. The maximum OOB emissions occur when the FOD FMCW signal spectrum is positioned near the spectrum edge of the EESS (passive) band just above 92 GHz.

## 7.4	Compatibility studies for Earth exploration-satellite service (passive)

The study results are included in Annex 2. Section A1 contains the static analysis and section A2 contains the dynamic analysis.

Annex 1

Interference analysis for Earth exploration satellite service (active)

# A1-1	Interference from foreign object debris detection systems into Earth exploration satellite service (active)

## A1-1.1	Static analysis based on interference with specific geometric scenarios

At least three different geometrical scenarios have to be considered in assessing the potential for interference from FOD detection systems into EESS active systems. The first is coupling of the antenna mainlobe of a nadir-looking EESS (active) satellite with the sidelobes of the FOD detection system antenna. The second geometrical scenario is when coupling occurs between the sidelobes of both the EESS active sensor antenna and the FOD detection system antenna; and the third, and worst case, geometrical scenario is when coupling occurs between the mainbeam of the FOD detection system antenna and the EESS active sensor antenna sidelobes at a time when the EESS active satellite is on the horizon with respect to the FOD detection system.

The peak interfering signal power level, I (dBW), received by a spaceborne radar from a terrestrial source is calculated from:

I    10 log Pt    Gt    Gr  –  (32.44    20 log ( f R ))  –  La – FDRIF	(A1-1)

where:

Pt :	peak terrestrial source transmitter power (W);

Gt :	terrestrial source antenna gain towards spaceborne sensor (dBi);

Gr :	spaceborne radar antenna gain towards terrestrial source (dBi);

f :	frequency (MHz);

R :	slant range between spaceborne sensor and terrestrial source (km);

La :	attenuation due to atmospheric absorption (dB).

FDRIF: 	frequency-dependent rejection produced by the receiver IF selectivity curve on an unwanted transmitter emission spectra (dB).

Attenuation due to atmospheric absorption, La, is dependent upon the path length to the satellite through the Earth’s atmosphere, and hence upon the elevation angle from the terrestrial source to the satellite. At frequencies around 94 GHz, La decreases rapidly from about 100 dB at 0º elevation angles to 1.5 dB at 90º elevation angles. La and FDRIF were not included in the following tables of calculation of the interference levels and margins.

The interference due to CPR antenna mainlobe coupling with the FOD detection system antenna sidelobes allows for the highest value of RFI levels of these three geometrical scenarios. For CPR L-1 and CPR L-2, the CPR L-1 system was found to be more sensitive to interference due to its narrower receiver bandwidth of 300 kHz. For geometric scenario 1 for coupling between the CPR L-1 mainlobe and the FOD radar sidelobes (elevation angle at 90º), preliminary calculations of two Cases examined are provided in Tables 3 (Case 1) and 4 (Case 2) which indicate the amount of attenuation needed to apply to in-band FOD detection systems emissions in order that they meet the EESS (active) interference protection criteria for CPR L-1. The two cases were examined with the FOD detection system antenna gain in the sidelobes at –10 dBi and 14 dBi, and the density of FOD detection systems set at 10 and 50 in the CPR footprint, respectively. The EESS active system noise figure was 7 dB, which resulted in the interference protection criteria threshold of –153.2 dBW. The preliminary calculated attenuations needed to meet the EESS (active) interference criteria, based on the hypothetical scenarios studied, are 22.5 dB for Case 1 and 53.5 dB for Case 2.

For geometric scenario 2 for coupling between the CPR L-1 sidelobes and the FOD detection sidelobes (elevation angle at 45º), preliminary calculations of two Cases examined are provided in Tables 5 (Case 1) and 6 (Case 2) which indicate the amount of attenuation needed to apply to in-band FOD detection systems emissions in order that they meet the EESS (active) interference protection criteria for CPR L-1. The two cases were examined with the FOD detection system antenna gain in the sidelobes at 0 dBi and 14 dBi, and the density of FOD detection systems set at 10 and 50 in the CPR footprint, respectively. The EESS active system noise figure was 7 dB which resulted in the interference protection criteria threshold of -153.2 dBW. The preliminary calculations show that the EESS (active) interference criteria are met with margins of 45.1 dB for the hypothetical Case 1 and 24.1 dB for Case 2.

For geometric scenario 3 for coupling between the CPR L-1 sidelobes and the FOD radar mainlobe (elevation angle at 0º), preliminary calculations of two Cases examined are provided in Tables 7 (Case 1) and 8 (Case 2) which indicate the amount of attenuation needed to apply to in-band FOD detection systems emissions in order that they meet the EESS (active) interference protection criteria for CPR L-1. The two cases were examined with the FOD detection system antenna gain in the sidelobes at 44 dBi, and the density of FOD detection systems set at 10 and 50 in the CPR footprint, respectively. The EESS active system noise figure was 7 dB which resulted in the interference protection criteria threshold of -153.2 dBW. The preliminary calculations show that the EESS (active) interference criteria are met with margins of 11.3 dB for Case 1 and 4.3 dB for Case 2.

For EESS (active) systems, when initially considering three different geometrical interaction scenarios between FOD detection systems and CPR L-1, the RFI levels at the CPR are highest for the geometrical situation of coupling between the nadir-looking CPR antenna coupling and the sidelobes of the FOD detection system antenna. The RFI levels are lower than for this first scenario when considering the second geometrical scenario which examines coupling between the sidelobes of both the CPR antenna and the FOD detection system antenna. The third geometrical scenario is coupling between the mainbeam of the FOD detection system antenna and the CPR antenna sidelobes with the CPR satellite on the horizon with respect to the FOD detection system. The RFI levels in this third scenario are also lower than for the first scenario. The CPR L-1 with the narrower bandwidth of 300 kHz, is more sensitive to the impact of interference than is CPR L-2.

[Editor’s note: An explanation is needed for the FOD detection system antenna gain in the sidelobes at 14 dBi is used for geometric scenario 1 and 2 (Tables 4 and 6 below). The value of 
-10 dBi, 0 dBi and 44 dBi for geometric scenario 1, 2 and 3 respectively seems reasonable. (Tables 3, 5, and 7 (and 8); respectively)]

[Editor’s note: Worst case FOD detection system density deployment information should be provided at the next meeting of WP 5B.  This information should be provided on a per airport basis and should take into account that the number of FOD detection system transmitting elements will vary in relation to the size of the airport where the system is deployed.]

Table A1-1

Radio frequency interference from foreign object debris detection systems into 
Earth exploration satellite service (active) systems at 94 GHz (Case 1) 
first geometric scenario: cloud profile radar mainlobe to foreign
object detection sidelobe

| Case 1: Calculation of rcv power at 90 degrees elevation (G_t=-10 dBi, 10 FODs in beam) |  |  |  |
| --- | --- | --- | --- |
|  | Units | CPR-L1 |  |
|  |  | Value | dB |
| Transmit power | W | 0.2 | -6.99 |
| Gain transmit antenna | dBi |  | -10.00 |
| e.i.r.p | dBW |  | -16.99 |
| Gain receive antenna | dBi |  | 65.20 |
| 1/R2 | km | 705 | -56.96 |
| 1/f2 | MHz | 94 050 | -99.47 |
| L-prop | dB |  | -188.87 |
| No. of FODs in beam |  | 10 | 10.00 |
| Interference power | dBW |  | -130.66 |
| K |  | 1.38E-23 | -228.60 |
| Temp | K | 290 | 24.62 |
| EESS Bandwidth | MHz | 0.3 | 54.77 |
| EESS noise floor | dB |  | 7.00 |
| Noise power | dBW |  | -143.17 |
| I/N | dB |  | 12.51 |
| I/N criteria | dB |  | -10.00 |
| Margin (attenuation) | dB |  | -22.51 |

Table  A1-2

Radio frequency interference from foreign object debris detection systems into 
Earth exploration satellite service (active) systems at 94 GHz (Case 2) 
first geometric scenario: cloud profile radar mainlobe to foreign
object detection sidelobe

| Case 2: Calculation of rcv pwr at 90 deg elev (G_t=14 dBi, 50 FODs in beam) |  |  |  |
| --- | --- | --- | --- |
|  | Units | CPR-L1 |  |
|  |  | Value | dB |
| Transmit power | W | 0.2 | -6.99 |
| Gain transmit antenna | dBi |  | 14.00 |
| e.i.r.p | dBW |  | 7.01 |
| Gain receive antenna | dBi |  | 65.20 |
| 1/R2 | km | 705 | -56.96 |
| 1/f2 | MHz | 94 050 | -99.47 |
| L-prop | dB |  | -188.87 |
| No. of FODs in beam |  | 50 | 16.99 |
| Interference power | dBW |  | -99.67 |
| K |  | 1.38E-23 | -228.60 |
| Temp | K | 290 | 24.62 |
| EESS Bandwidth | MHz | 0.3 | 54.77 |
| EESS noise figure | dB |  | 7.00 |
| Noise power | dBW |  | -143.17 |
| I/N | dB |  | 43.50 |
| I/N criteria | dB |  | -10.00 |
| Margin (attenuation) | dB |  | -53.50 |

Table A1-3

Radio frequency interference from foreign object debris detection systems into 
Earth exploration satellite service (active) systems at 94 GHz (Case 1) 
second geometric scenario: cloud profile radar sidelobe to foreign
object detection sidelobe

| Case 1: Calculation of rcv power at 45 degrees elevation (G_t=0 dBi, 10 FODs in beam) |  |  |  |
| --- | --- | --- | --- |
|  | Units | CPR-L1 |  |
|  |  | Value | dB |
| Transmit power | W | 0.2 | -6.99 |
| Gain transmit antenna | dBi |  | 0.00 |
| e.i.r.p | dBW |  | -6.99 |
| Gain receive antenna | dBi |  | -9.80 |
| 1/R2 | km | 952 | -59.57 |
| 1/f2 | MHz | 94 050 | -99.47 |
| L-prop | dB |  | -191.48 |
| No. of FODs in beam |  | 10 | 10.00 |
| Interference power | dBW |  | -198.27 |
| K |  | 1.38E-23 | -228.60 |
| Temp | K | 290 | 24.62 |
| EESS Bandwidth | MHz | 0.3 | 54.77 |
| EESS noise figure | dB |  | 7.00 |
| Noise power | dBW |  | -143.17 |
| I/N | dB |  | -55.10 |
| I/N criteria | dB |  | -10.00 |
| Margin (attenuation) | dB |  | 45.10 |

Table A1-4

Radio frequency interference from foreign object debris detection systems into 
Earth exploration satellite service (active) systems at 94 GHz (Case 2) 
second geometric scenario: cloud profile radar sidelobe to foreign
object detection sidelobe

| Case 2: Calculation of rcv power at 45 degrees elevation (G_t=14 dBi, 50 FODs in beam) |  |  |  |
| --- | --- | --- | --- |
|  | Units | CPR-L1 |  |
|  |  | Value | dB |
| Transmit power | W | 0.2 | -6.99 |
| Gain transmit antenna | dBi |  | 14.00 |
| e.i.r.p | dBW |  | 7.01 |
| Gain receive antenna | dBi |  | -9.80 |
| 1/R2 | km | 952 | -59.57 |
| 1/f2 | MHz | 94 050 | -99.47 |
| L-prop | dB |  | -191.48 |
| No. of FODs in beam |  | 50 | 16.99 |
| Interference power | dBW |  | -177.28 |
| K |  | 1.38E-23 | -228.60 |
| Temp | K | 290 | 24.62 |
| EESS Bandwidth | MHz | 0.3 | 54.77 |
| EESS noise figure | dB |  | 7.00 |
| Noise power | dBW |  | -143.17 |
| I/N | dB |  | -34.11 |
| I/N criteria | dB |  | -10.00 |
| Margin (attenuation) | dB |  | 24.11 |

Table A1-5

Radio frequency interference from foreign object debris detection systems into 
Earth exploration satellite service (active) systems at 94 GHz (Case 1) 
third geometric scenario: cloud profile radar sidelobe to foreign
object detection mainlobe

| Case 1: Calculation of rcv power at 0 degrees elevation (G_t=44 dBi, 10 FODs in beam) |  |  |  |
| --- | --- | --- | --- |
|  | Units | CPR-L1 |  |
|  |  | Value | dB |
| Transmit power | W | 0.2 | -6.99 |
| Gain transmit antenna | dBi |  | 44.00 |
| e.i.r.p | dBW |  | 37.01 |
| Gain receive antenna | dBi |  | -9.80 |
| 1/R2 | km | 3 081 | -69.77 |
| 1/f2 | MHz | 94 050 | -99.47 |
| L-prop | dB |  | -201.68 |
| No. of FODs in beam |  | 10 | 10.00 |
| Interference power | dBW |  | -164.47 |
| K |  | 1.38E-23 | -228.60 |
| Temp | K | 290 | 24.62 |
| EESS Bandwidth | MHz | 0.3 | 54.77 |
| EESS noise figure | dB |  | 7.00 |
| Noise power | dBW |  | -143.17 |
| I/N | dB |  | -21.30 |
| I/N criteria | dB |  | -10.00 |
| Margin (attenuation) | dB |  | 11.30 |

Table A1-6

Radio frequency interference from foreign object debris detection systems into 
Earth exploration satellite service (active) systems at 94 GHz (Case 2) 
third geometric scenario: cloud profile radar sidelobe to foreign
object detection mainlobe

| Case 2: Calculation of rcv power at 0 degrees elevation (G_t=44 dBi, 50 FODs in beam) |  |  |  |
| --- | --- | --- | --- |
|  | Units | CPR-L1 |  |
|  |  | Value | dB |
| Transmit power | W | 0.2 | -6.99 |
| Gain transmit antenna | dBi |  | 44.00 |
| e.i.r.p | dBW |  | 37.01 |
| Gain receive antenna | dBi |  | -9.80 |
| 1/R2 | km | 3 081 | -69.77 |
| 1/f2 | MHz | 94 050 | -99.47 |
| L-prop | dB |  | -201.68 |
| No. of FODs in beam |  | 50 | 16.99 |
| Interference power | dBW |  | -157.48 |
| K |  | 1.38E-23 | -228.60 |
| Temp | K | 290 | 24.62 |
| EESS Bandwidth | MHz | 0.3 | 54.77 |
| EESS noise figure | dB |  | 7.00 |
| Noise power | dBW |  | -143.17 |
| I/N | dB |  | -14.31 |
| I/N criteria | dB |  | -10.00 |
| Margin (attenuation) | dB |  | 4.31 |

## A1-1.2	Dynamic analysis based on interference observed with spacecraft orbit simulation

To obtain a more realistic assessment of the interference expected to be observed from FOD detection systems into EESS active systems, an analysis was conducted in which the orbits of the EESS spacecraft under investigation (i.e., CPR-L1 and CPR-L2 described in Table 2 of section A2-1.2) will be dynamically simulated. This approach corresponds to the EESS ITU-R Recommendations and is the only type of analysis where conclusions can be drawn.

Based on time series values for the peak interfering signal power level, complementary cumulative distribution function (CCDF) curves were generated in order to assess the percentage of time for which violations of the limit specified in Recommendation ITU-R RS.1166-4 occurred. From Recommendation ITU-R RS.1166-4, the interference criteria for a cloud profiling type radar, under conditions of systematic interference source, is I/N of -10 dB with a data availability of 99% of time.

### A1-1.2.1	Deployment of foreign object debris detection systems

[Editor’s note: Scenarios for deployment are to be verified with ICAO. ]

### A1-1.2.2	Characteristics of foreign object debris detection systems

[Note: This section will be edited according to the new FOD characteristics described in Section 2 of the main text.]

GTXα

### A1-A1-1.2.3	Characteristics of Earth exploration satellite service (active) system

#### A1-1.2.3.1 Earth exploration satellite service (active) orbital characteristics

The following table shows EESS satellite orbit parameters used by CloudSat.

Table 1-9

Earth exploration satellite service (active) satellite orbit parameters

| Parameter | Units | Value |
| --- | --- | --- |
| Altitude (mean) | km | 800 |
| Inclination | deg | 98.2 |

#### A1-1.2.3.2	Earth exploration satellite service (active) technical and operational characteristics

The following table summarizes the EESS parameters.

Table 1-10

Earth exploration satellite service (active) technical and operational characteristics

| Parameter | Units | Value |
| --- | --- | --- |
| Max gain | dBi | 65 |
| bandwidth | MHz | 0.3 |
| Antenna pattern | -- | Fig X |
| Temp | K | 290 |
| Noise Figure | dB | 7 |
| Noise Power | dBW | -143.17 |
| Scan Type | -- | Fixed |
| Pointing | -- | Nadir |

Figure A1-5

Earth exploration satellite service (active) antenna pattern normalized to 0dB

Figure A1-6

Earth exploration satellite service (active) small angle antenna pattern normalized to 0dB

#### A1-1.2.3.2	Sensor area of interest test cases

A single test case was explored which was a global area of interest. Consequently, all visible Earth ground area within the sensor field of view is considered.

### A1-1.2.4	Simulation parameters and results

The peak interfering signal power level, I/N (dB), received by a spaceborne radar from a terrestrial source is calculated from (A1-3):

I    10 log Pt    Gt    Gr  –  (32.44    20 log ( f R ))  –  La – FDRIF – Xr	(A1-2)

T =	Tb (10NF/10 - 1)		(K)

N =	10 log (T b BW)	(dBW)

I/N = I – N	(dB)	(A1-3)

where:

Pt :	peak terrestrial source transmitter power (W);

Gt :	terrestrial source antenna gain towards spaceborne sensor (dBi);

Gr :	spaceborne radar antenna gain towards terrestrial source (dBi);

f :	frequency (MHz);

R :	slant range between spaceborne sensor and terrestrial source (km);

La :	attenuation due to atmospheric absorption (dB).

FDRIF: 	frequency-dependent rejection produced by the receiver IF selectivity curve on an unwanted transmitter emission spectra (dB).

Xr:	losses due to polarization mismatch (dB)

Tb:	noise temperature w/o noise figure (K)

NF:	noise figure (dB)

b:	Boltzman constant 1.38 x 10-23 (K)

BW:	bandwidth of receiver (Hz)

N:	Noise power with noise figure (dBW)

#### A1-1.2.4.1	General simulation parameters

The following table gives the relevant aspects of the simulation.

Table A1-11

[TBD]

#### A1-1.2.4.2		Simulation results

#### [TBD]

# A1-2	Summary of Results

[TBD]

Annex 2

Interference analysis for Earth exploration satellite service (passive)

# A2-1	Interference from foreign object debris detection systems into Earth exploration satellite service (passive)

## A2-1.1	Static analysis based on interference with specific geometric scenarios

At least three different geometrical scenarios have to be considered in assessing the potential for interference from FOD detection systems into EESS passive systems. The first is coupling of the antenna mainlobe of a nadir-looking EESS (passive) satellite with the sidelobes of the FOD detection system antenna. The second geometrical scenario is when coupling occurs between the sidelobes of both the EESS passive sensor antenna and the FOD detection system antenna; and the third geometrical scenario is when coupling occurs between the mainbeam of the FOD detection system antenna and the EESS passive sensor antenna sidelobes at a time when the EESS passive satellite is on the horizon with respect to the FOD detection system.

The peak interfering signal power level, I (dBW), received by a spaceborne radiometer from a terrestrial source is calculated from:

I    10 log Pt    Gt    Gr  –  (32.44    20 log ( f R ))  –  La – FDRIF	(A2-1)

where:

Pt :	peak terrestrial source transmitter power (W);

Gt :	terrestrial source antenna gain towards spaceborne sensor (dBi);

Gr :	spaceborne radar antenna gain towards terrestrial source (dBi);

f :	frequency (MHz);

R :	slant range between spaceborne sensor and terrestrial source (km);

La :	attenuation due to atmospheric absorption (dB);

FDRIF: 	frequency-dependent rejection produced by the receiver IF selectivity curve on an unwanted transmitter emission spectra (dB).

Attenuation due to atmospheric absorption, La, is dependent upon the path length to the satellite through the Earth’s atmosphere, and hence upon the elevation angle from the terrestrial source to the satellite. At frequencies around 94 GHz, La decreases rapidly from about 100 dB at 0º elevation angles to 1.5 dB at 90º elevation angles. La and FDRIF were not included in the following tables of calculation of the interference levels and margins.

The interference due to spaceborne antenna mainlobe coupling with the FOD detection system antenna sidelobes allows for the highest value of RFI levels of these three geometrical scenarios. For geometric scenario 1 for coupling between the radiometer L-8 ( see ITU-R RS. 1861) mainlobe (L-8 is 2 GHz away from the FOD center frequency. At that frequency separation the FOD emission mask will be greatly reduced) and the FOD radar sidelobes (elevation angle at 55º), preliminary calculations of two Cases examined are provided in Tables 10 (Case 1) and 11 (Case 2) which indicate the amount of attenuation needed to apply to OOB FOD detection systems emissions in order that they meet the EESS (passive) interference protection criteria for radiometer L-8. The two cases were examined with the FOD detection system antenna gain in the sidelobes at –10 dBi and 14 dBi, and the density of FOD detection systems set at 10 and 50 in the radiometer L-8 footprint, respectively. The EESS passive system interference protection criteria threshold is -169.0 dBW. The calculated attenuations needed to meet the EESS (passive) interference criteria are 41.75 dB for Case 1 and 62.74 dB for Case 2.

For geometric scenario 2 for coupling between the radiometer L-8 sidelobes and the FOD sidelobes (elevation angle at 90º), preliminary calculations of two Cases examined are provided in Tables 12 (Case 1) and 13 (Case 2) which indicate the amount of attenuation needed to apply to in-band FOD detection systems emissions in order that they meet the EESS (passive) interference protection criteria for radiometer L-8. The two cases were examined with the FOD detection system antenna gain in the sidelobes at 0 dBi and 14 dBi, and the density of FOD detection systems set at 10 and 50 in the radiometer L-8 footprint, respectively. The EESS passive system interference protection criteria threshold is -169.0 dBW. The calculations show that the EESS (passive) interference criteria are met with margins of 36.41 dB for Case 1 and 5.42 dB for Case 2.

For geometric scenario 3 for coupling between the radiometer L-8 sidelobes and the FOD radar mainlobe (elevation angle at 0º), preliminary calculations of two Cases examined are provided in Tables 14 (Case 1) and 15 (Case 2) which indicate the amount of attenuation needed to apply to in-band FOD detection systems emissions in order that they meet the EESS (passive) interference protection criteria for radiometer L-8. The two cases were examined with the FOD detection system antenna gain in the sidelobes at 44 dBi, and the density of FOD detection systems set at 10 and 50 in the CPR footprint, respectively. The EESS passive system interference protection criteria threshold is -169.0 dBW. The calculated attenuations needed to meet the EESS (passive) interference criteria are 4.75 dB for Case 1 and 11.74 dB for Case 2.

[Chairman’s note: Which tables are referenced above as there is no table 14 or 15?]

For EESS (passive) systems, when initially considering three different geometrical interaction scenarios between FOD detection systems and radiometer L-8, the RFI levels at the radiometer L-8 are highest for the geometrical situation of coupling between the main-beam of the radiometer L-8 and the sidelobes of the FOD detection system antenna. The RFI levels are lower than for this first scenario when considering the second geometrical scenario which examines coupling between the sidelobes of both the radiometer L-8 antenna and the FOD detection system antenna. The third geometrical scenario is coupling between the main-beam of the FOD detection system antenna and the radiometer L-8 antenna sidelobes with the radiometer L-8 satellite on the horizon with respect to the FOD detection system. The RFI levels in this third scenario are also lower than for the first scenario.

These are the static analysis results and no conclusion on compatibility can be drawn from those.

[Editor’s note: An explanation is needed for the FOD detection system antenna gain in the sidelobes at 14 dBi is used for geometric scenario 1 and 2 (Tables 4 and 6 below). The value of 
-10 dBi, 0 dBi and 44 dBi for geometric scenario 1, 2 and 3 respectively seems reasonable. (Tables 3, 5, and 7 (and 8); respectively)] [Chairman’s note: What are the correct table references?]

[Editor’s note: Worst case FOD system density deployment information should be provided at the next meeting of WP 5B.  This information should be provided on a per airport basis and should take into account that the number of FOD system transmitting elements will vary in relation to the size of the airport where the system is deployed. ]

Table 2-1

Radio frequency interference from foreign object debris detection systems into 
Earth exploration satellite service (passive) at 92 GHz (Case 1) 
first geometric scenario: L-8 mainlobe to foreign
object debris sidelobe

| Case 1: Calculation of rcv power at 55 degrees elevation (G_t=-10 dBi, 10 FODs in beam) |  |  |  |
| --- | --- | --- | --- |
|  | Units | Radiometer L-8 |  |
|  |  | Value | dB |
| Transmit power | W | 0.2 | -6.99 |
| Gain transmit antenna | dBi |  | 0.00 |
| e.i.r.p | dBW |  | -6.99 |
| Gain receive antenna | dBi |  | 62.40 |
| 1/R2 | km | 1 114.9 | -60.94 |
| 1/f2 | MHz | 92 000 | -99.28 |
| L-prop | dB |  | -192.66 |
| No. of FODs in beam |  | 10 | 10.00 |
| Interference power | dBW |  | -127.25 |
| Interference power criteria | dBW |  | -169.00 |
| Margin | dB |  | -41.75 |

Table A2-2

Radio frequency interference from foreign object debris detection systems into 
Earth exploration satellite service (passive) at 92 GHz (Case 2) 
first geometric scenario: L-8 mainlobe to foreign
object debris sidelobe

| Case 2: Calculation of rcv power at 55 degrees elevation (G_t=14 dBi, 50 FODs in beam) |  |  |  |
| --- | --- | --- | --- |
|  | Units | CPR-L1 |  |
|  |  | Value | dB |
| Transmit power | W | 0.2 | -6.99 |
| Gain transmit antenna | dBi |  | 14.00 |
| e.i.r.p | dBW |  | 7.01 |
| Gain receive antenna | dBi |  | 62.40 |
| 1/R2 | km | 1 114.9 | -60.94 |
| 1/f2 | MHz | 92 000 | -99.28 |
| L-prop | dB |  | -192.66 |
| No. of FODs in beam |  | 50 | 16.99 |
| Interference power | dBW |  | -106.26 |
| Interference power criteria | dBW |  | -169.00 |
| Margin | dB |  | -62.74 |

Table A2-3

Radio frequency interference from foreign object debris detection systems into 
Earth exploration satellite service (passive) at 92 GHz (Case 1) 
second geometric scenario: L-8 sidelobe to foreign
object debris sidelobe

| Case 1: Calculation of rcv power at 90 degrees elevation (G_t=0 dBi, 10 FODs in beam) |  |  |  |
| --- | --- | --- | --- |
|  | Units | CPR-L1 |  |
|  |  | Value | dB |
| Transmit power | W | 0.2 | -6.99 |
| Gain transmit antenna | dBi |  | -10.00 |
| e.i.r.p | dBW |  | -16.99 |
| Gain receive antenna | dBi |  | -9.80 |
| 1/R2 | km | 700 | -56.90 |
| 1/f2 | MHz | 92 000 | -99.28 |
| L-prop | dB |  | -188.62 |
| No. of FODs in beam |  | 10 | 10.00 |
| Interference power | dBW |  | -205.41 |
| Interference power criteria | dBW |  | -169.00 |
| Margin | dB |  | 36.41 |

Table A2-4

Radio frequency interference from foreign object debris detection systems into 
Earth exploration satellite service (passive) at 92 GHz (Case 2) 
second geometric scenario: L-8 sidelobe to foreign
object debris sidelobe

| Case 2: Calculation of rcv power at 90 degrees elevation (G_t=14 dBi, 50 FODs in beam) |  |  |  |
| --- | --- | --- | --- |
|  | Units | CPR-L1 |  |
|  |  | Value | dB |
| Transmit power | W | 0.2 | -6.99 |
| Gain transmit antenna | dBi |  | 14.00 |
| e.i.r.p | dBW |  | 7.01 |
| Gain receive antenna | dBi |  | -9.80 |
| 1/R2 | km | 700 | -56.90 |
| 1/f2 | MHz | 92 000 | -99.28 |
| L-prop | dB |  | -188.62 |
| No. of FODs in beam |  | 50 | 16.99 |
| Interference power | dBW |  | -174.42 |
| Interference power criteria | dBW |  | -169.00 |
| Margin | dB |  | 5.42 |

Table A2-5

Radio frequency interference from foreign object debris detection systems into 
Earth exploration satellite service (passive) at 92 GHz (Case 1) 
third geometric scenario: L-8 mainlobe to foreign
object debris mainlobe

| Case 1: Calculation of rcv power at 0 degrees elevation (G_t=44 dBi, 10 FODs in beam) |  |  |  |
| --- | --- | --- | --- |
|  | Units | CPR-L1 |  |
|  |  | Value | dB |
| Transmit power | W | 0.2 | -6.99 |
| Gain transmit antenna | dBi |  | 44.00 |
| e.i.r.p | dBW |  | 37.01 |
| Gain receive antenna | dBi |  | -9.80 |
| 1/R2 | km | 3 069 | -69.74 |
| 1/f2 | MHz | 92 000 | -99.28 |
| L-prop | dB |  | -201.46 |
| No. of FODs in beam |  | 10 | 10.00 |
| Interference power | dBW |  | -164.25 |
| Interference power criteria | dBW |  | -169.00 |
| Margin | dB |  | -4.75 |

Table A2-6

Radio frequency interference from foreign object debris detection systems into 
Earth exploration satellite service (passive) at 92 GHz (Case 2) 
third geometric scenario: L-8 mainlobe to foreign
object debris mainlobe

| Case 2: Calculation of rcv power at 0 degrees elevation (G_t=44 dBi, 50 FODs in beam) |  |  |  |
| --- | --- | --- | --- |
|  | Units | CPR-L1 |  |
|  |  | Value | dB |
| Transmit power | W | 0.2 | -6.99 |
| Gain transmit antenna | dBi |  | 44.00 |
| e.i.r.p | dBW |  | 37.01 |
| Gain receive antenna | dBi |  | -9.80 |
| 1/R2 | km | 3 069 | -69.74 |
| 1/f2 | MHz | 92 000 | -99.28 |
| L-prop | dB |  | -201.46 |
| No. of FODs in beam |  | 50 | 16.99 |
| Interference power | dBW |  | -157.26 |
| Interference power criteria | dBW |  | -169.00 |
| Margin | dB |  | -11.74 |

## A2-1.2 Dynamic analysis based on interference observed with spacecraft orbit simulation

The interfering signal power level, I (dBW), received by a spaceborne radiometer from a terrestrial source is calculated from:

I    10 log Pt  Gt  Gr – (32.44  20 log ( fR )) – La – FDRIF - Xr	(A2-2)

where:

Pt :	peak terrestrial source transmitter power (W);

Gt :	terrestrial source antenna gain towards spaceborne sensor (dBi);

Gr :	spaceborne radar antenna gain towards terrestrial source (dBi);

f :	frequency (MHz);

R :	slant range between spaceborne sensor and terrestrial source (km);

La :	attenuation due to atmospheric absorption (dB);

FDRIF: 	frequency-dependent rejection produced by the receiver IF selectivity curve on an unwanted transmitter emission spectra (dB);

Xr:	losses due to polarization mismatch (dB).

An assessment of the RFI expected from FOD detection systems into EESS passive systems is achieved by dynamic simulation. The analysis was conducted in which the orbit of the EESS spacecraft under investigation (i.e., the radiometer L8 described in Table X) as well as the FOD pointing were dynamically simulated.I

Based on time series values for the interfering signal power level, CCDF curves will be generated in order to assess the percentage of time for which violations of the limit specified in Recommendation ITU-R RS.2017-0 occurred. From Recommendation ITU-R RS.2017-0, the following limits should hold:

–	For frequency band: 86-92 GHz, reference bandwidth: 100 MHz,

•	Maximum interference level: -169 dBW,

•	Percentage of area or time permissible interference level may be exceeded: 0.01%. The area analyzed should be 2 000 000 km2.

–	For frequency band: 100-102 GHz, reference bandwidth 10 MHz,

•	Maximum interference level: -189 dBW,

•	Percentage of time 1% over a 24h measurement time.

Recommendation ITU-R RS.1861-0 contains the operating characteristics for the 86-92 GHz and the 100-102 GHz band EESS passive sensors.

### A2-1.2	.1 Deployment of foreign object debris detection systems

### [Note: This section will be edited according to the new FOD deployment described in Section 6 of the main text.]

With respect to the passive band, this study used the deployment over the Japan. Runways which consisted of grass, water, or lakebed were removed, as the FOD systems were assumed to be used on asphalt or concrete only. The FOD detection systems were spread along the runway with a maximum of 48 per runway, evenly spaced along the runway. In total, a selection of XXX runways fit the criteria with a total of XXX FOD detection units along those runways. An example of the deployment is shown in Figure A2-1.

Figure A2-1:

[TBD]

### A2-1.2.2	Characteristics of foreign object debris detection systems

### [Note: This section will be edited according to the new FOD characteristics described in Section 2 of the main text.]

### A2-1.2.3	Characteristics of Earth exploration satellite service (passive) system

In the 86-92 GHz band there currently operate two main types of EESS (passive) sensor, the conical scan and mechanical nadir scan sensors. EESS (passive) sensors L8 and L5 ( see ITU-R RS. 1861) were simulated in this this study to verify compatibly with both a conical scan and mechanical nadir scan sensor. The operating parameters of the L8 and L5 sensors are such that they will receive a higher power from the FOD deployment for their respected scan types. The L8 sensor in this incidence is the best model to protect the EESS (passive) service for present and future missions. EESS (passive) satellite orbit parameters and technical and operational characteristics of Sensors L8 and L5 are shown in ITU-R RS.1861, which operates in the 86-92 GHz band. EESS (passive) receive filters frequency response is assumed to provide sufficient attenuation outside of the 86-92 GHz band to allow limiting the sharing study within the 86-92 GHz band. ITU-R RS.1813-1 was used to generate the antenna patterns for the passive sensors of the study.

### A2-1.2.4 Simulation parameters and results

#### A2-1.2.4.1 General simulation parameters

For the 86-92 GHZ band, ITU-R RS.2017 prescribes interference shall not exceed the power spectral density of -169 dBW/100MHz over any part of the band for 0.01% of the time.  A simulation frequency of 91 950 MHz, 50 MHz from the band edge, with a 100 MHz bandwidth was chosen to be in line with the EESS (passive) protection criteria of -169 dBW/100 MHz. Analysis was done along the band edge to determine the level of unwanted emissions into the EESS (passive) band. Table XX gives the remaining simulation parameters that were assumed for this simulation.

Table XX

General simulation parameters

| Parameter | Units | Value |
| --- | --- | --- |
| Simulation Frequency | MHz | 91 950 |
| Duration | days | [TBD] |
| Time Step | S | [TBD] |
| Atmospheric Losses |  | P.676 |
| Polarization Losses | dB | 3 |
| FDR | dB | [TBD] |
| EESS (passive) Band Power Limit | dBW/100MHz | [TBD] |

The simulation was run for a [TBD] day duration with a [TBD] second time step to encompass the repeat period of the sensor L8 or L5 respectively. This duration and time step included an appropriate amount of sample points to achieve statistical significance of results. Atmospheric losses (La ) were calculated using Recommendation ITU-R P.676. The irrational time step of [TBD] was chosen to create a random non-uniform distribution of the EESS locations and azimuth pointing angles during satellite orbit within the simulation run time.

#### A2-1.2.4.2 Simulation results

#### [TBD]

# A2-2	Summary of Results

[TBD]

______________

____________________

Descending.

The sensor uses an unmodulated pulse.