| Radiocommunication Study Groups |  |
| --- | --- |
| Received: 29 June 2023 Source: Document 5B/731 (Annex 3 to Annex 15) Subject: WRC-23 agenda item 1.10 | Document 5B/784-E |
|  | 30 June 2023 |
|  | English only |
| United States of America |  |
| Working document towards a preliminary draft new Report ITU-R M.[NON-SAFETY AM(OR)S CHARACTERISTICS AND SHARING STUDIES] |  |
| ANNEX 3 |  |
| Sharing of the frequency band 15.4-15.7 GHz between RLS radars and future non-safety AM(OR)S systems |  |

# 1	Introduction

The United States of America would like to finalize the sharing study between non-safety AM(OR)S and Radiolocation in the frequency band 15.4-15.7 GHz by providing some editorial and technical corrections for the multiple clusters analysis.

The United States proposals are in track changes. Attachment revisions are presented for consideration.

Attachment

WORKING DOCUMENT TOWARDS A PRELIMINARY DRAFT NEW 
REPORT ITU-R M.[NON-SAFETY AM(OR)S CHARACTERISTICS 
AND SHARING STUDIES]

ANNEX 3

Sharing of the frequency band 15.4-15.7 GHz between RLS radars 
and future non-safety AM(OR)S systems

There are no proposed changes prior to this point in the document.

…

## A3.4	Study D

This section assesses sharing between multiple clusters of AM(OR)S stations operating in the frequency band 15.4-15.7 GHz and an RLS system operating in the band 15.4-17.3 GHz. It determines minimum separation zones around the RLS receiver to meet the protection criterion I/N = −6 dB.

### A3.4.1	Methodology

The study evaluates the aggregate I/N variable at the RLS receiver, and determines the required separation distance to meet the protection criterion. The impact of GDTs (if some are present in a given scenario, for instance 6.2.1 and 6.2.3) is not taken into account. The simulation setup is described in Table A3-5 below. Figures A3-14 through A3-17 depicts the sharing scenarios between multiple clusters of AM(OR)S and radiolocation.

### A3.4.2	Results

Figures A3-18 through A3-21 provide the ECDF of I/N measured at the RLS receiver in the four operational scenarios, in two different configurations of the AM(OR)S clusters. Figures A3-22 to A3-25 repeat the simulation by introducing separation distances indicated in the title of each figure.

### A3.4.3	Summary

Depending on the interference scenario under consideration and the AMS system characteristics, a separation distance is required between AM(OR)S and RLS stations.

TABLE A3-5

Simulation setup of Study D

|  |  | Units | Scenario |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6.2.1 |  | 6.2.2 |  | 6.2.3 |  |  |  | 6.2.4 |
| Clusters deployment |  |  |  |  |  |  |  |  |  |  |  |
| Number of clusters |  | - | 5 |  |  | 3 |  |  | 4 |  | 8 |
| Location |  |  | All clusters randomized within 254 km from the radius of the simulations area |  |  | All clusters randomized within 484 km from the radius of the simulations area |  |  | All clusters randomized within 467 km from the radius of the simulations area |  | All clusters randomized within 332 km from the radius of the simulations area |
| AM(OR)S stations deployment within a cluster |  |  |  |  |  |  |  |  |  |  |  |
| Position of AM(OR)S stations inside a cluster |  | - | ADTs randomized within 70 km from the GDT |  |  | According to Table 6-1 |  |  | Relay ADT randomized 300 km from the GDT; observation ADTs randomized 5 km from the relay ADT |  | ADT#3 is fixed; ADT#2 and ADT#4 randomized within 500 km from ADT#3; ADT#1 within 500 km from ADT#2; ADT#5 within 500 km from ADT#4 |
| Altitude of AM(OR)S stations AGL |  | km | According to Table 6-1 |  |  |  |  |  |  |  |  |
| TPO |  | dBm | Adapt the ATPC algorithm described in section A11.6.1 |  |  |  |  |  |  |  |  |
| Antennas of AM(OR)S stations |  | – | According to section A1.2 |  |  |  |  |  |  |  |  |
| ADTs BW |  | MHz | 10 |  | 10 |  | 10 |  |  |  | 10 |
| GDTs BW |  |  | 10 |  | – |  | 10 |  |  |  | – |
| AM(OR)S stations centre frequency |  | GHz | Randomized in 15.4-15.7 |  |  |  |  |  |  |  |  |
| Simulation parameters |  |  |  |  |  |  |  |  |  |  |  |
| Simulation area radius |  | km | 400 |  |  | 800 |  |  | 900 |  | 1 500 |
| PL ADTs – RLS |  | dB | According to Rec. ITU-R P.528-5 (with 5% time) |  |  |  |  |  |  |  |  |
| PL AM(OR)S Tx – AM(OR)S Rx |  | dB | According to Rec. ITU-R P.528-5 (with 95% time) |  |  |  |  |  |  |  |  |
| Number of snapshots |  | – | 106 |  |  | 106 |  |  | 106 |  | 106 |
| RLS receiver deployment |  |  |  |  |  |  |  |  |  |  |  |
| Location |  | – | Randomized within the simulation area |  |  |  |  |  |  |  |  |
| Altitude AGL |  | km | Uniformly randomized between 0.3 and 13.7, see Table A2-1 |  |  |  |  |  |  |  |  |
| Antenna pointing |  | – | Uniformly randomized within ±45° in azimuth and between +5° and −45° in elevation as per Table A2A-1 |  |  |  |  |  |  |  |  |
| Protection criterion |  | – | I/N = −6 dB according to section A2.1.3 |  |  |  |  |  |  |  |  |
| Antenna |  | – | According to section A2.1.2 and Attachment A to Annex 2 |  |  |  |  |  |  |  |  |
| Centre frequency |  | GHz | Randomized in 15.4-15.7 |  |  |  |  |  |  |  |  |
| BW |  | MHz | 25 | 25 |  |  |  | 25 |  | 25 |  |

Figure A3-14

Sharing studies between multiple AM(OR)S clusters and radiolocation 
based on the wildfire observation scenario

Figure A3-15

Sharing studies between multiple AM(OR)S clusters and radiolocation based 
on the search and rescue scenario

Figure A3-16

Sharing studies between multiple AM(OR)S clusters and radiolocation based 
on the surveillance mission scenario

Figure A3-17

Sharing studies between multiple AM(OR)S clusters and radiolocation based 
on the Internet above the clouds scenario

Figure A3-18

ECDF (%) of I/N at the RLS receiver in scenario 6.2.1; the protection criterion of RLS (I/N = −6 dB) 
is indicated by a vertical red line; the left figure is without ATPC, and the right figure is with ATPC

Figure A3-19

As in Figure A3-18, in scenario 6.2.2

Figure A3-20

As in Figure A3-18, in scenario 6.2.3

Figure A3-21

As in Figure A3-18, in scenario 6.2.4

Figure A3-22

ECDF (%) of I/N at RLS receiver in scenario 6.2.1; RLS protection criterion is shown 
as a vertical red line; in the left figure, 560 km exclusion zone in green and 565 km in blue; in the right figure, 555 km exclusion zone in green and 560 km in blue

Figure A3-23

As in Figure A3-22, in scenario 6.2.2; in the left figure, 1 200 km exclusion zone in green and 1 205 km in blue; in the right figure, 1 005 km exclusion zone in green and 1 010 km in blue

Figure A3-24

As in Figure A3-22, in scenario 6.2.3; in the left figure, 875 km exclusion zone in green and 880 km in blue; 
in the right figure, 715 km exclusion zone in green and 720 km in blue

Figure A3-25

As in Figure A3-22, in scenario 6.2.4; in the left figure, 1 435 km exclusion zone in green and 1 440 km in blue; in the right figure, 1 330 km exclusion zone in green and 1 335 km in blue

Table A3-6 below provides the separation distance between non-safety AM(OR)S and radiolocation.

Table A3-6

Separation distance between systems operating in the aeronautical mobile service (off-route) 
and radiolocation service

|  |  |  | Separation distance between Radiolocation and the center of non-safety AM(OR)S cluster deployment (km) without ATPC feature | Separation distance between Radiolocation and the center of non-safety AM(OR)S cluster deployment(km) with ATPC feature |
| --- | --- | --- | --- | --- |
| Figure 4-2 – Wildfire observation scenario |  |  | 565 | 560 |
| Figure 4-3 – Search and rescue scenario |  |  | *1 205 | 1 010 |
| Figure 4-4 – Surveillance mission scenario |  |  | 880 | 720 |
| Figure 4-5 – Internet above the clouds scenario |  |  | 1 440 | 1 335 |

* Editor’s note: For the search and rescue scenario, the previous separation distance (720 km) is from the edge of the AM(OR)S cluster to Radiolocation. The correct value is 1 205 km for the distance between the center of the cluster to Radiolocation.

There are no proposed changes prior to this point in the document.

______________

____________________