| U.S. Radiocommunications Sector Fact Sheet |  |  |
| --- | --- | --- |
| Working Party: ITU-R WP 5C | Document No: USWP5C-04-FAA |  |
| Ref: WRC-27 AI 1.19 | Date: March 6, 2024 |  |
| Document Title: Proposed draft liaison statement to Working Party 7C |  |  |
| Author(s)/Contributors(s): Chris Tourigny FAA Spectrum Engineering Services Sandra Wright FAA Spectrum Engineering Services Amir Uzzaman FAA Spectrum Engineering Services Michael Tran MITRE Andrew Meadows AFSMO Arminder Singh eSimplicity support AFSMO |  | Phone: 202-267-3071 Email: chris.tourigny@faa.gov Phone: 202-603-7094 Email: sandra.a.wright@faa.gov Phone: 860-794-2025 Email: amir.uzzaman@faa.gov Phone: 703-593-9969 Email: mtran@mitre.org Phone: 334-467-4720 Email: andrew.meadows.1@us.af.mil Phone: 281-865-8678 Email: arminder.singh@esimplicity.com |
| Purpose/Objective: This contribution proposes a draft liaison statement to WP 7C regarding AI 1.19, which will allow WP 7C to proceed with its planning for sharing and compatibility studies. |  |  |
| Abstract: Pursuant to Resolution 674 (WRC-23), in preparation for Agenda Item 1.19 (WRC-27), this contribution proposes a draft reply liaison statement to WP 7C with relevant technical information of fixed systems used for the provision of air navigation services operating in the frequency bands 4 400-4 800 MHz and 8 400-8 500 MHz. |  |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: None Subject: WRC-27 Agenda Item 1.19 | Document 5C/ |
|  | 14 May 2024 |
|  | English only |
| United States of America |  |
| proposed draft reply liaison statement to Working party 7c Relevant technical information for sharing studies under WRC-27 Agenda Item 1.19 |  |

Introduction

WRC-27 Agenda Item 1.19 considers possible primary allocations in all Regions to the Earth exploration-satellite service (passive) in the frequency bands 4 200-4 400 MHz and 8 400-8 500 MHz, in accordance with Resolution 674 (WRC-23).  This contribution proposes a draft reply liaison statement to WP 7C with relevant technical information of fixed systems used for the provision of air navigation services operating in the frequency bands 4 400-4 800 MHz and 8 400-8 500 MHz.

Attachment:  1

ATTACHMENT

# Working Party 5C

PROPOSAL DRAFT REPLY LIAISON STATEMENT TO WORKING PARTY 7C

Fixed service characteristics for use in sharing
 studies under WRC-27 agenda item 1.19

Working Party (WP) 5C thanks WP 7C for its liaison statement (Document 5C/XX), requesting the characteristics and protection criteria of the fixed service (FS) systems operating in the frequency bands 4 400-4 800 MHz and 8 400-8 500 MHz, for sharing/compatibility studies between the FS and earth exploration-satellite service (EESS) (passive) under WRC-27 agenda item 1.19.

In response to a request from WP 7C for FS characteristics, WP 5C highlights the following ITU-R recommendations and revised recommendations that provide relevant technical information about FS systems, which may be further revised:

Recommendation ITU-R F.758-7	System parameters and considerations in the development of criteria for sharing or compatibility between digital fixed wireless systems in the fixed service and systems in other services and other sources of interference (11/2019)

Document 5C/384 Annex 7     Preliminary Draft Revision of Recommendation (PDRR) ITU-R F.758-7 where technical characteristics of FS systems in Tables 6 to 11 are updated/added.  Amendments were made throughout the document to improve the clarity of the text.

This recommendation and its proposed revision contain the principles for the development of sharing criteria of digital systems in the FS. Representative technical characteristics of digital fixed wireless systems (FWS) are in Annex 2 Table 8 for the 4 400-4 800 MHz band and Table 9 for the 8 400-8 500 MHz band of Document 5C/384 Annex 7. Table 5 of Annex 2 provides guidance in the choice of I/N values for long-term sharing criteria/interference.

Recommendation ITU-R F.699-8 	Reference radiation patterns for fixed wireless system antennas for use in coordination studies and interference assessment in the frequency range from 100 MHz to 86 GHz (01/2018)

Document 5C/384 Annex 2     Preliminary Draft Revision of Recommendation (PDRR) ITU-R F.699-8 provides updated antenna patterns and extends the upper frequency from 86 to 174.8 GHz.

This recommendation and its proposed revision provide reference radiation patterns for, and information on, FWS antennas in the frequency range from 100 MHz to 174.8 GHz. This information may be used in coordination studies and interference assessments when particular information concerning the FWS antenna is not available.

Recommendation ITU-R F.1245-3    Mathematical model of average and related radiation patterns for point-to-point fixed wireless system antennas for use in interference assessment in the frequency range from 1 GHz to 86 GHz (01/2019)

This recommendation provides average and related reference radiation patterns for point-to-point FWS antennas in the frequency range from 1 GHz to 86 GHz. This information may be used in interference assessments when particular information concerning the FWS antenna is not available. Recommendation ITU-R F.1245-3 could be used to predict the aggregate interference from many geostationary satellites or from one or more non-geostationary satellites, taking into account considering c).

WP 5C requests to be kept informed on the progress of the studies under WRC-27 agenda item 1.19 and will provide to WP 7C updated relevant information, before the 31 December 2024 deadline, for studies under this WRC-27 agenda item.

Status:  For information and action, as appropriate

Contact: TBD						E-mail: 	TBD