| U.S. Radiocommunications Sector Fact Sheet |  |
| --- | --- |
| Working Party: ITU-R WP 4C | Document No: USWP4C-11 |
| Ref: N/A | Date: 22 Mar 2024 |
| Document Title: Working Document [AI 1.13 sharing and compatibility studies] |  |
| Author(s)/Contributors(s): Jameson Dempsey SpaceX Zahid Islam SpaceX Brett Tarnutzer SpaceX Pascale Dumit T-Mobile | Email: Jameson.Dempsey@spacex.com Phone: Email: km.islam@spacex.com Phone: Email: brett.tarnutzer@spacex.com Phone: Email: Pascale.Dumit@T-Mobile.com Phone: |
| Purpose/Objective: Initiate the work under WRC-27 Agenda Item 1.13 |  |
| Abstract: The proposed U.S. contribution aims at getting the work started under WRC-27 Agenda Item 1.13. The document also includes a skeleton of various sections for the Working Document for the sharing and compatibility studies to be conducted under WRC-27 Agenda Item 1.13. |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: - Subject: WRC-27 agenda item 1.13 | Document 4C/TBD |
|  | TBD May 2024 |
|  | English only |
| United States of America |  |
| WORKING DOCUMENT [AI 1.13 SHARING AND COMPATIBILITY STUDIES] |  |

Introduction

The proposed U.S. contribution aims at getting the work started under WRC-27 Agenda Item 1.13. The attached document includes a skeleton of the envisaged sections for conducting the sharing and compatibility studies under WRC-23 Agenda Item 1.13 in bands between 694 - 2 700 MHz identified for IMT in the Radio Regulations and reflected in Recommendation ITU-R M.1036.

Attachment:	1

Attachment 1

WORKING DOCUMENT [AI 1.13 SHARING AND COMPATIBILITY STUDIES]

- Introduction

Resolution 253 (WRC-23) calls for studies on possible new allocations to the mobile-satellite service for direct connectivity between space stations and International Mobile Telecommunications (IMT) user equipment to complement terrestrial IMT network coverage.

Studies on sharing and compatibility between incumbent services, including in adjacent frequency bands are essential to ensure the protection of incumbent services in accordance with the Radio Regulations.

- Information on IMT frequency arrangements contained in

Recommendation ITU-R M.1036 and the corresponding incumbent and adjacent band services

The following tables provide information on the IMT frequency arrangements contained in Recommendation ITU-R M.1036 with all the corresponding incumbent and adjacent band services for bands between 694 - 2 700 MHz identified for IMT in the Radio Regulations.

The tables for the Earth-to-space direction have been included for information. Depending on further analysis, studies for this direction might not be necessary given that the characteristics of the IMT mobile stations (i.e. user equipment) are envisaged to remain unchanged.

[Editor's note: The table for the Earth-to-space direction is included for information. Depending on further analysis, studies for this direction might not be necessary.]

| FDD Frequency arrangements | Mobile station transmitter/ (Earth-to-space direction) (MHz) |
| --- | --- |
| A1 | 824-849 |
| A2 | 880-915 |
| A3 | 832-862 |
| A4 | 698-716 |
|  | 776-793 |
| A5 | 703-748 |
| A7 | 703-733 |
| A8 | 698-703 |
| A9 | 733-736 |
| A11 | 703-733 |

| FDD Frequency arrangements | Base station transmitter/ (space-to-Earth direction) (MHz) | Incumbent Services | Adjacent band Services |
| --- | --- | --- | --- |
| A1 | 869-894 | FIXED MOBILE MOBILE except aeronautical mobile BROADCASTING Radiolocation | FIXED MOBILE MOBILE except aeronautical mobile BROADCASTING Radiolocation |
| A2 | 925-960 | FIXED MOBILE MOBILE except aeronautical mobile BROADCASTING Radiolocation Amateur | FIXED MOBILE MOBILE except aeronautical mobile BROADCASTING Radiolocation Amateur AERONAUTICAL MOBILE (R) AERONAUTICAL RADIONAVIGATION |
| A3 | 791-821 | FIXED MOBILE except aeronautical Mobile MOBILE BROADCASTING | FIXED MOBILE except aeronautical Mobile MOBILE BROADCASTING |
| A4 | 728-746 | FIXED MOBILE except aeronautical Mobile MOBILE BROADCASTING | FIXED MOBILE except aeronautical Mobile MOBILE BROADCASTING |
|  | 746-763 | FIXED MOBILE except aeronautical Mobile MOBILE BROADCASTING | FIXED MOBILE except aeronautical Mobile MOBILE BROADCASTING |
| A5 | 758-803 | FIXED MOBILE except aeronautical Mobile MOBILE BROADCASTING | FIXED MOBILE except aeronautical Mobile MOBILE BROADCASTING |
| A7 | 758-788 | FIXED MOBILE except aeronautical Mobile MOBILE BROADCASTING | FIXED MOBILE except aeronautical Mobile MOBILE BROADCASTING |
| A8 | 753-758 | FIXED MOBILE except aeronautical Mobile MOBILE BROADCASTING | FIXED MOBILE except aeronautical Mobile MOBILE BROADCASTING |
| A9 | 788-791 | FIXED MOBILE except aeronautical Mobile MOBILE BROADCASTING | FIXED MOBILE except aeronautical Mobile MOBILE BROADCASTING |
| A10 | 738-758 | FIXED MOBILE except aeronautical Mobile MOBILE BROADCASTING | FIXED MOBILE except aeronautical Mobile MOBILE BROADCASTING |
| A11 | 758-788 | FIXED MOBILE except aeronautical Mobile MOBILE BROADCASTING | FIXED MOBILE except aeronautical Mobile MOBILE BROADCASTING |
|  | 738-758 | FIXED MOBILE except aeronautical Mobile MOBILE BROADCASTING | FIXED MOBILE except aeronautical Mobile MOBILE BROADCASTING |

| TDD Frequency arrangements | Un-paired arrangement (MHz) | Incumbent Services | Adjacent band Services |
| --- | --- | --- | --- |
| A6 | 698-806 | FIXED MOBILE except aeronautical Mobile MOBILE BROADCASTING | FIXED MOBILE except aeronautical Mobile MOBILE BROADCASTING |

[Editor's note: The table for the Earth-to-space direction is included for information. Depending on further analysis, studies for this direction might not be necessary.]

| FDD Frequency arrangements | Mobile station transmitter/ (Earth-to-space direction) (MHz) |
| --- | --- |
| G2 | 1 427-1 470 |

| FDD Frequency arrangements | Base station transmitter/ (space-to-Earth direction) (MHz) | Incumbent Services | Adjacent band Services |
| --- | --- | --- | --- |
| G1 | 1 427-1 517 | SPACE OPERATION (Earth-to-space) FIXED MOBILE except aeronautical MOBILE BROADCASTING BROADCASTING-SATELLITE | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) FIXED MOBILE except aeronautical MOBILE MOBILE-SATELLITE (space-to-Earth) |
| G2 | 1 475-1 518 | FIXED MOBILE except aeronautical MOBILE BROADCASTING BROADCASTING-SATELLITE | BROADCASTING BROADCASTING-SATELLITE FIXED MOBILE MOBILE except aeronautical mobile MOBILE-SATELLITE (space-to-Earth) |

| TDD Frequency arrangements | Un-paired arrangement (MHz) | Incumbent Services | Adjacent band Services |
| --- | --- | --- | --- |
| G3 | 1 427-1 517 | SPACE OPERATION (Earth-to-space) FIXED MOBILE except aeronautical MOBILE BROADCASTING BROADCASTING-SATELLITE | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) FIXED MOBILE except aeronautical MOBILE MOBILE-SATELLITE (space-to-Earth) |

[Editor's note: The table for the Earth-to-space direction is included for information. Depending on further analysis, studies for this direction might not be necessary.]

| FDD Frequency arrangements | Mobile station transmitter/ (Earth-to-space direction) (MHz) |
| --- | --- |
| B1 | 1 920-1 980 |
| B2 | 1 710-1 785 |
| B3 | 1 850-1 920 |
| B4 | 1 710-1 785 |
|  | 1 920-1 980 |
| B5 | 1 850-1 920 |
|  | 1 710-1 780 |
| B6 | 1 980-2 010 |
| B7 | 2 000-2 020 |

| FDD Frequency arrangements | Base station transmitter/ (space-to-Earth direction) (MHz) | Incumbent Services | Adjacent band Services |
| --- | --- | --- | --- |
| B1 | 2 110-2 170 | FIXED MOBILE SPACE RESEARCH (deep space) (Earth-to-space) MOBILE-SATELLITE (space-to-Earth) Mobile-Satellite (space-to-Earth) | SPACE OPERATION (Earth-to-space) (space-to-space) EARTH EXPLORATION-SATELLITE (Earth-to-space) (space-to-space) FIXED MOBILE SPACE RESEARCH (Earth-to-space) (space-to-space) MOBILE-SATELLITE (space-to-Earth) |
| B2 | 1 805-1 880 | FIXED MOBILE | FIXED MOBILE |
| B3 | 1 930-2 000 | FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) Mobile-Satellite (Earth-to-Space) | FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) |
| B4 | 1 805-1 880 | FIXED MOBILE | FIXED MOBILE |
|  | 2 110-2 170 | FIXED MOBILE SPACE RESEARCH (deep space) (Earth-to-space) MOBILE-SATELLITE (space-to-Earth) Mobile-Satellite (space-to-Earth) | SPACE OPERATION (Earth-to-space) (space-to-space) EARTH EXPLORATION-SATELLITE (Earth-to-space) (space-to-space) FIXED MOBILE SPACE RESEARCH (Earth-to-space) (space-to-space) MOBILE-SATELLITE (space-to Earth) |
| B5 | 1 930-2 000 | FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) Mobile-Satellite (Earth-to-Space) | FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) |
|  | 2 110-2 180 | FIXED MOBILE SPACE RESEARCH (deep space) (Earth-to-space) MOBILE-SATELLITE (space-to-Earth) Mobile-Satellite (space-to-Earth) | SPACE OPERATION (Earth-to-space) (space-to-space) EARTH EXPLORATION-SATELLITE (Earth-to-space) (space-to-space) FIXED MOBILE SPACE RESEARCH (Earth-to-space) (space-to-space) MOBILE-SATELLITE (space-to Earth) |
| B6 | 2 170-2 200 | FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) | SPACE OPERATION (space-to-Earth) (space-to-space) EARTH EXPLORATION-SATELLITE (space-to-Earth) (space-to-space) FIXED MOBILE SPACE RESEARCH (space-to-Earth) (space-to-space) |
| B7 | 2 180-2 200 | FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) | SPACE OPERATION (space-to-Earth) (space-to-space) EARTH EXPLORATION-SATELLITE (space-to-Earth) (space-to-space) FIXED MOBILE SPACE RESEARCH (space-to-Earth) (space-to-space) |

| TDD Frequency arrangements | Un-paired arrangement (MHz) | Incumbent Services | Adjacent band Services |
| --- | --- | --- | --- |
| B1 | 1 880-1 920 | FIXED MOBILE | FIXED MOBILE |
|  | 2 010-2 025 | FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) | MOBILE-SATELLITE (Earth-to-space) SPACE OPERATION (Earth-to-space) (space-to-space) EARTH EXPLORATION-SATELLITE (Earth-to-space) (space-to-space) FIXED MOBILE SPACE RESEARCH (Earth-to-space) (space-to-space) |
| B3 | 1 920-1 930 | FIXED MOBILE | FIXED MOBILE |
| B4 | 1 880-1 920 | FIXED MOBILE | FIXED MOBILE |
|  | 2 010-2 025 | FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) | MOBILE-SATELLITE (Earth-to-space) SPACE OPERATION (Earth-to-space) (space-to-space) EARTH EXPLORATION-SATELLITE (Earth-to-space) (space-to-space) FIXED MOBILE SPACE RESEARCH (Earth-to-space) (space-to-space) |
| B5 | 1 920-1 930 | FIXED MOBILE | FIXED MOBILE |

| TDD Frequency arrangements | Un-paired arrangement (MHz) | Incumbent Services | Adjacent band Services |
| --- | --- | --- | --- |
| E1 | 2 300-2 400 | FIXED MOBILE RADIOLOCATION Amateur | FIXED MOBILE MOBILE except aeronautical mobile SPACE RESEARCH (deep space) (space-to-Earth) RADIOLOCATION |

[Editor's note: The table for the Earth-to-space direction is included for information. Depending on further analysis, studies for this direction might not be necessary.]

| FDD Frequency arrangements | Mobile station transmitter/ (Earth-to-space direction) (MHz) |
| --- | --- |
| C1 | 2 500-2 570 |
| C2 | 2 500-2 570 |

| FDD Frequency arrangements | Base station transmitter/ (space-to-Earth direction) (MHz) | Incumbent Services | Adjacent band Services |
| --- | --- | --- | --- |
| C1 | 2 620-2 690 | FIXED MOBILE except aeronautical mobile BROADCASTING-SATELLITE FIXED-SATELLITE (Earth-to-space) (space-to-Earth) Earth exploration-satellite (passive) Radio astronomy Space research (passive) | FIXED MOBILE except aeronautical mobile BROADCASTING-SATELLITE FIXED-SATELLITE (space-to-Earth) EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) |
| C2 | 2 620-2 690 | FIXED MOBILE except aeronautical mobile BROADCASTING-SATELLITE FIXED-SATELLITE (Earth-to-space) (space-to-Earth) Earth exploration-satellite (passive) Radio astronomy Space research (passive) | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) |
|  | 2 570-2 620 | FIXED MOBILE except aeronautical mobile BROADCASTING-SATELLITE FIXED-SATELLITE (space-to-Earth) | FIXED MOBILE except aeronautical mobile BROADCASTING-SATELLITE FIXED-SATELLITE (space-to-Earth) |

| TDD Frequency arrangements | Un-paired arrangement (MHz) | Incumbent Services | Adjacent band Services |
| --- | --- | --- | --- |
| C3 | 2500-2690 | FIXED FIXED-SATELLITE (space-to- Earth) MOBILE except aeronautical mobile MOBILE-SATELLITE (space-to-Earth) BROADCASTING-SATELLITE FIXED-SATELLITE (Earth-to-space) (space-to-Earth) Earth exploration-satellite (passive) Radio astronomy Space research (passive) | FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) RADIODETERMINATIONSATELLITE (space-to-Earth) RADIOLOCATION EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) |

- Detailed list of sharing studies to be conducted

TBD

- Orbit and RF emission characteristics of systems and networks intended to provide direct connectivity between space stations and IMT user equipment

The following tables summarize orbit configurations of systems and networks intended to provide direct connectivity between space stations and IMT user equipment.

Table X: TBD

| Altitude (km) | Inclination (deg) | # Planes | Sats per plane | RAAN spacing (deg) | Total number of sats |
| --- | --- | --- | --- | --- | --- |

TBD

4.1 Emissions in the space-to-Earth direction

This section will contain relevant emission characteristics of the space station to conduct sharing studies. Relevant parameters should include, among others:

- Power Flux Density on the ground

- Satellite antenna pattern

- Cell edge definition (relative to cell center)

- TBD

TBD

4.2 Out-of-band emissions to be used for out-of-band studies

This section contains relevant information on out-of-band emissions.

TBD

4.3 Emissions in the Earth-to-space direction

The intended operations foresee communications from unmodified user equipment. Consequently, the receive/transmit characteristics of the user equipment will remain unchanged with respect to standard terrestrial operations.

4.4 Modelling of operations and the concept of topology

This section will contain important assumptions on how to accurately model operations of systems and networks implementing direct-to-device communications.

4.4.1 Satellite selection mechanism

TBD

4.4.2 The “topology” function

Satellite operators use software-based systems to dynamically manage interference and remain within applicable limits to protect all relevant services.

Modelling topology is fundamental and should be duly taken into account in studies, as this is something satellite systems will implement continuously to manage interference into neighboring countries.

4	Results of sharing studies

TBD