| U.S. Radiocommunications Sector Fact Sheet |  |
| --- | --- |
| Working Party: ITU-R WP 4C | Document No: US4C-23R0 |
| Ref: Document 4C/77, Annex 14 | Date: 12 July 2024 |
| Document Title: Proposed Revision to Preliminary Draft Revision of Report ITU-R M.2305-0, Consideration of aggregate radio frequency interference event potentials from multiple Earth exploration-satellite service systems on radionavigation-satellite service receivers operating in the 1 215-1 300 MHz frequency band |  |
| Author(s)/Contributors: Rick Merchant SSC/CGEP USSF Los Angeles AFB, CA Tiange (George) Fan, for GPS The Aerospace Corporation El Segundo, CA Tom L. Hayden, for GPS TLH Consulting Seattle, WA Stephen Baruch, for GPSIA New Wave Spectrum Partners LLC Dallas, TX Mark Rentz, for GPSIA John Deere Torrance, CA | Phone: (310) 653-1871 Email : rick.merchant.2@spaceforce.mil Phone : (310) 336-1252 Email : Tiange.Fan@aero.org Phone : (425) 443-1837 Email : Tom.Hayden@live.com Phone : (240) 476-2600 Email : sbaruch@newwavespectrum.com Phone : (310) 381-2607 Email : RentzMarkL@JohnDeere.com |
| Purpose/Objective: To provide an update to the Preliminary Draft Revision of Report ITU-R M.2305-0 that was developed during the April 2024 meeting of WP 4C. |  |
| Abstract: Working Party 4C initiated work on a potential revision to Report ITU-R M.2305 at its June/July 2023 meeting, and elevated the work to PDR status at its April 2024 meeting (see Doc. 4C/77, Annex 14). This contribution is intended to progress that work by proposing updates to the PDR of Report ITU-R M.2305-0 on aggregate interference from EESS (active) spaceborne SAR instruments into RNSS earth station receivers operating in the 1 215-1 300 MHz frequency band. |  |
| Fact Sheet prepared by: Steve Baruch |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Received: __ September 2024 Source: Document 4C/77, Annex 14 | Document 4C/__-E |
|  | __ September 2024 |
|  | English only |
| United States of America |  |
| PROPOSED REVSIONS TO PRELIMINARY DRAFT REVISION OF REPORT ITU-R M.2305-0 |  |

At its April 2024 meeting, Working Party 4C continued work on a preliminary draft revision of Report ITU-R M.2305-0, on the subject of aggregate radio frequency interference event potentials from multiple Earth exploration-satellite service systems on radionavigation-satellite service receivers operating in the 1 215-1 300 MHz frequency band.  See Annex 14 to Doc. 4C/77.

In this contribution, the United States proposes revisions and updates to PDR Report ITU-R M.2305-0.  Proposed U.S. changes are highlighted in green.

Attachments:  	1

ATTACHMENT

PRELIMINARY DRAFT REVISION OF REPORT ITU-R M.2305-0

Consideration of aggregate radio frequency interference event potentials from multiple Earth exploration-satellite service systems on radionavigation-satellite service receivers operating in the 1 215-1 300 MHz frequency band

(2014)

TABLE OF CONTENTS

Page

1	Introduction		2

2	Pulsed RFI effects		2

2.1	Effects of pulsed RFI from a single source		2

2.2	Aggregate RFI cases		3

3	EESS (active) sensors in the 1 215-1 300 MHz frequency band		4

3.1	General characteristics		4

3.2	Antenna characteristics		5

4	Aggregate RFI impingement analysis		8

4.1	Satellite models		8

4.2	Single sensor received isotropic power		8

4.3	Receiver models		9

4.4	Single-Sensor interference power into receivers		10

4.5	Aggregate RFI impingement statistics		11

5	Analysis results		14

5.1	Study A		14

5.2	Study B		15

5.3	Discussion		16

6	Summary		17

Annex 1 – Background on choice of time-step value for Scatterometer 2 calculations		18

Annex 2 – Example of evaluating the aggregate pulsed radio frequency interference from multiple EESS (active) spaceborne synthetic aperture radars to RNSS earth station receivers operating in the 1 215-1 300 MHz band		21

# [NOC]

Annex 1

[NOC]

Аnnex 2

Example of evaluating the aggregate pulsed radio frequency interference from multiple EESS (active) spaceborne synthetic aperture radars 
to RNSS earth station receivers operating in the 1 215-1 300 MHz band

# 1	Introduction

This annex provides an example of calculating the aggregate interference from multiple EESS (active) SAR sensors operating simultaneously over the same territory.

# 2	Example of evaluating the aggregate pulsed radio frequency interference from multiple spaceborne synthetic aperture radars

As an example, consider the impact of interference from the SAR1 system from Table 1 of Report ITU-R RS.2537-0 on an SBAS receiving RNSS earth station from Column 1 to Table 7. The characteristics of SAR1 are presented in Table 2-1 below.

Table 2-1

Technical characteristics of SAR1

| Parameter | Value |
| --- | --- |
| RF centre frequency, MHz | 1 257.5 |
| RF bandwidth, maximum, MHz | 40 |
| RF pulse width, µs | 33.8 |
| Pulse repetition frequency maximum, Hz | 1 736 |

The effective pulsed RFI duty cycle (PDCLIM) for SAR 1 is computed as follows:

PDCLIM=(PWSAR1,EFF+τr)PRFSAR1

where:

PWSAR1,EFF=PWSAR1(∆fChirpwidth)

The assumed SBAS receiver recovery time (τr) is 1.0 µs and the SBAS receiver pre-correlator filter bandwidth is 20.5 MHz centred at 1 227.6 MHz. Considering this PDCLIM  for SAR1 will be:

PDCLIM,  SAR1=0.00225

Using equation (8) from Annex 1 of Report ITU-R RS. 2537-0 the degradation ratio of the pulsed interference caused by SAR1 to the SBAS receiver is the following:

N0, EFF+YN0, EFF= 1(1-PDCY)2≈1.0045

Or in logarithmic form 10log10(N0,EFF+Y/N0,EFF) = 0.019 dB.

In accordance with Table 9 in Annex 1 of Report ITU-R RS.2537-0 the allowable degradation ratio of the SBAS receiver is 10log10(N0,EFF+Y/N0,EFF) = 0.2 dB. Thus, the SAR1 system meets the SBAS protection criteria.

Suppose that after some time a new SARA system appears, which is identical in characteristics to the SAR1 system, except that the central frequency of the signal will be 1 243.85 MHz. Due to the greater overlap of frequency bands, the effective pulse duration of such a system will be longer. Applying the same equations presented above, it turns out that the value of the effective pulse duty cycle for SARA is:

PDCLIM,  SARA=0.0223

Thus, the degradation ratio of the SARA interference impact on the SBAS receiver is 10log10(N0,EFF+Y/N0,EFF) = 0.196 dB. This system also meets the protection requirements of the SBAS receiver, since the degradation does not exceed 0.2 dB.

Now consider the cumulative impact of two systems – SAR1 and SARA – on the SBAS receiver in question if they operate simultaneously. Using equations (3), (4), (7) and (8) from Annex 1 of Report ITU-R RS.2537-0, the degradation for the SBAS receiver with simultaneous operation of SAR1 and SARA will be determined by the following equation:

N0,EFF+A+BN0,EFF=11-PDCA2*1-PDCB2

Or in logarithmic form:

10log10(N0,EFF+A+B/N0,EFF) = -20log10(1-PDCA) - 20log10(1-PDCB)

Thus, the degradation of simultaneous pulsed interference impact from the two new systems is equal to the sum of the degradations of the systems while they operate separately.

10log10(N0,EFF+A+B/N0,EFF) = 10log10(N0,EFF+A /N0,EFF) + 10log10(N0,EFF+B /N0,EFF)

Considering that for SAR1 and SARA systems the degradation is 0.019 dB and 0.196 dB, respectively, the total degradation will be 0.215 dB. This means that with simultaneous exposure to pulsed interference from SAR1 and SARA systems, the permissible degradation level for the SBAS receiver will be exceeded.

Thus, taking into account the increasing number of sources of pulsed interference, in order to correctly assess the interference effect of new pulsed systems on RNSS receivers, it is necessary to take into account the current cumulative interference from all operating pulsed systems.

3	Summary and proposals

This annex shows that when evaluating the impact of possible pulsed interference from new spaceborne synthetic aperture radars of the EESS on RNSS receivers based on the methodology of Recommendation ITU-R M.2030 as used in Report ITU-R RS.2537-0, it is necessary to take into account the cumulative simultaneous effect of pulsed interference from multiple sources. The methodology presented in Recommendation ITU-R M.2030 can be used for a preliminary assessment of the impact of pulsed interference to the RNSS receiver.

The issue of possible mechanisms to avoid or mitigate aggregate interference from multiple EESS (active) SAR systems requires further study, taking into account the examples in this Report.