| U.S. Radiocommunications Sector Fact Sheet |  |
| --- | --- |
| Working Party: ITU-R WP-5B | Document No: USWP5B33-13 |
| Ref: 5B/96 Annex 13 | Date: 15 August 2024 |
| Document Title: Update to the working document towards a preliminary draft new Report ITU-R M.[FOD_EESS_SHARE] |  |
| Author(s)/Contributors(s): | Contact |
| Ryan McDonough GRC NASA | ryan.s.mcdonough@nasa.gov |
| Purpose/Objective: To update the working document towards a draft new Report ITU-R M.[FOD_EESS_SHARE] document. This work will continue previous work on coexistence between Foreign Object Detection (FOD) systems operating in the 92-100 GHz band with EESS (passive) service in the 86-92 GHz and the EESS (active) service in the 94-94.1 GHz for radiofrequency interference. Summary of simulation results in section A1-2 will be finalized. |  |
| Abstract: This study provides will provide analyses of possible radiofrequency interference between the Foreign Object Detection (FOD) system placed along runways in the 92-100 GHz band with EESS (passive) in the adjacent band and EESS (active) in the 94-94.1 GHz band. This document contains two dynamic analyses of potential in-band interference to a spaceborne cloud profile radar in EESS (active) and OOB interference to a spaceborne radiometer in EESS (passive) in the 86-92 GHz band from FOD detection systems in the Radiolocation Service to be included in the Report. |  |

Radiocommunication Study Groups. Source: 5B/96 Annex 13 Subject: Report ITU-R M.[FOD_EESS_SHARE]; Document 3J/xx-E. 31 June 2024. English only. United States of America.

WORKING DOCUMENT TOWARDS A PRELIMINARY DRAFT NEW REPORT ITU-R M.[FOD_EESS_SHARE]

Sharing and compatibility studies between foreign object debris detection system and other services in the frequency ranges 92-100 GHz

Summary:

This study provides will provide analyses of possible radiofrequency interference between the Foreign Object Detection (FOD) system placed along runways in the 92-100 GHz band with EESS (passive) in the adjacent band and EESS (active) in the 94-94.1 GHz band. This document contains two dynamic analyses of potential in-band interference to a spaceborne cloud profile radar in EESS (active) and OOB interference to a spaceborne radiometer in EESS (passive) in the 86-92 GHz band from FOD detection systems in the Radiolocation Service to be included in the Report.

Proposal:

[TBD]

Attachments 1

ATTACHMENT 1

[TBD]

____________________