| US Radiocommunications Sector Fact Sheet |  |
| --- | --- |
| Working Party: WP 5B | Document No: USWP5B33-01 |
| Ref: Report ITU-R M.2530-0 (09/2023), Draft New Question Doc. 5-30 E | Date: 27 September 2024 |
| Document Title: Elements Toward a Working Document Toward Preliminary Draft New Recommendation ITU-R M.[DIGITAL-VOICE] Digital voice communications in the VHF maritime mobile band |  |
| Author(s)/Contributors(s): Jerry Ulcek US Coast Guard, Washington, DC Johnny Schultz Sev1Tech, Inc. Ross Norsworthy REC, Inc. | Phone : (202) 475-3607 E-mail: Jerry.l.Ulcek@uscg.mil Phone : (727) 403-4029 E-mail: johnny.schultz@sev1tech.com Phone : (727) 515-8025 E-mail: Ross_Norsworthy@msn.com |
| Purpose/Objective: This document provides “Elements Toward a Working Document Toward a Preliminary Draft New Recommendation ITU-R M.[DIGITAL-VOICE]” that describes the technical characteristics of a proposed new system for transmission of digital voice communications in the VHF maritime mobile band. |  |
| Abstract: Report ITU-R M.2530-0 describes candidate technologies and technical considerations for digital voice communications in the VHF maritime mobile band. Draft New Question ITU-R [DIGITAL VOICE]/5, Document 5-30 E, describes conditions intended to ensure compatibility with incumbent services that utilize the frequencies in Appendix 18 of the Radio Regulations (RR). To ensure compatibility with incumbent services, the new digital voice system should not use channels identified for GMDSS, AIS, VDES, safety communications, and other incumbent services. It should also be interoperable with the legacy linear FM voice communications on a secondary basis. |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Ref: Report ITU-R M.2530-0 (09/2023), Draft New Question Doc. 5-30 E | USWP5B33-01 |
|  | 26 September 2024 |
|  | English only |
| United States Elements Toward a Working Document Toward Preliminary Draft New Recommendation ITU-R M.[DIGITAL-VOICE] Digital voice communications in the VHF maritime mobile band |  |
| Digital voice communication in the VHF maritime mobile band |  |

Purpose

The purpose of this document is to provide Elements Toward a Working Document Toward a Preliminary Draft New Recommendation ITU-R M.[DIGITAL-VOICE] that describes the technical characteristics of a proposed new system for transmission of digital voice communications in the VHF maritime mobile band.

Abstract

Report ITU-R M.2530-0 described candidate technologies and technical considerations for digital voice communications in the VHF maritime mobile band. The new system should be designed such that it does not utilize the channels used for the GMDSS, AIS, VDES, and safety communications. It should also be interoperable with the legacy linear FM voice communications.

Attachment:	1

ATTACHMENT Elements Toward a Working Document Toward Preliminary Draft New Recommendation ITU-R M.[DIGITAL-VOICE] Digital voice communications in the VHF maritime mobile band

Digital voice communication in the VHF maritime mobile band

Scope

This recommendation describes a new technology that will permit the possible expansion of the number of VHF maritime voice channels. Studies are underway concerning operational reliability, impacts to the GMDSS, mode of operation (simplex/duplex), bandwidth, range, etc. which are the necessary objectives to determine the feasibility of implementation of digital voice radio telephony in the VHF maritime mobile band.

Keywords

Analogue communications, digital communications, migration, GMDSS, VHF radio, DSC

Abbreviations/Glossary (including terms used in the referenced Report ITU-R M.2530-0)

ACELP	Algebraic code-excited linear prediction

ADPCM	Adaptive differential pulse-code modulation

AIS	Automatic identification system

AMBE	Advanced multi-band excitation

AMR-WB	Adaptive multi-rate wideband

ATIS	Automatic transmitter identification system

CIRM	Comité International Radio-Maritime

CEPT	European Conference of Postal and Telecommunications Administrations

CODEC	Coder-decoder

CS-ACELP 	Conjugate-structure algebraic-code excited linear prediction

DSC	Digital selective call

DMR	Digital mobile radio

DPMR	Digital private mobile radio

ETSI	European Telecommunications Standards Institute

EU	European Union

FDMA	Frequency division multiple access

FEC	Forward error correction

FRAND 	Fair, reasonable and non-discriminatory

FSK	Frequency shift keying

GNSS	Global navigation satellite service

GMDSS	Global maritime distress and safety system

HF	High frequency

IALA	International Association of Marine Aids to Navigation and Lighthouse Authorities

IMO	International Maritime Organization

LD-CELP	Low delay code excited linear prediction

MF	Medium frequency

MMSI	Maritime mobile service identity

MoU	Memorandum of understanding

MSC	Maritime Safety Committee

NXDN	Next generation digital narrowband

PAMR	Public access mobile radio

PESQ	Perceptual evaluation of speech quality

PMR	Private mobile radio

POTS	Plain old telephone service

PSTN	Public switched telephone network

RAINWAT	Regional Arrangement on the Radiocommunication Service for Inland Waterways

RALCWI 	Robust advanced low complexity waveform interpolation

RR	Radio Regulations

SB-ADPCM 	Sub-band adaptive differential pulse code modulation

SMS	Short message service

SNR	Signal to noise ratio

SOLAS	Safety of Life at Sea

TETRA	Trans-European trunked radio system

TDMA	Time division multiple access

TCP/IP	Transmission control protocol/internet protocol

TWELP	Tri-wave excited linear prediction

Tx	Transmission

VDES	VHF data exchange system

VHF	Very high frequency

VTS	Vessel traffic service

WRC	World Radiocommunication Conference

Related ITU Recommendations and Reports

Recommendations:

ITU-R M.493 	Digital selective-calling system for use in the maritime mobile service

ITU-R M.541 	Operational procedures for the use of digital selective-calling equipment in the maritime mobile service

ITU-R M.585	 Assignment and use of identities in the maritime mobile service

ITU-R M.1084 	Interim solutions for improved efficiency in the use of the band 156-174 MHz by stations in the maritime mobile service

ITU-R M.1171 	Radiotelephony procedures in the maritime mobile service

ITU-R M.1309	 Digitally coded speech in the land mobile service

ITU-R M.1808	 Technical and operational characteristics of conventional and trunked land mobile systems operating in the mobile service allocations below 869 MHz to be used in sharing studies in bands below 960 MHz

Reports:

ITU-R BT.2140	 Transition from analogue to digital terrestrial broadcasting

ITU-R M.2010 	Improved efficiency in the use of the band 156-174 MHz by stations in the maritime mobile service

ITU-R M.2231 	Use of Appendix 18 to the Radio Regulations for the maritime mobile service

ITU-R M.2288	 Digital voice communication system on MF/HF radio channels of the maritime mobile service for shore-to-ship/ship-to-shore applications

ITU-R M.2474	 Conventional digital land mobile radio systems

ITU-R M.2530	 Digital voice communication in the VHF maritime frequency band

ITU-R SM.2022 	The effect on digital communications systems of interference from other modulation schemes

The ITU Radiocommunication Assembly,

considering

a)	that the International Maritime Organization (IMO) has established performance standards for shipborne radio equipment:

–	Resolution A.694(17): General requirements for shipborne radio equipment forming part of the Global Maritime Distress and Safety System (GMDSS) and for electronic navigational aids;

–	Resolution MSC.511(105): Performance standards for shipborne VHF radio installations capable of voice communication and digital selective calling;

–	SOLAS Convention Chapter IV/5.1.3;

–	SOLAS IV 7.1.1;

–	SOLAS IV 7.1.2;

b)	that the main performance standard for VHF shipborne radio installations is IMO Resolution MSC.511(105) which defines the minimal requirements for the use, availability, installation, robustness, etc. of VHF radios on board ships;

c)	that the Radio Regulations (RR Appendix 18) have incorporated ITU technical standards that apply to VHF marine radios: ITU-R M.493; ITU-R M.541; ITU-R M.585; ITU-R M.1084;

d)	that the International Electrotechnical Commission (IEC) has established test standards for compliance with technical requirements defined by IMO, ITU, and the Radio Regulations, and that these IEC standards have been incorporated in IMO carriage requirements for ships,

recognizing

- that the implementation of digital voice communications should not impair the functions of safety systems that operate in RR Appendix 18, i.e., digital selective calling (DSC), automatic identification system (AIS), VHF data exchange system (VDES), and voice distress, safety and calling communication (Channel 16), bridge-bridge communication (Chennel 13), and other safety-related communications, and that it should not utilize channels that are not designated for maritime service by the various Administrations,

- that some frequencies in the band used by MMS in Appendix 18 are allocated to the fixed and mobile services on a co-primary basis,

- that a need exists to protect existing and planned in-band and adjacent-band services with no additional regulatory or technical constraints on these co-primary incumbent services when considering any potential modifications to MMS channelling arrangements.

recommends

1	that general elements of VHF digital voice communication should be in accordance with Annex 1;

2	that the operations of VHF digital voice communications should be compatible with and should not interfere with the operations of IMO/ITU/IEC certified marine equipment;

3	that equipment that utilizes VHF digital voice communication should comply with IMO/ITU/IEC requirements;

4	that equipment that utilizes VHF digital voice communication should use DSC for calling in accordance with IMO/ITU/IEC standards;

5	that technical characteristics VHF digital voice communication should be in accordance with Annex 1.

.

Annex 1

Technical characteristics of the VHF digital voice communication operating system

# 1	General description of VHF digital voice communication system

In essence, the VHF digital voice communication system provides a means for increasing the number of available voice channels in RR Appendix 18 by multiplexing an available 25 kHz voice channel to provide four 6.25 kHz voice subchannels. The system senses the 25 kHz channel to determine availability. If the 25 kHz channel is occupied by 25 kHz voice communications, then the system considers that channel unavailable. If the 25 kHz channel is in use by digital voice communications, then the system determines which, if any, of the 6.25 kHz subchannels is available and selects an unused subchannel. The system will not implement digital voice communications on channels that are designated for safety systems in RR Appendix 18, such as digital selective calling (DSC), automatic identification system (AIS), applications specific messages (ASM), VHF data exchange system (VDES), voice distress, safety and calling communication (Channel 16), bridge-bridge communication (Channel 13), and other safety-related communications,

- Key technical characteristics

2.1 Simplex and duplex operation

The VHF digital voice communication system may operate in either simplex or duplex mode in accordance RR Appendix 18.

2.2 Channel usage

The VHF digital voice communication system will use only those channels that are not designated  for safety systems in RR Appendix 18, such as digital selective calling (DSC), automatic identification system (AIS), applications specific messages (ASM), VHF data exchange system (VDES), voice distress, safety and calling communication (Channel 16), bridge-bridge communication (Channel 13), and other safety-related communications. RR Appendix 18 may in the future update an existing footnote to elaborate on which channels are not available for VHF digital voice communication.

### 2.2.2	Performance standards

The main performance standards for VHF shipborne radio installations are contained in IMO Resolution MSC.511(105) and in Recommendation ITU-R M.493.

These performance standards define the minimal requirements for the use, availability, installation, robustness, etc. of VHF radios on board ships.

## 2.2.3	Channel access and channel numbering

1	Channel number assignments for implementing digital voice communications with 6.25 kHz channel spacing between the four subchannels on 25 kHz channels in the VHF maritime frequency band should be in accordance with Table 1.

2	The channel centre frequency retains its channel number in the VHF maritime frequency band, and it is used exclusively for 25 kHz voice communications. Digital voice communications should not be used when the 25 kHz channel is being utilized for normal 25 kHz voice communications.

Table 1

Example of channel number assignments with 6.25 kHz channel spacing

| Channel No. (6.25 kHz spacing between subchannels) |  |  |  |  | Ship | Ship and coast | Coast |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 801 |  |  | 156.059375 |  | 160.659375 |
|  | 701 |  |  |  | 156.053125 |  | 160.653125 |
| 01 |  |  |  |  | 156.050 |  | 160.650 |
|  | 601 |  |  |  | 156.046875 |  | 160.646875 |
|  |  | 501 |  |  | 156.040625 |  | 160.640625 |

- Technical characteristics of marine VHF radios

Marine VHF radios with DSC (DSC, digital selective calling, is used for distress calling and general automated calling purposes, and it is required by most administrations) capability are divided into three main classes:

- Class A, mandatory carriage for SOLAS ships under GMDSS SOLAS Chapter IV.

- Class D, voluntary carriage by non-SOLAS vessels.

- Class H, handheld radios for non-SOLAS voluntary carriage.

Some of these have integral Global Navigation Satellite System (GNSS)NSS, and some also have texting capability (RTCM SC123 standard).

Digital voice-capable radios would need to meet the current requirements of their respective classes.

- Technical characteristics of the VHF maritime band, RR Appendix 18

The VHF maritime band covers the frequency ranges, 156.025 to 157.425 MHz, and 160.625 to 162.025 MHz, with a gap in the middle frequency range between 157.425 and 160.625 MHz, and it is channelized in 25 kHz channels with channel numbers in two digits and four digits in accordance with Recommendation ITU-R M.1084-5.

RR Appendix 18 contains simplex and duplex channels, and some of the duplex channels may be used in simplex mode, designated by four-digit channel numbers.

Footnotes are used in RR Appendix 18 to designate how the channels are used, for example:

- For DSC, channel 70, footnotes j) and f).

- For AIS, channels AIS 1 and AIS 2, footnotes f), l), and p).

- For VDES, numerous channels in both the lower and upper frequency ranges, footnote w).

# 5	Technical parameters

The technical parameters of the transmitting apparatus should comply with the ITU regulations.

# 6	Technical requirements for voice communication and associated digital selective calling in the VHF maritime mobile band

The following requirements should be considered based on the regulations for VHF DSC radios (the sources of the regulations are given in brackets):

–	Should be capable of operating on single-frequency channels or on single- and two frequency channels (IMO);

–	Operating in the bands 156.025 MHz to 157.325 MHz and 160.625 MHz to 161.925 MHz on single-frequency and two-frequency channels (ITU/IMO);

–	Should provide at least three priorities of communications using voice (IMO);

–	A dedicated DSC watchkeeping facility to maintain a continuous watch on channel 70 (IMO);

–	DSC facility should be capable of operating on Channel 70 (IMO).

# 7	Voice coder-decoders (CODECs)

Appendix 18 of the Radio Regulations offers channels with a spectrum bandwidth of 25 kHz for analogue speech communications. Using technologies available today this can be split up to improve the spectral efficiency by applying digital encoding techniques to the speech signals.

The VHF digital communication system uses a frequency division multiple access (FDMA) approach, splitting the 25 kHz radio channel into four separate radio channels to yield a channel bandwidth of 6.25 kHz. This technology allows for an over air data rate of 4800 bps, using 4 FSK modulation and still remaining within the adjacent channel power limits. Removing the signalling overhead, this results in a speech channel of 3600 bps, of which approx. 1/3 is used for error correction, so that the data channel available for encoding the speech waveform is approx. 2 400 bps.

Numerous voice CODECs are available to perform this function, and one should be selected for this application. The selected CODEC should also conform to the patent policy of ITU.