| U.S. Radiocommunications Sector Fact Sheet |  |  |
| --- | --- | --- |
| Working Party: ITU-R WP 5C | Document No: USWP5C 31-05 |  |
| Ref: WRC-27 AI 1.13 | Date: August 23, 2024 |  |
| Document Title: Proposed draft liaison statement to Working Party 4C |  |  |
| Author(s)/Contributors(s): Andrew Meadows AFSMO Arminder Singh eSimplicity support AFSMO Dominic Nguyen eSimplicity support AFSMO |  | Phone: 334-467-4720 Email: andrew.meadows.1@us.af.mil Phone: 281-865-8678 Email: arminder.singh@esimplicity.com Phone: 703-606-7394 Email: dominic.nguyen@esimplicity.com |
| Purpose/Objective: This contribution proposes a draft liaison statement to WP 4C regarding AI 1.13, which will allow WP 4C to proceed with its planning for sharing and compatibility studies. |  |  |
| Abstract: Pursuant to Resolution 253 (WRC-23), in preparation for Agenda Item 1.13 (WRC-27), this contribution proposes a draft reply liaison statement to WP 4C with relevant technical information of systems operating in the FS frequency band 1 780-1 850 MHz. |  |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: None Subject: WRC-27 Agenda Item 1.13 | Document 5C/ |
|  | 12 September 2024 |
|  | English only |
| United States of America |  |
| proposed draft reply liaison statement to Working party 5D Relevant technical information for sharing studies under WRC-27 Agenda Item 1.13 |  |

Introduction

WRC-27 Agenda Item 1.13 considers possible new allocations to the mobile-satellite service in the frequency range between 694/698 MHz and 2.7 GHz, for direct connectivity between space stations and International Mobile Telecommunications (IMT) user equipment to complement terrestrial IMT network coverage, in accordance with Resolution 253 (WRC-23). This contribution proposes a draft reply liaison statement to WP 4C with relevant technical information of fixed systems operating in the fixed service frequency band 1 780-1 850 MHz.

Attachment:  1

ATTACHMENT

# Working Party 5C

PROPOSAL DRAFT REPLY LIAISON STATEMENT TO WORKING PARTY 4C

Fixed service characteristics for use in sharing
 studies under WRC-27 agenda item 1.13

Working Party (WP) 5C has sent WP 4C a liaison statement (Document 4C/88), providing the characteristics and protection criteria of the fixed service (FS) systems operating in the frequency band 1 780-1 850 MHz, for sharing and compatibility studies between the FS and mobile satellite service (MSS) under WRC-27 agenda item 1.13. In addition to the characteristics provided in the ITU-R Rec. F.758, Table 1 below provides characteristics for FS that were not included in the Recommendation. WP 5C requests consideration of these characteristics in the sharing and compatibility studies for WRC-27 AI 1.13.

Table 1

| Frequency range (GHz) | 1 780-1 850 MHz |
| --- | --- |
| Modulation |  |
| Channel spacing and receiver noise bandwidth (MHz) |  |
| Tx output power range (dBW) |  |
| Tx output power density range (dBW/MHz) |  |
| Feeder/multiplexer loss range (dB) |  |
| Antenna gain range (dBi) |  |
| e.i.r.p. range (dBW) |  |
| e.i.r.p. density range (dBW/MHz) |  |
| Receiver noise figure typical |  |
| Receiver noise power density typical (=NRX) (dBW/MHz) |  |