| U.S. Radiocommunications Sector Fact Sheet |  |
| --- | --- |
| Working Party: ITU-R WP 5B | Document No: USWP5B33FS-05 |
| Ref: Annex 3 to Document 5B/96-E | Date: 3rd October2024 |
| Document Title: preliminary draft new Recommendation Itu-r m.[cnpc_char_5GHz] - Characteristics and protection criteria of terrestrial and satellite unmanned aircraft system control and non-payload communications links operating in the aeronautical mobile (route) service and aeronautical mobile satellite (R) service in the band 5 030-5 091 MHz. |
| Author(s)/Contributors(s): Name: Don Nellis Org: Federal Aviation Administration Name: Michael Neale Org: ACES Corporation for the FAA | Phone: (202) 267-9779 Email: Donald.Nellis@faa.gov Phone: (858) 705-8978 Email: michael.neale@aces-inc.com |
| Purpose/Objective: The purpose of this contribution is to propose an update to the terrestrial and satellite characteristics based on a recent update to the RTCA MOPS DO-362A and EUROCAE MOPS ED-265 that standardize and define this CNPC Link. To enable the finalization of the provisional nature of RR No. 5.443C it includes (in an attachment) a proposed liaison statement to WP 4C requesting characteristics, protection criteria and operating scenarios for RNSS systems using the 5 010 - 5 030 MHz frequency band. |  |
| Abstract: This contribution contains characteristics and protection criteria for terrestrial and satellite based systems that can be used for remote control of unmanned aircraft. |  |

CHANGES SINCE FIRST DRAFT HIGHLIGHTED IN YELLOW

| Radiocommunication Study Groups |  |
| --- | --- |
| Received : XXX Source: Annex 3 to Document 5B/96 Subject: New Recommendation ITU-R M.[CNPC_CHAR_5GHZ] | Document 5B/XXX-E |
|  | XXX |
|  | English only |
| United Sates of America |  |
| preliminary draft new Recommendation Itu-r m.[cnpc_char_5GHz] |  |
| Characteristics and Protection Criteria of Terrestrial and Satellite Unmanned Aircraft System Control and Non-Payload Communications Links operating in the aeronautical mobile (route) service and aeronautical mobile satellite (R) Service in the band 5 030-5 091 MHz |  |

Introduction

At WRC-2012 it was agreed, under No. 5.443C, that the frequency band 5 030-5 091MHz could be used by the aeronautical mobile (R) service limited to internationally standardized aeronautical systems.  Industry, international standards development organizations and ICAO have been working since then to develop the technology and standards necessary to use that allocation.  Consequently, it is now possible to provide characteristics and protection criteria for such systems for use in any future sharing studies within ITU-R.

Proposal

The United States of America proposes to assist in answering the above need by providing characteristics for such Control and Non-Payload Communications (CNPC) links operating in the AM(R)S allocation under No. 5443C and used in air-ground applications between Unmanned Aircraft (UA) and their Control Station (CS) where the Remote Pilot (RP) is located.

Attachment:	1

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Annex 3 to Document 5B/96 Subject: New Recommendation ITU-R M.[CNPC_CHAR_5GHz] | Document 5B/XXX-E |
|  | XXX |
|  | English only |
| PRELIMINARY DRAFT NEW RECOMMENDATION ITU-R M.[CNPC_CHAR_5GHZ] |  |
| Characteristics and protection criteria of terrestrial and satellite unmanned aircraft system control and non-payload communications links operating in the aeronautical mobile (route) service and aeronautical mobile-satellite (R) service in the band 5 030-5 091 MHz |  |

Scope

This Recommendation specifies the characteristics of control and non-payload communications (CNPC) links, carrying command and control (C2) information, operating in the aeronautical mobile (route) service (AM(R)S) and aeronautical mobile-satellite (route) service (AMS(R)S) in the frequency band 5 030-5 091 MHz in order to be used in analyzing compatibility between unmanned aircraft systems (UAS) CNPC C2 Links operating in the AM(R)S, as well in the AMS(R)S and other services.

Keywords

Unmanned aircraft systems

Abbreviations/Glossary

AM(R)S:	Aeronautical mobile (route) service

AMS(R)S:	Aeronautical mobile-satellite (route) service

ARRS:	Airborne radio relay system

ARS:	Airborne radio system

BGRS:	Beyond line-of-sight ground radio system

C2:	Command and control

CNPC:	Control and non-payload communication

CRC:	Cyclic redundancy check

DVB-RCS:	Digital video broadcasting - return channel via satellite

FDMA:	Frequency division multiple access

GMSK:	Gaussian minimum shift keying

GRS:	Ground radio system

ICAO:	International Civil Aviation Organization

QPSK:	Quadrature phase shift keying

RPA:	Remotely piloted aircraft

RPAS:	Remotely piloted aircraft system

RR:	Radio Regulation

TCC:	Turbo code comparison

TDD:	Time division duplex

UA:	Unmanned aircraft

UACS:	Unmanned aircraft control station

UAS:	Unmanned aircraft system

Related ITU Reports

Reports

ITU-R M.2205	Results of studies of the AM(R)S allocation in the band 960-1 164 MHz and of the AMS(R)S allocation in the band 5 030-5 091 MHz to support control and non-payload communications links for unmanned aircraft systems

ITU-R M.2233	Examples of technical characteristics for unmanned aircraft control and non-payload communications links

ITU-R M.2171	Characteristics of unmanned aircraft systems and spectrum requirements to support their safe operation in non-segregated airspace

The ITU Radiocommunication Assembly,

considering

a)	that …

recognizing

a)	that the frequency band 5 030-5 091 MHz is allocated to both the aeronautical mobile (route) service (AM(R)S) and the aeronautical mobile-satellite (route) service (AMS(R)S) and are planned to be used for unmanned aircraft (UA) control and non-payload communication (CNPC) command and control (C2) links to support the safe operation of UA;

b)	that the frequency band 5 030-5 091 MHz is also allocated to the aeronautical radionavigation service;

c)	that some internationally standardized microwave landing systems operate in this band in accordance with Radio Regulations (RR) No. 5.444;

d)	that from RR No. 5.444, in the frequency band 5 030-5 091 MHz, the requirements of microwave landing system have priority over other uses of this frequency band;

e)	that use of the frequency band 5 030-5 091 MHz by the AM(R)S and AMS(R)S is limited to internationally standardized aeronautical systems in accordance respectively with RR No. 5.443C and RR No. 5.443D.

recommends

1	that the technical and operational characteristics of the UA CNPC links for C2 operating in the AM(R)S and AMS(R)S described in the Annex should be considered representative of AM(R)S and AMS(R)S systems operating in the frequency band 5 030-5 091 MHz and should be used in studies of compatibility with systems operating under an allocation to another service;

2	that an aggregate interference protection criterion I/N for terrestrial system unmanned aircraft system (UAS) CNPC receiving stations of −6 dB should be used.

Annex

# 1	Introduction and scope

The characteristics and protection criteria of UAS CNPC C2 links and their spectrum requirements must support the safe operation of unmanned aircraft (UA) in non-segregated airspace. There is a strong and growing demand for the use of UAS (also known as remotely piloted aircraft systems (RPAS) within the international civil aviation organization (ICAO)) in civil applications. These UA flights will share airspace with passenger carrying aircraft, so their operation needs to be managed to safely allow the introduction of this new paradigm in aviation.

As these communications are critical for a safe management of the controlled airspaces, especially in terminal approach areas with high density of aircraft, future ICAO standards are obviously mandatory for these kinds of communications.

The CNPC C2 Link between the unmanned aircraft control station (UACS) and the UA support the following two ways of communication:

–	The forward link: To send telecommands to the unmanned aircraft for flight and navigation equipment control.

–	The return link: To send telemetry (e.g. flight status) from the UA to the UACS. It is anticipated that in some flight conditions or in specific airspaces it may be necessary to downlink video streams.

The potential types of C2 information exchanges carried over the CNPC C2 link system are:

The UA control – To support the remote pilot's activity to fly the UA, status information from the aircraft back to the remote pilot is essential on a frequent basis relative to the dynamics of the UA.

The UA avionics – Avionics systems send information (e.g. flight guidance system, flight management system, air traffic control communication, detect and avoid, weather radar, status reporting system) over the CNPC C2 link system from the UA to the UACS.

One aspect of the management of safe UAS operations is the management of the interference received by the receivers that link the UA and the UA CS. To enable this interference analysis to be undertaken the characteristics and protection criteria for these links operating in the AM(R)S and AMS(R)S allocations under RR Nos. 5.443C and 5.443D are proposed. This recommendation contains those characteristics and protection criteria based on systems which are currently under development and that will have to comply with the international standardization being developed by ICAO.

The CNPC C2 link consists of a suite of air-to-ground links that can be used simultaneously or independently, as required, to provide operational coverage and performance.  In total it consists of a terrestrial based component, a high-altitude relay-based component and a satellite-based component.

The terrestrial component uses an airborne radio system (ARS) on the UA to communicate with a ground radio system (GRS) that connects to the UACS. The high-altitude relay component uses a similar ARS on the UA, [which communicates via the airborne radio relay system (ARRS)] to a beyond line-of-sight ground radio system (BGRS) on the ground that connects to the UACS. The satellite component uses an airborne earth station on the UA to communicate with a ground earth station (via the geostationary satellite) that connects to the UACS.

[Figure 1

Example command and non-payload communications link system components]

[Editor’s note: Concerns were raised with respect to the Airborne radio relay system in the figure above. According to the explanations given by ICAO, the relay system is under consideration by ICAO for standardization. It is not clear if the standardization will be completed before the adoption of this recommendation]

# 2	Characteristics of command and non-payload communication links at 5 GHz

## 2.1	Unmanned aircraft and control station characteristics for terrestrial control 
and non-payload communication link

TABLE 1

Transmission and reception characteristics for the terrestrial control 
and non-payload communication link System 1

| Terrestrial command and non-payload communication System 1 |  |  |  |
| --- | --- | --- | --- |
|  | Units | UA ARS | GRS |
| Frequency of operation | MHz | 5 030 to 5 091 | 5 030 to 5 091 |
| Duplexing |  | Time division duplex (TDD) | TDD |
| Transmit/receive duration | msec | 60 Receive + 2.5 guard time 65 Transmit + 2.5 guard time | 60 Transmit + 2.5 guard time 65 Receive + 2.5 guard time |
| Modulation |  | GMSK or QPSK | GMSK or QPSK |
| Modulation symbol rates | ksps | GMSK: TBD including error correction/detection, guard times and synchronization overhead | GSMK: TBD including error correction/detection, guard times and synchronization overhead |
| Forward error correction |  | GMSK: Rate 5/8 Turbo Conv. Code QPSK: Rate 5/9 and Rate 1/3 TCC | GMSK: Rate 5/8 Turbo Conv. Code QPSK: Rate 5/9 and Rate 1/3 TCC |
| Error detection |  | 32-bit CRC | 32-bit CRC |
| Baseband Input/Output Signal |  | User Data | User Data |
| User Data Bit Rates | kbps | GMSK: TBD QPSK: TBD Includes TDD duty cycle overhead | GMSK: TBD QPSK: TBD Includes TDD duty cycle overhead |
| Occupied bandwidth, C | kHz | Variable per application with a maximum of 250 | Variable per application with a maximum of 250 |
| Antenna gain | dBi | 2 | Maximum 20 |
| Cable loss | dB | 2 | 1 |
| Antenna pattern |  | Constant azimuth Constant elevation | Constant azimuth Tailored in elevation See Table 2 |
| Antenna polarization |  | Vertical with aircraft flying straight and level | Vertical |
| Maximum antenna height | m | 22 860 (mean sea level) Typical 6 000 | 2 to 50 Typical 10 |
| Service range | km | 550 Typical 80 | 550 Typical 80 |
| Transmitter conducted power | dBm | 40 | 40 |
| Transmitter in band emission limits | dBc/1 MHz | −66 at 2 MHz offset See Table 3 | −66 at 2 MHz offset See Table 3 |
| Receiver noise figure | dB | 7 | 7 |
| Receiver sensitivity | dBm | GMSK: TBD QPSK: TBD | GMSK: TBD QPSK: TBD |
| Receiver in band rejection – except the operating channel | dB | One channel separation: 23 Two channel separation: 43 Three channel separation: 57 2 MHz or more separation: 63 | One channel separation: 23 Two channel separation: 43 Three channel separation: 57 2 MHz or more separation: 63 |
| Protection criteria (aggregate) I/N | dB | –6 | –6 |

TABLE 2

GRS elevation antenna pattern envelope
is constant in azimuth for System 1 with 20 dBi antenna with elevation 3 dB beamwidth equal to 9° and azimuth 3 dB beamwidth equal to 32°

| System 1 |  |
| --- | --- |
| Elevation degrees | Gain dBi |
| 0.5 | 19.0 |
| 1.5 | 19.5 |
| 2.5 | 20.0 |
| 3.5 | 19.5 |
| 7 | 17.0 |
| 11.5 | 14.0 |
| 16 | 11.5 |
| 32 | 6.5 |
| 64 | 1.5 |
| >75 | 0.5 |

TABLE 3

Transmitter out of band emission limits in the 5 030-5 091 MHz frequency band for system 1

| System 1 |  |
| --- | --- |
| Offset from carrier frequency | dBc/kHz |
| Channel width ÷ 2 | −54 |
| 1.5 × channel width | −74 |
| 500 kHz | −90 |
| 2 000 kHz | −96 |

TABLE 4

Transmitter spurious emission limits

|  | Maximum command and non-payload communication link system power spectral density in the spurious domain |  |
| --- | --- | --- |
|  | UA ARS | GRS |
| System 1 | TBD | TBD |

[Editor’s note: It is envisioned that the proposed Recommendation will eventually include the spurious emission characteristics of AM(R)S transmissions into adjacent allocations including those below 5 030 MHz that would be necessary for sharing studies to resolve the provisional nature of the -75 dBW/MHz protection value in RR No. 5.443C. See attached proposed liaison statement to WP 4C requesting the latest information on the RNSS operating in the 5 010 to 5 030MHZ frequency band]

TABLE 5

Terrestrial System 1 link budget

| Link Budget Element | GRS to UA | UA to GRS |
| --- | --- | --- |
| Maximum Slant Range | 80 km | 80 km |
| Typical UA Altitude (AGL) at Slant Range | 6,000 m | 6,000 m |
| GRS to UA LOS Path Elevation Angle | 3.0 deg | 3.0 deg |
| Transmitter Antenna Gain at Path Elevation Angle | 19.75 dBi | 2 dBi |
| EIRP (10W transmitter conducted power, plus Antenna Gain minus cable loss) | 58.75 dBm | 41.0 dBm |
| Free Space Loss at Slant Range at 5 091 MHz | 144.7 dB | 144.7 dB |
| Multipath Fading plus Airframe Obstruction for 99.8% Availability | 13.0 dB | 13.0 dB |
| Receiver Antenna Gain at Path Elevation Angle | 2 dBi | 19.75 dBi |
| Received Signal Level, C (including cable loss) | -98.0 dBm | -98.0 dBm |
| Total On Channel Interference Power Density from other CNPC Systems | -138.3 dBm/kHz | -129.0 dBm/kHz |
| Receiver Noise Power for 7dB Noise Figure in 500 kHz Noise Bandwidth | -140.0 dBm/kHz | -140.0 dBm/kHz |
| Total On Channel Interference Power Density from other non-CNPC Systems at -6dB Aggregate I/N Protection Criteria | -146.0 dBm/kHz | -146.0 dBm/kHz |
| Combined On Channel Interference and Noise Power Density | -135.6 dBm/kHz | -128.6dBm/kHz |
| ICAO 6dB Aeronautical Safety Margin added to Combined On Channel Interference and Noise Power Density, Io + No | -129.6 dBm/kHz | -122.6 dBm/kHz |
| Maximum Modulation Symbol Rate, Rs | 170 kHz | 170 kHz |
| Es/No including 3 dB implementation Loss, for required BER assuming GMSK with rate 5/8 Turbo Convolutional Code for FEC | 2.3 dB | 2.3 dB |
| Required Cmin/(Io + No), equals Es/No plus 10 x Log Rs | 24.6 dBm-kHz | 24.6 dBm-kHz |
| Available C/(Io + No) | 31.6 dBm-kHz | 24.6 dBm-kHz |
| Excess Link Margin @ Maximum Modulation Symbol Rate | 7.0 dB | 0.0 dB |

## [2.2	Unmanned aircraft and ground radio station characteristics for control and non-payload communication link via high altitude relay platform

Command and non-payload communications link can also be used via high altitude relay platform. Such high-altitude relay system consists of:

a)	A BGRS on the ground, with user data and link management interfaces to the host ground system.

b)	An airborne relay radio system (ARRS) with user data and link management interfaces hosted on the airborne relay RPA.

c)	An ARS with user data and link management interfaces hosted on the RPA.

This high-altitude relay system based C2 link system (“system 3” in this document) utilizes the frequency band 5 030–5 091 MHz to provide two-way communication between a beyond line-of-sight GRS (BGRS) and RPA flying at lower altitude via an airborne relay RPA. By using radios on the airborne relay-RPA as a relay point, C2 Link for other RPA can be extensively transmitted without deploying many radios stations on the ground.

The ARRS relays the C2 link between the ARSs and a BGRS within the coverage area of the system. As shown in Figure XX the ARRS simultaneously forms multiple beams (using one or more antennas) for the User Links between the ARRS and ARS and for the Feeder Link between the ARRS and BGRS.

Figure XX

Overview of the high-altitude relay system

The feeder link can be used as a C2 link for the airborne relay-RPA equipped with the ARRS. The airborne relay-RPA can also be controlled by another C2 link system. Communication protocols of a forward link of the User Link and a reverse link of the feeder link are both the same as those of a link from the GRS to the RPA which is an uplink of the terrestrial system. Likewise, the communication protocols of a reverse link of the user link and a forward link of the feeder link are both the same as those of a link from the RPA to the GRS which is a downlink of the terrestrial system. Therefore, the ARRS of the high-altitude relay system can be viewed as equivalent to the terrestrial system GRS, but in the sky. The ARS for the Terrestrial system can be connected to the ARRS of the high-altitude relay system without any particular modification.]

[TABLE XX

Transmission and reception characteristics of the Feeder link for the BGRS and the ARRS

|  | Units | BGRS | ARRS |
| --- | --- | --- | --- |
| Frequency of operation | MHz | 5 030 to 5 091 | 5 030 to 5 091 |
| Duplexing |  | Time division duplex (TDD) | TDD |
| Transmit/receive duration | msec | 60 Receive + 2.5 guard time 65 Transmit + 2.5 guard time | 60 Transmit + 2.5 guard time 65 Receive + 2.5 guard time |
| Modulation |  | GMSK | GMSK |
| Modulation symbol rates | ksps | 5 300 (max) | 5 300 (max) |
| Occupied bandwidth, C | kHz | 5 600 (max) | 5 600 (max) |
| Antenna gain | dBi | 22.5 | 18.7 |
| Cable loss | dB | 1 | 2 |
| Antenna pattern |  | less than 15 degree for the wider of elevation direction and azimuth direction (3 dB bandwidth) | less than 45 degree (3 dB bandwidth) |
| Maximum antenna height | m | 2 to 50 Typical 10 | Approximately between 60,000ft to 65,000ft |
| Service range | km | 77.6 | 74 |
| Transmitter conducted power | dBm | 30 (max) | 30 (max) |
| Transmitter in band emission limits | dBc/kHz | See Table XX (See Table 3, when C is 250kHz ) | See Table XX (See Table 3, when C is 250kHz ) |
| Receiver noise figure | dB | 7 | 7 |
| Receiver sensitivity | dBm | ‒111~‒96 | ‒111~‒96 |
| Receiver in band rejection – except the operating channel | dB | TBD | TBD |
| Protection criteria (aggregate) I/N | dB | TBD | TBD |

TABLE XX

Transmission and reception characteristics of the User link for the ARRS and the ARSs

|  | Units | ARSs | ARRS |
| --- | --- | --- | --- |
| Frequency of operation | MHz | 5 030 to 5 091 | 5 030 to 5 091 |
| Duplexing |  | Time division duplex (TDD) | TDD |
| Transmit/receive duration | msec | 60 Receive + 2.5 guard time 65 Transmit + 2.5 guard time | 60 Transmit + 2.5 guard time 65 Receive + 2.5 guard time |
| Modulation |  | GMSK or QPSK | GMSK or QPSK |
| Modulation symbol rates | ksps | GMSK: TBD including error correction/detection, guard times and synchronization overhead | GSMK: TBD including error correction/detection, guard times and synchronization overhead |
| Forward error correction |  | GMSK: Rate 5/8 Turbo Conv. Code QPSK: Rate 5/9 and Rate 1/3 TCC | GMSK: Rate 5/8 Turbo Conv. Code QPSK: Rate 5/9 and Rate 1/3 TCC |
| Error detection |  | 32-bit CRC | 32-bit CRC |
| Baseband Input/Output Signal |  | User Data | User Data |
| User data block size transmitted per TDD frame | bits | GMSK: TBD QPSK @ TBD QSPK @ TBS | GMSK: TBD QPSK @ TBD QSPK @ TBD |
| User data rates | kbps | GMSK Tx: 7.04, 16.0, 25.6 and 34.56 GSMK Rx: 7.04, 16.0 and 25.6 QPSK Tx/Rx @ 20 ksps: 20.64 and 34.88; Includes TDD duty cycle overhead | GMSK Rx: 7.04, 16.0, 25.6 and 34.56 GSMK Tx: 7.04, 16.0 and 25.6 QPSK Tx/Rx @ 20 ksps: 20.64 and 34.88; Includes TDD duty cycle overhead |
| Occupied bandwidth, C | kHz | Variable per application with a maximum of 250 | Variable per application with a maximum of 250 |
| Antenna gain | dBi | 2 | 18.7 |
| Cable loss | dB | 2 | 1 |
| Antenna pattern |  | Constant azimuth Constant elevation | Constant azimuth Tailored in elevation See Table 2 |
| Antenna polarization |  | Vertical with aircraft flying straight and level | Vertical |
| Maximum antenna height | m | 22 860 (mean sea level) Typical 8 000 | Approximately between 60,000ft to 65,000ft |
| Service range | km | 74 | 74 |
| Transmitter conducted power | dBm | 40 | 30 |
| Transmitter in band emission limits | dBc/kHz | −96 at 2 MHz offset See Table 3 | −96 at 2 MHz offset See Table 3 |
| Receiver noise figure | dB | 7 | 7 |
| Receiver sensitivity | dBm | GMSK: TBD QPSK @ TBD QSPK @ TBD | GMSK: TBD QPSK @ TBD QSPK @ TBD |
| Receiver in band rejection – except the operating channel | dB | One channel separation: 23 Two channel separation: 43 Three channel separation: 57 2 MHz or more separation: 63 | One channel separation: 23 Two channel separation: 43 Three channel separation: 57 2 MHz or more separation: 63 |
| Protection criteria (aggregate) I/N | dB | ‒6 | ‒6 |

[Editor’s note: The high-altitude relay system is a system that is compatible with the system described in paragraph 2.1 and most of the communication characteristics of the user link are the same. The specific aspects of communication between the ARRS and UA ARS flying at high altitudes are highlighted in yellow.]

TABLE XX

Transmitter out of band emission limits in the 5 050-5 091 MHz frequency band 
for the BGRS and ARRS (Feeder link)

| Offset from carrier frequency | dBc/kHz |
| --- | --- |
| Channel width ÷ 2 | −54 |
| Channel width ×1.1 | −90 |
| Channel width ×1.5 | −96 |

]

[Editor’s note: during discussion in July 2023 and May 2024 meetings of WP 5B, some administration(s) raised a concern that compatibility studies of the high altitude relay system had not been performed in ITU-R during the study cycle in preparation of WRC-12. In order to respond to the concern, it was agreed that detailed technical specifications of the high-altitude relay system and the result of the compatibility study will be provided at a future meeting. Further clarifications and information are welcome to address this concern.]

## 2.3	Unmanned aircraft and control station characteristics for control and non-payload communication link via satellite

### 2.3.1	Control and non-payload communication via geostationary orbiting satellite systems

It is to be noted that:

–	feeder links between the UACS and the satellite are assumed to be in the frequency band 5 030-5 091 MHz, but may also be accommodated in other frequency bands;

–	a QPSK 1/2 digital video broadcasting - return channel via satellite (DVB-RCS) type waveform is considered;

–	the availability (link availability from the ground earth station to the UA and from the UA to the ground earth station) considered in this example is 99.99%;

–	the link budgets are performed for UA and UACS located in Western Europe, corresponding to the worst case in terms of sharing with microwave landing system (according to ICAO database used in Report ITU-R M.2205). On other areas more favourable from a sharing point of view, additional margin is available;

–	the path loss includes the degradation due to atmospheric effects. The multipath and scintillation effects are included in the 3 dB link budget margin. Such a value is consistent with the margins needed for multipath and scintillation in the propagation channel of the 1.5/1.6 GHz aeronautical band;

–	the link budget is carried out considering rain loss on the satellite – UA link, this representing the worst case compared to the UACS – Satellite link.

The feeder link is assumed to be in the frequency band 5 030-5 091 MHz, this case being the most restrictive one. A QPSK 1/2 DVB-RCS type waveform is considered. The availability (link availability from the ground earth station to the UA and from the UA to the ground earth station) that is considered is 99.99%.

TABLE 6

Aeronautical mobile-satellite (route) service return link budget for system 1

| System |  |  | Repeater |  |
| --- | --- | --- | --- | --- |
| Availability (%) | 99.99% |  | Repeater gain (dB) | 110.5 |
| Satellite longitude (degrees) | –2.8 |  | Tx feeder loss (dB) | 1.0 |
| Conditions | Rain UL |  | Amplifier BO (OBO) (dB) | 3.5 |
| Modulation | QPSK 1/2 |  | Amplifier NPR (dB) | 17.0 |
| Useful bit rate per carrier (kbps) | 44.0 |  | C/IM0 degradation (dB/Hz) | 67.2 |
| Duplex ratio | 0.5 |  |  |  |
| Symbol rate per carrier (kbauds) | 103.5 |  | Satellite Tx antenna |  |
| Minimum bandwidth per carrier (kHz) | 139.8 |  | Tx antenna diameter (m) | 6.0 |
|  |  |  | Tx e.i.r.p. per carrier (dBW) | 14.1 |
| Aircraft Earth stations |  |  | Max Tx e.i.r.p. per carrier (dBW) | 17.1 |
| Frequency (MHz) | 5 000 |  | Downlink C/I inter-spots (dB) | 17.0 |
| Elevation (degrees) | 39.5 |  | Downlink C/I0 inter-spots (dB/Hz) | 67.2 |
| Carrier HPA power (W) | 20.0 |  |  |  |
| Antenna gain (dBi) | 3.0 |  | Downlink propagation |  |
| Tx loss (dB) | 2.0 |  | Total path loss (dB) | 198.0 |
| Power control uncertainty (dB) | 0.5 |  |  |  |
| Tx e.i.r.p. per carrier (dBW) | 13.5 |  | Ground Earth station |  |
|  |  |  | Downlink frequency (MHz) | 5 000 |
| Uplink propagation |  |  | Elevation (deg) | 39.5 |
| Total path loss (dB) | 198.5 |  | Antenna diameter (m) | 3.8 |
|  |  |  | G/T (dB/K) | 18.8 |
| Satellite Rx antenna |  |  | Downlink C/N0 (dB/Hz) | 63.5 |
| Rx antenna diameter (m) | 6.0 |  |  |  |
| Rx antenna gain (dBi) | 45.1 |  | Demodulation |  |
| Rx feeder loss (dB) | 0.5 |  | MLS degradation (dB) | 1.0 |
| Satellite G/T (dB/°K) | 18.7 |  | Total C/(N0+IM0+I0) (dB/Hz) | 57.0 |
| Uplink C/N0 (dB/Hz) | 62.4 |  | Total C/(N+IM+I) (dB) | 6.8 |
| Uplink C/I0 inter-spots (dB/Hz) | 67.2 |  | Required C/(N0+IM0+I0) (dB/Hz) | 54.0 |
| Uplink C/I inter-spots (dB) | 17.0 |  | Required C/(N+IM+I) (dB) | 3.8 |
|  |  |  | Margin (dB) | 3.0 |

TABLE 7

Aeronautical mobile-satellite (route) service forward link budget for system 1

| System |  |  | Repeater |  |
| --- | --- | --- | --- | --- |
| Availability (%) | 99.99% |  | Repeater gain (dB) | 104.5 |
| Satellite longitude (degrees) | –2.8 |  | Tx feeder loss (dB) | 1.0 |
| Conditions | Rain DL |  | Amplifier BO (OBO) (dB) | 4.0 |
| Modulation | QPSK ½ |  | Amplifier NPR (dB) | 17.0 |
| Useful bit rate per carrier (kbps) | 7.0 |  | C/IM0 degradation (dB/Hz) | 59.2 |
| Duplex ratio | 0.5 |  |  |  |
| Symbol rate per carrier (kbauds) | 16.5 |  | Satellite Tx antenna |  |
| Minimum bandwidth per carrier (kHz) | 22.2 |  | Tx antenna diameter (m) | 6.0 |
|  |  |  | Tx e.i.r.p. per carrier (dBW) | 44.7 |
| Ground Earth station |  |  | Max Tx e.i.r.p. per carrier (dBW) | 47.7 |
| Frequency (MHz) | 5.000 |  | Downlink C/I inter-spots (dB) | 17.0 |
| Elevation (degrees) | 39.5 |  | Downlink C/I0 inter-spots (dB/Hz) | 59.2 |
| Number of carriers | 20 |  |  |  |
| HPA power (W) | 100.0 |  | Downlink propagation |  |
| Antenna diameter (m) | 3.8 |  | Total path loss (dB) | 198.5 |
| Antenna gain (dBi) | 44.1 |  |  |  |
| Tx loss (dB) | 1.0 |  | Aircraft Earth station |  |
| Power control uncertainty (dB) | 0.5 |  | Downlink frequency (MHz) | 5.000 |
| Tx e.i.r.p. per carrier (dBW) | 49.6 |  | Elevation (deg) | 39.5 |
|  |  |  | G/T (dB/K0 | –23.0 |
| Uplink propagation |  |  | Downlink C/N0 (dB/Hz) | 51.9 |
| Total path loss (dB) | 198.0 |  | Downlink C/N (dB) | 9.7 |
| Satellite Rx antenna |  |  | Demodulation |  |
| Rx antenna diameter (m) | 6.0 |  | MLS degradation (dB) | 1.0 |
| Rx antenna gain (dBi) | 45.1 |  | Total C/(N0+IM0+I0) (dB/Hz) | 49.0 |
| Rx feeder loss (dB) | 0.5 |  | Total C/(N+IM+I) (dB) | 6.8 |
| Satellite G/T (dB/K) | 18.7 |  | Required C/(N0+IM0+I0) (dB/Hz) | 46.0 |
| Uplink C/N0 (dB/Hz) | 98.9 |  | Required C/(N+IM+I) (dB) | 3.8 |
| Uplink C/I0 inter-spots (dB/Hz) | 59.2 |  | Margin (dB) | 3.0 |
| Uplink C/I inter-spots (dB) | 17.0 |  |  |  |

The previous table was quoted from Report ITU-R M.2233 (Annex 3 § 6).

### 2.3.2	Control and non-payload communication via low Earth orbiting non-geostationary satellite orbiting systems

This section describes the link between the satellite and the aircraft.

TABLE 8

Aircraft earth station characteristics

| Parameters | Units | Values |
| --- | --- | --- |
| Altitude | km | 0-20 |
| Frequency range | MHz | 5030-5091 |
| Overall Tx necessary bandwidth | MHz | 15 |
| Tx e.i.r.p. | dBW | 13.5 |
| Tx loss | dB | 2 |
| Tx bandwidth per carrier | kHz | 140 |
| OoB emissions | dBm | TBD |
| Max antenna gain | dBi | 3 |
| Type antenna gain | / | Omni |
| Overall Rx bandwidth | MHz | 15 |
| Rx bandwidth carrier | kHz | 22 |
| Protection criteria (aggregated interference): I/N | dB | –6 |
| Rx blocking | dBm | TBD |

TABLE 9

Space station characteristics

| Parameters | Units | Values |
| --- | --- | --- |
| Altitude | km | 1200 |
| Frequency range | MHz | 5030-5091 |
| Overall Tx necessary bandwidth | MHz | 15 |
| Tx e.i.r.p. at 30° elevation | dBW | 25 |
| Tx bandwidth per carrier | kHz | 22 |
| OoB emissions | dBm | TBD |
| Max antenna gain | dBi | 15 |
| Type antenna gain | / | ITU-R S.1528 |
| Overall Rx bandwidth | MHz | 15 |
| Rx bandwidth carrier | kHz | 140 |
| Protection criteria (aggregated interference): I/N | dB | -6 |
| Rx blocking | dBm | TBD |

PROPOSED LIAISON STATEMENT TO WP 4C - NEXT PAGE ATTACHED

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: 4C/104-E 12 November 2012 | Document 5B/XXX |
|  | XXX 2024 |
|  | English only |
| Working Party 5B |  |
| REPLY LIAISON STATEMENT TO WORKING PARTY 4C |  |
| PROTECTION OF RNSS RECEIVERS OPERATING IN 5 010-5 030 MHz FROM AM(R)S TRANSMITTERS OPERATING IN 5 030-5 091 MHz |  |

Working Party 5B would like to follow up on our previous response to Working Party 4C’s liaison requesting WP 5B’s views and intentions concerning the Radio Regulations (RR) No. 5.443C created by WRC-12:

“5.443C	The use of the frequency band 5 030-5 091 MHz by the aeronautical mobile (R) service is limited to internationally standardized aeronautical systems. Unwanted emissions from the aeronautical mobile (R) service in the frequency band 5 030-5 091 MHz shall be limited to protect RNSS system downlinks in the adjacent 5 010-5 030 MHz band. Until such time that an appropriate value is established in a relevant ITU-R Recommendation, the e.i.r.p. density limit of −75 dBW/MHz in the frequency band 5 010-5 030 MHz for any AM(R)S station unwanted emission should be used. (WRC-12)”

Like WP 4C, WP 5B is also of the view that studies should be progressed to address the provisional e.i.r.p. density limit of –75 dBW/MHz.

WP 5B is now actively developing a set of characteristics and protection criteria for terrestrial unmanned aircraft system control and non-payload communications links operating in the aeronautical mobile (route) service in the band 5 030-5 091 MHz.

Consequently, to begin the required studies on the provisional e.i.r.p. density limit in (RR) No. 5.443C, WP 5B requests that WP 4C provide it with the latest characteristics, protection criteria, and operating scenarios for RNSS using the 5 010-5 030 MHz frequency band.

WP 5B also acknowledges WP 4C’s interest in any studies addressing the e.i.r.p. density limit in RR No. 5.443C and encourages WP 4C to continue working in close cooperation with WP 5B on this subject.

Status:	For action

Contact:	Tom Hayden					E-mail:   tom.hayden@live.com

______________

____________________