| U.S. Radiocommunications Sector Fact Sheet |  |
| --- | --- |
| Working Party: ITU-R WP-5B | Document No: USWP5B35-18 |
| Ref: 5B/315 Annex 3.1 | Date: 11 August 2025 |
| Document Title: Working Document Towards a Preliminary Draft New Report ITU-R M.[MODERNIZATION OF HF AM(OR)S] |  |
| Author(s)/Contributors(s): Fumie Wingo DON CIO Taylor King ACES for DON CIO Robert Leck ACES for DON CIO Hank Beard HII for DON CIO | Phone: 703-697-0066 E-mail: fumie.n.wingo.civ@us.navy.mil Phone: 443-966-0550 E-mail: taylor.king@aces-inc.com Phone: 321-332-2111 E-mail: robert.leck@aces-inc.com Phone: 410-991-4920 E-mail: armand.beard@hii.com |
| Purpose/Objective: The purpose of this document is to continue work on the WDPDN Report ITU-R M. [MODERNIZATION OF HF AM(OR)S] to progress Agenda Item 1.9 in accordance with Resolution 411 (WRC-23). |  |
| Abstract: The study provides an assessment of the technical characteristics, protection criteria, measured emission characteristics, and sharing methodologies for consideration in the modernization of high-frequency spectrum for Aeronautical Mobile (OR) Service within the 3.025 to 18.030 MHz frequency range as defined under Agenda Item 1.9. The study is intended to demonstrate the feasibility of modernizing the Aeronautical Mobile (OR) Service through the implementation of wideband channel bandwidths of 6, 12, 24 and 48 kHz. |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Received: Source: Document 5B/315 Annex 3.1 | Document 5B/XX-E |
|  | XX November 2025 |
|  | English only |
| United States of America |  |
| WORKING DOCUMENT TOWARDS A PRELIMINARY DRAFT NEW REPORT ITU-R M.[MODERNIZATION OF HF AM(OR)S] |  |
| Technical characteristics, protection criteria, and compatibility and sharing studies for the modernization of high-frequency systems operating in the aeronautical mobile (OR) service within 3.025 to 18.030 MHz |  |

TBD

Attachment:	1

| Radiocommunication Study Groups |  |
| --- | --- |
| Subject: Document 5B/TEMP/104 | Annex 3.1 to Document 5B/315-E |
|  | 15 May 2025 |
|  | English only |
| Annex 3.1 to Working Party 5B Chair’s Report |  |
| WORKING DOCUMENT TOWARDS A PRELIMINARY DRAFT NEW REPORT ITU-R M.[MODERNIZATION OF HF AM(OR)S] |  |
| Technical characteristics, protection criteria, and compatibility and sharing studies for the modernization of high-frequency systems operating in the aeronautical mobile (OR) service within 3.025 to 18.030 MHz |  |

[Editor’s Note: There is a need to review the usage of the terms occupied and channel bandwidths.]

[Editor’s Note: Due to absence of established international standards and a designated entity responsible for spectrum planning and coordination of existing AM(OR)S allocations under Appendix 26 and given the nature of HF radio wave propagation. It is necessary to establish clear criteria for managing and mitigating potential interference arising from the proposed WBHF system configurations. Additionally, adherence to agreed upon standards is crucial to ensure protection of existing systems from harmful interference.]

(202X)

Scope

This Report identifies technical characteristics, protection criteria, and compatibility and sharing studies required for the modernization of high frequency spectrum use in the aeronautical mobile (OR) service. It assesses compatibility with incumbent services that are allocated on a primary basis in-band and adjacent band.

Abbreviations/Glossary

ALE	Automatic Link Establishment

AM	Amplitude Modulation

AM(OR)S	Aeronautical Mobile Off-Route Service

AM(R)S	Aeronautical Mobile Route Service

BLOS	Beyond Line of Sight

CSMA	Carrier Sense Multiple Access

CW	Continuous Wave

ASK	Amplitude-Shift Keying

DSB-AM	Double Sideband Amplitude Modulation

FSK	Frequency-Shift Keying

FM	Frequency Modulation

HF	High Frequency

LQA	Link Quality Analysis

LUF	Lowest Usable Frequency

MUF	Maximum Usable Frequency

PM	Phase Modulation

PSK	Phase-Shift Keying

QAM	Quadrature Amplitude Modulation

RF	Radio Frequency

RR	Radio Regulations

SNR	Signal to Noise Ratio

SSB	Single-sideband

WBHF	Wideband High Frequency

Definitions

Allotment (of a radio frequency or radio frequency channel):  Entry of a designated frequency channel in an agreed plan, adopted by a competent conference, for use by one or more administrations for a terrestrial or space radiocommunication service in one or more identified countries or geographical areas and under specified conditions. (RR 1.17).

Aeronautical mobile route (R) service AM(R)S:  An aeronautical mobile service reserved for communications relating to safety and regularity of flight, primarily along national or international civil air routes. (RR 1.34)

Aeronautical mobile off-route (OR) service AM(OR)S:  An aeronautical mobile service intended for communications, including those relating to flight coordination, primarily outside national or international civil air routes. (RR 1.XX)

Related ITU Recommendations and Reports

Recommendation ITU-R BS.415 – Minimum performance specifications for low-cost sound-broadcasting receivers

Recommendation ITU-R BS.560 – Definitions of radiation in LF, MF and HF broadcasting bands

Recommendation ITU-R BS.705 – HF transmitting and receiving antennas characteristics and diagrams

Recommendation ITU-R BT.1895 – Protection criteria for terrestrial broadcasting systems

Recommendation ITU-R F.240 – Signal-to-interference protection ratios for various classes of emission in the fixed service below about 30 MHz

Recommendation ITU-R F.339 – Bandwidths, signal-to-noise ratios and fading allowances in complete systems

Recommendation ITU-R F.1761 – Characteristics of HF fixed radiocommunication systems

Recommendation ITU-R F.1762 – Characteristics of enhanced applications for high frequency (HF) radiocommunication systems

Recommendation ITU-R F.1821 – Characteristics of advanced digital high frequency (HF) radiocommunication systems

Recommendation ITU-R F.2119 – Guidance on technical parameters and methodologies for sharing and compatibility studies related to fixed and land mobile services in the frequency range 1.5-30 MHz

Recommendation ITU-R M.627 – Technical characteristics for HF maritime radio equipment using narrow-band phase-shift keying (NBPSK) telegraphy

Recommendation ITU-R M.1765 – Technical and operational characteristics of land mobile MF/HF systems

Recommendation ITU-R M.1825 – Guidance on technical parameters and methodologies for sharing studies related to systems in the land mobile service

Recommendation ITU-R P.372 – Radio noise

Recommendation ITU-R P.533 – Method for the prediction of the performance of HF circuits

Recommendation ITU-R P.1149 – Guide to the application of propagation methods of Radiocommunication Study Group 3

Recommendation ITU-R P.2108 – Prediction of clutter loss

Recommendation ITU-R TF. 2487 – Protection criteria for systems in the standard frequency and time signal services

Report ITU-R BS.458 – Characteristics of systems in LF, MF and HF broadcasting

Report ITU-R F.2061 – HF fixed radiocommunications systems

Report ITU-R F.2087 – Requirements for high frequency (HF) radiocommunication systems in the fixed service

# 1	Introduction

The Aeronautical Mobile (OR) Service (AM(OR)S) utilizing the Appendix 26 (Rev.WRC-15) frequency range between 3 025 kHz and 18 030 kHz has been a very important radio communication system for aircraft when communications is needed beyond the range of terrestrial radio systems operating at higher frequencies.. For this document, systems that use a 3 kHz channel bandwidth (2.8 kHz occupied bandwidth) within  Appendix 26 (Rev.WRC-15) allotments are referred to as “legacy AM(OR)S”. To differentiate, a wideband implementation of AM(OR)S where channel bandwidths range from 6 to 48 kHz are referred to as WB AM(OR)S. Next generation High Frequency (HF) can accommodate digital technologies for aeronautical systems operating under AM(OR)S. The next generation of Wideband HF (WBHF) radio systems are expected to address the limitations of today’s HF radio communications systems to enable broadband applications and significantly improving HF data rate, voice clarity, and link availability. To support inclusion of WBHF systems, revision of the Appendix 26 (Rev.WRC-15) is being studied to consider appropriate regulatory actions to support modernization of systems operating in the AM(OR)S within the frequency range defined in Appendix 26 (Rev.WRC-15) without modifying the existing area allotments or altering current channelization.

# 2	Aeronautical Mobile OFF-Route (OR)

AM(OR)S is intended for communications, including those relating to flight coordination, primarily outside national or international civil air routes. AM(OR)S is used specifically for aircraft communications when an aircraft is not flying on a standard published airway. HF communications are used where line of sight communication at very high frequency (VHF) and much higher frequencies are not possible due to range limitations.

HF systems, in accordance with Appendix26 (Rev.WRC-15), are used for non-safety related communications, such as administrative coordination, logistical support, and the exchange of meteorological or operational information that does not directly impact flight safety. The long-range capabilities of HF communications make them valuable for maintaining connectivity across large distances.

WBHF technologies have been identified to improve the performance of the  legacy AM(OR)S operations. These technologies allow for wider channel bandwidths, typically on the order of 6 to 48 kHz, resulting in improved voice quality, higher data rates, enhanced reliability, improved ground station monitoring, and improved handover in the presence of signal degradation.

# 3	Appendix 26 (Rev.WRC-15) overview

Aircraft use specific HF frequencies allocated for AM(OR)S as defined in the ITU Radio Regulations, Appendix 26 (Rev.WRC-15). Appendix 26 (Rev.WRC-15) specifically refers to the "frequency allotment plan for the aeronautical mobile service," essentially outlining the designated frequency bands allocated for air traffic communication within the AM(OR)S, including details on channel usage and related information.

Frequency allotments within Appendix 26 (Rev.WRC-15) are based upon channel bandwidths that are limited to 3 kHz.   Implementation of wider channel bandwidths will need to be accomplished within the scope of the current Appendix 26 (Rev.WRC-15) allotment plan.

The AM(OR)S frequencies within Appendix 26 (Rev.WRC-15) are listed below in Table 1, taken from Parts I and II of Appendix 26 (Rev.WRC-15).

TABLE 1

Appendix 26 (Rev.WRC-15) Frequency ranges exclusive to AM(OR)S (kHz)

| 3025-3155 | 8965-9040 |
| --- | --- |
| 3900-3950 (Region 1 only) | 11175-11275 |
| 4700-4750 | 13200-13260 |
| 5680-5730 | 15010-15100 |
| 6685-6765 | 17970-18030 |

A frequency allotment in the aeronautical mobile (OR) service which comprises:

–	a frequency channel from the channels appearing in the channelling arrangement in RR No. 26/3;

–	a bandwidth of up to 2.8 kHz, situated wholly within the frequency channel concerned;

–	a power within the limits laid down in RR No. 26/4.4 or specified against the allotted frequency channel;

–	an allotment area which is the area in which the aeronautical station can be situated and which coincides with all or part of the territory of the country, or of the geographical area, as indicated against the frequency channel concerned in the Frequency Allotment Plan.

26/3.4	The carrier (reference) frequencies 3 023 kHz and 5 680 kHz are intended for worldwide common use (see also Appendix 27, Nos. 27/232 to 27/238).

26/3.6	The channelling arrangement specified in RR No. 26/3.1 does not prejudice the rights of administrations to establish, and to notify assignments to stations in the aeronautical mobile (OR) service other than those using radiotelephony, provided that:

–	the occupied bandwidth does not exceed 2 800 Hz and is situated wholly within one frequency channel;

–	the limits of unwanted emission are met (see Appendix 27, No. 27/74).

# 4	In-band and adjacent band incumbent services

Table 2 lists the in-band and adjacent band primary allocated services per Article 5 of the ITU Radio Regulations that will need to be studied in accordance with Resolution 411 (WRC-23).

TABLE 2

Incumbent primary services in-band and adjacent to AM(OR)S allocations

| Frequency Band (kHz) Appendix 26 (Rev. WRC-15) Bands (Bold) | Region 1 | Region 2 |  |  |  | Region 3 |
| --- | --- | --- | --- | --- | --- | --- |
| 2850-3025 | AM(R)S 5.111 5.115 |  |  |  |  |  |
| 3025-3155 | AM(OR)S |  |  |  |  |  |
| 3155-3200 | FIXED MOBILE (except AM(R)) 5.116, 5.117 |  |  |  |  |  |
| 3800-3900 | FIXED AM(OR)S LAND MOBILE | 3750-4000 AMATEUR FIXED MOBILE (except AM(R)) 5.122 5.125 |  |  |  | 3500-3900 AMATEUR FIXED MOBILE |
| 3900-3950 | AM(OR)S 5.123 |  |  |  |  | AMS BROADCASTING |
| 3950-4000 | FIXED BROADCASTING |  |  |  |  | FIXED BROADCASTING 5.126 |
| 4650-4700 | AM(R)S |  |  |  |  |  |
| 4700-4750 | AM(OR)S |  |  |  |  |  |
| 4750-4850 | FIXED AM(OR)S LAND MOBILE BROADCASTING 5.113 | FIXED MOBILE (except AM(R)) BROADCASTING 5.113 |  | FIXED BROADCASTING 5.113 |  |  |
| 5480-5680 | AM(R)S 5.111 5.115 |  |  |  |  |  |
| 5680-5730 | AM(OR)S 5.111 5.115 |  |  |  |  |  |
| 5730-5900 | FIXED LAND MOBILE |  | FIXED MOBILE (except AM(R)S) |  | FIXED MOBILE (except AM(R)S) |  |
| 6525-6685 | AM(R)S |  |  |  |  |  |
| 6685-6765 | AM(OR)S |  |  |  |  |  |
| 6765-7000 | FIXED MOBILE (except AM(R)) 5.138 |  |  |  |  |  |
| 8815-8965 | AM(R)S |  |  |  |  |  |
| 8965-9040 | AM(OR)S |  |  |  |  |  |
| 9040-9305 | FIXED | 9040-9400 FIXED |  | FIXED |  |  |
| 10150-11175 | FIXED |  |  |  |  |  |
| 11175-11275 | AM(OR)S |  |  |  |  |  |
| 11275-11400 | AM(R)S |  |  |  |  |  |
| 12230-13200 | MARITIME MOBILE 5.109 5.110 5.132 5.137A 5.145 |  |  |  |  |  |
| 13200-13260 | AM(OR)S |  |  |  |  |  |
| 13260-13360 | AM(R)S |  |  |  |  |  |
| 15005-15010 | STANDARD FREQUENCY AND TIME SIGNAL |  |  |  |  |  |
| 15010-15100 | AM(OR)S |  |  |  |  |  |
| 15100-15600 | BROADCASTING |  |  |  |  |  |
| 17900-17970 | AM(R)S |  |  |  |  |  |
| 17970-18030 | AM(OR)S |  |  |  |  |  |
| 18030-18052 | FIXED |  |  |  |  |  |

# 5	Technical characteristics

Section 5.1 contains all technical characteristics for HF AM(OR)S systems. Subsections 5.1.1 and 5.1.2 present the technical parameters associated with AM(OR)S and WB AM(OR)S. Subsection 5.1.3 presents an overview of technology called Automatic Link Establishment (ALE) that is used by HF services to automatically find and select the best frequency for communication. Lastly, Section 5.2 presents the receiver technical parameters associated with adjacent band incumbent services.

## 5.1	AM(OR)S Characteristics

## 5.1.1	Legacy AM(OR)S technical characteristics

Tables 3-6 lists typical parameters for [legacy] AM(OR)S aeronautical and aircraft stations. These parameters vary depending on the aircraft type and operating regions regulations.

Additional technical parameters are listed below:

–	–	Aeronautical radio stations are limited to single-side-band emissions

–	The upper sideband shall be employed, and the assigned frequency shall be 1400 Hz higher than the carrier frequency

–	–	A channel bandwidth of up to 3 kHz (occupied bandwidth of 2.8 kHz) are contained within the given frequency channel power limits as defined in Table 3.

TABLE 3

[Legacy] AM(OR)S power limits for aeronautical and aircraft stations

| Class of emission | Power limit values (peak envelope power supplied to the antenna) |  |
| --- | --- | --- |
|  | Aeronautical station | Aircraft stations |
| J3E | 36 dBW | 23 dBW |
| A1A, A1B | 30 dBW | 17 dBW |
| F1B | 30 dBW | 17 dBW |
| A2A, A2B | 32 dBW | 19 dBW |
| H2A, H2B | 33 dBW | 20 dBW |
| (R, J) 2 (A, B, D) | 36 dBW | 23 dBW |
| J (7,9) (B, D, X) | 36 dBW | 23 dBW |

Legacy AM(OR)S transmitter parameters and emission mask parameters are included in Table 4, Table 5, and in Figure 1. Legacy AM(OR)S receiver parameters are included in Table 6.

TABLE 4

Legacy AM(OR)S typical transmitter parameters

| Parameter | Aeronautical Ground and Aircraft Station |
| --- | --- |
| Frequency Range (MHz) | 3.025-18.030 |
| Occupied Bandwidth (kHz) | 2.8 |
| Channel Separation (kHz) | 3 |
| Signal to Noise Ratio (dB) | 10 |
| Antenna Type | Directional |
| Power (dBW) | 30 |
| Emission Type (Telephony) | J3E |
| Emission Type (Telegraphy) | A1A, A1B, F1B, (A, H) 2 (A, B) (R, J) 2 (A, B, D), J (7, 9) (B, D, X) |
| Modulation Type | Upper Sideband Single Sideband (SSB) Quadrature Amplitude Modulation (QAM) Frequency-Shift Keying (FSK) Phase-Shift Keying (PSK) |
| Propagation Mode | Skywave |
| Range (km) | Aeronautical Ground Stations: >1000 Aircraft Stations: <500 |
| Protection Criteria (dB) | 15 |

TABLE 5

[Legacy] AM(OR)S emission mask*

| Frequency separation Δ from the assigned frequency (kHz) | Minimum attenuation below peak envelope power (PX) (dB) |
| --- | --- |
| 1.5 ≤ Δ < 4.5 | 30 |
| 4.5 ≤ Δ < 7.5 | 38 |
| 7.5 ≤ Δ | 43** |
| * For aircraft station transmitters first installed after 1 February 1982 and for aeronautical stations transmitters in use after 1 February 1983 ** For transmitter power up to and including 50W: 43+10log10 (PX) (W). For transmitter powers more than 50W, the attenuation shall be at least 60 dB. |  |

FIGURE 1

Legacy AM(OR)S emission mask

TABLE 6

Legacy AM(OR)S Typical Receiver Parameters

| Parameter | Aeronautical ground and aircraft stations |
| --- | --- |
| Frequency range (MHz) | 3.025-18.030 |
| Receiver bandwidth (kHz) | 2.8 |
| Receiver sensitivity (dB) | –113 to –110 at 10 dB SINAD |
| Receiver noise figure (dB) | 14-19 |
| IF rejection (dB) | >80 |
| I/N Protection Criteria (dB) | -6 |
| SINAD | >12 |
| Antenna gain | 0 |
| Antenna type | Aeronautical ground station horizontal dipole Aircraft Tail cap, trailing wire, notch antenna. Multi-loop |
| Antenna polarization | Vertical/horizontal |
| Antenna height | Aeronautical ground (15-30) Aircraft (9000 to 13000) |

## 5.1.2	WBHF AM(OR)S technical characteristics

WBHF AM(OR)S systems with wider channel bandwidths will utilize the existing AM(OR)S allotments within the frequency range 3.025 to 18.030 MHz; therefore no additional spectrum will need to be allocated to accommodate WBHF AM(OR)S systems. New modulation waveforms consistent with the emission designators as defined in Appendix 26 (Rev.WRC-15) will be implemented.

Except for channel bandwidths, modulation types, and the emission mask, the wideband receiver and transmitter parameters are the same as those listed in Tables 3 through 6 for legacy AM(OR)S.

The following emission mask, Table 7, specifications are based upon the 3 kHz channel bandwidth emission masks as shown in Figure 1 above. The only difference between the mask in Figure 1 above and the mask in Figure 2, below, is the channel bandwidth. In Figure 2, the channel bandwidth can vary between 3 and 48 kHz. That is, the WBHF AM(OR)S mask supports channel bandwidths of up to 48 kHz(contiguous or non-contiguous) in 3 kHz increments. Adherence to this mask ensures non-interference with adjacent HF services.

TABLE 7

WBHF AM(OR)S emission mask

| Frequency separation Δ from the assigned frequency (kHz) | Minimum attenuation below peak envelope power (PX) (dB) |
| --- | --- |
| BW/2 ≤ Δ < BW/2 +3 | 30 |
| BW/2 +3 ≤ Δ < BW/2 +6 | 38 |
| BW/2 +6 ≤ Δ | 43* |
| * For transmitter power up to and including 50W: 43+10log10 (PX) (W). For transmitter powers more than 50W, the attenuation shall be at least 60 dB. BW is the channel bandwidth (defined by Nx3 kHz, where N is the number of contiguous bonded 3 kHz channels) |  |

FIGURE 2

WBHF AM(OR)S emission mask

## 5.1.3	Automatic Link Establishment (ALE)

Automatic Link Establishment (ALE) is a technology used in radio communication, especially high-frequency (HF) radio, to automatically find and select the best frequency for communication. ALE addresses the challenges of communicating over long distances by enabling the radio system to adapt to varying signal conditions. Technologies for automatic link establishment have been in development since the 1970s, evolving through various stages from 2G and 3G ALE to 4G ALE.     ALE is designed for global use and is a widely adopted standard for initiating and sustaining HF communications.

The primary difference between 4G ALE and 2G and 3G ALE is 4G ALE ability to identify unoccupied spectrum prior to a link setup. 4G ALE was developed specifically to support WBHF (contiguous and non-contiguous), including the ability to negotiate and utilize channels up to a contiguous 48 kHz segment or 3 kHz non-contiguous segments over a 200 kHz block. 4G ALE uses dynamic frequency selection, wideband waveform adaptation, and real-time spectrum sensing to avoid interference and optimize communication. The system continuously monitors the spectrum, assessing occupancy and link quality, before initiating a call assuring that unoccupied frequencies will be selected for use.  This process protects incumbents that are occupying the spectrum that is being considered for use by wideband channel bandwidths.

## 5.2	Adjacent band incumbent services receiver technical characteristics

In-band and adjacent band incumbent services include AM(R)S, Fixed, Mobile (except AM(R), Land Mobile, Broadcasting, Maritime Mobile and Standard Frequency and Time. The in-band and adjacent band technical parameters are listed in Table 8.

TABLE 8

Adjacent Band Incumbent Services Receiver Technical Parameters

| Technical parameters | AM(R)S | Fixed | Mobile (except (R) | Land mobile | Broadcasting | Maritime Mobile | Standard frequency and time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth (kHz) | 3-48 | 2.8 | 2.8 to 30 MHz | 2.8 to 30 MHz | 3 | 10 to 20 | 1 to 10 |
| Modulation | SSB with Supressed Carrier | AM, FM, PM, QAM, ASK, FSK, PSK | PSK, FSK, QAM | SSB, AM, FM, ASK, FSK | SSB, CW, PSK, FSK | SSB with Supressed Carrier | DSB-AM |
| Antenna Type | Vertical Whip | Horizontal Dipole | Vertical Whip | Vertical Whip | Vertical monopole antenna | Vertical Whip | Dipole |
| Antenna Gain | 0 to 3 | 6 to 12 | 0 to 2 | 0 to 3 | 2 to 10 | 0 | 3 to 10 |
| Polarization | Horizontal | Horizontal/Vertical | Horizontal | Vertical | Horizontal/Vertical | Vertical | Vertical |
| Receiver Sensitivity | –113 to –110 at 10 dB SINAD] | –113 to –110 | –125 to –130 | -110 to -110 | –110 to –120 | –110 to –130 | –110 to –125 |
| Noise Environment | 2 to 15 | 2 to 10 | 2 to 10 | 10 to 15 | 10-20 | 10 to 15 | 4 to 6 |

# 6	Protection criteria and maximum interference levels

This section addresses protection criteria for legacy AM(OR)S and incumbent systems that could be operating in frequency bands that are adjacent to or in-band with WBHF AM(OR)S. Section 6.1 considers the protection criteria for legacy AM(OR)S, Section 6.2 lists the protection criteria for incumbent adjacent band and in-band services and Section 6.3 addresses the derivation of environmental noise, and Section 6.4 presents the derivation of the maximum interference levels based on the protection criteria and environmental noise.

## 6.1	Legacy AM(OR)S protection criteria

Legacy AM(OR)S off-route protection criteria is based upon specific regulations and standards that have been put in place to protect radio receivers from interference that would impact communications between ground stations and aircraft that are flying off designated air routes.

Protection for AM(OR)S is achieved through the implementation of dedicated frequency allocations, power limitations, frequency reuse techniques and strict coordination between different users within the AM(OR)S service frequency bands. Applying these concepts, as defined by International Telecommunication Union (ITU) standards, ensures that clear and reliable AM(OR)S communications, especially in situations where [safety of life is critical], will be assured.

Editor’s Note: Review AM(OR)S usage and edit accordingly.

An I/N value -6 dB, equivalent to the I/N for AM(R)S, will provide a level of protection that would assure that an interfering signal would be significantly weaker than the background noise level to avoid disrupting communication with aircraft.

6.2	Adjacent Band Incumbent Services protection criteria

Table 9 lists the I/N protection criteria for in-band and adjacent band services. These I/N values were obtained from ITU-R reports and recommendations as provided by the responsible working parties of each incumbent service.

TABLE 9

Protection criteria for services that are in-band and adjacent band to
the allocated AM(OR)S frequency bands

| Service | I/N (dB) |
| --- | --- |
| AM(R)S | –6 |
| AM(OR)S | –6 |
| Fixed | –6 |
| Land Mobile | –10 |
| Broadcasting | –20 |
| Maritime mobile | –10 |
| Standard frequency and time | –20 |

[Editor’s Note: All values for each service should also be confirmed and updated if necessary. ]

## 6.3	Derivation of external noise levels

The external noise is a combination of three components: man-made noise, galactic noise, and atmospheric noise.

Man-made noise depends on the frequency and the environment. Section 5 in Recommendation ITU-R P.372-17 shows how to derive median values of man-made noise energy, Nman, for several environments:

Nman=c-d(log10f)-204 (dBWHz)

where f is the operational frequency expressed in MHz and c and d are environmental constants as defined in Table 10 from Recommendation ITU-R P.372-17, section 6.1.1.

TABLE 10

Values of the constants c and d

| Environmental category | c | d |
| --- | --- | --- |
| City (curve A) | 76.8 | 27.7 |
| Residential (curve B) | 72.5 | 27.7 |
| Rural (curve C) | 67.2 | 27.7 |
| Quiet rural (curve D) | 53.6 | 28.6 |
| Galactic noise (curve E) | 52.0 | 23.0 |

Nman

With regards to galactic noise and atmospheric noise, Figure 3 shows the noise energy contributions of noise sources across the 3-30 MHz band.

FIGURE 3

Noise energy vs. frequency

Galactic noise Ngal only depends on frequency. The galactic noise component will not be observed at frequencies below the ionospheric critical frequency of 10 MHz (Recommendation ITU-R P.372-17).

Atmospheric noise Natm depends on frequency, time of day, and season. The Geneva 0 UT represents atmospheric noise energy during midnight hours vs. the 12 UT which represents atmospheric noise energy closer to noon.

For this study, the constant values, Table 10, associated with rural environments were used to calculate the man-made noise.

Applying this formula for “rural” environments, one can then derive the resulting man-made noise level Nman.

### 6.4	Derivation of maximum interference level for in-band legacy AM(OR)S and adjacent band services

Once the external noise, N, is known, the maximum interference level, Imax , into a given receivers’ bandwidth is shown in the formula below:

Imax=N+(IN)long term+10log10(RBW)

where:

Imax =	the maximum interference level in the incumbent service receiver (dBW)

(IN)long term  =	I/N for a given service type (dB)

N = 	the external noise value in dBW/Hz as a function of the selected noise environment (dBW/Hz)

RBW= 	Receiver Bandwidth (Hz).

The maximum interference level, Imax, is derived using the rural noise levels for center frequencies of 8.5 MHz, 10.7 MHz, and 15 MHz. Incumbent in band maximum interference levels are shown in Tables 11, 12, and 13.

TABLE 11

Incumbent maximum interference levels at 8.5 MHz
(Rural)

| Service | I/N (dB) | Receiver bandwidth (kHz) | Maximum interference level (dBW/channel bandwidth) |  | Maximum interference level (dBm/channel bandwidth) |
| --- | --- | --- | --- | --- | --- |
| AM(R)S (3 kHz) | -6 | 3 | -134.23 |  | -104.23 |
| AM(R)S (48 kHz) | -6 | 48 | -122.19 |  | -92.19 |
| Fixed | –6 | 3 | -134.23 |  | -104.23 |
| Land mobile | –10 | 12 | -132.21 |  | -102.21 |
| Broadcasting | –20 | [10] | -143.00 |  | -113.00 |
| Maritime mobile | –10 | 3 | -138.23 |  | -108.23 |
| Standard frequency and time | –20 | 10 | -143.00 |  | -113.00 |

TABLE 12

Incumbent maximum interference levels at 10.7 MHz
(Rural)

| Service | I/N (dB) | Receiver bandwidth (kHz) | Maximum interference level (dBW/channel bandwidth) | Maximum interference level (dBm/channel bandwidth) |
| --- | --- | --- | --- | --- |
| AM(R)S (3 kHz) | –6 | 3 | -137.23 | -107.23 |
| AM(R)S (48 kHz) | –6 | 348 | -125.19 | -95.19 |
| Fixed | –6 | 3 | -137.23 | -107.23 |
| Land mobile | –10 | 12 | -135.21 | -105.21 |
| Broadcasting | –20 | [10] | -146.00 | -116.00 |
| Maritime mobile | –10 | 3 | -141.23 | -111.23 |
| Standard frequency and time | –20 | 10 | -146.00 | -116.00 |

TABLE 13

Incumbent maximum interference levels at 15 MHz
(Rural)

| Service | I/N (dB) | Receiver bandwidth (kHz) | Maximum interference level (dBW/channel bandwidth) | Maximum interference level (dBm/channel bandwidth) |
| --- | --- | --- | --- | --- |
| AM(R)S (3 kHz) | -6 | 3 | -140.23 | -110.23 |
| AM(R)S (48 kHz) | -6 | 48 | -128.19 | -98.19 |
| Fixed | –6 | 3 | -140.23 | -110.23 |
| Land mobile | –10 | 12 | -138.21 | -108.21 |
| Broadcasting | –20 | [10] | -149.00 | -119.00 |
| Maritime mobile | –10 | 3 | -144.23 | -114.23 |
| Standard frequency and time | –20 | 10 | -149.00 | -119.00 |

# 7	Propagation models

ITU-R P.1144-9 contains a list of all ITU-R propagation Reports, Recommendations and analysis software.

For the AM(OR)S analysis Recommendation ITU-R P.533, as noted in Recommendation ITU-R P.1144-9, provides basic maximum usable frequency (MUF), sky-wave field strength, available receiver power, signal-to-noise ratio, lowest usable frequency (LUF), and circuit reliability for frequencies between 2 to 30 MHz over a range of 0 to 40 000 kilometres. The aeronautical station (ground station) and the aircraft station (aircraft in flight) transmit via Skywave propagation and will be used in such a manner within these studies, where applicable.

# 8	Compatibility and sharing studies

Maintaining consistency with the operational factors as defined in Appendix 26 (Rev.WRC-15) while preserving legacy AM(OR)S, with the exception of channel width, system parameters will provide compliance with the HF spectral mask regarding adjacent channel power. Maintaining compliance with the HF emission mask will assure that WBHF AM(OR)S can coexist without conflict to HF voice and data transmissions, as well as with existing systems in frequency band that are in-band and adjacent to the Appendix 26 (Rev.WRC-15) AM(OR)S allocations. The objective of this analysis is therefore to show that WBHF AM(OR)S systems comply with the power limitations and emission masks of the legacy AM(OR)S systems.

Should additional studies be required the protection criteria outlined in Section 6.3.1 would be used for analysis and/or simulation.

Article 5 of the ITU Radio Regulations show that there are situations where an incumbent service and a WB AM(OR)S transmission have the potential to operate within the same frequency space. To assure the availability of a clear channel, any implementation of WB AM(OR)S would require the use of ALE technology.

## 8.1	Analysis methodology

### 8.1.1	In-band Measurement Analysis Methodology

Emission measurements will be used to conduct a comparison between the in-band measured emission spectrum of a WB AM(OR)S transmission and the WB Emission mask.  The objective of making this comparison is to show that the in-band  emission of a WB AM(OR)S transmitter is compliant with the WB AM(OR)S emission mask.

Figure 4 is a block diagram of the setup that was used to measure the emission spectrum of a typical aeronautical WB radio transmitter.

Figure 4

Simplified Emission Measurement Block Diagram

Compliance with the mask requirements within the in-band WB AM(OR)S transmission coupled with the utilization of 3 or 4 G ALE will assure that interference to any in-band transmissions will be avoided.  Any deviation from the emission mask will be noted and the impact of that deviation will be assessed.

### 8.1.2	Adjacent band Measurement Analysis Methodology

### [Internal US note: This section and previous section are intended to be merged for final draft]

Adjacent band measurement analysis consists of demonstrating that typical WBHF aeronautical and aircraft station transmitters meet the requirements of the WBHF emission mask, as shown in Figure 2 and that emissions in bands that are adjacent to the passband do not exceed incumbent interference levels as defined in Section 6.3.1.

To accomplish this, spectrum analyser measurements of a typical aeronautical transmitter was measured over a range of channel bandwidths (6, 12, 24 and 48 kHz), modulation types (QAM and PSK) and frequencies spanning the 3 to 18 MHz frequency range.

### 8.1.3  Adjacent Band Emission Static Analyses Methodology

The static analysis will consider three target frequencies (8.5, 10.7, and 15 MHz) for the following incumbents (AM(R)S, Fixed, Land Mobile, Broadcasting, Maritime Mobile and Standard Frequency and Time.

The selection of 3, 10 and 18 MHz covers the AM(OR)S frequency band where WBHF could be implemented. The selection of incumbent services covers the services that could be impacted in the lower and upper adjacent bands associated by a WB AM(OR)S transmission. Since the frequencies that are allotted to AM(OR)S are for AM(OR)S only adjacent band transmissions will be considered. The signal level at the incumbent’s receiver for path lengths of 500, 1000, 5000 and 10000 km will be calculated. The analysis will include plots of frequency versus received signal power at the incumbent receiver for each of the target frequencies. The plots will also show the incumbent protection criteria range and the average operational noise floor across the 3 to 18 MHz frequency range.

The result of the static pathloss analysis will provide minimum, mid-range and maximum received signal levels at an incumbents’ receiver.  The pathloss calculations will take incumbent system characteristics, operating frequency, noise factors and miscellaneous signal propagation losses (these include fading margin, polarization mismatch, losses associated with the medium through which signal is travelling and other losses into consideration.

The analysis methodology is as follows. Maximum interference levels for each of the incumbent systems under study will be calculated. (See Tables 11,12 and 13). Utilizing various path lengths, the Signal levels at the incumbent service receivers will be derived using equations 1 and 2 below. Losses due to absorption will be included in the calculation. The resultant signal level will be compared to the maximum incumbent interference levels as calculated in Section 6.4 to determine if the maximum interference level has been exceeded.

Equations that will be used to derive the received signal from an WB AM(OR) transmitter to an incumbent receiver are listed below.

Free Space Path Loss

FSL=32.4+20logf+20logd  				(1)

Where:

F     = Frequency (MHz)

D     = Distance (km)

FSL = dB

Using equation (1) and considering operational frequencies of 8.5, 10.7, and 15 MHz will yield pathlosses for each of the target frequencies.

The received signal level at each of the incumbent receivers for various path lengths will be calculated using equation 2.

Interference Signal Level

Pr=TXp+TXg+RXg-TXL-FSL-Lp-RXL dBm                         (2)

Where:

Pr    =  received power (dBm)

TXp  = transmitter output power (dBm)

TXg  = transmitter antenna gain (dBi)

RXg  = receiver antenna gain (dBi)

TXL  = transmit feeder and associated losses (feeder, connectors, etc.) (dB)

FSL  = free space loss or path loss (dB)

Lp    = miscellaneous signal propagation losses (these include fading margin, polarization      mismatch, losses associated with medium through which signal is travelling, other losses...) (dB) For these calculations absorption loss values mid-latitude regions were used and were equal to an average of 5 dB for the mid-latitude regions.

RXL = receiver feeder and associated losses (feeder, connectors, etc.) (d)B Typically 3 dB

The results of the calculations will be tabulated. (Table 14 – Example Results )  for each of the target frequencies.

Table 14

Example Results

TBD

# 9	Analysis and Results

## 9.1	In-band analysis results

Interference to WB AM(OR)S transmission from incumbent transmissions within a desired spectrum segment would impact a WB AM(OR)S transmission that is attempting to utilize a portion of the available channel with the incumbent or incumbents, see Figure 5.

FIGURE 5

Potential interference to a WB channel

As shown in Section 5.1.3, 4G ALE has the capability of sensing the radio environment during the establishment of the link and will select a WB channel (6 to 48 kHz) that will occupy whatever segment of the allocated channel is available during the link setup time, see Figure 6. (NOTE: Add a footnote here and a reference)

FIGURE 6

4G ALE adaption to channel availability

From an operational perspective, a WB AM(OR)S radio transmits a sounding signal on multiple frequencies to assess the availability of a given channel and selects the best one for the link effectively eliminating any potential for interference. Because of this, implementation of WB AM(OR)S will implement the use of 3G or 4G ALE and will enre that interference will be avoided.

## 9.2	Adjacent band measurement analysis results

Figure 7 shows the results of a QAM emission measurement taken at the output of an aeronautical WB radio transmitter at a center frequency of 10.7 MHz and a channel bandwidth of 24 kHz.

Figure 7

Example of measurement overlayed with mask and typical noise floor at 10.7 MHz

The measurement also shows that noise levels adjacent to the passband are below  maximum incumbent interference levels  and are below the average HF noise floor as shown in Figure 8.

FIGURE 8

Typical HF emissions in the 3 to 18 MHz frequency band

Additional emission measurements were taken several center frequencies of 8MHz and 15 MHz at channel bandwidths of 3,6,12,24 and 48 kHz using both QAM and PSK modulations. The results of these measurements and can be found in Appendix 1.

9.2.2 	Static analysis results

[Editors’ Note: A summary of the static analysis results will be added here as a Table. The detailed result will be included in Appendix 2.]

Adjacent band adherence to mask requirements (legacy AM(R)S)

TBD

# 10	Summary

[The comparative measurements analysis results showed that adjacent band emissions and noise floors were below incumbent maximum interference levels, below the average noise floor across the 3 to 18 MHz frequency band, did not contain any anomalous emissions and met the HFWB emission mask as noted in Appendix 27.

Static analysis results showed that…. TBD

Given these results, and the fact that frequency allotments within Appendix 26  only apply to  AM(OR)S, incumbent services operating in frequency band adjacent to AM(OR)S transmissions would operate in an interference free environment, making WBHF AM(OR)S feasible in the context of Appendix 26.]

APPENDIX 1

Emission measurement RESULTS

TBD

APPENDIX 1

Static Analysis RESULTS

TBD

--

-

-

-

-

-

-

-

For the purposes of this document, the frequency range utilized by Appendix 26 (Rev.WRC-15) (3 205 kHz to 18 030 kHz) will also be called the “HF” frequency range.  This is done because the frequency range of Appendix 26 falls within the generally accepted “HF” frequency range which is between 3 MHz and 30 MHz.

The emission mask in Figure 1 is sourced from Annex 3.2 to Document 5C/152.

The emission mask in Figure 2 is sourced from Annex 3.2 to Document 5C/152

Handbook Frequency-Adaptive Communication Systems and Networks in the MF/HF Bands, Edition 2002, ITU-R

Under Appendix 27 Section 27/18A “Individual contiguous or non-contiguous channels complying with the provisions of the Plan3 contained in this Appendix may be aggregated to provide wideband communication without changing the Plan of individual channels” Appendix 27 does not address specific bandwidths. Typical wideband HF channel vary from 6 to 48 kHz. While a single 3 kHz channel has an occupied bandwidth of 2.8 kHz.