NOTE:  Changes to the Second Draft of this Document are highlighted in turquoise.

| U.S. Radiocommunications Sector Fact Sheet |  |
| --- | --- |
| Working Party: ITU-R WP-5B | Document No: USWP5B35-14 |
| Ref: Annex 2.7 to Document 5B/315-E | Date: 22 September 2025 |
| Document Title: PRELIMINARY DRAFT NEW RECOMMENDATION ITU-R M.[24.45-24.65_GHz_ARNS] - Characteristics of and protection criteria for radars operating in the aeronautical radionavigation service in the frequency band 24.45-24.65 GHz |  |
| Author(s)/Contributors(s): Don Nellis Federal Aviation Administration 800 Independence Ave., S.W. Washington, DC 20591 | Phone: (202) 267-9779 e-mail: Donald.Nellis@faa.gov Phone: e-mail: |
| Purpose/Objective: The purpose of this contribution is to develop a new recommendation for radionavigation systems, including unmanned aircraft systems (UAS) Detect and Avoid (DAA) radar systems, in the 24.45-24.65 GHz band. This contribution will update and/or address comments on the technical parameters of DAA radars in Table A-1 as applicable. Due to the stability of the technical characteristics data for the airborne and ground based DAA system this contribution proposes to upgrade the Working Document into Preliminary Draft New Recommendation. This contribution will be an update to the new report found in Annex 2.7 of Document 5B/315-E, the Chairman’s Report of the 29 April – 8 May WP-5B meeting. |  |
| Abstract: This contribution is a new recommendation for UAS Detect and Avoid (DAA) systems that operate in the 24.45-24.65 GHz Radionavigation Service allocation. This contribution contains characteristics and protection criteria for DAA radar that can be used both on airborne and grounds based unmanned aircraft. |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 5B/315-E Annex 2.7 Subject: New Recommendation ITU-R M.[24.45-24.65_GHz_ARNS] | Document 5B/XX-E |
|  | 22 September 2025 |
|  | English only |
| United States of America |  |
| WORKING DOCUMENT TOWARDS A PRELIMINARY DRAFT NEW RECOMMENDATION ITU-R M.[24.45-24.65_GHZ_ARNS] |  |
| Characteristics of and protection criteria for radars operating in the radionavigation service in the frequency band 24.45-24.65 GHz |  |

Introduction

This document proposes new Recommendation with characteristics and protection criteria for radionavigation systems, including unmanned aircraft (UA) detect and avoid (DAA) radar system operating in the radionavigation service (RNS) in the frequency band 24.45-24.65 GHz. These technical and operational characteristics are to be used as a guideline in analysing compatibility between radars operating in the radionavigation service and systems in other services within this band.

Proposal

The United States proposes that ITU-R Working Party (WP) 5B consider the updates to the working document towards a preliminary draft New Recommendation ITU-R M.[24.45-24.65_GHz_RNS] attached to the Chairman’s Report. The proposed updates seek to address the editor’s notes and square brackets in this document.

It is also proposed to elevate the status of this document to Preliminary Draft New Recommendation.

Attachment:

ATTACHMENT

preliminary draft new Recommendation ITU-R M.[24.45-24.65_GHz_ARNS]

Characteristics of and protection criteria for radars operating in the radionavigation service in the frequency band 24.45-24.65 GHz

(202X)

Scope

This document specifies the characteristics and protection criteria of radars operating in the radionavigation service (RNS) in the frequency band 24.45-24.65 GHz. The technical and operational characteristics should be used in analysing compatibility between radars operating in the radionavigation service and systems in other services.

Keywords

24.45-24.65 GHz, radar, characteristics, protection.

Abbreviations/Glossary

RNS	radionavigation service

DAA	Detect and avoid

ESA	Electronically scanned array

FMCW	Frequency-modulated continuous wave

RCS	Radar cross-section

RR	Radio Regulations

SNR	Signal-to-noise power ratio

UA	Unmanned aircraft

UAS	Unmanned aircraft system

Related ITU Recommendations, Reports

Recommendations

ITU-R M.1372	Efficient use of the radio spectrum by radar stations in the radiodetermination service

Report

ITU-R M.2204	Characteristics and spectrum considerations for sense and avoid systems use on Unmanned Aircraft Systems (UAS)

The ITU Radiocommunication Assembly,

considering

a)	that the technical characteristics of detect and avoid radars operating in the radionavigation service (RNS) are driven by the performance requirement and depend on the frequency band;

b)	that the performance characteristics of receivers should be adequate to ensure that they do not suffer from interference due to transmitters situated at a reasonable distance and which operate in accordance with the Radio Regulations,

recognizing

a)	that the frequency band 24.45-24.65 GHz is allocated on a primary basis to the radionavigation, fixed, mobile except aeronautical mobile, and inter-satellite services in ITU-R Region 2;

b)	that the frequency band 24.45-24.65 GHz is allocated on a primary basis to the radionavigation, inter-satellite, fixed, and mobile services in ITU-R Region 3;

c)	that the radionavigation service is a safety service as specified by No. 4.10 of the Radio Regulations (RR);

d)	that some interference suppression techniques between radars are contained in Recommendation ITU-R M.1372 “Efficient use of the radio spectrum by radar stations in the radiodetermination service”;

that

e)	that the inter-satellite service operating in the frequency band 24.45-24.65 GHz shall not claim protection from harmful interference from airport surface detection equipment stations of the radionavigation service according to RR No. 5.533;

f)	that the frequency band 24.45-24.65 GHz is not allocated to radionavigation service in Region 1;

g)	that this frequency band is harmonised for IMT in large number of countries in Region 1;

h)	that based on recognizing f) and g) it is not possible to have a worldwide airborne DAA application in the frequency band 24.45-24.65 GHz in particular in Region 1;

i)	that this frequency band is used for Airport Surface Detection Equipment Radar in some countries in Region 3,

recommends

1	that the technical and operational characteristics of detect and avoid radars operating in the radionavigation service described in the Annex should be considered representative of those operating in the frequency band 24.45-24.65 GHz and used in studies of compatibility with systems in other services;

2	that, the protection criterion of interfering signal power to radar receiver noise power level (I/N) of −6 dB, should be used as the required protection level for detect and avoid radars , and that this represents the aggregate protection level if multiple interferers are present.

Annex

Technical and operational characteristics of radars operating in the
radionavigation service in the
frequency band 24.45-24.65 GHz

# A-1	Introduction

RNS system operates in Regions 2 and 3 on a primary basis in the frequency band 24.45-24.65 GHz. This Annex presents the technical and operational characteristics of representative RNS radars operating in this frequency band.

RNS systems are installed in unmanned aircraft (UA) or on the ground to detect non-cooperative aircraft as a component of an UA Detect and Avoid (DAA) system. These radars are used for collision avoidance on-board UA and can be used as a part of the integration of unmanned aircraft system (UAS).

# A-2	Characteristics of radionavigation detect and avoid radar

The technical parameters are provided in Table A-1.

TABLE A-1

Technical parameters of detect and avoid radar

| Parameter | Units | Radar 1 (Note 1) | Radar 2 (Note 1) | Radar 3 (Note 1) |
| --- | --- | --- | --- | --- |
| Platform |  | Airborne/Ground | Airborne/Ground | Airborne/Ground |
| [Platform height | km | Up to 3 / 20 | Up to 3 / 20 | Up to 3 / 20 |
| Radar type |  | FMCW | FMCW | FMCW |
| Operating range (Note 2) | km | 6 | 6 | 3.5 |
| Maximum number of systems within the same operating area |  | 3 to 8 | 3 to 10 | 3 to 10 |
| Ground relative speed (Note 2) | m/s | 120 | 120 | 40 (for typical waveform) |
| Target speeds (max) | m/s | 200 | 200 | 94 (for typical waveform) |
| Transmitter |  |  |  |  |
| Frequency tuning range (Note 3) | GHz | 24.45-24.65 | 24.45-24.65 | 24.45-24.65 |
| Channel selection method between radars (Note 4) |  | Software selectable | Software selectable | Software selectable |
| RF bandwidth | MHz | 45 | 10 to 50 (Note 4) | 20 or 40 |
| Pulse width | s | 200 | 50 to 200 (Note 5) | 33 (chirp time) |
| Pulse rise and fall times | s | 1 | 1 | 0.1 |
| RF emission bandwidth at −3 dBc −20 dBc −40 dBc | MHz | 47.1 55 75 | 47.1 55 75 | 37 40.75 42.9 |
| Pulse repetition frequency | kHz | 4.7 | 2 to 10 | 30.3 |
| Pulse repetition interval | s | 5 (Note 6) | 5 (Note 6) | 33 |
| Average transmitter power | W | 2 | 3.2 | 0.8 |
| Out-of-band emission characteristics [Editor’s note: to harmonize with RF emission bandwidth at −3 dBc −20 dBc −40 dBc | dBc | Measured at 84 dBuV/m at 3m through 4th harmonics. | Measured at 84 dBuV/m at 3m through 4th harmonics. | 70 |
| Spurious emission characteristics (conducted) | dBc/dB/dBm in 1MHz BW | 65 | 65 | 84 |
| Receiver |  |  |  |  |
| Receiver IF bandwidth −3 dB −20 dB −60 dB | MHz | 9.8 20 60 | 9.8 20 60 | 48.5 65 78 |
| Sensitivity (MDS) (at RX input. SNR = 12 dB) | dBm | −131 | −131 | −141 |
| Receiver noise figure | dB | 6 | 6 | 6.5 |
| Calculated Rx noise power (Note 7] | dBW | −131 | −134 | −121 |
| Saturation level | dBm | −40 | −40 | −50 |
| Antenna |  |  |  |  |
| Antenna type |  | ESA (Note 8) | ESA (Note 8) | Phased array (Note 8) |
| Antenna placement (Note 1) |  | Fixed (internally sealed package) Pointing to be indicated | Fixed (internally sealed package) Pointing to be indicated | Front panel, integrated in the module (internally sealed package) |
| Antenna gain | dBi | 21 | 21 | 17 |
| Antenna pattern |  | Electronically Scanned | Electronically Scanned | Electronically Scanned |
| Antenna pointing |  | Forward | Forward | Forward |
| First antenna sidelobe | dBi | 16 | 16 | 5 |
| Horizontal beamwidth (2-way at 0,0) | degrees | 12 degrees one-way | 12 degrees one-way | 28 |
| Vertical beamwidth (2-way at 0,0) | degrees | 12 degrees one-way | 12 degrees one-way | 28 |
| Polarization |  | Horizontal | Horizontal | Vertical |
| Field of Regard | degrees | −40/40 el −60/60 az | −40/40 el −60/60 az | ±118 |
| Horizontal antenna scan (from boresight) | degrees | ±60 | ±60 | ±45 |
| Vertical antenna scan (from boresight) | degrees | ±40 | ±40 | ±45 |
| Notes: 1 These radars can also be deployed on the ground. 2 These radars have similar detection range on the same aircraft even if it flies at a different speed as long as the radar tracking software is expecting and designed for the correct aircraft speeds. What matters is radar cross section (RCS, i.e. “size”) of the target. 3 Utilized bandwidth - Inclusive of frequency-channel guard-bands. 4 Channel selection is purely SW-defined and can be changed on-the-fly. Some settings may allow radar to self-configure based on detected spectrum-conflict. 5 Waveform is software-defined on a CPI-by-CPI basis, and optimized for targets, and spectral environments. 6 The radar is dynamic/cognitive radar and the PRI/PRR can be changed dynamically based on a number of characteristics (terrain, target, etc.). The PRR/PRI can be higher with future software revisions for specific mission sets. 7 Compressed bandwidth before processing gain. 8 High T/R ESA RF beamforming on both transmit and receive. |  |  |  |  |

____________________

The criterion of protection does not include aeronautical safety margin.