| U.S. Radiocommunications Sector Fact Sheet |  |  |
| --- | --- | --- |
| Working Party: ITU-R WP 5B | Document No: USWP5B35-10 |  |
| Ref: 5B/315 Annex 3.3 | Date: August 6, 2025 |  |
| Document Title: Preliminary Draft New Recommendation ITU-R M.[AMRS-VDL], “Characteristics and protection criteria for the International Civil Aviation Organization standardized VHF datalink Mode 2 systems operating in the aeronautical mobile (route) service in the frequency band 136-137 MHz” |  |  |
| Author(s)/Contributors(s): Chris Tourigny FAA Spectrum Engineering Services Sandra Wright FAA Spectrum Engineering Services Reza Amini MITRE Michael Tran MITRE |  | Phone: 202-267-3071 Email: chris.tourigny@faa.gov Phone: 202-603-7094 Email: sandra.a.wright@faa.gov Phone: 703-983-7145 Email : ramini@mitre.org Phone: 703-983-1295 Email : mtran@mitre.org |
| Purpose/Objective: The purpose of this contribution is to elevate this Preliminary Draft New Recommendation to Draft New Recommendation. |  |  |
| Abstract: This Recommendation provides the technical characteristics and protection criteria for the International Civil Aviation Organization (ICAO) standardized VHF datalink (VDL) Mode 2 (VDL Mode 2) communications systems operating in the aeronautical mobile (route) service (AM(R)S) in the frequency band 136-137 MHz. The purpose of this contribution is to elevate this Preliminary Draft New Recommendation to Draft New Recommendation. |  |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 5B/315 Annex 3.3 Subject: New Recommendation ITU-R M.[AMRS-VDL] | Document 5B/xxx |
|  | 17 November 2025 |
|  | English only |
| United States of America |  |
| PRELIMINARY DRAFT NEW RECOMMENDATION ITU-R M. [AMRS-VDL] |  |
| Characteristics and protection criteria for the International Civil Aviation Organization standardized VHF datalink Mode 2 systems operating in the aeronautical mobile (route) service in the frequency band 136-137 MHz |  |

Introduction

This contribution proposes to elevation the Preliminary Draft New Recommendation ITU-R M.[AMRS-VDL], “Characteristics and protection criteria for the International Civil Aviation Organization standardized VHF datalink Mode 2 systems operating in the aeronautical mobile (route) service in the frequency band 136-137 MHz” to the Draft New Recommendation.

ATTACHMENT

DRAFT NEW RECOMMENDATION ITU-R M. [AMRS-VDL]

Characteristics and protection criteria for the International Civil Aviation Organization standardized VHF datalink Mode 2 systems operating in the aeronautical mobile (route) service in the frequency band 136-137 MHz

(202X)

Scope

This Recommendation provides the technical characteristics and protection criteria for the International Civil Aviation Organization (ICAO) standardized VHF datalink (VDL) Mode 2 (VDL Mode 2) communications systems operating in the aeronautical mobile (route) service (AM(R)S) in the frequency band 136-137 MHz. These technical characteristics and protection criteria should be used for sharing and compatibility studies with VDL Mode 2 systems.

Keywords

AM(R)S, VHF, VDL, protection criteria, air-to-ground communications, ground-to-air communications

Abbreviations/Glossary

AM(R)S:		Aeronautical mobile (route) service

ICAO:		International Civil Aviation Organization

VDL Mode 2:	VHF data link mode 2

VHF:		Very high frequency

RR:		Radio Regulations

Related ITU Recommendations and Reports

Recommendation

ITU-R SM.1535	The protection of safety services from unwanted emissions

The ITU Radiocommunication Assembly,

considering

a)	that the frequency band 136-137 MHz is currently used by International Civil Aviation Organization (ICAO)-standardized VHF data link Mode 2 (VDL Mode 2) data communications worldwide for air-to-ground, air-to-air, and ground-to-air aeronautical safety communications;

b)	that aeronautical safety communications are used in all areas that aircraft operate and land, and in all phases of flight;

c)	that aircraft may be equipped with up to three aeronautical mobile (route) service (AM(R)S) radio stations utilizing a combination of voice and data radios,

recognizing

a)	that in high aircraft density areas, such as in Europe and North America, the utilization of VHF channels in the 117.975-137 MHz is highly congested;

b)	that the ICAO develops standards and recommended practices for civil aviation;

c)	that Annex 10 to the Convention on International Civil Aviation contains standards and recommended practices for aeronautical radiocommunication systems used by civil aviation;

d)	that the AM(R)S is a safety service;

e)	that No. 4.10 of Radio Regulations stipulates “Member States recognize that the safety aspects of radionavigation and other safety services require special measures to ensure their freedom from harmful interference; it is necessary therefore to take this factor into account in the assignment and use of frequencies”;

f)	that Recommendation ITU-R SM.1535 provides a guideline for the protection of safety services from unwanted emissions,

recommends

1	that the technical and operational characteristics of the VDL Mode 2 systems operating in the 136-137 MHz frequency band, allocated to the AM(R)S and described in Annex 1, should be considered for sharing and compatibility studies;

2	that the criterion of interfering signal power to receiver noise power level, I/N = −6 dB, should be considered to protect the VDL Mode 2 systems operating in the AM(R)S in the frequency range 136-137 MHz, and that this represents the aggregate protection level if multiple interferers are present.

Annex 1

Technical and operational characteristics of the VHF datalink mode 2 systems operating in the aeronautical mobile (route) service
in the frequency band 136-137 MHz

## A1.1	Introduction

The frequency band 136-137 MHz is allocated to the AM(R)S and is one of the communications band for aeronautical safety data communications in the air-to-ground, air-to-air, and ground-to-air directions. These systems are internationally standardized by the ICAO for VDL Mode 2. These communications are used where air traffic services are available and in all phases of flight.

## A1.2	Technical characteristics of the VHF datalink mode 2 systems operating in the aeronautical mobile (route) service in the frequency band 136-137 MHz

The technical characteristics of representative VDL Mode 2 systems operating in the frequency band 136-137 MHz are presented in Table A1-1. Some stations use different antennas to transmit and to receive signals.

Table A1-1

Characteristics of VHF data link mode 2 systems operating in the frequency band 136-137 MHz

| Platform | Units | Aircraft | Base station |
| --- | --- | --- | --- |
| Type of emission |  | Data | Data |
| Modulation type |  | D8PSK | D8PSK |
| Type of operation |  | Simplex | Simplex |
| Max antenna height | m | 15 240 (MSL) | 15-50 (AGL) (15 typical) |
| Transmitter |  |  |  |
| Power | W | 18 to 25 | Typical 25 |
| Coverage radius | km | 370 | 370 |
| Bandwidth | kHz | 25 | 25 |
| Antenna gain | dBi | 0 | 2.2 |
| Radiation pattern |  | Omni | Omni |
| Antenna polarization |  | Vertical | Vertical |
| Emission mask |  | ICAO SARPs, Annex 10, Vol. III, Part 1, Sections 6.3.3 (RR App. 3) and 6.3.4 | ICAO SARPs, Annex 10, Vol. III, Part 1, Sections 6.2.3 (RR App. 3) and 6.2.4 |
| Receiver |  |  |  |
| Noise figure | dB | 6 | 6 |
| IF bandwidth | kHz | 25 | 25 |
| Antenna gain | dBi | 0 | 2.2 |
| Radiation pattern |  | Omni | Omni |
| Antenna polarization |  | Vertical | Vertical |

This protection criterion does not include a safety margin.