| US Radiocommunication Sector FACT SHEET |  |
| --- | --- |
| Working Party: ITU-R WP 5C | Document No: USWP5C34-xx |
| Reference: WRC-27 AI 1.10, Annex 2.4 Doc 5C/271 | Date: March 11, 2026 |
| Document Title: Working document on sharing studies under Agenda Item 1.10 |  |
| Author(s)/Contributor(s): Andrew Meadows AFSMO Victory Nguyen eSimplicity for AFSMO | Phone: 334-467-4720 E-mail: andrew.meadows.1@us.af.mil Phone: 443-535-3942 E-mail: victory.nguyen@esimplicity.com |
| Purpose/Objective: The purpose of this document is to continue the sharing and compatibility studies for WRC-27 Agenda Item 1.10 in accordance with Resolution 775 (WRC-23). |  |
| Abstract: This contribution continues the sharing studies between the FS/AMS systems and FSS GSO system in the 71-76 and 81-86 GHz range. |  |
| Fact Sheet Preparer: Victory Nguyen |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Annex 2.4 Doc 5C/271 Subject: WRC-27 Agenda Item 1.10 | Document 5C/XX |
|  | XX May 2026 |
|  | English only |
| United States of America |  |
| Working document on sharing studies under agenda item 1.10 |  |

Introduction

WRC-27 Agenda Item 1.10 considers developing power flux-density and equivalent isotropically radiated power limits for inclusion in Article 21 of the Radio Regulations for the fixed-satellite, mobile-satellite and broadcasting-satellite services to protect the fixed and mobile services in the frequency bands 71-76 GHz and 81-86 GHz, in accordance with Resolution 775 (Rev.WRC-23).

This contribution provides an update to the working document presented in Annex 2.4 of the WP 5C Chair’s Report, Document 5C/271. Particular emphasis on the studies in Sections 8.1.1.1.2 and 8.2.2.1.

Attachment:  1

ATTACHMENT Annex 2.1 to Working Party 5C Chair’s Report

WORKING DOCUMENT ON SHARING STUDIES UNDER WRC-27 AGENDA ITEM 1.10

#### 8.1.1.1.2	Scenario 2: Single Interferer (GSO Satellite, Dynamic Analysis)

{from 5C/188, with amendments from 5C/257}

The following relevant GSO FSS characteristics were extracted from the liaison statement by Working Party 4A in document 5C/142. The specific system used in this study is System C.

Table 6

Parameters of the GSO FSS System

| Parameter | System C (Satellite) |
| --- | --- |
| Frequency (GHz) | 71-76 |
| Altitude (km) | 35,786 |
| Number of planes | 1 |
| Satellites per plane | 1 |
| Inclination angle (deg) | 0 |
| RAAN | N/A |
| Number of co-frequency beams (N_co) | 1 |

The analysis was conducted assuming that the FS  receiver was operating at locations at the following latitude/longitude: 39.673° N, and 104.6° W. The FS receiver antenna’s azimuth is fixed at a specific angle. Multiple simulations were conducted with azimuth angles of 0, 45, 90. 135, and 180 degrees. . The elevation angle of the receiver antenna is fixed at a specific angle. Multiple simulations were conducted with elevation angles of 0, 3, 5, 7.5 and 10 degrees.

FIGURE X

Methodology Flow Chart

The analysis produced a complementary cumulative distribution function (CCDF) curve for the I/N levels received by the FS which was then compared to the I/N protection criteria of FS.

The following assumptions were made during the analysis:

–	There are 53 GSO satellites with 3 degree spacing between satellites between 182.6° W and 26.6° W.

–	The antenna of the GSO satellites is pointing nadir.

–	The FS receiver’s azimuth and elevation angles  are fixed.

Study results

The results are presented in the following plots. In the following figures, the FS receiving station I/N is plotted as a CCDF plot.

TABLE 7

PFD Mask of GSO Satellite

| Frequency band |  | Service* |  | Limit in dB(W/m2) for angles of arrival (δ) above the horizontal plane |  |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° | 25°-90° |  |
| 71-76 GHz | Fixed-satellite (geostationary-satellite orbit) | −115 | −115 + 0.5(δ − 5) | −105 | 1 MHz |

Figure X

FS receiver I/N CCDF plot, Free Space

Figure X

FS receiver I/N CCDF plot, Dry Air

[No changes until Section 8.2.2.1]

#### 8.2.2.1	Scenario 1: Single Interferer (GSO FSS ES, Dynamic Analysis)

Station(s) of the fixed service are defined with the parameters of the following table. The FS parameters used are found in Table 4 of the document.

The analysis was conducted assuming that the FSS ES was operating at the following latitude/longitude: 39.73° N, and 104.75° W. The location of the FS was randomized with a fixed separation distance of 60 km radius of the FSS ES that is stationed at the aforementioned location.  The FS receiver antenna’s azimuth is pointing at another FS station whose location is randomized within a 0.4 to 3 km circle of the receiver.

The analysis produced a CCDF curve for the I/N levels received by the FS which was then compared to the I/N protection criteria of FS.

The following assumptions were made during the analysis:

–	The SRTM V3 (3 arc second, 90m) terrain profile data was used

–	A random percentage was used in the ITU-R P.452 propagation model

–	There is only 1 ES deployed at 39.73° N and 104.75° W

–	The FSS ES azimuth pointing is towards the FS system

–	The elevation pointing angle of the FSS ES is randomized between 0 and 10 degrees

–	The EIRP of the ES is adjusted in accordance with RR Nos. 21.8

–	The beamwidth of the FSS ES is 0.41 degrees

–	The FSS ES antenna height is 10 m.

–	The FS receiver’s elevation angle is randomized between [-5 and 5/-10 and 10] degrees

Study results

The results are presented in the following plots. In the following figures, the FS receiving station I/N is plotted as a CCDF plotThe results show that the FS protection criteria, short- and long-term, are met when the transmitting FSS ES and FS receiver are separated by 60 km.

FIGURE X

FS receiver I/N CCDF plot

____________________