| U.S. Radiocommunications Sector Fact Sheet |  |
| --- | --- |
| Working Party: ITU-R WP 5C | Document No: USWP5C-01 |
| Ref: Annex 2.3 of 5C/271 | Date: 17 Feb 2026 |
| Document Title: WORKING DOCUMENT TOWARDS DRAFT CPM TEXT ON WRC-27 AGENDA ITEM 1.10 |  |
| Author(s)/Contributors(s): Michael Mullinix CTIA Courtney Tolerico CTIA | Email: mmullinix@ctia.org Phone: Email: ctolerico@ctia.org Phone: |
| Purpose/Objective: To update the draft CPM text taking into account the result of U.S. studies under WRC-27 agenda item 1.10. |  |
| Abstract: ITU-R Working Party 5C is tasked with finalizing the draft CPM text on WRC-27 agenda item 1.10 at its upcoming meeting (May 2026). This input contribution will propose regulatory solutions to address Resolution 775 (Rev. WRC-23), including the development of pfd and eirp limits for inclusion in Article 21 of the Radio Regulations for satellite services to protect fixed and mobile services in the frequency bands 71-76 and 81-86 GHz. |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: N/A | Document TBD |
|  | TBD April 2026 |
|  | English only |
| United States WORKING DOCUMENT TOWARDS DRAFT CPM TEXT ON WRC-27 AGENDA ITEM 1.10 |  |  |

Radiocommunication Study Groups. Source: Annex 2.3 to 5C/271; Document USWP5C-01. TBD May 2026. English only.

As the responsible group for WRC-27 agenda item 1.10, Working Party 5C has been conducting the requisite studies called for in Resolution 775 (Rev. WRC-23).  This input contribution updates the draft CPM text to include regulatory considerations based on the results of studies.

Tracked changes, except for modifications to the Radio Regulations, were accepted from 5C/271(Annex 2.3), such that new proposals from the United States could be clearly illustrated.

Attachment

DRAFT CPM TEXT ON WRC-27 AGENDA ITEM 1.10

{

CHAPTER 2

Fixed, mobile and radiolocation issues

(Agenda items 1.7, 1.8, 1.9, 1.10)

Agenda item 1.10

(WP 5C* / WP 3J, WP 3M, WP 4A*, WP 4B, WP 4C*, WP 5A*, WP 5B, WP 6A, WP 7C, WP 7D)

1.10	to consider developing power flux density and equivalent isotropically radiated power limits for inclusion in Article 21 of the Radio Regulations for the fixed-satellite, mobile-satellite and broadcasting-satellite services to protect the fixed and mobile services in the frequency bands 71-76 GHz and 81-86 GHz, in accordance with Resolution 775 (Rev.WRC23);

Resolution 775 (Rev.WRC-23) – Power flux-density and equivalent isotropically radiated power limits for inclusion in Article 21 for the fixed-satellite, mobile-satellite and broadcasting-satellite services to protect the fixed and mobile services in the frequency bands 71-76 GHz and 81-86 GHz

# 2/1.10/1	Executive summary

{Executive summary to describe briefly the purpose of the agenda item, summarize the results of the studies carried out and, most importantly, provide a brief description of the method(s) identified that may satisfy the agenda item; should be limited to no more than half a page of text (see also §§ A2.1, A2.3 , A2.5 and A2.6 of Annex 2 to Resolution ITU-R 2-9)}

# 2/1.10/2	Background

WRC-2000 adopted co-primary allocations in the 71-76 GHz and 81-86 GHz band for terrestrial (fixed and mobile services) and the fixed-satellite, mobile-satellite and broadcasting-satellite services.  The sharing conditions between the fixed service, mobile service, and satellite services in the frequency bands 71-76 GHz and 81-86 GHz could not be fully defined at WRC-2000 due to limited information on the types of service applications at the time.

In the last two decades, there have been a number of significant technology advances and changes in network requirements in the fixed and mobile services. Regarding fixed use, beyond high capacity backhaul links, advancement of smaller, lighter backhaul antennas to facilitate 5G deployments have resulted in a rise in fixed service use in these bands globally. In addition, new mobile use for point-to-point communications to aircraft and ships has been authorized.

Satellite systems in the 71-76 GHz and 81-86 GHz bands have also significantly matured since WRC-2000.  Thousands of operational satellites use the 71-76 and 81-86 GHz bands today, supported by a ground network of high-capacity gateway earth stations. Similar to high-capacity fixed wireless backhaul systems, these earth stations use high-gain, directional beams that can enable coexistence with fixed service deployments in close proximity.

Recognizing the growing terrestrial use of the 70/80 GHz bands to support future mobile networks, and the increasing number of satellite filings in the frequency bands, WRC-23 agreed to study the protection of terrestrial services from satellites services, under agenda item 1.10 and Resolution 775.  Accordingly, studies were performed to determine the power flux-density (pfd) and equivalent isotropically radiated power (e.i.r.p.) limits to be included in Article 21 for satellite services to protect the current and planned fixed and mobile services in the frequency bands 71-76 GHz and 81-86 GHz.

# 2/1.10/3	Summary and analysis of the results of ITU-R studies

## 2/1.10/3.1	Study to determine PFD limits of satellite systems operating in the 71-76 GHz frequency range

The frequency band 71-76 GHz has co-primary allocations to fixed service, mobile service, fixed-satellite service (space-to-Earth), mobile-satellite service (space-to-Earth), and broadcasting-satellite service.

A number of studies were conducted to determine the maximum permissible PFD limits for each individual service and their respective orbits. A summary of those studies is provided below.

### 2/1.10/3.1.1	Fixed Satellite Service into Fixed Service

#### 2/1.10/3.1.1.1	Geostationary Orbit Satellite

[TBD]

#### 2/1.10/3.1.1.2	Non-Geostationary Orbit Satellite

[TBD]

### 2/1.10/3.1.2	Fixed Satellite Service into Mobile Service

[TBD]

### 2/1.10/3.1.3	Mobile Satellite Service into Fixed Service

No studies were performed as no characteristics were provided for the mobile satellite service.

### 2/1.10/3.1.4	Mobile Satellite Service into Mobile Service

No studies were performed as no characteristics were provided for the mobile satellite service.

### 2/1.10/3.1.5	Broadcasting Satellite Service into Fixed Service

No studies were performed as no characteristics were provided for the broadcasting satellite service.

### 2/1.10/3.1.6	Broadcasting Satellite Service into Mobile Service

No studies were performed as no characteristics were provided for the broadcasting satellite service.

## 2/1.10/3.2	Study to determine EIRP limits of satellite systems operating in the 81-86 GHz frequency range

The frequency band 81-86 GHz has co-primary allocations to fixed service, mobile service, fixed-satellite service (Earth-to-space) and mobile-satellite service (Earth-to-space).

### 2/1.10/3.2.1	Fixed Satellite Service into Fixed Service

[TBD]

### 2/1.10/3.2.2	Fixed Satellite Service into Mobile Service

[TBD]

### 2/1.10/3.2.3	Mobile Satellite Service into Fixed Service

No studies were performed as no characteristics were provided for the mobile satellite service.

### 2/1.10/3.2.4	Mobile Satellite Service into Mobile Service

No studies were performed as no characteristics were provided for the mobile satellite service.

### 2/1.10/3.2.5	Broadcasting Satellite Service into Fixed Service

No studies were performed as no characteristics were provided for the broadcasting satellite service.

### 2/1.10/3.2.6	Broadcasting Satellite Service into Mobile Service

No studies were performed as no characteristics were provided for the broadcasting satellite service.

# 2/1.10/4	Methods to satisfy the agenda item

{This section should contain the brief description of the Method or Methods to satisfy the agenda item (see §§  A2.3, A2.4, A2.5 and A2.6 of Annex 2 to Resolution ITU-R 2-9)}

## 2/1.10/4.1 	Method A: Modification of Article 21

–	81-86 GHz, modifications to Table 21-3 to include the 81-86 GHz band for the fixed satellite service. Consequential modifications to Appendix 7 are made to facilitate coordination.

–	71-76 GHz, modifications to Table 21-4 to include a power-flux density mask for fixed satellite geostationary satellite and non-geostationary satellite systems.

–	Resolution 775 (Rev. WRC-23) is suppressed.

# 2/1.10/5	Regulatory and procedural considerations

{Example(s) of regulatory text relating to the Method(s) to satisfy the agenda item (see §§  A2.3, A2.4.6 and A2.5 of Annex 2 to Resolution ITU-R 2-9)}

## 2/1.10/5.1 	For Method A

ARTICLE 21

Terrestrial and space services sharing frequency bands above 1 GHz

Section III − Power limits for earth stations

21.12		5)	The limits given in No. 21.8 apply, where applicable, to the services and frequency bands indicated in Table 21-3 below for transmission by earth stations where the frequency bands are shared with equal rights with the fixed or mobile service:

MOD

TABLE  21-3     (Rev.WRC-27)

| Frequency band |  | Services |  |
| --- | --- | --- | --- |
| 2 025-2 110 MHz 5 670-5 725 MHz 5 725-5 755 MHz 6 | (for the countries listed in No. 5.454 with respect to the countries listed in Nos. 5.453 and 5.455) (for Region 1 with respect to the countries listed in Nos. 5.453 and 5.455) | Earth exploration-satellite Fixed-satellite Meteorological-satellite Mobile-satellite Space operation |  |
| 5 755-5 850 MHz 6 | (for Region 1 with respect to the countries listed in Nos. 5.453 and 5.455) | Space research |  |
| 5 850-7 075 MHz |  |  |  |
| 7 190-7 250 MHz |  |  |  |
| 7 900-8 400 MHz |  |  |  |
| 10.7-11.7 GHz 6 | (for Region 1) |  |  |
| 12.5-12.75 GHz 6 | (for Region 1 with respect to the countries listed in No. 5.494) |  |  |
| 12.7-12.75 GHz 6 | (for Region 2) |  |  |
| 12.75-13.25 GHz |  |  |  |
| 14.0-14.25 GHz | (with respect to the countries listed in No. 5.505) |  |  |
| 14.25-14.3 GHz | (with respect to the countries listed in Nos. 5.505 and 5.508) |  |  |
| 14.3-14.4 GHz 6 | (for Regions 1 and 3) |  |
| 14.4-14.8 GHz |  |  |
| 17.7-18.1 GHz |  | Fixed-satellite |
| 22.55-23.15 GHz |  | Earth exploration-satellite |
| 27.0-27.5 GHz 6 | (for Regions 2 and 3) | Mobile-satellite |
| 27.5-29.5 GHz |  | Space research |
| 31.0-31.3 GHz | (for the countries listed in No. 5.545) |  |
| 34.2-35.2 GHz | (for the countries listed in No. 5.550 with respect to the countries listed in No. 5.549) |  |
| 51.4-52.4 GHz 81-84 GHz 84-86 GHz |  | Fixed-satellite Fixed-satellite Fixed-satellite |

MOD

Section V − Limits of power flux-density from space stations

21.16	§ 6	1)	The power flux-density at the Earth’s surface produced by emissions from a space station, including emissions from a reflecting satellite, for all conditions and for all methods of modulation, shall not exceed the limit given in Table 21-4. The limit relates to the power flux-density which would be obtained under assumed free-space propagation conditions and applies to emissions by a space station of the service indicated where the frequency bands are shared with equal rights with the fixed or mobile service, unless otherwise stated.

MOD

TABLE  21-4     (Rev.WRC-27)

-

-

-

-

| Frequency band | Service* | Limit in dB(W/m2) for angles of arrival (δ) above the horizontal plane |  |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° |  | 25°-90° |  |
| 40.5-42 GHz | Fixed-satellite (geostationary-satellite orbit) Broadcasting-satellite (geostationary-satellite orbit) | −120 21 | 5°-15° | 15°-25° | −105 21 | 1 MHz |
|  |  |  | −120  ( − 5) 21 | −110  0.5 ( − 15) 21 |  |  |
| 42-42.5 GHz | Fixed-satellite (non-geostationary-satellite orbit) Broadcasting-satellite (non-geostationary-satellite orbit) | −120 11, 21 | 5°-25° |  | −105 11, 21 | 1 MHz |
|  |  |  | −120 + 0.75( − 5) 11, 21 |  |  |  |
| 42-42.5 GHz | Fixed-satellite (geostationary-satellite orbit) Broadcasting-satellite (geostationary-satellite orbit) | −127 21 | 5°-20° | 20°-25° | −105 21 | 1 MHz |
|  |  |  | −127 + (4/3) ( − 5) 21 | −107  0.4 ( − 20) 21 |  |  |
| In Region 1: 47.5-47.9 GHz 48.2-48.54 GHz 49.44-50.2 GHz | Fixed-satellite (geostationary-satellite orbit) | −115 | 5°-25° |  | −105 | 1 MHz |
|  |  |  | −115  0.5(δ − 5) |  |  |  |
| 71-76 GHz | Fixed-satellite (space-to-Earth) (geostationary satellite orbit) | [TBD] | [TBD] |  | [TBD] | 1 MHz |
| 71-76 GHz | Fixed-satellite (space-to-Earth) (non-geostationary satellite orbit) | [TBD]27 | [TBD] 27 |  | [TBD] 27 | 1 MHz |

_______________

*	The references to services are those services which have allocations in Article 5.

27 21.16.20 The limits [tbd]. (WRC-27)

MOD

APPENDIX 7 (REV.WRC-27)

Methods for the determination of the coordination area around an earth
station in frequency bands between 100 MHz and 105 GHz

ANNEX 7

System parameters and predetermined coordination distances for determination of the coordination area around an earth station

MOD

TABLE 7c    (Rev.WRC-27)

Parameters required for the determination of coordination distance for a transmitting earth station

| Transmitting space radiocommunication service designation |  | Fixed- satellite | Fixed- satellite | Fixed- satellite 2 | Fixed- satellite | Space research | Earth exploration-satellite, space research | Fixed-satellite, mobile-satellite, radionavigation-satellite | Fixed- satellite 2 | Fixed-satellite | Fixed-satellite, mobile-satellite |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency bands (GHz) |  | 24.65-25.25 27.0-29.5 | 24.65-25.25 27-27.5 | 28.6-29.1 | 27.5-28.6 29.1-29.53 | 34.2-34.7 | 40.0-40.5 | 42.5-47 47.2-50.2 50.4-51.4 | 47.2-50.2 | 51.4-52.4 | 81-86 GHz |  |
| Receiving terrestrial service designations |  | Fixed (except HAPS), mobile | Fixed (HAPS ground station) | Fixed, mobile | Fixed, mobile | Fixed, mobile, radiolocation | Fixed, mobile | Fixed, mobile, radionavigation | Fixed, mobile | Fixed, mobile | Fixed, Mobile |  |
| Method to be used |  | § 2.1 | § 2.1 | § 2.2 | § 2.2 |  | § 2.1, § 2.2 | § 2.1, § 2.2 | § 2.2 | § 2.1 | § 2.1, § 2.2 |  |
| Modulation at terrestrial station 1 |  | N | N | N | N |  | N | N | N | N | N |  |
| Terrestrial station interference parameters and criteria | p0 (%) | 0.005 | 0.01 | 0.005 | 0.005 |  | 0.005 | 0.005 | 0.001 | 0.005 | [0.00128] |  |
|  | n | 1 | 1 | 2 | 1 |  | 1 | 1 | 1 | 1 | [1] |  |
|  | p (%) | 0.005 | 0.005 | 0.0025 | 0.005 |  | 0.005 | 0.005 | 0.001 | 0.005 | [0.00128] |  |
|  | NL (dB) | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | [0] |  |
|  | Ms (dB) | 25 | 10 | 25 | 25 |  | 25 | 25 | 25 | 25 | [11] |  |
|  | W (dB) | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | [0] |  |
| Terrestrial station parameters | Gx (dBi) 4 | 50 | 05 | 50 | 50 |  | 42 | 42 | 46 | 42 | [41.5, 51] |  |
|  | Te (K) | 2 000 | 350 | 2 000 | 2 000 |  | 2 600 | 2 600 | 2 000 | 2 600 | [TBD] |  |
| Reference bandwidth | B (Hz) | 106 | 106 | 106 | 106 |  | 106 | 106 | 106 | 106 | 106 |  |
| Permissible interference power | Pr( p) (dBW) in B | −111 | −134 | −111 | −111 |  | −110 | −110 | –111 | −110 | [‒125] |  |
| 1 A: analogue modulation; N: digital modulation. 2 Non-geostationary satellites in the fixed-satellite service. 3 Feeder links to non-geostationary-satellite systems in the mobile-satellite service. 4 Feeder losses are not included. 5 Maximum HAPS ground station antenna gain toward the horizon. |  |  |  |  |  |  |  |  |  |  |  |

SUP

RESOLUTION 775 (REV.WRC-23)

Power flux-density and equivalent isotropically radiated power limits 
for inclusion in Article 21 for the fixed-satellite, mobile-satellite 
and broadcasting-satellite services to protect the fixed and mobile services 
in the frequency bands 71-76 GHz and 81-86 GHz

_________________

____________________

* 	Studies should be carried out in close collaboration between the indicated WPs.

-