| U.S. Radiocommunication Sector Fact Sheet |  |
| --- | --- |
| Working Party: ITU-R WP-5B | Document No: USWP5B-36-XX |
| Ref: WRC-27 Agenda Item 1.8/Res. 663 (WRC-23), 5B 435/Annex 1.8 | Date: 19 Feb 2026 |
| Document Title: [WORKING DOCUMENT TOWARDS A] PRELIMINARY DRAFT NEW REPORT ITU-R M.[RLS_231.5-700GHZ]/SUPPORTING MATERIAL FOR WRC-27 AGENDA ITEM 1.8 |  |
| Author(s)/Contributors(s): |  |
| Ryan McDonough (NASA) | Ryan.S.McDonough@nasa.gov |
| Purpose/Objective: To continue development for compatibility and sharing studies performed under WRC-27 Agenda Item 1.8 in accordance with Resolution 663 (WRC-23). |  |
| Abstract: Pursuant to Resolution 663 (Rev.WRC-23), Working Party (WP) 5B is the responsible group for WRC-27 Agenda Item 1.8 request to consider possible additional spectrum allocations to the radiolocation service on a primary basis in the frequency range 231.5-275 GHz and possible new identifications for radiolocation service applications in frequency bands within the frequency range 275-700 GHz for millimetric and sub-millimetric wave imaging systems. This document will update sharing and compatibility studies and their associated results for section A7 (A7.4.1 STUDY A) on EESS (passive). |  |
| Fact Sheet Preparer: Ryan McDonough, NASA |  |

Attachment:	1

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 5B/TEMP/149 | Annex 1.8 to Document 5B/435-E |
|  | 17 December 2025 |
|  | English only |
| Annex 1.8 to Working Party 5B Chair’s Report |  |
| [WORKING DOCUMENT TOWARDS A] PRELIMINARY DRAFT NEW REPORT ITU-R M.[RLS_231.5-700GHZ]/SUPPORTING MATERIAL FOR WRC-27 AGENDA ITEM 1.8 |  |
| Frequency bands and technical characteristics for sharing and compatibility studies in support of possible allocation/identification for the radiolocation service in the frequency range 231.5-700 GHz |  |

TABLE OF CONTENTS

1	Introduction		4

2	Provisions of the Radio Regulations		5

3	Atmospheric attenuation in the frequency bands for EESS and RAS		8

3.2	Considerations for RAS		10

4	Characteristics of RLS systems		11

4.1	Description and operational systems characteristics of radar systems to operate in the frequency band 231.5-450 GHz for close proximity object detection		12

4.2	Description and operational systems characteristics of vehicular radar systems operating in the near ranges around vehicles to operate in the frequency band 231.5-450 GHz or parts thereof			18

4.3	Description and operational systems characteristics of radar systems to operate in the frequency range 252-[439] GHz for security applications		24

4.4	Description and operational systems characteristics of security radar systems to operate in the frequency band 252-700 GHz		28

5	Spectrum needs of RLS systems		36

5.1	Systems for close proximity object detection		36

5.2 	Vehicular radar systems		36

5.3 	security applications (section 4.3)		36

5.4 	security applications (section 4.4)		36

6	Propagation model		36

6.1	Recommendations applicable for all sharing geometries		36

6.2	Recommendations applicable for sharing between stations on the surface of the Earth	39

6.3	Recommendations applicable for frequencies above 200 GHz		39

7	Considerations relative to incumbent service systems		40

7.1	Summary related to fixed service systems		40

7.2	Summary related to mobile service systems		40

7.3	Summary related to fixed satellite service systems		41

7.4	Summary related to fixed amateur and amateur satellite service systems		41

7.5	Summary related to radionavigation service systems		41

7.6	Summary related to radionavigation satellite service systems		41

7.7	Summary related to earth exploration satellite service systems		41

7.8	Summary related to radioastronomy service systems		41

8	Conclusions		41

Annex 1		42

A1 	Characteristics and technical analysis related to the fixed service systems		42

A1.1	Frequency bands		42

A1.2	Interference criteria		42

A1.3	Technical and operational characteristics		42

A1.4	Sharing and compatibility analysis		43

A1.4	Summary of Results		48

Annex 2		49

A2 	Characteristics and technical analysis related to the mobile service systems		49

A2.1	Frequency bands		49

A2.2	Interference criteria		49

A2.3	Technical and operational characteristics		49

A2.4	Sharing and compatibility analysis		50

A2.4	Summary of Results		56

Annex 3		57

A3	Characteristics and technical analysis related to the mobile satellite service systems	57

Annex 4		57

A4 	Characteristics and technical analysis related to the fixed amateur and amateur satellite service systems		57

A4.1	Frequency bands		57

A4.2	Interference criteria		57

A4.3	Technical and operational characteristics		57

A4.4	Sharing and compatibility analysis		57

Annex 5		58

A5	Characteristics and technical analysis related to radionavigation service systems	58

A5.1	Frequency bands		58

A5.2	Interference criteria		58

A5.3	Technical and operational characteristics		58

A5.4	Sharing and compatibility analysis		58

Annex 6		58

A6	Characteristics and technical analysis related to radionavigation satellite service systems	58

A6.1	Frequency bands		58

A6.2	Interference criteria		58

A6.3	Technical and operational characteristics		58

A6.4	Sharing and compatibility analysis		58

Annex 7		59

A7 	Characteristics and technical analysis related to earth exploration satellite service systems	59

A7.1	Frequency bands		59

A7.2	Interference criteria		59

A7.3	Technical and operational characteristics		60

A7.4	Sharing and compatibility analysis		69

Annex 8		89

A8 	Characteristics and technical analysis related to radioastronomy service systems	89

A8.1	Recommendations and Reports		90

A8.2	Representative radio astronomy sites operating in relevant frequency bands		92

A8.3	Protection criteria		96

A8.4	Graphical representations		98

A8.5	Sharing and compatibility analysis		99

A8.5.1 	Study 1		99

Annex 9		101

A9 	Common characteristics related to the RLS applications		101

A9.1	Antenna Pattern		101

Scope

TBD

Keywords

TBD

List of Abbreviations/Glossary

TBD

Related ITU-R Recommendations and Reports

TBD

# 1	Introduction

WRC-23 approved WRC-27 agenda item 1.8 ‘to consider possible additional spectrum allocations to the radiolocation service on a primary basis in the frequency range 231.5-275 GHz and possible new identifications for radiolocation service applications in frequency bands within the frequency range 275-700 GHz for millimetric and sub-millimetric wave imaging systems, in accordance with Resolution 663 (Rev.WRC-23)’.

Resolution 663 (Rev.WRC-23) calls to conduct the following studies:

1	the description of the technical and operational characteristics, including required protection criteria, for those receive-only and active millimetric and sub-millimetric wave RLS systems and applications in the categories listed in recognizing a);

[Editor’s Note: the result of discussion of 5B/180 should be reflected here]

2	studies on globally harmonized spectrum for the RLS, in particular for those millimetric and sub-millimetric wave RLS systems and applications above 231.5 GHz;

3	sharing and compatibility studies (in-band and adjacent bands) for active millimetric and sub-millimetric wave RLS systems and applications with other services in the frequency range 231.5-275 GHz, while ensuring protection for the current use and further development of the incumbent services allocated to this frequency range;

4	sharing and compatibility studies (in-band and adjacent bands) for RLS applications with EESS (passive), space research service (passive) and RAS applications in the frequency range 275-700 GHz, while maintaining protection for the passive service applications identified in RR No. 5.565;

5	sharing and compatibility studies (in-band and adjacent bands) for RLS applications with fixed service and land mobile service applications in the frequency range 275-450 GHz, as identified in RR No. 5.564A.

This document contains the result of the studies in response to these five bullets.

# 2	Provisions of the Radio Regulations

The extract from Article 5 of Radio Regulations, edition 2024, is presented in Table 1 for the frequency range 231.5-3 000 GHz.

Table 1

Extract from Article 5 of Radio Regulations

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 231.5-232 FIXED MOBILE Radiolocation |  |  |
| 232-235 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE Radiolocation |  |  |
| 235-238 EARTH EXPLORATION-SATELLITE (passive) 5.563AA FIXED FIXED-SATELLITE (space-to-Earth) MOBILE SPACE RESEARCH (passive) 5.563A 5.563B |  |  |
| 238-239.2 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE RADIOLOCATION RADIONAVIGATION RADIONAVIGATION-SATELLITE |  |  |
| 239.2-240 EARTH EXPLORATION-SATELLITE (passive) FIXED-SATELLITE (space-to-Earth) RADIOLOCATION RADIONAVIGATION RADIONAVIGATION-SATELLITE |  |  |
| 240-241 EARTH EXPLORATION-SATELLITE (passive) RADIOLOCATION |  |  |
| 241-242.2 EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite 5.149 |  |  |
| 242.2-244.2 RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite 5.138 5.149 |  |  |
| 244.2-247.2 EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite 5.138 5.149 |  |  |
| 247.2-248 RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite 5.149 |  |  |
| 248-250 AMATEUR AMATEUR-SATELLITE Radio astronomy 5.149 |  |  |
| 250-252 EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) 5.340 5.563A |  |  |
| 252-265 FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) RADIO ASTRONOMY RADIONAVIGATION RADIONAVIGATION-SATELLITE 5.149 5.554 |  |  |
| 265-275 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE RADIO ASTRONOMY 5.149 5.563A |  |  |
| 275-3 000 (Not allocated) 5.564A 5.565 |  |  |

The allocations on the primary basis in the frequency range 231.5-275 GHz are shown schematically in Figure 1, only allocation to the radiolocation service in the frequency band 231.5-235 GHz is on the secondary basis. The identifications in the frequency range 275-700 GHz are shown in Figure 2. The use of applications of other services is not precluded in the frequency range 275-450 GHz, the frequency range 450-700 GHz can be used by active services, including fixed and mobile service applications.

FIGURE 1

Allocations in the frequency range 231.5-275 GHz

* RR No. 5.340 (250-252 GHz)

Note: No studies under WRC-23 AI 1.14 for RLS versus EESS (passive) around 240 GHz

FIGURE 2

Identifications in the frequency range 275-700 GHz

Article 1 of Radio Regulations contains the following terms and definitions in relation to radiolocation systems:

1.9	radiodetermination:  The determination of the position, velocity and/or other characteristics of an object, or the obtaining of information relating to these parameters, by means of the propagation properties of radio waves.

1.11	radiolocation:  Radiodetermination used for purposes other than those of radionavigation.

1.40	radiodetermination service:  A radiocommunication service for the purpose of radiodetermination.

1.48	radiolocation service:  A radiodetermination service for the purpose of radiolocation.

1.100	radar:  A radiodetermination system based on the comparison of reference signals with radio signals reflected, or retransmitted, from the position to be determined.

1.101	primary radar:  A radiodetermination system based on the comparison of reference signals with radio signals reflected from the position to be determined.

1.102	secondary radar:  A radiodetermination system based on the comparison of reference signals with radio signals retransmitted from the position to be determined.

[According to these definitions, radar may work only based on the comparison of reference signals with radio signals reflected (primary radar), or retransmitted (secondary radar), from the position to be determined. The receive-only use of systems and applications aimed to detect naturally radiated power is outside of the radiolocation service as it is defined in the Radio Regulation.]

# 3	Atmospheric attenuation in the frequency bands for EESS and RAS

The frequency range 231.5-700 GHz is characterized by important atmospheric attenuation due to dry air and water vapour. The specific attenuation and attenuation for zenith path are shown in Figures 3 and 4, respectively, for ITU-R reference atmosphere (See Recommendation ITU-R P.835).

FIGURE 3

Specific attenuation by atmospheric gases for ITU-R reference atmosphere based on 
Recommendation ITU-R P.676

FIGURE 4

Zenith attenuation by atmospheric gases for ITU-R reference atmosphere based on
Recommendation ITU-R P.676

Frequency bands allocated or identified for EESS (passive) and RAS were selected in the frequency ranges corresponding to spectral lines for EESS (passive) and to the ranges between spectral lines (windows) for RAS.

## 3.1	General considerations on EESS frequency dependent sensitivities

The following figure demonstrates a baseline interference margin applied to the 20 proposed frequency ranges by taking into account max protection criteria (according to A7.2) and basic propagation losses: atmospheric and free-space-loss. The figure assumes a nominal EESS (passive) receiver at 800 km altitude, nadir mode, and ground reference altitude of 300 m AMSL. The margin refers to maximum EIRP (in dBW) in the relevant reference bandwidth directed back up to the orbiting sensor. Note that this scenario takes into account integrated losses through the entire atmosphere and does not take into account antenna gains of the sensor or radar systems, additional losses due to clutter, building exit losses, etc. The intention of this figure is that it should provide a rough baseline of remaining margin available to avoid exceeding protection levels for the targeted frequency ranges of study. The red shaded panels in the figure refer to RR No. 5.340 bands and the blue panels bound the frequency ranges of study.

The amateur and amateur-satellite services have a primary allocation covering the 248-250 GHz frequency band. The generic characteristics for stations of the amateur and amateur-satellite services using those bands are given in Tables 1B, 2B, 3B, 5, 6, 7 and 8 in Recommendation ITU-R M.1732-3. It should be noted that the available transmitter power for typical amateur transmitters using those bands is quite limited and is generally no more than a few milliwatts, to make up for this limitation typical maximum antenna gain of 52 dBi may be considered, where applicable, as listed in Recommendation ITU-R M.1732.

Figure 5 demonstrates that there is a very wide range of margin values depending on the operational frequency due almost entirely to attenuation effects. It is meant to identify how much margin is available that could become closed mostly due to aggregate interference and pointing antenna alignment and its associated signal enhancements. This figure is not intended to draw conclusions about sharing and compatibility.

figure 5

Blue above 275 GHz are RR No. 5.565 identifications

Blue between 235-275 GHz EESS(p) bands

## 3.2	Considerations for RAS

FIGURE 6

Specific attenuation at the 5 km elevation ALMA site under good conditions

Shown in Figure 6 are measured values of the specific attenuation (dB/km) applicable under good conditions at the 5 km elevation ALMA site. These can be derived from the empirically measured zenith atmospheric opacities at ALMA shown in Figure 4-2 of Annex 8 using a scale height of 4.2 km, or taken from the lowest curve in Figure 2 of Report ITU-R RA.2189. The values shown here are typically two orders of magnitude smaller than those in Figure 3 for the standard atmosphere, for instance 0.04 versus 4 dB/km at 275 GHz.

# 4	Characteristics of RLS systems

[Editor’s note:

The following questions were received, and contributions are invited to address those items at the next meeting:

–	Address RLS spectrum requirement i.e. how many channel/bands is needed for this RLS application? There are at least 2 different systems. Will they all use the same spectrum or need separate one?

–	The protection of the potential RLS allocation needs to be addressed including protection criteria (I/N = −6 dB)

–	to confirm that systems operating under no protection approach (example: short range devices) are outside the scope of this agenda item

–	special care would be considered on how the duty cycle and activity factor may be used in particular if expected to apply attenuation in dB that is not applicable in all cases.

]

Table 2 provides a summary of the various RLS applications considered in the studies of this Report.

Table 2

Summary of radars studied

| Radar name | Units | Radar A | Radar B | Radar C | Radar D | Radar E | Radar F |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Short name | - | Scanning mode | Focusing/tracking mode | IMRAP | IMRAS | DORAS | AISAT |
| Frequency band that are proposed/ targeted to be studied for this application | GHz | 252-296 356-439 | 252-296 356-439 | 356-386 | 400-430 | 252-257 | 252-255.5 |

| Radar name | Units | Radar G | Radar H | Radar I | Radar X | Radar Y |
| --- | --- | --- | --- | --- | --- | --- |
| Short name | - | Security Radar | Indoor non-destructive inspection | Non-destructive inspection | vehicularShort range | vehicularParking (support) |
| Frequency band that are proposed/ targeted to be studied for this application | GHz | 252-296 | 252-296 | 325-700 | 252-275 275-296 356-439 | 252-275 275-296 356-439 |

## 4.1	Description and operational systems characteristics of radar systems to operate in the frequency band 231.5-450 GHz for close proximity object detection

### 4.1.1	Introduction

A description and operational system characteristics of radar systems to operate in parts of the frequency band 231.5-450 GHz is presented here.

Radiolocation Service (RLS) radar systems include three primary aspects: sensing, positioning, and localization where:

–	Sensing involves the reception and processing of radio signals to gather information pertinent to a specific service. This process can utilize the geometry of the transmitter, receiver, and the environment, as typically seen in radar applications. However, some sensing tasks, like pollution monitoring, may not depend on such spatial information.

–	Positioning determines the location coordinates of a connected device in a certain frame of reference using signal metrics, so it relies on sensing.

–	Localization extends beyond positioning by also estimating the location coordinates of passive objects or targets in a certain frame of reference, enabling what is known as device-free localization.

The radars under considerations here are monostatic and can be mobile and/or fixed. It is used for measuring different physical parameters like presence, distance, velocity, or material properties of a target object. The obtained information is further processed and used for example to aid automation purposes. The radar application is intended for private use. They can be operational indoor and/or outdoor with a predominant usage indoor. Examples are given below. The sensing radar does not operate all the time and its activity, i.e. when it is transmitting, is dependent on the combination of the duty cycle (in %) of the technology and the activity factor (in %) from the user or the system.

Considering pointing direction, it is assumed random pointing for any simulation time step subject to elevation mask constraints specified.

With regard to determining the active state of a single device it modelled as a random variable with probability for any given time step a fixed value according the simple product of activity factor with duty cycle. For instance, a duty cycle of 10% combined with an activity factor of 4% indicates that the probability that a single device is actively transmitting at a given time step is 0.4%.

Mobile or fixed RLS in this frequency range will contribute to object detection in close proximity by using radar-based functions with high resolution.

### 4.1.2	Technical characteristics of radar systems operating in the frequency range 231.5-450 GHz or parts thereof

The technical parameters of RLS radar systems operating in the frequency range 231.5-450 GHz or parts thereof are presented in Table 3. These RLS radar types will be mounted on individual mobile and /or fixed devices.

TABLE 3

RLS radar technical parameters in the frequency range between 231.5 and 450 GHz or parts thereof

| Parameter | Units | Radar A Scanning mode | Radar B Focusing/tracking mode |
| --- | --- | --- | --- |
| Frequency band that are proposed/ targeted to be studied for this application (Note 1) | GHz | 252-296 356-439 GHz | 252-296 356-439 GHz |
| Typical operating range | m | Up to 15 | Up to 10 |
| Typical number of RLS radars per handheld (note 2a) | - | 2 | 2 |
| Range resolution | cm | 10 | 1-2 |
| Typical emission type |  | FMCW, FMCW-variants | FMCW, FMCW-variants |
| Max occupied bandwidth | GHz | 3 | 20 |
| Chirp bandwidth (necessary bandwidth) | GHz | max 2.5 | max 18 |
| Typical sweep time | ms | 0.1-1 | 0.1-1 |
| Maximum e.i.r.p. | dBm | 25 | 25 |
| Max power density of unwanted emission | dBm/MHz | –30 | –30 |
| Duty cycle (Note 3) | % | 10 | 10 |
| Activity factor (Note 3) | % | 10 | 1 |
| Receiver IF bandwidth | MHz | 15 | 100 |
| Receiver sensitivity | dBm | –173.9 dBm/Hz + 10*log10(15 MHz) = –102 | –94 |
| Receiver noise figure | dB | 10 | 10 |
| Typical antenna main beam gain | dBi | TX: 15 RX: 15 | TX: 15 RX: 15 |
| Antenna height | m | 1.5 (mobile) 2.5 (fixed) | 1.5 (mobile) 2.5 (fixed) |
| Downtilt for fixed device only | deg | –25 (wall mounted) –90 (ceiling mounted) | –25 (wall mounted) –90 (ceiling mounted) |
| Antenna azimuth 3 dB beamwidth | degrees | ±3 | ±3 |
| Antenna elevation 3 dB beamwidth | degrees | ±10 | ±10 |
| Motion | - | Mobile (indoor/outdoor) Or fixed indoor | Mobile (indoor/outdoor) Or fixed indoor |
| Urban deployment density (Note 4) (Note 2) | devices/km2 | 1 000-2 500 (mobile) 1 000-2 500 (fixed) | (Note 5) |
| Suburban deployment density (Note 4) (Note 2) | devices/km2 | 100-250 (mobile) 100-250 (fixed) | (Note 5) |
| Rural deployment density (Note 4) (Note 2) | devices/km2 | 10-25 (mobile) 10-25 (fixed) | (Note 5) |
| Antenna polarization |  | Linear | Linear |
| Average Antenna polarization loss | dB | 3 | 3 |
| Average body/casing loss | dB | 3 (without body/casing shielding) 20 (full body/casing shielding) | 3 ( without body/casing shielding) 20 (full body/casing shielding) |
| Antenna efficiency loss | dB | 3 | 3 |
| Indoor mobile deployment |  | 90% | 90% |
| Frequency reuse |  | 1 | 1 |
| Note 1: The frequency bands listed in RR No. 5.340 are excluded from the studies. Note 2a: on the same device only one radar is ever active. It switches between them. Note 2: These numbers of device densities (i.e. devices/km2) would represent a penetration rate from 5% to 12.5% of the population density when taking the example of a large dense urban city like Paris (e.g. 20000 inhabitants/ km2). For population density-based simulations, it is preferred to use the penetration rate of 5% to 12.5% associated to the real population density from database. Note 3: Duty cycle (in %) is technology dependent while the activity factor (in %) is user or system dependent (with reference to 1 day). The activity factor is to be understood as the average of the percentage of devices that are active (i.e. being used by the user) at the same time, same frequency and in the same simulation area. The product of the duty cycle and the activity factor is equivalent to a ratio that is used to determine the number of active devices to be simulated from a density. Note 4: It is expected that the RLS use case will be distributed over 3 bands: 60 GHz, 140 GHz and above 231.5 GHz. Depending on the actual implementation requirements, a different band may be chosen (power/efficiency, integration size and bandwidth/resolution). This makes it difficult to foresee a definitive deployment number. Therefore, a range is provided. Note 5: The number of devices is the same whether it is in scanning or focus/tracking mode, i.e. there should not be doubled counted. The scanning or focus/tracking mode are not active at the same time. |  |  |  |

### 4.1.3	Antenna pattern

The equations for the antenna pattern are contained in Annex 9.1

The antenna patterns presented in in Figure 7 describes the envelope antenna pattern of the radar sensor. The function of the radar sensor is provided within the envelope antenna pattern. The patterns are theoretically designed based on the model given in the equations.

The radar sensor does not transmit over full envelope antenna pattern at any time. The radar sensor is scanning within the envelope pattern with an antenna pattern that has a smaller beamwidth. The scanning (with possible different aiming/radiation directions) is performed within the envelope antenna pattern and is used to detect and track objects. Transmission of the radar is only in the direction of a detected object, there is no transmission into the other areas of the envelope pattern.

[Editor's Note: More information on the instantaneous patter would be helpful for simulations]

Antenna pattern using these formulas for the sensing radar defined in Table 3 is presented in Figure 7.

Figure 7

Antenna pattern examples in transmission for close proximity object detection

### 4.1.4	Operational characteristics of radar systems operating in the frequency bands 231.5-450 GHz

Sensing of passive objects will be instrumental in creating digital representations of physical entities or environments. Sensing with 20 GHz of bandwidth or more enables more accurate modelling with extreme fine ranging. In this case of monostatic sensing, the transmitter and receiver are co-located.

For the implementation of the envisaged radar-based functions, one or multiple radar sensors will be mounted on the mobile/fixed device so that the coverage of the near range of the handheld is achieved. These radars switches between them and only one is ever active.

The mounting positions for the radar sensors vary to facilitate coverage of the different ranges of a handheld. The device might be stationary (fixed) or moving (mobile). There is a scanning and focusing mode that is explained below in connection with Figure 8

Mobile or fixed RLS in this frequency range will contribute to object detection in close proximity by using radar-based functions with high resolution. For the implementation of the envisaged radar-based functions, one or multiple radar sensors will be mounted on the mobile/fixed device so that the coverage of the near range is achieved (on the same device only one radar is ever active. It switches between them). The number of sensors that are mounted on a mobile/fixed device depend on the implementation of the required functions; a typical number of sensors is given in Table 3. An illustrative example of the mounting position of the RLS on the mobile/fixed device is given in Figure 8.

FIGURE 8

Example of a multiple RLS in a mobile/nomadic device

The sensors will be deployed in object-detection and personal-assistance functions, where personal-safety/personal-protection is the target. Objects that might be a hazard to the person may be stationary or may be moving/approaching the person carrying the mobile/fixed device. The following is a non-exhaustive list of scenarios describing where the sensors are intended to be used: standing/walking on the pavement, street crossing and generally in urban traffic, where vulnerable road users or small objects need to be detected.

Figure 9 shows several examples of the usage of the RLS when the device is mobile while depicting the concept of scanning and focusing mode. In the case of a bicycle as a possible hazard to a pedestrian, to detect any danger, the scanning mode rate is faster (i.e. higher duty cycle) than for the focusing mode. Once the bike is detected in the scanning mode the object/hazard approaching will be tracked in the focus mode with higher range resolution capability (i.e. with a higher bandwidth, but lower duty cycle). The number of RLS in the device is the same whether it is in scanning or focus/tracking mode and should not be doubled counted. The scanning or focus/tracking mode are not active at the same time. Note that in the moving case, indoor or outdoor, the person carrying a mobile/nomadic device might be approaching an obstacle or object, or vice versa and obstacle or object might be approaching the person.

Another example in Figure 9, is for instance in indoor the capability of the RLS device to detect objects so that human do not hurt themselves when they have vision impairment, or whether they are immersed in virtual reality. This is a similar principal to AR/VR headset that project environment digitally using several cameras and lidar to detect objects, so people do not hurt while immersed in a virtual environment. These technologies cannot detect some objects like glass for instance. The RLS would instead be able to detect all objects.

FIGURE 9

Example of a mobile scenario to protect people from active and passive object

For the fixed device scenario, in indoor environment there may be obstruction or hazard as shown in Figure 10. The fixed device will detect them and will provide feedback to a user or a group of users so that they can be re-routed. This may be more effective than having distributed communication between devices. This is important for safety reason during live emergency evacuation since people can be notified of obstructed area like blockage due to a collapsing ceiling or building structure collapse.

FIGURE 10

Example of a fixed scenario helping crowd to evacuate efficiently and not be blocked to a ceiling/building structure collapse

The maximum necessary bandwidth indicated for the radar sensors will mainly be used in urban scenarios, where the mobile device radar(s) is(are) scanning its surrounding for objects, but by its nature it is not limited to urban only and can be used in suburban and rural at a lower deployment density.

In the indoor environment there may be obstructions, and a fixed device would have a better chance detection of an object. This may be more effective than each mobile device scanning on its own.

Since the mobile device is most likely battery operated, its transmission is sporadic, i.e. not continuous in the time domain. The need for the spectrum is dependent on the activity factor and duty cycle.

## 4.2	Description and operational systems characteristics of vehicular radar systems operating in the near ranges around vehicles to operate in the frequency band 231.5-450 GHz or parts thereof

### 4.2.1	Introduction

The present document provides an example of an application under the Radio Location Service, technical and operational characteristics and deployment information for vehicular radars operating in the near ranges around vehicles operating in the frequency band 231.5-450 GHz or parts thereof.

Vehicular radars in this frequency range will contribute to traffic-safety (the provisions of RR No. 4.10 do not apply) for vehicle passengers and road users by realizing additional radar-based functions that require a high resolution.

### 4.2.2	Technical characteristics of vehicular radars operating frequency
band 231.5-450 GHz or parts thereof

Multiple vehicular radar sensors of type X and type Y are envisioned to be installed at one vehicle.

Considering pointing direction, it is assumed random pointing for any simulation time step subject to elevation mask constraints specified.

With regard to determining the active state of a single device it modelled as a random variable with probability for any given time step a fixed value according to the simple product of activity factor with duty cycle. For instance, a duty cycle of 10% combined with an activity factor of 4% indicates that the probability that a single device is actively transmitting at a given time step is 0.4%.

The following subchapters provide information about the individual vehicular radar sensor types and vehicles that are equipped with a certain number of the described radar sensors.

#### 4.2.2.1	Technical parameters for vehicular radar sensors operating in the frequency range 231.5-450 GHz or parts thereof

Table 4 provides the technical parameters for two types of vehicular radar sensors that will cover the near ranges of a vehicle.

TABLE 4

Technical parameters for vehicular radars in the frequency band between 231.5-450 GHz or parts thereof

| Parameter | Units | Radar X Short Range | Radar Y Parking (support) |
| --- | --- | --- | --- |
| Frequency bands that are proposed/ targeted to be studied for this application (Note 1) | GHz | 252-275 GHz 275-296 GHz 356-439 GHz | 252-275 GHz 275-296 GHz 356-439 GHz |
| Typical operating range | m | 25 | 10 |
| Typical number of sensors per vehicle (Note 2) |  | ≤ 6 | ≤ 8 |
| Range resolution | cm | 3 | 1-2 |
| Typical emission type |  | Digital Modulation – SW defined | Digital Modulation – SW defined |
| Max necessary bandwidth (Note 3) | GHz | 10 | 25 |
| Minimum occupied bandwidth (for vehicle speeds >50 km/h) | GHz | 1 | 1 |
| Signal bandwidth | GHz | max 5 | max 12.5 |
| Typical sweep time | ms | 10-100 | 10-100 |
| Maximum e.i.r.p. | dBm | MIMO 4 × 4: 26 MIMO 8 × 8: 32 MIMO 16 × 16: 38 | MIMO 4 × 4: 22 MIMO 8 × 8: 28 MIMO 16 × 16: 34 |
| Mean power spectral density | dBm/MHz | MIMO 4 × 4: −13 MIMO 8 × 8: −10 MIMO 16 × 16: −7 | MIMO 4 × 4: −21 MIMO 8 × 8: −18 MIMO 16 × 16: −15 |
| Maximum transmit power to antenna | dBm | 3 | 3 |
| Mean power density of unwanted emissions | dBm/MHz | ‒20 | ‒30 |
| Duty cycle (Note 4) | % | Less than 50 | Less than 50 |
| Activity factor (Note 5) | % | Passenger cars 4,2 Truck 37.5-45.8 | Passenger cars 4,2 Truck 37.5-45.8 |
| Receiver IF bandwidth (–3 dB) | MHz | Less or equal than signal BW | Less or equal than signal BW |
| Receiver sensitivity | dBm | –105 | –100 |
| Receiver noise figure | dB | 15 | 15 |
| Antenna main beam gain | dBi | Single antenna element: 10 Group antenna (max): 35 | Single antenna element: 6 Group antenna (max): 31 |
| Antenna height Above the road | m | 0.2-2 for passenger car, typical 0.5 0.5-3.5 for Truck | 0.2-2 for passenger car typical 0.5 0.5-3.5 for Truck |
| Antenna azimuth 3 dB beamwidth | degrees | +/–60° | +/–60° |
| Antenna elevation 3 dB beamwidth | degrees | +/–10° | +/–40° |
| Urban areas (Note 6,7) | vehicles/ km2 | 453 | 453 |
| Urban/ suburban areas (Note 6,7) | vehicles/ km2 | 330 | 330 |
| Highway scenario (outside urban/ suburban areas) (Note 6,7) | vehicles/ km2 | 123 | 123 |
| Note 1: The frequency bands listed in RR No. 5.340 are excluded from the studies. Note 2: The transmitted output signals of all radar sensors mounted on a vehicle will be arranged so that they operate in the indicated maximum necessary bandwidth. For the maximum number of sensors also see section 4.2.2.2. Note 3: The maximum necessary bandwidth for the described applications depends on the intended application of the radar-based function and the speed of the vehicle. All sensors deployed at one vehicle will be synchronized in order to operate simultaneously in the indicated maximum necessary bandwidth. For each of the radar types, the required maximum necessary bandwidth must be in a contiguous spectrum segment. The indicated maximum necessary bandwidth will only be used at vehicle speeds below 50 km/h down to standstill where the full resolution of the radar sensors is needed. At vehicle speeds above 50 km/h, the necessary bandwidth will be significantly lower (approx. 1 GHz). See section 4.2.2.2 for further details. Note 4: Duty cycle (in %) of the sensors is technology dependent, not all envisaged technologies will have a dedicated duty cycle, in some implementations this is related to the implemented signal processing and modulation. Note 5: Activity factor (in %) is the typical operation time of a vehicle per day. The activity factor is to be understood as the average of the percentage of devices that are active (i.e. being used by the user) at the same time, same frequency and in the same simulation area. The product of the duty cycle and the activity factor is equivalent to a ratio that is used to determine the number of active devices to be simulated from a density. Note 6: Depending on the geographical area under consideration the number of vehicles or trucks per inhabitants varies. The timeframe for the deployment of new vehicles that are equipped with the described radar sensors will vary depending on the geographical area under consideration. The values provided here are based on Report ITU-R SM.2057-0 (Annex 6, Table 254) Note 7: vehicles equipped with the described radar sensors are typically deployed on roads. |  |  |  |

#### 4.2.2.2	Additional information and parameters for vehicles equipped with the described radar sensors

For the implementation of the envisaged radar-based functions, multiple radar sensors will be mounted on the vehicle in order that the full coverage of the near range of the vehicle is achieved.

Depending on the required functions either only radar sensors of type X or Y or combinations of both radar types will be mounted on a vehicle. The number of sensors that are mounted on a vehicle depend on the implementation of the required functions, the combination between Radar X and Y depends on the type of vehicle; the maximum number of sensors per vehicle is 8.

The output signals of the radar sensors that are mounted on a vehicle are synchronized, so that all radar sensors will be transmitting within the indicated maximum necessary bandwidth.

Figure 11 shows the mounting positions for radar sensors type X and Y to facilitate full coverage of the near ranges of a vehicle. The maximum number of sensor devices per vehicle is 8. The number/ and combination of radar X and Y devices can vary over different vehicle types.

FIGURE 11

Mounting positions of the radar sensors and coverage of the near ranges around the vehicle

The maximum necessary bandwidth indicated for the radar sensors type X and Y that are mounted on a vehicle will only be used in urban scenarios, such as parking (support) and functions that cover the near range of the vehicle. The vehicle speed is below a certain threshold in these scenarios (typically 50 km/h). Outside these urban scenarios, the vehicle speed is greater than 50 km/h and the occupied bandwidth of the radar sensors is only around approximately 1 GHz.

The total required amount of spectrum for all sensors mounted at a vehicle is 35 GHz with 10 GHz for radar type X and 25 GHz for radar type Y. To implement the described vehicular radar systems, it would be beneficial to have these 35 GHz available in a contiguous block. If the requested 35 GHz is not available in a contiguous segment, the spectrum blocks for radar type X and radar type Y should be located closely together.

Interference mitigation between several radar equipped vehicles is achieved through various measures: The radar sensors have interference mitigation measures implemented in their receiver signal processing.

### 4.2.3	Antenna pattern and description of antenna function

#### 4.2.3.1	General remarks

In this chapter, a description of an envelope set of parameters is provided, that covers the implementations for the presented use cases in near-range and parking applications.

#### 4.2.3.2	Transmit scheme

In state-of-the-art vehicular radar sensors, all transmitters are active simultaneously. Multiplexing techniques such as DDM (Doppler Division Multiplexing) or CDM (Code Division Multiplexing) are employed to allow for separation of the transmitters in the receiver. The above-mentioned multiplexing techniques employ phase changes of the transmitters throughout one transmit cycle. Consequently, the distribution of the radiated power in free space also changes over one transmit cycle. The instantaneous distribution of the radiated power in free space is always bound by the transmit antennas’ pattern but is otherwise not essential for the radar’s performance since the radar signal processing is done digitally in the receiver. The instantaneous radiated power is thus usually unknown.

The described transmit scheme contrasts with mobile technology where for some applications transmit beamforming is performed to establish a link to a UE. In radar applications the desired information is contained in the received signals.

#### 4.2.3.3	Radiated emissions

Given the transmit scheme described in the previous section, the only practical way to describe the radiated emissions for coexistence considerations is to make use of worst-case (Maximum e.i.r.p.) and mean values (Mean e.i.r.p., mean PSD e.i.r.p. in-band and out-of-band).

The figure below illustrates the relationship between the Maximum e.i.r.p., instantaneous e.i.r.p. and mean e.i.r.p. for an exemplary state-of-the-art vehicular radar system with 16 transmitters.

Figure 12

Radiated powers over one transmit cycle

The worst-case maximum e.i.r.p. (dashed red line) is reached when all transmitters interfere constructively. The absolute maximum e.i.r.p. occurs in boresight and can be calculated as follows (see also 4.2.2.1 Table 4):

Maximum e.i.r.p=Ptx+20log10ntx+G=3 dBm+20log1016+10 dBi≤38 dBm

Since the exact phases at the transmitter inputs are not known, full constructive interference happens only by chance. The worst-case maximum e.i.r.p. can be considered an upper bound for the radiated emissions.

The instantaneous e.i.r.p. (gray lines) changes according to the chosen multiplexing scheme. In the presented case, there are 16 sets of transmitter phases that are toggled throughout the transmit cycle. This results in 16 different instantaneous e.i.r.p. profiles. The envelop over all instantaneous e.i.r.p. profiles gives the observable maximum e.i.r.p. (solid red line). As can be seen in the figure, it stays well below the worst-case values. In fact, it is exceedingly rare for a radar sensor with 16 transmitters to achieve full constructive transmitter interference in any of the instantaneous e.i.r.p. profiles in any direction.

Despite the exact instantaneous e.i.r.p. values being usually unknown, the mean e.i.r.p. (blue line) is well defined as the sum of each single transmitter’s radiated power profile.

Mean e.i.r.p=Ptx+10log10ntx+G=3 dBm+10log1016+10 dBi≤26 dBm

#### 4.2.3.4	MIMO structure

Depending on the required sensor performance 3 MIMO configurations will be used:

4 × 4, 8 × 8 or 16 × 16.

The MIMO – antenna configuration for a sensor device is fixed in the device hardware and cannot be reconfigured. The relevant parameters for the foreseen antenna configurations are provided in Table 4 in section 4.2.2.1.

#### 4.2.3.5	Antenna pattern

The equations for the antenna pattern are contained in Annex 9.1.

The equations provide the resulting antenna radiation pattern that could be used in the analysis of interference. The resulting antenna pattern is described as one antenna beam with a high gain. The patterns are theoretically designed based on the model given in the equations.

The antenna patterns presented in Figures 13 and 14 describe the envelope antenna pattern of the radar sensor. The function of the radar sensor is provided within this envelope antenna pattern.

Figure 13

Envelope antenna pattern for radar X

Figure 14

Envelope - Antenna pattern for radar Y

### 4.2.4	Operational characteristics of vehicular radars operating frequency band 231.5-450 GHz

The main use of described vehicular radar sensors is envisaged for urban scenarios. The sensors will be deployed in driver-assistance functions and higher-level automated vehicles where high resolutions are required. In urban scenarios with vehicle speeds between standstill and approx. 50 km/h high resolution is required to have sufficient target separation capability to detect small objects and other road users in the near ranges (up to 20 metres) around the vehicle.

The following non exhaustive gives a list of scenarios where the described sensors are intended to be used: parking, parking support, road crossings, pedestrian crossings and generally in urban traffic where vulnerable road users or small objects need to be detected.

The market deployment and the availability of vehicles that are equipped with the described radar sensors will increase over a certain timeframe. The timing for deployment and the density of vehicles that are equipped with the described radar sensors will vary depending on the geographical regions.

## 4.3	Description and operational systems characteristics of radar systems to operate in the frequency range 252-[439] GHz for security applications

### 4.3.1	Introduction

Primarily, most of these systems are imaging radars for body scanning. This incorporates amongst others concealed weapon and Person-Borne Improvised Explosive Device (PB-IED) detection capabilities. Due to its favourable propagation characteristics this technology can be extrapolated to numerous other applications of interest, as elaborated further in this document. In general, the range coverage of the systems extent up to a few hundred meters, depending on the application.

The low-power active RLS systems offer high resolution operational capabilities, with operational frequencies ranging from 231.5 to 700 GHz and a range resolution of 7.5 to 0.5 cm.

The findings described in this document identify the potential of RLS systems. Additionally, experimental validation has provided insight in the possible operational capabilities for security purposes.

The frequency range 0.1-1 THz is suitable for very high-resolution imaging applications due to large fractional bandwidth availability. Also, for a given integration time, the sensors in this range provide finer Doppler resolution, along with greater Doppler shifts. For a given aperture size, the antenna beamwidth is narrower, allowing for compact design for high performance operation. There are number of applications where these systems are relevant, as listed below:

–	High-resolution imaging for concealed weapons detection

•	Sub-THz imaging allows to penetrate through clothing whilst reflecting off of hidden objects underneath, with very high resolution. These imaging radars can be used for aiding the security sector for autonomous body scanning at reasonable stand-off distances of up to few tens of meters.

–	Spectroscopy and imaging for Improvised Explosive Device (IED) detection

•	Similarly, spectroscopy and imaging technology can be utilized for IED detection at a safe distance. The spectral signatures provide better performance at secure distances compared to lower frequency bands, essential to alleviate threats such as identifying a suicide bomber. High explosive devices such as HMX, PETN, RDX, TNT, et cetera can be classified with higher accuracy using the sensor signatures.

–	Human activity monitoring in indoor/outdoor environment

•	High Doppler sensitivity at sub-THz frequencies can be used for more accurate monitoring of human activities. In conjunction with Artificial Intelligence (AI) algorithms, the radar signatures of different body part motions can be used to predict specific activities/intents that potentially pose threats.

–	Perimeter security of properties

•	Security of property/facilities can be enhanced by integrating the sensor capability with other sensors. Sub-THz radars will have the ability to produce detailed image of the surrounding with very high resolution. This complements other sensors such as cameras or IR sensors which are more prone to signal degradation in adverse weather conditions.

–	Imaging sensor for security operations in a Degraded Visual Environment (DVE)

•	A compact imaging radar can be mounted on a helicopter to act as a landing sensor for a helicopter. A technology solution for such application requires small physical size and high-resolution imaging through dust or snow.

–	Stand-off detection of chemical and biological agents

•	Spectroscopy and imaging can also be used for real-time, non-invasive inspection of harmful chemical and biological agents concealed by clothes, envelopes, packages et cetera. Spatial interferometric imaging provides characteristic signatures which can be used for such agent detection.

–	High resolution Synthetic Aperture Radar (SAR) imaging and terrain mapping for surveillance

•	Terrain mapping with RLS systems have the potential to be used for autonomous navigation, both in land and naval environments. High resolution image supplemented with Doppler information provides high fidelity target classification capabilities, which are difficult at lower bands.

### 4.3.2	Technical characteristics and description of active radar systems operating in frequency band 252-[439] GHz

In terms of the radar systems considered for the studies the systems are divided into four categories based on their typical use cases. The parameters are then calculated to be representative of such systems, taking into consideration that the values can be used for frequency allocation/identification decisions.

The four categories are as below:

Radar C: IMRAP (IMaging RAdar for Personnel screening)

‒	Concealed weapons detection

‒	PB-IEDs detection

Radar D: IMRAS (IMaging RAdar for Security surveillance)

‒	Human activity monitoring

‒	Imaging for DVE

•	helicopter landing in brown-out or white-out situations

•	maneuvering through smoke/fog

Radar E: DORAS (DOppler RAdar for Security surveillance)

‒	Environmental monitoring

‒	Drone detection

Radar F: AISAT (AIrborne SAr system for Tactical situational awareness)

‒	Moving target indicator

‒	Changes to stationary scene content.

TABLE 5

Technical parameters of radars for security applications in the frequency range of 252 and 430 GHz

| Parameter | Units | Radar C | Radar D | Radar E | Radar F |
| --- | --- | --- | --- | --- | --- |
| Frequency bands that are proposed/ targeted to be studied for this application (Note 1) | GHz | 356-386 | 400-430 | 252-257 | 252-255.5 |
| Typical operating range | m | up to 25 | up to 150 | Up to 300 | Up to 4 000 |
| Signal bandwidth | GHz | max 30 | max 30 | max 5 | max 3.5 |
| Range resolution | cm | 0.5 | 0.5 | 3 | 4.3 |
| Typical emission type |  | FMCW, FMCW variants | FMCW, FMCW-variants | FMCW, FMCW-variants | Linear Frequency Modulation (LFM) pulse |
| Typical sweep time | µs | ~100 | ~100 | ~100 | ~1 000 |
| Maximum e.i.r.p. | dBm | +30 | +57 | +52 | +90 |
| Power spectral density | dBm/MHz | ‒41 | –38 | ‒23 | +12 |
| Power spectral density (e.i.r.p.) | dBm/MHz | ‒15 | +12 | +15 | +55 |
| Maximum power density of unwanted emissions | dBm/MHz | ‒49 | ‒46 | ‒34 | +10 |
| Duty cycle | % | 90 | 90 | 90 | 45 |
| Activity factor (Note 2) | % | 30-75 | 30-75 | 30-75 | 5 |
| Receiver IF bandwidth (Note 3) | MHz | 50 | 50 | 50 | 50 |
| Receiver noise figure | dB | 10 | 10 | 10 | 10 |
| Typical antenna main beam gain | dBi | TX: 26 RX: 26 | TX: 50 RX: 50 | TX: 38 RX: 38 | TX: 43 RX: 43 |
| Antenna azimuth 3 dB beamwidth (FWHM) | degrees | 10 | 0.5 | 2 | 1.4 |
| Antenna elevation 3 dB beamwidth (FWHM) | degrees | 10 | 0.5 | 2 | 1.4 |
| Image frame rate (Note 4) | Hz | ~ 5 | - | - | - |
| Interference threshold level | dBm | ‒103 | ‒103 | ‒103 | ‒103 |
| Motion | - | Mobile (indoor/outdoor) Or fixed indoor/outdoor | Mobile (indoor/outdoor) Or fixed indoor/outdoor | Fixed outdoor (close to ground or top of buildings) | Mobile outdoor Altitude ~ 2000 m |
| Device deployment density | devices/km2 | 1-50 (indoor) 1-10 (outdoor) | 1-5 (outdoor) | 1-6 | <1 |
| Antenna polarization |  | Linear | Linear | Linear | Linear |
| Note 1: The frequency bands listed in RR No. 5.340 are excluded from the studies. Note 2: For Radar C, D and E, estimation here is that systems can be operational about one third to three quarter of the day (e.g. an airport operating for three quarter of the day). For Radar F, the estimation is that the airborne SAR operation in one day will take about 1-2 hours. Note 3: Approximated from expected sampling rate to cover the instrumented range. Here, the maximum receiver IF bandwidth is considered to be the maximum expected beat frequency of the FMCW radar. Note 4: Approximate median value considered. |  |  |  |  |  |

### 4.3.3 	Operational characteristics and foreseen use cases

The deployment of sensors in these scenarios is limited in space and in time, e.g. the sensors are used only at specific locations, either indoors or outdoors, and/or at specific times, e.g. to protect a specific event.

–	Airport security - Example: If the Amsterdam Schiphol airport is considered (27.87 km2), it has 130 security lanes (currently being replaced by five large central checkpoints). However, the concentration of the checkpoints is not evenly distributed. So, around the checkpoint areas, the sensor density can be in the range of ~50 per km2 (~10 times more than the simple average). It should be noted that currently the airport has not any THz sensor deployed.

–	Entrance to critical infrastructures, government facilities, E-commerce distribution centers, public landmark buildings, large scale events, prison security- Numbers can vary depending on the type of infrastructure. For instance, public landmark buildings can be mostly concentrated in the city center area. Example: Amsterdam-Centrum district is 8.04 km2 and there are about 15-20 landmarks within this area where a high concentration of visitors is observed. This means there would be about 5 sensors per km2 if two sensors are placed in each building. The values would be even less for the other more sparse infrastructures.

–	Security surveillance/Drone detection - A THz radar system considered here for surveillance has an instrumented range of ~150 m. However, for drone detection, the system bandwidth can be reduced for larger range coverage. This is because drone detection system uses propeller blade Doppler information for which finer range resolution is not essential. The system then can operate at 2 GHz bandwidth, increasing the instrumented range to ~300 m. The main deployment area is in urban scenarios. In this case, the sensor density will not be evenly distributed as the most likely case would be that these will be concentrated around security critical areas. Examples: If just Amsterdam-Centrum district needs to be covered, ~50 sensors can be used to roughly cover the area of 8.04 km2 (corresponding to ~6 sensors per km2). The number can significantly vary depending on the line of sight, which depends on the building structures within that area. Around a large-scale event (e.g. music festival, sports event), there can be ~1-5 sensors to cover all the directions around it. Again, just outside that area, it is unlikely that such sensors will be present.

–	Helicopter landing in brown-out/white-out - These are usually remote areas, so expected sensor density would be less than 1 per km2.

It should be noted that the sensor density is mostly limited by the practical deployment capability/requirement, not by signal interference, as these are predominantly very short range (e.g. THz imaging radars for body scanning have stand-off distance of up to around 25 m only).

## 4.4	Description and operational systems characteristics of security radar systems to operate in the frequency band 252-700 GHz

### 4.4.1	Introduction

Radars for security check or non-destructive inspection operating in this frequency range will make a significant contribution to public safety, such as counterterrorism and the security of high-risk/high-value assets or areas in the imaging and localization categories which is addressed in considering c) in Resolution 663 (Rev.WRC-23) .

Aiming to create safe and secure communities, there is a strong need to restrict the bringing of hazardous materials into public spaces. This is particularly important for long-distance trains and buildings that handle critical information or functions, where it is desirable to check the belongings or luggage of passengers and visitors for hazardous materials. Given the large number of individuals involved in these scenarios, walk-through security gates that can perform measurements without stopping the subjects are gaining attention. These gates need to conduct inspections in a short time, matching the general walking speed of passengers. Due to the nature of the application, it is assumed that access to and from the security area will be strictly controlled, and the use case is expected to be indoors.

In response to advanced factory automation, efficiency and quality improvements, applications such as non-destructive inspection to detect scratches, impurities, or internal structural defects in production lines are highly anticipated. While non-destructive inspection using X-rays has been utilized, its application is limited due to concerns about exposure to living organisms. To enable broader use across various products, transmission-type imaging radars using high-frequency bands like terahertz waves are gaining attention, with early practical implementation being highly anticipated. This application is also expected to be operated indoors, considering the quality inspection on production lines.

Figure 15 shows Concept and scenario of operations on security radar and indoor non-destructive inspection.

FIGURE 15

Concept and scenario of operations on security radar and indoor non-destructive inspection

### 4.4.2	Technical characteristics of security radar systems operating in the frequency range 231.5-450 GHz or parts thereof

The technical parameters of RLS radar systems operating in the frequency range 231.5-450 GHz or parts thereof are presented in Table 6.

TABLE 6

RLS radar technical parameters in the frequency range between 231.5 and 450 GHz or parts thereof

| Parameter | Units | Radar G Security Radar | Radar H Indoor non-destructive inspection | Radar I Non-destructive inspection |
| --- | --- | --- | --- | --- |
| Frequency bands that are proposed/ targeted to be studied for this application (Note 1) | GHz | 252-296 | 252-296 | 325-700 |
| Typical operating range | m | Up to 30 | Up to 30 | Up to 10 |
| Typical emission type |  | FMCW, FMCW-variants | FMCW, FMCW-variants | FMCW, FMCW-variants |
| Max occupied bandwidth | GHz | 30 | 30 | 50 |
| Chirp bandwidth (necessary bandwidth) | GHz | 27 | 27 | 35 |
| Typical sweep time | s | 0.2/GHz | 0.2/GHz | 0.2/GHz |
| Maximum e.i.r.p. | dBm | 40 | 40 | 40 |
| Max power density of unwanted emission | dBm/MHz | –20 | –20 | –20 |
| Duty cycle (Note 2) | % | 2 (15.3 ms/sec) | 2 | 5 |
| Activity factor (Note 2) | % | 10 | 30 | 20 |
| Receiver IF bandwidth | MHz | 0.2（Specifically, 167 kHz） | 0.2 | 0.2 |
| Receiver sensitivity | dBm/Hz | –143 | –143 | –143 |
| Receiver noise figure | dB | 20 | 20 | 20 |
| Antenna main beam gain | dBi | TX: 24 RX: 24 | TX: 24 RX: 24 | TX: 27 RX: 27 |
| Antenna height | m | 0.0 – 2.0 | 1.0 – 2.0 | 1.0 – 2.0 |
| downtilt | deg | 0 (wall mounted) ‒45 (ceiling mounted) +45 (floor mounted) | ‒90 - 0 | –90 - 0 |
| Antenna azimuth 3 dB beamwidth | degrees | ±3 | ±3 | ±2 |
| Antenna elevation 3 dB beamwidth | degrees | ±3 | ±3 | ±2 |
| Urban deployment density | devices/km2 | 26.67 | 7.45 | - |
| Suburban deployment density | devices/km2 | 0.63 | 7.45 | - |
| Rural deployment density | devices/km2 | 0.4 | 0.46 | - |
| Antenna polarization |  | Linear | Linear | Linear |
| Average Antenna polarization loss | dB | 3 | 3 | 3 |
| Average body loss | dB | 0 | 0 | 0 |
| Antenna efficiency loss | dB | 3 | 3 | 3 |
| Indoor deployment |  | 100% | 100% | 100% |
| Frequency reuse |  | 1 | 1 | 1 |
| Note 1: The frequency bands listed in RR No. 5.340 are excluded from the studies. Note 2: Duty cycle (in %) is technology dependent while the activity factor (in %) is user or system dependent (with reference to 1 day). The activity factor is to be understood as the average of the percentage of devices that are active (i.e. being used by the user) at the same time, same frequency and in the same simulation area. The product of the duty cycle and the activity factor is equivalent to a ratio that is used to determine the number of active devices to be simulated from a density. |  |  |  |  |

Regarding indoor deployment:

For security radar, human intervention is required when hazardous materials are detected. Additionally, since moisture on the antenna can significantly affect performance, it is difficult to envision deployment in environments without a roof where it might rain. Therefore, 100% indoor operation is assumed.

For indoor non-destructive inspection, while there may be some use cases such as at construction sites, it is unlikely to be used outdoors to detect millimeter-scale scratches while moving. Similar to security radar, moisture on the antenna can significantly affect performance, making it difficult to envision deployment in environments without a roof. Therefore, 100% indoor operation is assumed.

The basis for calculating deployment density is shown below.

The following provides an example based on Japan. Given the high density of buildings and population in Japan, it is considered that conducting studies using deployment density data based on the Japanese context would provide the necessary and sufficient conditions for ensuring sharing and compatibility with incumbent services.

Regarding security radar, potential deployment targets include train stations, airports, museums, stadiums, event venues (concert halls and arenas) and important buildings.

Train stations

Regarding train stations, it is unlikely that baggage inspections would be conducted in subways, trams, or suburban areas where stations are close together and passengers can quickly disembark. Therefore, the target is limited to long-distance trains such as the Shinkansen and Maglev – magnetic levitation – railway in Japan.

However, there are only 120 Shinkansen stations, including mini-Shinkansen stations, which is a small number. Therefore, as the maximum assumed deployment density, we referred to the number of automatic ticket gates across Japan. According to statistical data from the Japanese Ministry of Land, Infrastructure, Transport and Tourism, the number of automatic ticket gates is as follows:

‒	(Private and public railways except Japan Railways Group (JR)) 2 743 stations, 17 123 gates.

‒	(JR total) 2,109 stations, 11 887 gates.

Dividing these totals by Japan's land area of 378 000 km2 and rounding to the fourth decimal place, we get 0.077 gates/km2. Even if four gates are installed at one location, it would be approximately 0.31 gates/km2.

Airports

There are only 97 airports in Japan, which is not a significant number, so they are not considered.

Museums

There are only about 480 large museums in Japan (floor area ≥ 3 000 m2) for which the introduction of security gates is anticipated, which is not a significant number, so they are not considered.

Stadiums

There are only about 160 large stadiums in Japan, including certified athletics stadiums that host international, national, or regional competitions and ballparks with a seating capacity of 10 000 or more that host Nippon Professional Baseball official games, which is not a significant number. Furthermore, because the operating hours of security gates at such large sports facilities are typically limited to entry periods during events, their utilization is extremely low compared with the important buildings mentioned next, so they are not considered.

Event venues (concert halls and arenas)

There are only about 20 large indoor event venues in Japan with a seating capacity of 10 000 or more, which is not a significant number, so they are not considered.

Important buildings

The minimum floor area per worker stipulated by law is 4.8 square meters or more. However, in practice, the average floor area per worker in Tokyo in 2022 was 12.1 m2/person. Therefore, a floor area of 10 000 m2 would accommodate approximately 826 people.

For a building of this scale, it can be assumed that some important operations are being conducted. Thus, office buildings with a floor area of 10 000 m2 or more are considered important buildings where security gates are likely to be installed.

Assuming the processing capacity of a security gate is 5 seconds per person, it can handle 12 people per minute, and with 4 gates, 48 people per minute. Therefore, 800 people can be processed in about 16 minutes, which means 4 security gates per 10 000 m2. The deployment density is calculated based on this assumption.

In Table 7, three major cities are categorized as Urban, Sendai City, Kyoto City, and Nagoya City are extracted as Suburban, and other regional cities are selected as Rural.

TABLE 7

Estimated deployment density of relevant security radar

| Area | Total floor area of large office buildings (total floor area of 3 000 m2 or more) [10 000 m2] | Total floor area of large office buildings (total floor area of 10 000 m2 or more) [10 000 m2] | Regional area [km2] | Deployment density [devices/km2] |
| --- | --- | --- | --- | --- |
| Tokyo's 23 wards | 8 460 | 5 837.4 | 619 | 37.72 |
| Osaka City | 2 001 | 1 360.68 | 223 | 24.41 |
| Nagoya City | 803 | 497.86 | 326 | 6.11 |
| Urban | 11 264 | 7 695.94 | 1 168 | 26.36 |
| Sendai City | 298 | 169.86 | 786 | 0.86 |
| Kyoto City | 145 | 65.25 | 828 | 0.32 |
| Fukuoka City | 528 | 295.68 | 4971 | 0.24 |
| Suburban | 971 | 530.79 | 6585 | 0.32 |
| Rural | 910 | 354.9 | 16385 | 0.09 |

Here:

‒	Total floor area of large office buildings [m2] = Total floor area of office buildings [m2] × Proportion of floor area that is 10 000 m2 or more.

‒	Deployment density = (4 [units] / 10 000 [m2]) × Total floor area of large office buildings [m2] / Regional area [km2].

The results are rounded to the third decimal place.

Finally, the deployment density for train stations is uniformly set at 0.31 [devices/km2] for each region. This value is added to the deployment density for important buildings in Urban, Suburban, and Rural areas, and the results are recorded in Table 6 as the deployment density.

Next, regarding Indoor non-destructive inspection:

Based on statistical data from the Japanese Ministry of Economy, Trade and Industry, the number of production lines is estimated from the number of employees, and the number of non-destructive inspection devices is estimated accordingly. Here, it is assumed that there are approximately 30-50 employees per production line, and establishments with 30 or fewer employees are considered to have one inspection device. Additionally, since manufacturing establishments are not necessarily more concentrated in urban areas, Urban and Suburban are treated as the same.

‒	Urban/Suburban: Tokyo, Kanagawa, Aichi, and Osaka were selected as representative industrial regions of Japan.

Table 8

Estimated number of devices in urban area

| Area | 29 or fewer people (1 device) | 30-99 people (2 devices) | 100-299 people (4 devices) | 300 or more people (10 devices) |
| --- | --- | --- | --- | --- |
| Tokyo | 13 903 | 1 189 | 232 | 76 |
| Kanagawa | 7 988 | 1 264 | 465 | 194 |
| Aichi | 14 455 | 2 745 | 927 | 382 |
| Osaka | 17 933 | 2 367 | 529 | 142 |
| Total | 54 285 devices | 15 130 devices | 8 612 devices | 7 940 devices |

A total of 85 967 devices are distributed across the four prefectures, with a combined area of Tokyo (2 103.97 km2) + Kanagawa (2 415.86 km2) + Aichi (5 116.22 km2) + Osaka (1 901.42 km2) = a total of 11 537.47 km2. The deployment density is 7.45 devices/km2.

‒	Rural: Aomori, Ishikawa, Yamaguchi, and Miyazaki were selected as regions with relatively low concentrations of manufacturing industries.

Table 9

Estimated number of devices in rural area

| Regions | 29 or fewer people (1 device) | 30-99 people (2 devices) | 100-299 people (4 devices) | 300 or more people (10 devices) |
| --- | --- | --- | --- | --- |
| Aomori | 1118 | 277 | 88 | 24 |
| Ishikawa | 2562 | 451 | 146 | 46 |
| Yamaguchi | 1450 | 345 | 139 | 59 |
| Miyazaki | 1164 | 255 | 89 | 29 |
| Total | 6 294 devices | 2 656 devices | 1 848 devices | 1 580 devices |

A total of 12 378 devices are distributed across the four prefectures, with a combined area of Aomori (9 644.70 km2) + Ishikawa (4 186.16 km2) + Yamaguchi (6 114.13 km2) + Miyazaki (6 794.69 km2) = a total of 26 739.68 km2. The deployment density is 0.46 devices/km2.

### 4.4.3	Antenna pattern

The equations for the antenna pattern are contained in Annex 9.1 Antenna pattern using these formulas for the sensing radar defined in Table A9-1 is presented in Figures 16, and 17 for Radar G and H and 18 for Radar I.

[Editor’s Note: Add text on the antenna pattern ]

Figure 16

Simplified antenna pattern in transmission for sensing radar G and H ( 128 × 1 elements )

Figure 17

Simplified antenna pattern in transmission for sensing radar G and H ( 16 × 16 elements )

*Regarding Figure 2, the azimuth and elevation are consistent.

Figure 18

Simplified antenna pattern for radar I (16 × 16 elements )

(a) 325 GHz                                                 (b) 470 GHz

(c) 700 GHz

### 4.4.4	Operational characteristics of radar systems operating in the frequency bands [252]-700 GHz

The primary application of the described terahertz near-field non-destructive testing radar systems is envisaged for high-precision industrial inspection and scientific research scenarios. These sensors will be deployed in quality control, research and development, and failure analysis functions where extreme resolutions at microscopic levels are required. In inspection scenarios involving critical components and advanced materials, the exceptional resolution of terahertz waves is required to provide sufficient contrast and detail resolution capability to detect sub-surface micro-cracks, delamination, and material inhomogeneities in the near-field ranges from the sensor.

The following non-exhaustive list gives examples of scenarios where the described sensors are intended to be used: quality control of composite material layers in aerospace components, thickness measurement and defect detection in multi-layer paint and coating systems, identification of moisture ingress and voids in dielectric materials, and generally in laboratory or production-line settings where internal integrity of non-conductive materials needs to be verified without contact.

The market deployment and the availability of such advanced terahertz inspection systems will increase over a certain timeframe as the technology matures and costs decrease. The timing for widespread industrial deployment and the density of adoption of the described terahertz radar sensors will vary significantly depending on the geographical regions and the technological advancement of their respective high-tech manufacturing sectors.

# 5	Spectrum needs of RLS systems

[If Report, this needs to be spectrum aspects]

## 5.1	Systems for close proximity object detection

The frequency band of interest are 252-296 GHz and 356-439 GHz, which in total is equivalent to 127 GHz. To have sufficient spectrum available for the foreseen deployment of radar sensors mounted on mobile/fixed device it would be favourable to make 40 GHz in total available for this application (Radar A and B). If possible, this should be one spectrum segment or at least two contiguous 20 GHz channels.

## 5.2 	Vehicular radar systems

The total required amount of spectrum for all sensors mounted at a vehicle is 35 GHz with 10 GHz for radar type X and 25 GHz for radar type Y. To implement the described vehicular radar systems, it would be beneficial to have these 35 GHz available in a contiguous block. If the requested 35 GHz is not available in a contiguous segment, the spectrum blocks for radar type X and radar type Y should be located closely together.

## 5.3 	Security applications (section 4.3)

The total amount of required spectrum for security applications is 65 GHz in total, with systems corresponding to RADAR C needing contiguous 30 GHz, RADAR D contiguous 30 GHz, and RADAR E and RADAR F 5 GHz (contiguous) in total, considering the frequency ranges for these two categories overlap.

## 5.4 	security applications (section 4.4)

It is necessary to ensure sufficient spectrum availability for the planned deployment of radar sensors mounted on security radar systems (Radar G and H). In this application, a resolution of less than 1 cm is required for the detection of hazardous materials. To achieve such depth resolution, a contiguous bandwidth of 30 GHz is necessary. Operation in the 252-296 GHz frequency band is envisioned for this purpose.

[Editor’s Note: Spectrum Requirement for Radar I is still needed]

[Editor’s Note: The spectrum requirement per radar has been explained, but explanation is needed on the use of the same subband]

# 6	Propagation model

## 6.1	Recommendations applicable for all sharing geometries

### 6.1.1	ITU-R P.2108 Prediction of clutter loss

This Recommendation provides a method for calculating additional loss due to one or both of the radio terminals being embedded in local clutter (e.g., buildings, foliage) in an urban or suburban environment.

The height gain terminal correction model, section 3.1, gives the median of losses due to different terminal surrounding environments for frequencies between 30 MHz and 3 GHz. The possible propagation mechanisms include obstruction loss and reflections due to clutter objects at representative heights, scattering and reflection from the ground and smaller clutter objects. The method used in section 3.1 distinguishes between several general cases including for woodland, open areas, suburban, and urban categories. It is assumed that the dominant mechanisms are diffraction over clutter, reflection or scattering. When using this model, the basic transmission loss should be calculated to/from the height of the representative clutter height used.

In addition, two statistical models are provided in sections 3.2 and 3.3 to estimate clutter loss in urban and suburban environments as a function of probability. The statistical output of these models includes a predicted proportion of events, based on empirical data, where there is line-of-sight and therefore negligible clutter loss:

1	one model (in section 3.2) addresses terrestrial paths for frequencies between 0.5 and 67 GHz;

2	the model in section 3.3 is for an inclined path for elevation angles in the range 0 to 90 degrees and frequencies in the range 10 to 100 GHz where the terrestrial end of the path is within the clutter.

These models have been developed using measurement results as well as analytical models over the range of frequencies, particularly in urban environments.

Report ITU-R P.2402 – A method to predict the statistics of clutter loss for earth-space and aeronautical paths describes the development of a stochastic model for the inclined path in this Recommendation based on a typical cluttered terminal height of 4 to 6 metres.

There is currently no statistical clutter loss model for a rural environment.

This Recommendation should be used in addition to one of the Recommendations for the appropriate geometry. Please refer to comments in the text for each Recommendation as to when and how this should be included.

Working Parties 3J, 3K and 3M are working on updating Recommendation ITU-R P.2108-1 (section 3.3). In the interim Working Parties 3J, 3K and 3M would encourage Working Parties of other Study Groups to wait for the conclusion of the work, the results of which will be liaised from the next WPs 3J, 3K, and 3M meetings in February 2025.

See additional note on the usage of Recommendation ITU-R P.2108 in section 6.1.3.

### 6.1.2	ITU-R P.2109	Prediction of building entry loss

This Recommendation provides a method for calculating additional loss due to one of the radio terminals being inside a building while the other is outside. Inputs to the model are frequency, probability that the loss is not exceeded, building class and elevation angle of the path at the building façade.

The current model is based on measurement data collated in Report ITU-R P.2346 in the range 80 MHz to 73 GHz and is nominally valid ‘at frequencies between about 80 MHz and 100 GHz’. Since the formulation of the present model, new data at 47 MHz and at 97, 158 and 300 GHz were submitted. Working Parties 3J, 3K and 3M anticipate that it will be possible to extend both the upper and lower frequency limits of the Recommendation, the latter to around 200 GHz, prior to July 2025. A robust extension to 700 GHz is unlikely, but it is anticipated to offer guidance for such frequencies in the same timescale. Working Parties 3J, 3K and 3M would welcome any further measurement data that may be available.

Coefficients are provided for two building types, traditional and thermally efficient; these are categorized dependent on whether thermally efficient building methods and materials are used or not.

The statistics given by the model of Recommendation ITU-R P.2109 relate to the loss exceeded at any point in any building (of the ‘traditional’ or ‘thermally efficient’ types), globally.

In most cases of practical interest, it will be necessary to combine the predicted statistics of building loss with those of local environmental clutter (e.g. other buildings) or with other spatial loss distributions. It is important that this combination takes account of the entire loss distributions, rather than, for example, just considering median or other quantile values.

The statistics of building entry loss as given by Recommendation ITU-R P.2109 are currently assumed to be independent with respect to other loss distributions and should be combined accordingly. For use in Monte Carlo models, this combination reduces to the simple process of using independent draws to sample building entry loss and, e.g. clutter loss (from Recommendation ITU-R P.2108) and combining the two by addition in decibel form.

Retrieving the overall loss, for a specific probability, is less straightforward, because the model of Recommendation ITU-R P.2109 is an empirical fit with a non-analytical form. In this case, numerical convolution should be used to derive the joint distribution.

This Recommendation should be used in addition to one of the ITU-R P-series Recommendations described below for specific geometries. Please refer to comments below as to when and how this should be included.

See additional note on the usage of Recommendation ITU-R P.2109 in section 6.1.3.

### 6.1.3 	Additional note on using Recommendations ITU-R P.2108 and P.2109

The expert group, Working Parties 3J, 3K and 3M, recognized that the current Recommendations ITU-R P.2108-1 and ITU-R P.2109-2 are empirical models derived by fitting data from measurements or simulations. They are applicable up to 100 GHz partly due to the measurement capabilities available at the time of the development of the models. It is observed that the current model of Recommendation ITU-R P.2108-1 in section 3.2 (terrestrial path), validated up to 67 GHz, presents a constant loss beyond certain frequencies, while the current model in section 3.3 (Earth-space and aeronautical paths), validated up to 100 GHz, presents a monotonic increase of the losses with frequency. Moreover, the current model of Recommendation ITU-R P.2109-2 presents, above 10 GHz, a monotonic increase of losses with frequency for the two building type environments (i.e., “traditional” and “thermally efficient”).

The physical phenomena affecting clutter loss or building entry loss (e.g. reflections, diffraction, scattering, material losses, etc.) are not expected to show drastic changes in trends at frequencies below and above 100 GHz. It would therefore be reasonable to assume that, in the absence of evidence to the contrary, the trends observed in Recommendations ITU-R P.2108 and ITU-R P.2109 would continue above 100 GHz. Nevertheless, extrapolating those empirical formulas outside their domain of validity introduces significant uncertainty. Even if the extrapolated results appear physically reasonable, unvalidated extrapolation can produce results with unknown reliability.

Working Parties 3J, 3K, and 3M recommend applying the loss value computed at the model’s upper frequency limit (67 GHz for Recommendation ITU-R P.2108, section 3.2; 100 GHz for Recommendation ITU-R P.2108, section 3.3 and Recommendation ITU-R P.2109) across all higher frequencies. It would be reasonable to assume that losses will not decrease with increasing frequency, as this is consistent with the expected behaviour of most propagation losses at higher frequencies. This approach would ensure that losses are not overestimated and the interference not underestimated for sharing and compatibility studies.

Until measurement data are available to extend and validate these models above 100 GHz, this approximation offers a practical solution. Experts in WP 5B are invited to provide any available propagation measurement data above 100 GHz to WPs 3J, 3K, and 3M, which will support future revisions and possible extensions of those models based on validated evidence.

## 6.2	Recommendations applicable for sharing between stations on the surface of the Earth

### 6.2.1	ITU-R P.1411 – Propagation data and prediction methods for the planning of short-range outdoor radiocommunication systems and radio local area networks in the frequency range 300 MHz to 100 GHz

This Recommendation provides methods for predicting loss on outdoor short-range (typically less than 1 km) paths. The prediction includes site-specific and site-general models for line-of-sight (LOS) and non-line-of-sight (NLOS) environments, multipath models, polarization characteristics and fading conditions. The physical environments discussed in this Recommendation include urban very high rise, urban high rise, urban low-rise/suburban, residential, and rural.

The site-specific models of this recommendation require measured empirical constants to complete the calculations. Many of the prediction methods are based on measurements at specific frequencies within the range of interest. However, site-general prediction methods in the Recommendation are applicable to sharing studies using statistical methods within the frequency ranges, environments, and distances specified in the Recommendation.

It must be emphasized that the measurements and modelling in this Recommendation intrinsically include the effect of clutter over the full length of the path, so it would be inappropriate to add a calculation of clutter separately. Section 4.5.2 of the Recommendation notes the requirement to consider building entry loss separately, and this can be done by using Recommendation ITU-R P.2109 described in section 2 of the WP 3M Chair’s Report (Document 3M/106, Annex 1).

Working Party 3K is currently working to increase the frequency limit of this recommendation.

### 6.2.2	ITU-R P.1238 – Propagation data and prediction methods for the planning of indoor radiocommunication systems and radio local area networks in the frequency range 300 MHz to 450 GHz

This Recommendation provides prediction methods for paths contained entirely within a single building, at frequencies between 300 MHz and 450 GHz. It would be of use in interference scenarios where the dominant paths between the interfering and interfered-with systems remain inside the same building. For obvious reasons, the addition of clutter loss or building entry loss would not be appropriate in this case.

## 6.3	Recommendations applicable for frequencies above 200 GHz

For frequencies above 200 GHz, for unobstructed line-of-sight paths the two dominant propagation mechanisms are free-space basic transmission loss and gaseous attenuation. For frequencies above 200 GHz where obstructions along the path lead to diffraction loss, these losses are likely very high.

### 6.3.1	ITU-R P.525 – Calculation of free-space attenuation

Recommendation ITU-R P.525 provides methods to calculate the attenuation in free space.

### 6.3.2	ITU-R P.676 – Attenuation by atmospheric gases and related effects

This recommendation provides methods in its Annex 1 to calculate the slant path gaseous attenuation, phase nonlinearity, atmospheric bending, excess atmospheric path length and downwelling and upwelling noise temperatures due to oxygen and water vapour for the frequency range from 1 to 1 000 GHz for arbitrary known pressure, temperature and water vapour height profiles. Alternatively, the approximate method in Annex 2 may be used to estimate the instantaneous slant path gaseous attenuation due to oxygen and water vapour for the frequency range from 1 to 350 GHz.

# 7	Considerations relative to incumbent service systems

The present section provides the summary results of the technical analysis performed under this agenda item for each of the incumbent service systems.

The detailed characteristics and technical analysis are described in the following annexes:

Annex 1: 	Characteristics and technical analysis related to the fixed service

Annex 2: 	Characteristics and technical analysis related to the mobile service

Annex 3:	 Characteristics and technical analysis related to the mobile-satellite service

Annex 4: 	Characteristics and technical analysis related to the amateur and amateur satellite service systems

Annex 5: 	Characteristics and technical analysis related to the radionavigation service systems

Annex 6: 	Characteristics and technical analysis related to the radionavigation-satellite service

Annex 7: 	Characteristics and technical analysis related to the earth exploration satellite service systems

Annex 8: 	Characteristics and technical analysis related to the radioastronomy service systems

[TBD]

## 7.1	Summary related to fixed service systems

Study with FS stations considers Radars A and B and Radars X and Y.

Part of the study on Radars A and B is an aggregate statistical study between one FS station and uniformly distributed (urban deployment) Radars A or B. The results shows that FS protection criterion is not exceeded under such assumptions. Clutter loss and building entry loss was not considered.

Part of the study on Radars X and Y is a single-entry statistical study between one FS station and one vehicle equipped with eight radars (four Radars Y on the corners, two Radars Y as parking radars, two Radars X as front and rear radars). The results which are distributions of protection distances between FS station and one uniformly rotating vehicle show that protection distances will be about 2-13 km for 252-296 GHz band and about 100-700 m for 356-439 GHz band.

## 7.2	Summary related to mobile service systems

Study with MS stations considers Radars A and B and Radars X and Y.

Part of the study on Radars A and B is an aggregate statistical study between one MS station and uniformly distributed (urban deployment) Radars A or B. The results shows that MS protection criterion may be exceeded for percentages of time up to 0.4%. Clutter loss and building entry loss was not considered.

Part of the study on Radars X and Y is a single-entry statistical study between one MS station and one vehicle equipped with eight radars (four Radars Y on the corners, two Radars Y as parking radars, two Radars X as front and rear radars). The results which are distributions of protection distances between MS station and one uniformly rotating vehicle show that protection distances will be between 200 m and 17 km.

## 7.3	Summary related to fixed satellite service systems

[TBD]

## 7.4	Summary related to fixed amateur and amateur satellite service systems

[TBD]

## 7.5	Summary related to radionavigation service systems

[TBD]

## 7.6	Summary related to radionavigation satellite service systems

[TBD]

## 7.7	Summary related to earth exploration satellite service systems

[TBD]

## 7.8	Summary related to radioastronomy service systems

[TBD]

# 8	Conclusions

[TBD]

List of Annexes

Annex 1:	Characteristics and technical analysis related to the fixed service

Annex 2:	Characteristics and technical analysis related to the mobile service

Annex 3: 	Characteristics and technical analysis related to the mobile-satellite service

Annex 4:	Characteristics and technical analysis related to the amateur and amateur satellite service systems

Annex 5: 	Characteristics and technical analysis related to the radionavigation service systems

Annex 6: 	Characteristics and technical analysis related to the radionavigation-satellite service

Annex 7: 	Characteristics and technical analysis related to the earth exploration satellite service systems

Annex 8: 	Characteristics and technical analysis related to the radioastronomy service systems

Annex 9: 	Common characteristics related to the RLS applications

Annex 1

# A1 	Characteristics and technical analysis related to the fixed service systems

[Editor’s note: All calculations below in this section are done based on the RLS characteristics as contained in Annex 6 to doc. 5B/216.]

## A1.1	Frequency bands

According to RR No. 5.564A the frequency bands 275-296 GHz, 306-313 GHz, 318-333 GHz and 356-450 GHz are identified for use by administrations for the implementation of fixed service applications where no specific conditions are necessary to protect Earth exploration-satellite service (passive) applications. The frequency bands 296-306 GHz, 313-318 GHz and 333-356 GHz may only be used by fixed service applications when specific conditions to ensure the protection of Earth exploration-satellite service (passive) applications are determined.

## A1.2	Interference criteria

The protection criterion is I/N –10 dB that should not be exceeded by more than 20% of the time according to Report ITU-R F.2416.

## A1.3	Technical and operational characteristics

Table A1-1 contains technical and operational characteristics of fixed service applications from Report ITU-R F.2416.

TABLE A1-1

Technical and operational characteristics of the fixed service applications

| Frequency band (GHz) | 275-325 | 380-445 |
| --- | --- | --- |
| Duplex Method | FDD/TDD | FDD/TDD |
| Modulation | BPSK/QPSK/8PSK/8APSK/16QAM/32QAM/64QAM BPSK-OFDM/QPSK-OFDM/ 16QAM-OFDM/32QAM-OFDM/64QAM-OFDM | BPSK/QPSK/8PSK/8APSK/ 16QAM/32QAM, 8PSK, 8APSK BPSK-OFDM/QPSK-OFDM/ 16QAM-OFDM/32QAM-OFDM |
| Channel bandwidth (GHz) | 2 to 25 (FDD) 2 to 50 (TDD) | 2 to 32.5 (FDD) 2 to 65 (TDD) |
| Tx output power range (dBm) | 0 to 20 | −10 to 10 |
| Tx output power density range (dBm/GHz) | −17 to 17 | −28 to 7 |
| Feeder/multiplexer loss range (dB) | 0 to 3 | 0 to 3 |
| Antenna gain range (dBi) | 24 to 50 | 24 to 50 |
| e.i.r.p. range (dBm) | 44 to 70 | 37 to 60 |
| e.i.r.p. density range (dBm/GHz) | 30 to 67 | 19 to 57 |
| Antenna pattern | Recommendation ITU-R F.699 (Single entry) Recommendation ITU-R F.1245 (Aggregate) | Recommendation ITU-R F.699 (Single entry) Recommendation ITU-R F.1245 (Aggregate) |
| Antenna type | Parabolic Reflector | Parabolic Reflector |
| Antenna height (m) | 6-25 | 10-25 |
| Antenna elevation (degree) | ±20 (typical) | ±20 (typical) |
| Receiver noise figure typical (dB) | 15 | 15 |
| Receiver noise power density typical (dBm/GHz) | −69 | −69 |
| Normalized Rx input level for 1×10-6 BER (dBm/GHz) | −61 to −54 | −61 to −54 |
| Link length (m) | 100 to 300 | 100 to 300 |
| Deployment Density | See § 8.1 of Report ITU-R F.2416 | See § 8.1 of Report ITU-R F.2416 |

## A1.4	Sharing and compatibility analysis

This study considers the following cases:

–	radar systems from section 4.1.2 (radars using scanning and focusing/tracking mode): Radar A and Radar B;

–	vehicular radar systems from section 4.2: Radar X and Radar Y.

Radar A and Radar B

This study considers aggregate interference from RLS devices (Radar A and Radar B) into one FS station. It uses Monte-Carlo method to calculate the distribution of I/N. Radars are distributed uniformly over the surface around FS station according to urban deployment density as provided in the table with Radar A and Radar B characteristics. The directions of the main beams of radars are also uniformly distributed. Only one type of the radar – Radar A or Radar B – is simulated in one scenario as scanning or focusing/tracking modes are not used by the same device in the same time. As the propagation loss is due to free space loss and atmospheric gaseous attenuation, the variability is caused only by geometry of the interference scenario.

In calculations an assumption was taken that all stations are working in the same plane so elevation angle was taken as 0 degrees.

FS station parameters: bandwidth 2 GHz for all frequencies, 50 GHz for 252-325 GHz band and 65 GHz for 380-445 GHz band.

Radar A parameters: e.i.r.p. 25 dBm, antenna gain 15 dBi, beamwidth ±3 deg.

Radar B parameters: e.i.r.p. 25 dBm, antenna gain 15 dBi, beamwidth ±3 deg.

The results of calculation are shown in Figures A1-1 to A1-4. Figures A1-1 and A1-2 show the results for Radar A distribution, but for two different FS station antennas, Figures A1-3 and A1-4 show the results for Radar B distribution, also for two different FS station antennas. The small red star on these figures is the FS protection criterion. Figures show different combinations of FS and RLS bandwidths.

Figure A1-1

CDF of I/N for FS station with 24 dBi antenna, Radar A

Figure A1-2

CDF of I/N for FS station with 50 dBi antenna, Radar A

Figure A1-3

CDF of I/N for FS station with 24 dBi antenna, Radar B

Figure A1-4

CDF of I/N for FS station with 50 dBi antenna, Radar B

It can be seen that the FS protection criterion is not exceeded for the case of Radar A or Radar B deployed uniformly.

Vehicular radar systems (Radars X and Y)

[Editor’s Note: These simulations were based on the information available after the May 2025 of WP 5B]

This study considers interference from one vehicle (equipped with radars) into one FS station. It uses Monte-Carlo method to calculate the distribution of protection distances based on FS protection criterion. Vehicle with radars is modelled as an example in Figure 1 of the main body of the document, i.e. radars on the corners (four Radars Y), parking radars (two Radars Y), front and rear radars (two Radars X). The radar radiation patterns are assumed to radiate only in the front hemisphere. The angular position of such vehicle with radiating radars in relation to FS station is randomized (angle θ in Figure A1-5) as well as azimuth of the orientation of the vehicle (angle φ in Figure A1-5). For each pair of randomly generated angles the protection distance is calculated. As the propagation loss is due to free space loss and atmospheric gaseous attenuation, the variability is caused only by geometry of the interference scenario.

Figure A1-1

Interference scenario geometry between vehicle with radars and FS

In calculations an assumption was taken that all stations are working in the same plane so elevation angle was taken as 0 degrees. The CDFs of protection distances are shown in Figures A1-2, A-23, A1-4 and A1.5 for frequencies 252 GHz, 275 GHz, 380 GHz, 439 GHz. Figures A1-6 and A1-7 show the case of 24 dBi FS antenna and Figures A1-8 and A1-9 – 50 dBi FS antenna. Figures A1-6 and A1-8 show frequencies 252 GHz and 275 GHz, Figures A1-7 and A1-9 – 380 GHz and 439 GHz. FS station parameters: bandwidth 2 GHz for all frequencies, 50 GHz for 252-325 GHz band and 65 GHz for 380-445 GHz band (the same as in the first part of the study).

Radar X parameters: e.i.r.p. 65 dBm, antenna gain 40 dBi (typical), beamwidth 30 deg.

Radar Y parameters: e.i.r.p. 55 dBm, antenna gain 30 dBi (typical) and beamwidth 60 deg.

The red horizontal line on these figures is the percentage of FS protection criterion (20%). Figures show different combinations of FS and RLS bandwidths.

Figure A1-6

CDF of protection distance for system of eight vehicular radars, FS with 24 dBi antenna, lower frequencies

Figure A1-7

CDF of protection distance for system of eight vehicular radars, FS with 24 dBi antenna, higher frequencies

Figure A1-8

CDF of protection distance for system of eight vehicular radars, FS with 50 dBi antenna, lower frequencies

Figure A1-9

CDF of protection distance for system of eight vehicular radars, FS with 50 dBi antenna, higher frequencies

So, depending on the scenario, the protection distance between only one vehicle (with eight radars) and FS station will be about 2-13 km for 252-296 GHz band and about 100-700 m for 356-439 GHz band to meet FS protection criterion.

## A1.4	Summary of results

The first part of this study which is an aggregate statistical study between one FS station and uniformly distributed (urban deployment) Radars A and Radars B shows that FS protection criterion is not exceeded under such assumptions.

The second part of the study which is a single-entry statistical study between one FS station and one vehicle equipped with eight radars and which gives the distribution of protection distances between FS station and uniformly rotating vehicle shows that the protection distances will be 2-13 km for 252-296 GHz band and about 100-700 m for 356-439 GHz band. As one single vehicle cannot exist in real situations, the only possibility to provide protection to FS service stations is to limit RLS systems to the upper band.

Annex 2

# A2 	Characteristics and technical analysis related to the mobile service systems

## A2.1	Frequency bands

According to RR No. 5.564A the frequency bands 275-296 GHz, 306-313 GHz, 318-333 GHz and 356-450 GHz are identified for use by administrations for the implementation of land mobile service applications where no specific conditions are necessary to protect Earth exploration-satellite service (passive) applications. The frequency bands 296-306 GHz, 313-318 GHz and 333-356 GHz may only be used by land mobile service applications when specific conditions to ensure the protection of Earth exploration-satellite service (passive) applications are determined.

## A2.2	Interference criteria

The protection criterion is I/N ‒6 dB according to Report ITU-R M.2517.

## A2.3	Technical and operational characteristics

Table A2-1 contains technical and operational characteristics of two types of CPMS applications from Report ITU-R M.2417.

TABLE A2-1

Technical and operational characteristics of the land mobile service CPMS applications

| Parameters | Values |  |
| --- | --- | --- |
|  | CPMS application | Enhanced CPMS application |
| Frequency band (GHz) | 275-325 | 275-450 |
| Deployment density (1) | 0.6 devices/km2 | 0.6 devices/km2 |
| Tx output power density (dBm/GHz) | −3.8….6.9 | −10.1…6.7 |
| Max. e.i.r.p. density(dBm/GHz) | 26.2…...36.9 | 19.9…36.7 |
| Duplex Method | FDD/TDD | FDD/TDD |
| Modulation | OOK-SC/BPSK-SC/QPSK-SC/16QAM-SC/64QAM-SC BPSK-OFDM/QPSK-OFDM/ 16QAM-OFDM/32QAM-OFDM/64QAM-OFDM | OOK-SC/BPSK-SC/QPSK-SC/16QAM-SC/64QAM-SC/8PSK-SC/8APSK-SC BPSK-OFDM/QPSK-OFDM/ 16QAM-OFDM/32QAM-OFDM/64QAM-OFDM |
| Average distance between CPMS fixed and mobile devices (m) | 0.1 | 0.1 |
| Maximum distance between CPMS fixed and mobile devices (m) | 1 | 1 |
| Antenna height (m) | 1…2 | - |
| Antenna beamwidth (degree) | 3…10 | 5…90 |
| Antenna elevation (degree) | ±90 | ±90 |
| Frequency reuse | 1 | 1 |
| Antenna type | Horn | Horn |
| Antenna pattern | Gaussian | Gaussian |
| Antenna polarization | Linear | Linear |
| Indoor CPMS fixed device deployment (%) | 100 | 90 |
| Feeder loss (dB) | 2 | 2 |
| Maximum CPMS fixed/mobile device output power (dBm) | 10 | 10 |
| Channel bandwidth (GHz) | 2.16/4.32/8.64/12.96/17.28/ 25.92/51.8 | 2.16/4.32/8.64/12.96/17.28/25.92/51.84/69.12/103.68 |
| Maximum CPMS fixed device antenna gain (dBi) | 30 | 30 |
| Maximum CPMS mobile device antenna gain (dBi) | 15 | 15 |
| Maximum CPMS fixed device output power (e.i.r.p.) (dBm) | 40 | 40 |
| Maximum CPMS mobile device output power (e.i.r.p.) (dBm) | 25 | 25 |
| Average activity factor (%) | 0.76 | 0.2 |
| Average CPMS fixed device power (dBm (e.i.r.p)) | 20 | 20 |
| Receiver noise figure typical (dB) | 15 | 15 |

## A2.4	Sharing and compatibility analysis

This study considers the following cases:

–	radar systems from section 4.1.2 (radars using scanning and focusing/tracking mode): Radar A and Radar B;

–	vehicular radar systems from section 4.2: Radar X and Radar Y.

Radars A and B

This study considers aggregate interference from RLS devices (Radar A and Radar B) into one MS station (CPMS and enhanced CPMS). It uses Monte-Carlo method to calculate the distribution of I/N. Radars are distributed uniformly over the surface around MS station according to urban deployment density as provided in the table with Radar A and Radar B characteristics. The directions of the main beams of radars are also uniformly distributed. Only one type of the radar – Radar A or Radar B – is simulated in one scenario as scanning or focusing/tracking modes are not used by the same device in the same time. As the propagation loss is due to free space loss and atmospheric gaseous attenuation, the variability is caused only by geometry of the interference scenario.

In calculations an assumption was taken that all is working in the same plane so elevation angle was taken as 0 degrees.

MS station parameters:

–	CPMS applications: beamwidth 10 deg, 15 dBi antenna for mobile device, 30 dBi antenna for fixed device, feeder loss 2 dB;

–	enhanced CPMS applications: beamwidth 40 deg, 15 dBi antenna for mobile device, 30 dBi antenna for fixed device, feeder loss 2 dB.

Radar A parameters: e.i.r.p. 25 dBm, antenna gain 15 dBi, beamwidth ±3 deg.

Radar B parameters: e.i.r.p. 25 dBm, antenna gain 15 dBi, beamwidth ±3 deg.

The results of calculations are shown in Figures A2-1 to A2-4. Figures A2-1 and A2-2 show the results for Radar A distribution, but for two different MS station antennas, Figures A2-3 and A2-4 show the results for Radar B distribution, also for two different MS station antennas. The red vertical dashed line on these figures is the MS protection criterion (−6 dB). Figures show different combinations of MS and RLS bandwidths.

Figure A2-1

CDF of I/N for MS station with 15 dBi antenna, Radar A

Figure A2-2

CDF of I/N for MS station with 30 dBi antenna, Radar A

Figure A2-3

CDF of I/N for MS station with 15 dBi antenna, Radar B

Figure A2-4

CDF of I/N for MS station with 30 dBi antenna, Radar B

So, depending on the scenario, the MS protection criterion (−6 dB) may be exceeded for percentages of time up to 0.4%.

Vehicular radar systems (Radars X and Y)

[Editor’s Note: These simulations were based on the information available after the May 2025 of WP 5B]

This study considers interference from one vehicle (equipped with radars) into one MS station. It uses Monte-Carlo method to calculate the distribution of protection distances based on MS protection criterion. Vehicle with radars is modelled as an example in Figure 1 of the main body of the document, i.e. radars on the corners (four Radars Y), parking radars (two Radars Y), front and rear radars (two Radars X). The radar radiation patterns are assumed to radiate only in the front hemisphere. The angular position of such vehicle with radiating radars in relation to the MS station is randomized (angle θ in Figure A2-5) as well as azimuth of the orientation of the vehicle (angle φ in Figure A2-5). For each pair of randomly generated angles the protection distance is calculated. As the propagation loss is due to free space loss and atmospheric gaseous attenuation, the variability is caused only by geometry of the interference scenario.

Figure A2-5

Interference scenario geometry between vehicle with radars and MS

In calculations an assumption was taken that all stations are working in the same plane so elevation angle was taken as 0 degrees.

Radar X parameters: e.i.r.p. 65 dBm, antenna gain 40 dBi (typical), beamwidth 30 deg.

Radar Y parameters: e.i.r.p. 55 dBm, antenna gain 30 dBi (typical), beamwidth 60 deg.

MS station parameters (the same as in the first part of the study):

–	CPMS applications: beamwidth 10 deg, 15 dBi antenna for mobile device, 30 dBi antenna for fixed device, feeder loss 2 dB;

–	enhanced CPMS applications: beamwidth 40 deg, 15 dBi antenna for mobile device, 30 dBi antenna for fixed device, feeder loss 2 dB.

The CDFs of protection distances are shown in Figures A2-6 to A2-19 Figure A2-6 shows the case of CPMS applications in 275 GHz frequency, Figure A2-7 – in 325 GHz frequency. Figure A2-18 shows the case of enhanced CPMS applications in 275 GHz frequency, Figure A2-9 – in 450 GHz frequency.

Figure A2-6

CDF of protection distance for system of eight vehicular radars, CPMS application, 275 GHz

Figure A2-7

CDF of protection distance for system of eight vehicular radars, CPMS application, 325 GHz

Figure A2-8

CDF of protection distance for system of eight vehicular radars, enhanced CPMS application, 275 GHz

Figure A2-9

CDF of protection distance for system of eight vehicular radars, enhanced CPMS application, 450 GHz

So, depending on the scenario, the protection distance between only one vehicle (with eight radars) and MS station will be between 200 m and 17 km.

## A2.4	Summary of results

The first part of this study which is an aggregate statistical study between one MS station and uniformly distributed (urban deployment) Radars A and Radars B shows that the MS protection criterion (−6 dB) may be exceeded for percentages of time up to 0.4%. Clutter loss and building entry loss was not considered.

The second part of the study which is a single-entry statistical study between one MS station and one vehicle equipped with eight radars and which gives the distribution of protection distances between MS station and uniformly rotating vehicle shows that the protection distances will be between 200 m and 17 km. As one single vehicle cannot exist in real situations, the only possibility to provide protection to MS service stations is to limit RLS systems to the upper band.

Annex 3

# A3	Characteristics and technical analysis related to the mobile satellite service systems

With respect to information on technical and operational characteristics as well as protection criteria of the systems relating to WRC-27 agenda item 1.8, no input was received on this regard and there are no relevant ITU-R Reports and Recommendations.

TBD

Annex 4

# A4 	Characteristics and technical analysis related to the fixed amateur and amateur satellite service systems

[From 5B/218]:

The amateur and amateur-satellite services have a primary allocation covering the 248-250 GHz frequency band. The generic characteristics for stations of the amateur and amateur-satellite services using those bands are given in Tables 1B, 2B, 3B, 5, 6, 7 and 8 in Recommendation ITU-R M.1732-3. It should be noted that the available transmitter power for typical amateur transmitters using those bands is quite limited and is generally no more than a few milliwatts, to make up for this limitation typical maximum antenna gain of 52 dBi may be considered, where applicable, as listed in Recommendation ITU-R M.1732.

## A4.1	Frequency bands

TBD

## A4.2	Interference criteria

TBD

## A4.3	Technical and operational characteristics

TBD

## A4.4	Sharing and compatibility analysis

TBD

Annex 5

# A5	Characteristics and technical analysis related to radionavigation service systems

With respect to information on technical and operational characteristics as well as protection criteria of the systems relating to WRC-27 agenda item 1.8, no input was received on this regard and there are no relevant ITU-R Reports and Recommendations.

TBD

## A5.1	Frequency bands

TBD

## A5.2	Interference criteria

TBD

## A5.3	Technical and operational characteristics

TBD

## A5.4	Sharing and compatibility analysis

TBD

Annex 6

# A6	Characteristics and technical analysis related to radionavigation satellite service systems

With respect to information on technical and operational characteristics as well as protection criteria of the systems relating to WRC-27 agenda item 1.8, no input was received on this regard and there are no relevant ITU-R Reports and Recommendations.

TBD

## A6.1	Frequency bands

TBD

## A6.2	Interference criteria

TBD

## A6.3	Technical and operational characteristics

TBD

## A6.4	Sharing and compatibility analysis

TBD

Annex 7

# A7 	Characteristics and technical analysis related to earth exploration satellite service systems

[Editor’s note: All calculations below in this section are done based on the RLS characteristics as contained in Annex 6 to doc. 5B/216.]

## A7.1	Frequency bands

Considering in-band and adjacent frequency bands scenarios, the following frequency bands allocated to EESS (passive) below 275 GHz need to be included in the relevant studies under this agenda item:

‒	226-231.5 GHz: This frequency band is subject to RR No. 5.340 (all emissions are prohibited) and is adjacent to the lower edge addressed under WRC-27 agenda item 1.8. Only adjacent frequency band compatibility studies are assumed to be performed.

‒	250-252 GHz: This frequency band is subject to RR No. 5.340 (all emissions are prohibited). Only adjacent frequency band compatibility studies are assumed to be performed.

‒	235-238 GHz: It is noted that this frequency band is only used by limb sounding instruments.

‒	239.2-242.2 GHz and 244.2-247.2 GHz: these two frequency bands have been allocated to EESS (passive) by WRC-23 and sharing studies with EESS (passive) should be undertaken once RLS characteristics are known.

Above 275 GHz, there is currently no frequency allocation in the RR, but RR No. 5.565 identifies several frequency bands that are relevant and are already in use by EESS (passive). When considering sharing and compatibility studies in the range 275-700 GHz, the following EESS (passive) frequency bands are to be taken into account:

‒	275-286 GHz, 296-306 GHz, 313-356 GHz, 361-365 GHz, 369-392 GHz, 397-399 GHz, 409-411 GHz, 416-434 GHz, 439-467 GHz, 477-502 GHz, 523-527 GHz, 538-581 GHz, 611-630 GHz, 634-654 GHz, 657-692 GHz.

Finally, the band 237.9-238 GHz is also allocated to EESS (active) through RR No. 5.563B, and its use is limited to spaceborne cloud radars.

## A7.2	Interference criteria

For EESS (passive), the interference criteria are given in Table 2 of Recommendation ITU-R RS.2017, as aggregate criteria for all sources of interference.

It should also be noted that this Recommendation does not yet include the protection criteria for the newly allocated bands 239.2-242.2 GHz and 244.2-247.2 GHz but interference criteria given in Report ITU-R RS.2535 should be used (i.e. the same protection criteria as specified in Recommendation ITU-R RS.2017 for the band 226-231.5 GHz).

The following Table summarises the protection criteria to be used for EESS (passive):

TABLE A7-1

| Frequency bands (GHz) | Reference bandwidth (MHz) | Maximum interference level (dBW) | Percentage of area or time permissible interference level may be exceeded(1) (%) |
| --- | --- | --- | --- |
| 226-231.5 239.2-242.2 244.2-247.2 296-306 | 200/3(2) | −160/−194(2) | 0.01/1(2) |
| 313.5-355.6 361.2-365 369.2-391.2 397.2-399.2 | 200/3(2) | −158/−194(2) | 0.01/1(2) |
| 416-433.46 439.1-466.3 | 200/3(2) | −157/−194(2) | 0.01/1(2) |
| 523-527 | 200 | −156 | 0.01 |
| 497-502 538-581 | 200/3(2) | −156/−194(2) | 0.01/1(2) |
| 634-654 656.9-692 | 200/3(2) | −155/−194(2) | 0.01/1(2) |
| 235-238 250-252 275-285.4 409-411 477.75-496.75 611.7-629.7 | 3 | −194 | 1 |
| (1) For a 0.01% level, the measurement area is a square on the Earth of 2 000 000 km2, unless otherwise justified; for a 1% level, the measurement time is 24 h, unless otherwise justified. (2) First number for nadir or conical scanning modes and second number for microwave limb sounding applications |  |  |  |

The protection criteria for EESS (active) in the band 237.9-238 GHz is given in Recommendation ITU-R RS.1166 as follows:

TABLE A7-2

| I/N (dB) | Data availability criteria |  |
| --- | --- | --- |
| –10 | 99 % (systematic) | 95 % (random) |

## A7.3	Technical and operational characteristics

### A7.3.1	EESS (passive) below 275 GHz

The 231-275 GHz frequency band is essential for the measurement of atmospheric trace gases using passive remote sensing satellite limb sounders. Specifically, at the 240 GHz part of the spectrum, data products include measurements of carbon monoxide, nitric acid in the upper and lower stratosphere, cloud ice water content (IWC), ice water path (IWP), ozone, volcanic sulfur dioxide, and improve the quality of upper tropospheric water vapor measurements.

The technical and operational characteristics of EESS (passive) systems in the range 231.5-275 GHz are given in section 6.20 of Recommendation ITU-R RS.1861, as follows:

TABLE A7-3

EESS (passive) sensor characteristics operating between 226 and 252 GHz

|  | Sensor T1 | Sensor T2 |
| --- | --- | --- |
| Sensor type | Conical scan | Limb sounder |
| Orbit parameters |  |  |
| Altitude (km) | 830 | 705 |
| Inclination (degree) | 98.7 | 98.2 |
| Eccentricity | 0.001 | 0 |
| Repeat period (days) | 29 | 16 |
| Sensor antenna parameters |  |  |
| Number of beams | 1 | 1 |
| Antenna size (m) | 0.255 | 1.6 (V) × 0.8 (H) |
| Maximum beam gain (dBi) | 52 | 67.5 |
| Polarization | V and H | H |
| −3 dB beamwidth (degree) | 0.5 | 0.060 × 0.123 |
| Instantaneous field of view (km) | 11 × 18 (155 km2) | 3.2 × 6.4 |
| Off-nadir pointing angle (degree) | 44.7 | N/A |
| Incidence angle at Earth (degree) | 52.7 | N/A |
| Swath width (km) | 1 700 | N/A |
| Antenna efficiency | 0.64 | 0.69 |
| Beam dynamics | 45 rpm (1.33 s) | Scans continuously in tangent height from the surface to ~92 km in 24.7 s, 240 scans/orbit |
| Sensor antenna pattern | Rec. ITU-R RS.1813 | Rec. ITU-R RS.1813 with minor mods (see NOTE) |
| Cold calibration ant. gain (dBi) | 47 | N/A |
| Cold calibration angle (degrees re. satellite track) | 130° to 135° | N/A |
| Cold calibration angle (degrees re. nadir direction) | 90° | N/A |
| Sensor receiver parameters |  |  |
| Sensor integration time | 2 to 3 ms | 0.166 s |
| Channel bandwidth | See Table A7-5 | See Table A7-6 |
| Measurement spatial resolution |  |  |
| Horizontal resolution (km) |  | 6.4 |
| Vertical resolution (km) |  | 3.2 |
| NOTE – The antenna model from Recommendation ITU-R RS.1813-1 can be adjusted to support elliptical reflectors with the following modifications: • The maximum antenna gain be defined as: Gmax=10log10ηπ2DmaxDminλ2. • The antenna diameter be defined as: D(α)=Dmax2cos2α+ Dmin2sin2α. Therefore, the antenna diameter becomes a function of the angle (α ϵ [0°, 90°]) in the plane that is perpendicular to the antenna boresight vector and between the intended direction of emission and the antenna beam’s major axis. • The existing functions for G(φ) and φm should be evaluated for each point in the alpha/phi space. |  |  |

TABLE A7-4

EESS (passive) sensor characteristics operating between 226 and 252 GHz

|  | Sensor T3 (MWS) |
| --- | --- |
| Sensor type | Nadir Scan |
| Orbit parameters |  |
| Altitude (km) | 830 |
| Inclination (degree) | 98.7 |
| Eccentricity | 0.001 |
| Repeat period (days) | 29 |
| Sensor antenna parameters |  |
| Number of beams | 1 |
| Antenna size (m) | 0.35 |
| Maximum beam gain (dBi) | 43 |
| Polarization | QV |
| −3 dB beamwidth (degree) | 1.15° |
| Instantaneous field of view (km) | Nadir FOV: 17 (218 km2) Outer FOV: 55 × 28 (1 225 km2) |
| Off-nadir pointing angle (degree) | ±49.31° cross-track |
| Incidence angle at Earth (degree) | 0 (nadir) 58.9 |
| Swath width (km) | 2 220 |
| Antenna efficiency | 0.60 |
| Beam dynamics (s) | 2.254 |
| Sensor antenna pattern | Rec. ITU-R RS.1813 |
| Cold calibration ant. gain (dBi) |  |
| Cold calibration angle (degrees re. satellite track) | 90° |
| Cold calibration angle (degrees re. nadir direction) | 78° to 83° |
| Number of beams |  |
| Sensor receiver parameters |  |
| Sensor integration time (ms) | 13.7 |
| Channel bandwidth | 2 000 MHz centred at 229 GHz |
| Measurement spatial resolution |  |
| Horizontal resolution | 17 km (nadir) |
| Vertical resolution | 17 km (nadir) |

TABLE A7-5

Sensor T1 passive sensor characteristics for channels between 239 and 248 GHz

| Centre frequency (GHz) (see NOTE below) | Frequency range (GHz) | Channel bandwidth (MHz) |
| --- | --- | --- |
| 243.2 ± 2.5 | 239.2-242.2 244.2-247.2 | 2 × 3 000 |
| NOTE – The T1 instrument has also multiple channels in bands above 275 GHz (three channels around 325 GHz, three channels around 448 GHz and one channel at 664 GHz). |  |  |

TABLE A7-6

Sensor T2 passive sensor characteristics for channels between 231 and 248 GHz

| Centre frequency (GHz) | Channel bandwidth (MHz) |
| --- | --- |
| 231.86 | 500 |
| 232.46 | 500 |
| 233.9515 | 1 250 |
| 234.86 | 500 |
| 235.7151 | 10 |
| 235.7151 | 1 250 |
| 236.66 | 500 |
| 242.66 | 500 |
| 244.46 | 500 |
| 246.86 | 500 |
| 247.46 | 500 |

Unless otherwise stated, antenna patterns for EESS (passive) are provided in Recommendation ITU-R RS.1813.

### A7.3.2	Characteristics of EESS (passive) sensors above 275 GHz.

The 275-700 GHz frequency band is essential for the measurement of atmospheric trace gases using passive remote sensing satellite limb sounders. Specifically, at the 640 GHz band data products include measurements of bromine monoxide, methyl chloride, methyl cyanide, methanol, chlorine monoxide, hydrogen chloride, hydroperoxyl, hypochlorous acid, volcanic sulfur dioxide, methyl cyanide, nitric acid, cloud ice water content (IWC), ice water path (IWP), nitrous oxide, ozone, and improve the quality of upper tropospheric water vapor measurements.

The technical and operational characteristics of EESS (passive) systems in the range 275-450 GHz are given in Report ITU-R RS.2431, as follows:

TABLE A7-7

| Instrument | ICI | TWICE | SMM | STEAMR | GOMAS |
| --- | --- | --- | --- | --- | --- |
| Type of Orbit | SSO LEO | SSO LEO | SSO LEO | SSO LEO | GSO |
| Altitude (km) | 817 | 400 | not available | 817 | 35 684 |
| Inclination (degrees) | 98.7 | High inclination | High inclination | 98.7 | 0 |
| Scanning mode | Conical | Conical | Conical or cross track (Fig. ee-1) | Limb (Fig. ee-2) | Conical |
| Observation Zenith Angle (OZA) (degrees) for conical scan, or Min. pointing altitude (km), for limb scan | Conical: 53 ± 2 | Conical: 53 | not available | Limb: 6 | not available |
| RF Centre Frequency (GHz) | 325.15 448 | 310 380.2 | 325 | 319.5 349.6 | 380.197 424.763 |
| RF Bandwidth (GHz) | 3.2 – 6 2.4 – 6 (Table A7-9) | 10 7.2 | not available | 12 12 | 0.3 – 4 0.06 – 1 (Table A7-10) |
| Antenna type | Offset reflector, multiple feeds | Broadband multi-flare horns | not available | Reflector antenna | Filled aperture scanning |
| Antenna Peak Gain (dBi) | 55 | 46-48 (TBC) | not available | 70 | not available |
| Antenna Diameter (m) | ~ 0.5 | not available | not available | not available | 3 |
| Antenna Beamwidth (degrees) | not available | 0.64o 0.56o | not available | See Fig. ee-2 | 0.019o 0.017o |
| FOV (km) Footprint area (km2) | 16 Area ≈ 200 km2 (Table 3) | FOV: 6.5 × 9.9 Area ≈ 50 km2 FOV: 5.8 × 8.7 Area ≈ 40 km2 (Fig. 6.2-2) | not available | N/A (See Fig. 15) | IFOV: 12 Area ≈ 110 km2 IFOV: 10 Area ≈ 75 km2 |

TABLE A7-8

| Instrument | GEM | CAMLS | MASTER | GMS |
| --- | --- | --- | --- | --- |
| Type of Orbit | GSO | LEO | SSO LEO | GSO |
| Altitude (km) | 35 684 | not available | 817 | 35 684 |
| Inclination (degrees) | 0 | not available | 98.7 | 0 |
| Scanning mode | Conical | Limb | Limb | wide strip and thin circle combined scan (Fig. ee-3) |
| Observation Zenith Angle (OZA) (degrees) for conical scan, or Min. pointing altitude (km), for limb scan | not available | Limb: 10 | Limb: 3 | N/A |
| RF Centre Frequency (GHz) | 380.197 425.763 | 340 | 299.75 320.0 345.6 | 338 380.197 424.763 |
| RF Bandwidth (GHz) | 0.05-18 (LSB) | 16 | 11.5 9.0 6.5 | 0.03-8 0.01-1 |
| Antenna type | Filled aperture scanning | not available | Elliptical Offset reflector | Reflector Antenna |
| Antenna Peak Gain (dBi) | not available | not available | not available | 76 |
| Antenna Diameter (m) | 2 | not available | 1 × 2 | 3 |
| Antenna Beamwidth (degrees) | 0.029o 0.026o | not available | not available | 0.027° |
| FOV (km) Footprint area (km2) | FOV: 20.5 Area ≈ 330 km2 FOV: 16.4 Area ≈ 210 km2 | N/A (See Table 13) | N/A (See Table 17) | IFOV: 16 |

TABLE A7-9

Channel specifications for Remote Sensor-1 (ICI)

| Channel No | Frequency fo±Δf (GHz) (*) | Bandwidth (MHz) (**) | Polarization | Utilization |
| --- | --- | --- | --- | --- |
| ICI-1 | 183.31±7.0 | 2 × 2 000 | V | Water vapour profile and snowfall |
| ICI-2 | 183.31±3.4 | 2 × 1 500 | V |  |
| ICI-3 | 183.31±2.0 | 2 × 1 500 | V |  |
| ICI-4 | 243.2±2.5 | 2 × 3 000 | V, H | Quasi-window, cloud ice retrieval, cirrus clouds |
| ICI-5 | 325.15±9.5 | 2 × 3 000 | V | Cloud ice effective radius |
| ICI-6 | 325.15±3.5 | 2 × 2 400 | V |  |
| ICI-7 | 325.15±1.5 | 2 × 1 600 | V |  |
| ICI-8 | 448±7.2 | 2 × 3 000 | V | Cloud ice water path and cirrus |
| ICI-9 | 448±3.0 | 2 × 2 000 | V |  |
| ICI-10 | 448±1.4 | 2 × 1 200 | V |  |
| ICI-11 | 664±4.2 | 2 × 5 000 | V, H | Quasi-window, cirrus clouds, cloud ice water path |
| (*) For each ICI channel, measurements are performed at both sides of the centre frequency (fo + Δf and fo - Δf) (**) Total bandwidth per channel, considering both sides around the centre frequency fo. |  |  |  |  |

TABLE A7-10

Channel specifications for Remote Sensor-6 (GOMAS) within the 275-450 GHz range

| Channel No. | Frequency (GHz) | Bandwidth (GHz) | Utilization |
| --- | --- | --- | --- |
| GOMAS-1 | 380.17 ± 0.3 | 0.3 | Water |
| GOMAS-2 | 380.17 ± 0.9 | 0.5 |  |
| GOMAS-3 | 380.17 ± 1.65 | 0.7 |  |
| GOMAS-4 | 380.17 ± 3 | 1 |  |
| GOMAS-5 | 380.17 ± 5 | 2 |  |
| GOMAS-6 | 380.17 ± 7 | 2 |  |
| GOMAS-7 | 380.17 ± 17 | 4 |  |
| GOMAS-8 | 424.76 ± 0.15 | 0.06 | Oxygen |
| GOMAS-9 | 424.76 ± 0.3 | 0.1 |  |
| GOMAS-10 | 424.76 ± 0.6 | 0.2 |  |
| GOMAS-11 | 424.76 ± 1 | 0.4 |  |
| GOMAS-12 | 424.76 ± 1.5 | 0.6 |  |
| GOMAS-13 | 424.76 ± 4 | 1 |  |

FIGURE ee-A7-1

Illustration of the SMM receiver array concept

FIGURE A7-2

STEAMR limb imaging principles

FIGURE A7-3

Typical wide strip and thin circle combined scanning configuration

Additional characteristics of EESS (passive) systems in the range 275-700 GHz are provided below:

TABLE A7-11

| Satellite | Metop-SG-B series of 3 satellites |  |  | AWS-PFM (1 satellite) / EPS Sterna constellation (18 + 2 satellites) |
| --- | --- | --- | --- | --- |
| Instrument | ICI |  |  | EPS Sterna Radiometer |
| Type of orbit | SSO |  |  | SSO |
| Altitude (km) | 830 |  |  | 595 |
| Inclination (degree) | 98.7 |  |  | 97.79 |
| Scanning geometry | Conical |  |  | Nadir (cross-track) |
| Centre frequency (GHz) | 1) 325.15 ± 9.5 2) 325.15 ± 3.5 3) 325.15 ± 1.5 | 1) 448 ± 7.2 2) 448 ± 3.0 3) 448 ± 1.4 | 664 ± 4.2 | 1) 325.15 ± 6.6 2) 325.15 ± 4.1 3) 325.15 ± 2.4 4) 325.15 ± 1.2 |
| Channel bandwidth (MHz) | 1) 2 × 3 000 2) 2 × 2 400 3) 2 × 1 600 | 1) 2 × 3 000 2) 2 × 2 400 3) 2 × 1 600 | 2 × 5 000 | 1) 2 × 2 800 2) 2 × 1 800 3) 2 × 1 200 4) 2 × 800 |
| Polarisation | H/V | H/V | H/V | QH/QV |
| Antenna Peak Gain (dBi) | 52 |  |  | 52 |
| Antenna 3 dB beamwidth | 0.5 |  |  | 0.8 |
| Footprint area / IFOV | 11 × 18 km (155 km2) |  |  | Nadir FOV: 8 km (54 km2) Outer FOV: 16 × 35 km (433 km2) |
| Off-Nadir angle | 44.7° |  |  | 54.4° |

TABLE A7-12

| Satellite | future non-GSO |
| --- | --- |
| Instrument | ice cloud detector |
| Type of orbit | SSO |
| Altitude (km) | 836 |
| Inclination (degree) | 98.75 |
| Scanning geometry | Conical |
| Centre frequency (GHz) | 664 ± 4.2 GHz |
| Channel bandwidth (MHz) | 2 × 5 000 |
| Polarisation | V/H |
| Antenna Peak Gain (dBi) | 54 |
| Antenna 3 dB beamwidth | 0.4°/0.6° |
| Footprint area / IFOV | Nadir FOV: 18 km2 |
| Off-Nadir angle | 53° |

Unless otherwise stated, antenna patterns for EESS (passive) are provided in Recommendation ITU-R RS.1813.

### A7.3.3	EESS (active) in the band 237.9-238 GHz

The characteristics of EESS (active) systems in the band 237.9-238 GHz are given in section 7.14 of Recommendation ITU-R RS.2105, as follows:

TABLE A7-13

Characteristics of EESS (active) missions in the 237.9-238 GHz band

| Parameter | CPR-N1 |
| --- | --- |
| Sensor type | Cloud profiling radar |
| Type of orbit | SSO |
| Altitude (km) | 705 |
| Orbital inclination (degrees) | 98.2 |
| Ascending node LST | 13:30 |
| Repeat period (days) | 16 |
| Antenna diameter (m) | 3 |
| Antenna (Transmit and Receive) peak gain (dBi) | 78 |
| Polarization | Linear |
| Azimuth scan rate (rpm) | 0 |
| Antenna beam look angle (degrees) | 0 |
| Antenna beam azimuth angle (degrees) | 0 |
| Antenna elevation beamwidth (degrees) | 0.024 |
| Antenna azimuth beamwidth (degrees) | 0.024 |
| RF centre frequency (GHz) | 237.95 |
| RF bandwidth (MHz) | 0.65 |
| Transmit Pk power (W) | 80 |
| Pulsewidth (μs) | 1.6 |
| Pulse repetition frequency (PRF) (Hz) | 4 000 |
| Range resolution (m) | 250 |
| Horizontal resolution | 0.1 × 0.7 km |
| System noise figure (dB) | 11 |

## A7.4	Sharing and compatibility analysis

### A7.4.1	Sharing and compatibility analysis of EESS (passive)

#### A7.4.1.1	STUDY A

##### A7.4.1.1.1 General Considerations

The study of sharing and compatibility of EESS (passive) sensors with regard to the proposed RLS radar systems is considered in several parts. This section covers general aspects and simulation methodology employed by A7.4. Section A7.4.1.1 describes reference Measurement Areas of Interest that the passive sensors and proposed radar system deployments for use in simulation studies. Section A7.4.2.1 covers scenarios involving frequency ranges below 275 GHz. This section includes allocated bands for EESS (passive) as well as RR No. 5.340 adjacency aspects. Section A7.4.2.2 covers scenarios involving frequency ranges above 275 GHz. This section includes consideration of frequency bands identified for use by EESS (passive) under RR No. 5.565.

##### A7.4.1.1.2 Simulation Methodology

Assessments of the aggregate RFI expected from the RLS radar systems (operating according to section 4.1 or 4.2) into EESS (passive) operating in the certain bands are achieved by dynamic simulations. The analysis is conducted in which the orbit of the EESS (passive) spacecraft under investigation is dynamically simulated, retaining only the data points when the EESS (passive) sensor antenna boresight points within a defined Measurement Area of Interest (MAI), as defined in section A7.2. Calculations are performed to determine the potential interference from each of the current active stations into the EESS (passive) sensors under study and will consider the aggregate effect from multiple active stations. The simulation will propagate the satellite based on its orbital parameters, and the simulation step size is selected to be an irrational number to ensure that the beam dynamics of the passive sensor are maximally representative and do not exhibit statistically reductive periodic behaviour. At each simulation step, a snapshot of the interference scenario is generated where the directional vectors from each active RLS radiative source to the EESS (passive) sensor is computed along with the apparent gain of the transmit and receive antennas using their respective antenna patterns. For the RLS systems the radiation patterns are modelled by section 4.1.3 and 4.2.3 for mobile/fixed radars and vehicular radars, respectively.

The interfering signal power level, Ii,n (W), received by a spaceborne sensor at the nth simulation step from the ith active station is calculated from:

Ii,n= PTX i,nGTX i,n GRX i,nLa i,n LFSPL i,n 	(A3-1)

where:

PTX i,n:	active station out of band transmitter power in the EESS (passive) band, accounting for frequency dependent rejection

GTX i,n:		active station antenna gain towards spaceborne sensor

GRX i,n:		spaceborne receive antenna gain towards terrestrial source

La i,n :		propagation losses in accordance with section 6.1

LFSPL i,n: 	Free Space Path Loss (if not already included in La i,n).

[Editor’s note: Working Parties 3J, 3K and 3M informed WP 5B that they intend to send an update regarding propagation advice. So this aspect may be refined in the methodology for EESS.]

The aggregate interference from radiolocation systems at the nth simulation step, AggIn (W), is calculated by the summation of the received interference from all active stations within line of sight of EESS (passive) satellite:

AggIn=iIi,n=iPTX i,n GTX i,n GRX i,nLa i,n LFSPL i,n 	(A3-2)

Thus, the aggregate interference can be represented in the logarithmic domain as:

AggIn|dB=10 log10i PTX i,n GTX i,n GRX i,nLa i,n LFSPL i,n  |dB	(A3-3)

Using the resulting data containing received interfering power levels, a CCDF curve is generated to assess interference received by the sensor undergoing observations over the MAI.

Since the occupied bandwidth of all radar systems is significantly larger than the reference bandwidth of the EESS(p) protection, for in-band assessments, it is assumed that emissions fully cover the reference bandwidth.

##### A7.4.1.1.3 Measurement Area of Interest (MAI) selection and RLS radar system deployments

MAI are chosen to represent typical EESS (passive) sensor targets that may include intersection with significant density of proposed RLS radar systems. Reference MAI will include one or more urban localities to explore the impact of deployment density categorization on the predicted interference environment. MAI A/B of figure A7.4.1.1-1 are 2 million square kilometers in area. MAI A includes the west coast of the United States of America and MAI B includes the eastern coast. MAI C in figure A7.4.1.1-1 has an area of 10 million square kilometers. All regions are commonly studied by EESS (passive) sensors.

[Editor’s Note: more MAI may be included. For instance, to examine cross border scenarios. The east coastal scenario (MAI B) mainly involves a domestic scenario for the purposes of demonstrating compatibility. A more detailed cross-border scenario such as in MAI C would be added in subsequent updates to this study.]

MAI A

39.0N 118W

MAI B

37.0N 79.8W

figure a7.4.1.1-1 Reference MAI A/B

FIGURE A7.4.1.1-2 REFERENCE MAI C

###### A7.4.1.1.3.1 Deployment and characterization of RLS radar systems

Deployment and operational characteristics are informed by the tables and description contained in sections 4.1 and 4.2. The deployment density entries in the tables of these sections require that categories are assigned to locations inside the MAI region. Therefore, methodology is presented below describing how categories are determined.

###### A7.4.1.1.3.2 Data sources for location-based categories

All systems in these sections consider deployment density values categorized by “urban”, “suburban”, and “rural” densities as specified in sections 4.1.1 and 4.1.2 for mobile/fixed radars and vehicular radars, respectively.

###### A7.4.1.1.4.3 Land cover-based Categorization

This is an alternative methodology to population-based categorization based on satellite imagery of ground cover that could be developed further as needed.

###### A7.4.1.1.4.4 Gridded Population-based Categorization

The Gridded Population of the World (GPW) dataset, developed by NASA's Socioeconomic Data and Applications Center (SEDAC), offers a globally consistent and spatially explicit representation of human population distribution. Now in its fourth version (GPWv4), this dataset provides population counts and densities at a 30 arc-second (~1 km at the equator) resolution for the years 2000, 2005, 2010, 2015, and 2020. The data are derived from approximately 13.5 million national and sub-national administrative units, ensuring high-resolution mapping of population distribution. This granularity makes GPWv4 an invaluable resource for simulations requiring detailed demographic inputs, such as urban planning, disaster response, and environmental impact assessments. By integrating GPWv4 into simulation models, researchers can accurately assess human exposure to various risks, optimize resource allocation, and inform policy decisions based on precise population metrics. See: https://www.earthdata.nasa.gov/data/projects/gpw

The categorization of urban/suburban/rural is computed from the density reported by GPW-4 extrapolated to 2020, and utilizing density ranges of:

Urban >1158 persons per square km

Suburban 386 < density <= 1158 persons per square km

Rural 0 < density <= 386 persons per square km.

This set of definitions derives from U.S. Department of Health and Human Services 42 CFR Parts 403 and 408, Federal Register /Vol. 68, No. 240 /Monday, December 15, 2003 /Rules and Regulations page 69851. See: https://www.cms.gov/Regulations-and-Guidance/Regulations-and-Policies/QuarterlyProviderUpdates/downloads/cms4063ifc.pdf

figure a7.4.1-X: Population Categories MAI East

Note: State borders are drawn in black. Urban is drawn red, suburban yellow, rural green. There are also regions without data which are also drawn in black. The MAI boundary is drawn in blue.

##### A7.4.1.4 RLS radar system characteristics according to section 4.1 (mobile + fixed systems)

###### A7.4.1.4.1 	Modelling mobile radar systems

The mobile radar systems described in section 4.1 are active inside and outside buildings with mixed use.

figure a7.4.1-X mobile radar System (Radar 1)

Note: This antenna pattern normalizes over all angles to average gain <=0 dBi.

###### A7.4.1.4.2 	Modelling fixed radar systems

The fixed radar systems described in section 4.1 are active inside and outside buildings with mixed use. These systems are notable in that they have down tilt attitude.

##### A7.4.1.5 RLS vehicular radar system characteristics according to section 4.2 (short range + parking systems)

###### A7.4.1.5.1 Modelling short range radar systems

The short-range vehicular radar systems described in section 4.2 have an added aspect associating their population on or near roadways and parking facilities. These systems are considered outdoor use with respect to estimating blocking losses from buildings, or other ground cover.

Short-distance systems were simulated as fixed positions with uniform spacing along roadways according to an inference between population category shared by the centre of the roadway combined with spatially nearby roadways to ensure that overall vehicle density (either car or truck) conforms with deployment density values in the tables of 4.2.

The source of the roadway spatial data was the North American Roads (NTAD) geospatial dataset. This source provides a digital single-line representation of major roads and highways for Canada, the United States, and Mexico. The North American Roads highway network has a number of intended uses including building national and regional-level maps where major highways and arterials are an important feature, national and regional transport corridor planning, national/regional traffic analyses including the routing of freight and passenger traffic flows within and between countries, and traffic simulations based on various disruption/diversion scenarios. The specific dataset used was published 27 October 2020. See: https://geodata.bts.gov/datasets/usdot::north-american-roads/about.

The assignment of category to roadways is complicated by the fact that some routes intersect multiple categories due to their location/length. At this time, assignment is sampled by the mean spatial location of the route. There is also a complication relating the implementation of radar deployment in adherence to the densities in the tables of 4.2. The road data include roadways with single and multi-lane routes. Moreover, the density of independent routes is highly location specific. As such, uniform spacing of vehicles on-route is selected as a simple metric for deployment. The determination of vehicle (and radar cluster) spacing is achieved by sampling 1 square kilometre area in representative land categories and adjusting the uniform spacing parameter to match the respective deployment densities per square kilometre indicated by the tables of 4.2.

Below are the representative spacing values:

Urban >1158 : 5 m,

Suburban 386 < density <= 1158 : 15 m,

Rural 0 < density <= 386 : 35 m.

For the purpose of simulation the former linear spacing between cars is used. An alternative approach is randomize allocation of systems within a spatial window and constrain deployment to roadways. So the roadways are populated with a budget of transmitters determined by the land category for the window. This procedure is repeated to cover the MAI region. The density of systems per square kilometer should be retained regardless of deployment methodology.

Time-dependent variations in spacing are expected, however, nominal values are used in simulations at this point.

figure a7.4.1-X: short-range radar (Radar X)

Note: This antenna pattern normalizes over all angles to average gain >0 dBi.

figure a7.4.1-X

figure a7.4.1-X

Note: All roadways are merged regardless of type (interstate, state, municipal, or local) and drawn in purple. State borders are drawn in green. The red dot is the MAI centroid.

###### A7.4.1.5.2 	Modelling parking radar systems

The radar systems described in section 4.2 are expected to be active in segregated regions of the roadways and building infrastructure.

#### A7.4.1.6 Sharing and compatibility analysis EESS (passive) below 275 GHz

[Editor’s Note: This study is based on the parameters available after the May 2025 Meeting of WP 5B]

The following results show observed co-frequency receive power from vehicular deployment assuming a co-channel use at 244.6 GHz. The orbiting sensor type consists of T1 type from ITU-R RS.1861, with conical scanning mode. Figures A7.4.1-X shows local aggregation of mobile short range elements.

figure a7.4.1-X

The following results show observed co-frequency receive power from fixed deployment assuming a co-channel use at 244.6 GHz. The orbiting sensor type consists of T1 type from ITU-R RS.1861, with conical scanning mode. Figures A7.4.1-X shows local aggregation of scanning (Radar A) and focused (Radar B) elements.

figure a7.4.1-X

These results are based on the application of the full EESS protection criterion for each of the radar categories.

figure a7.4.1-X

##### A7.4.1.7 Summary of results for study A

These calculations are addressing the adjacent band 250-252 GHz case, for the following radars: Radar A (scanning), Radar B (tracking), Vehicular radar X (short-range), and Vehicular radar Y (parking operation)

Study A shows that radars A and B (in aggregate) are maintaining compatibility with EESS (passive) with safe margin given the characteristics and deployment densities studied.

For vehicular radars X and Y this is assessed to be more potentially problematic with threshold power limit of [XX dBW/MHz] per unit radar to ensure compatibility with EESS (passive).

#### A7.4.1.8 Sharing and compatibility analysis EESS (passive) above 275 GHz

TBD

#### A7.4.1.2	STUDY B

##### A7.4.1.2.1	Maximum interference at the ground in the EESS footprint

The starting point of study B is to determine the “Maximum interference at the ground in the EESS footprint” that will have to be complied with the aggregation of all sources of interference.

Such a calculation takes into account the relevant atmospheric attenuation (from Rec. ITU-R P.676) and the following EESS (passive) parameters:

–	Center frequency (GHz)

–	Bandwidth (MHz)

–	Orbit altitude (km)

–	Nadir Angle (°)

–	Protection criteria (dBW in reference bandwidth).

At current stage, calculations are only performed to address systems described in Table A7-11 above, i.e. ICI on METOP-SG and AWS.

[Editor’s Note: Calculations for other sensors may be provided at a later stage]

These calculations consider the various measurement channels (at different frequencies) of these instruments with the corresponding free space and atmospheric attenuation (average over the bandwidth).

Table A7-12

Calculations for ICI

Table A7-13

Calculations for AWS

A more generic calculation is also provided below for a hypothetical sensor operating at 296 GHz.

Table A7-14

Calculations for Generic system at 296 GHz

##### A7.4.1.2.2	Compatibility analysis

[Editor’s Note: Since imaging systems around 340 GHz are not fully characterized in the current Report, these systems are not addressed within the present analysis.]

The initial calculations are limited to the adjacent band 296-356 GHz case, for the following radars:

Table A7-15

TBD

| Radar type | Section of the WP 5B Chair’s Report (Document 5B/315 - Annex 2.9) |
| --- | --- |
| Radar A (scanning) | Section 4.1.2 (Table 1) |
| Radar B (tracking) | Section 4.1.2 (Table 1) |
| Vehicular radar X (short range) | Section 4.2.2.1 (Table 1) |
| Vehicular radar Y (parking) | Section 4.2.2.1 (Table 1) |

From the previous section, the most relevant elements providing the “maximum interference at the ground in the EESS footprint” for the ICI, AWS and a “296 GHz generic” instruments are reproduced below.

Table A7-16

TBD

The calculations also requires to assess the total emissions of interferers present within the EESS (passive) sensor footprint, from possible future Radiolocation service (as well as other existing services (e.g. fixed and mobile services), if appropriate).

A way to consider the potential aggregation of all possible interferers could be to address the various RLS applications separately to which is given a portion of the maximum allowable interference level available; A decrease by 12 dB is proposed.

###### A7.4.1.2.2.1	Calculations for Radars A and B

Frequency band used by radars A and B = 252-296 GHz

Frequency band used by sensors

–	Generic sensor= 296.5 GHz

–	ICI channel 1 = 315.65 GHz

–	AWS channel 2 = 318.55 GHz.

Figure A7-1

TBD

Considering their characteristics and the way to assess interference to EESS (passive), it should be noted that the calculations for radars A or B are similar, given in the Table below:

Table A7-17

TBD

Note : the Indoor/outdoor ratio used in the above Table is calculated using the BEL model from Recommmendation ITU-R P.2109 at 35° elevation, with a combination of 70% “traditional” and 30% “thermally efficient” buildings. Being used for the aggregation of the interfering power of high number of interferers, this value corresponds, in dBs, to the average of the linear values of the BEL distribution. It should be stressed that since BEL is an attenuation, the linear values are to be derived from the dB values of the distribution taken in negative terms.

Explanations for some parameters above:

–	Duty cycle is considered but not the activity factor since it applies over 1 day and cannot hence be applied to a short-term interference.

–	The antenna gain relative to EESS of 5 dBi is assumed.

–	The indoor/outdoor losses are an average over indoor transmitters. It is also assumed.

–	The overall indoor-outdoor factor is the average over all radars, 90% of them with 20 dB attenuation and the other without. It is calculated.

It can here be seen that the calculation shows that there is a 6.8 dB exceeding of the maximum interference for the generic sensor within Urban areas.

[Editor’s note : Additional elements will be needed to address the following issues:

–	Clarify the notion of activity factor, in conjunction with the number of devices per km².

–	Consider whether it has a daily basis or whether it is more concentrated on certain period of the day

–	Provides additional calculations related to the activity factor

–	Application of clutter loss, where applicable]

###### A7.4.1.2.2.1	Calculations for vehicular radars X and Y

Frequency band used by radars X and Y = 252-296 GHz

Frequency band used by sensors

–	Generic sensor = 296.5 GHz

–	ICI channel 1 = 315.65 GHz

–	AWS channel 2 = 318.55 GHz

Figure A7-2

TBD

a)	Specific antenna system for automotive radars

It is understood that the antenna pattern in the current Report is an envelope and does not represent the single antenna pattern.

Additional information tends to show that each RLS antenna would be using 4 × 4, 8 × 8 or 16 × 16 elements with element gain of 10 dBi, hence leading to maximum antenna gain of 22 dBi, 28 dBi and 34 dBi, respectively.

One may assume that the radar pattern is described as a beamformed antenna based on M.2101. This is however complex to model in such a case, and, as a simple initial approach, it is proposed to model it using F.699. The figure below compares both pattern for a 16 × 16 antenna system and a 34 dBi antenna.

Figure A7-4

TBD

In addition, and more important, each car is expected to embark multiple radars (6 to 8), pointing in different direction. It seems more than likely that not all radars are pointing in the direction of an EESS (passive) sensor. A specific analysis of the “aggregate gain” toward EESS (passive) sensor is required.

The analysis consists of calculating the composite radar antenna gain in the direction of the EESS (sensor), pointing at 35° elevation, combining the relative gain of the 8 radars per car.

For simplification, each radar is assumed to cover a 45° angle azimuth and point randomly within excursion ranges −22.5 to 22.5° in azimuth and of 0 to 45° in elevation.

Figure A7-5

TBD

Subsequently, the calculation is the following:

−	For each radar, the pointing azimuth and elevation if chosen randomly within the corresponding range.

−	The discrimination angle with the EESS (passive) sensor pointing is calculated.

−	Allowing to calculate the relative gain of the radar in the direction of the EESS (passive) sensor.

−	The aggregate gain of the 8 radars in the direction of the EESS (passive) sensor is then calculated.

−	To obtain a representative distribution of the aggregate gain, the single calculation is repeated 10 000 times.

The figure below depicts the distribution of the composite antenna gain in the direction of the EESS (passive) sensor considering antenna system using 4 × 4 (22 dBi), 8 × 8 (28 dBi) and 16 × 16 (34 dBi).

Figure A7-6

TBD

These distributions corresponds of the composite gain of the 8 radars in the direction of the EESS, meaning that, to calculate the interference from a large population of cars (with random orientation) within the EESS (passive) footprint, the average gain of the distribution can be used.

It leads to the average gains as in table below:

Table A7-18

TBD

| Max radar antenna gain | Average gain over 10 000 samples |
| --- | --- |
| 4 × 4 (22 dBi) | 16.4 dBi |
| 8 × 8 (28 dBi) | 16.2 dBi |
| 16 × 16 (34 dBi) | 15.9 dBi |

It is interesting to note that, whatever is the maximum antenna gain, the average antenna gain over 10 000 samples is pretty consistent. It is hence proposed to retain a value of 16 dBi average radar gain.

b)	Calculations

Table A7-19

TBD

Explanations for some parameters above:

–	Duty cycle is considered but not the activity factor since it applies over 1 day and cannot hence be applied to a short-term interference.

–	The antenna gain relative to EESS of 16 dBi is calculated above.

–	The number of radars per vehicle and density of cars are given parameters.

It can here be seen that the calculation shows that there is an important exceeding (between 29.8 and 52 dB) of the maximum interference for all scenarios of EESS (passive) systems and geographical areas.

Compared with the calculation for radars A and B, the following parameters are mainly driving the large difference:

–	Maximum power density of unwanted emissions = –30 dBm/MHz versus –20 dBm/MHz.

–	Antenna gain relative to EESS = 5 dBi versus 16 dBi.

–	Indoor usage of 90 % deployment for radars A and B and only outdoor for vehicular radars.

From an EESS (passive) prospective, unwanted emissions limits given in 1 MHz is not relevant, and it is better to express it within larger bandwidth. This, in particular, would provide more flexibility to the RLS design.

The table below provides the possible a RLS maximum unwanted emission expressed in dBW/200 MHz or 1 GHz:

Table A7-20

TBD

|  | Radar X |  | Radar Y |  |
| --- | --- | --- | --- | --- |
|  | Max | Min | Max | Min |
| in dBW/200 MHz | −67.2 | −81.9 | −66 | −80.7 |
| in dBW/1 GHz | −60.2 | −74.9 | −59 | −73.7 |

[Editor’s note : Additional elements will be needed to address the following issues:

–	How the daily activity factor is distributed over the day and what could be its values during the potential periods of higher traffic.

–	Consider that car radars may not all be implemented in these frequency band, and that hence a band factor may need to further included. This may also help in drawing conclusions in term of possible maximum penetration ratio of radars in the concerned bands.

–	Provides additional calculations related to the activity factor and band factor.

–	Study how the above possible unwanted emission limits could constrain or be applied by vehicular radars

–	Application of clutter loss, where applicable]

###### A7.4.1.2.2.1	Conclusions for study B

These calculations are addressing the adjacent band 296-356 GHz case, for the following radars:

Table A7-21

TBD

Radar type

Radar A (scanning)

Radar B (tracking)

Vehicular radar X (short range)

Vehicular radar Y (parking)

They show that radars A and B are pretty close to compatibility with EESS (passive).

The situation is different for vehicular radars X and Y for which the limits could be proposed within the following ranges:

Table A7-22

TBD

|  | Radar X |  | Radar Y |  |
| --- | --- | --- | --- | --- |
|  | Max | Min | Max | Min |
| in dBW/200 MHz | −67.2 | −81.9 | −66 | −80.7 |
| in dBW/1 GHz | −60.2 | −74.9 | −59 | −73.7 |

### A7.4.2	Sharing and compatibility analysis of EESS (active)

#### A.7.4.2.1	Sharing and compatibility analysis of EESS (active) in the band 237.9-238 GHz

General considerations

TBD

Simulation methodology

Assessments of the aggregate RFI expected from the RLS radar systems (operating according to section 4.1 or 4.1) into EESS (active) operating in the certain bands are achieved by dynamic simulations. The analysis is conducted in which the orbit of the EESS (active) spacecraft under investigation is dynamically simulated, retaining only the data points when the EESS (active) sensor antenna boresight points within the intended service area. Calculations are performed to determine the potential interference from each of the current active stations into the EESS (active) sensors under study and will consider the aggregate effect from multiple active stations. The simulation will propagate the satellite based on its orbital parameters, and the simulation step size is selected to be an irrational number to ensure that the beam dynamics of the active sensor are maximally representative and do not exhibit statistically reductive periodic behaviour. At each simulation step, a snapshot of the interference scenario is generated where the directional vectors from each active RLS radiative source to the EESS (active) sensor is computed along with the apparent gain of the transmit and receive antennas using their respective antenna patterns. For the RLS systems the radiation patterns are modelled by section 4.1.3 and 4.2.3 for mobile/fixed radars and vehicular radars, respectively.

The interfering signal power level, Ii,n (W), received by a spaceborne sensor at the nth simulation step from the ith active station is calculated from:

Ii,n= PTX i,nGTX i,n GRX i,nLa i,n LFSPL i,n 	(A3-1)

where:

PTX i,n:	active station out of band transmitter power in the EESS (active) band, accounting for frequency dependent rejection

GTX i,n:		active station antenna gain towards spaceborne sensor

GRX i,n:		spaceborne receive antenna gain towards terrestrial source

La i,n :		propagation losses in accordance with section 6.1

LFSPL i,n: 	Free Space Path Loss (if not already included in La i,n).

[Editor’s note: Working Parties 3J, 3K and 3M informed WP 5B that they intend to send an update regarding propagation advice. So this aspect may be refined in the methodology for EESS.]

The aggregate interference from radiolocation systems at the nth simulation step, AggIn (W), is calculated by the summation of the received interference from all active stations within line of sight of EESS (active) satellite:

AggIn=iIi,n=iPTX i,n GTX i,n GRX i,nLa i,n LFSPL i,n 	(A3-2)

Thus, the aggregate interference can be represented in the logarithmic domain as:

AggIn|dB=10 log10i PTX i,n GTX i,n GRX i,nLa i,n LFSPL i,n  |dB	(A3-3)

Using the resulting data containing received interfering power levels, a CCDF curve is generated to assess interference received by the sensor undergoing observations over the service area.

Since the occupied bandwidth of all radar systems is significantly larger than the reference bandwidth of the EESS(p) protection, for in-band assessments, it is assumed that emissions fully cover the reference bandwidth.

##### Deployment and characterization of RLS radar systems

The deployment and characterization of RLS radar system types will adopt those described in section A7.4.1.2.

Annex 8

# A8 	Characteristics and technical analysis related to radioastronomy service systems

Working Party (WP) 7D thanks WP 5B for Document 7D/60 and is pleased to provide relevant technical information pertaining to the radio astronomy service to support studies under WRC-27 agenda item 1.8 concerning possible additional spectrum allocations to the radiolocation service on a primary basis in the frequency range 231.5-275 GHz and possible new identifications for radiolocation service applications in the frequency bands within the frequency range 275-700 GHz for millimetric and sub millimetric wave imaging systems, in accordance with Resolution 663 (Rev.WRC-23).

Radio astronomy observations have been conducted for many years at frequencies above 200 GHz and, unlike the situation for active services, operating conditions are well understood. As noted in Reports ITU-R RA.2189, ITU-R RA.2512 and ITU-R SM.2450, observations above 200 GHz are conducted mostly at high altitude sites possessing exceptional atmospheric transparency, where propagation conditions of dry air and low atmospheric pressure are appropriate, which should be taken into account when performing sharing and compatibility studies with the radio astronomy service.

Radio astronomy has primary allocations in the frequency bands 200-231.5 GHz, 241-248 GHz and 250-275 GHz. Frequency bands above 275 GHz are identified for use by radio astronomy in RR No. 5.565. All emissions are prohibited in the frequency bands 200-209 GHz, 226-231.5 GHz and 250-252 GHz according to RR No. 5.340.

Working Party 7D would like to bring to the attention of WP 5B that the radio astronomy systems are listed and described in the Master International Frequency Register together with the status of allocation and conditions/remarks thereto and published in BR IFIC Space.

The spectral lines of carbon monoxide (CO) at 330.588 GHz, 345.777 GHz, 440.765 GHz, 461.041 GHz, 661.067 GHz and 691.473 GHz are of exceptional importance to radio astronomy.

## A8.1	Recommendations and Reports

| Recommendation ITU-R RA.314 | “Preferred frequency bands for radio astronomical measurements below 1 THz” |
| --- | --- |
| Recommendation ITU-R RA.517 | “Protection of the radio astronomy service from transmitters operating in adjacent bands” |
| Recommendation ITU-R RA.611 | “Protection of the radio astronomy service from spurious emissions” |
| Recommendation ITU-R RA.769 | “Protection criteria used for radio astronomical measurements” |
| Recommendation ITU-R RA.1031 | “Protection of the radio astronomy service in frequency bands shared with active services” |
| Recommendation ITU-R RA.1272 | “Protection of radio astronomy measurements above 60 GHz from ground based interference” |
| Recommendation ITU-R RA.1513 | “Levels of data loss to radio astronomy observations and percentage-of-time criteria resulting from degradation by interference for frequency bands allocated to the radio astronomy service on a primary basis” |
| Report ITU-R RA.2126 | “Techniques for mitigation of radio frequency interference in radio astronomy” |
| Report ITU-R RA.2131 | “Supplementary information on the detrimental threshold levels of interference to radio astronomy observations in Recommendation ITU-R RA.769” |
| Report ITU-R RA.2188 | “Power flux-density and e.i.r.p. levels potentially damaging to radio astronomy receivers” |
| Report ITU-R RA.2189 | “Sharing between the radio astronomy service and active services in the frequency range 275-3 000 GHz” Updated versions of Tables 5 and 6 of Report ITU-R RA.2189 are given in Annex 3 here. |
| Report ITU-R RA.2508 | “Widely-distributed radio astronomy array systems operating above 200 GHz” |
| Report ITU-R RA.2512 | “Technical and operational characteristics of broadband, background-limited detectors operating in the millimetre-wave regime” Instruments described in Report RA.2512 operating above 200 GHz are excerpted for display in Annex 2 here. |
| Report ITU-R SM.2450 | “Sharing and compatibility studies between land-mobile, fixed and passive services in the frequency range 275-450 GHz” Updated and extended versions of Tables 9 and 10 (protection criteria) and 11 (sites) of Report ITU-R SM.2450 are given in Annexes 3 and 2 here, respectively. |

## A8.2	Representative radio astronomy sites operating in relevant frequency bands

Table A8-1 describes a representative list of radio astronomy sites currently operating heterodyne instruments above 200 GHz in the three ITU-R regions, with additional and more current information than the comparable Table 11 of Report ITU-R SM.2450.

Tables A8-2 and A8-3 describe, respectively, bolometer receivers in Chile and at the South Pole using information extracted from Report ITU-R RA.2512.

Table A8-1

Radio astronomy sites operating heterodyne instruments

ITU-R Region 1

| Observatory name, place, administration | Longitude (E) Latitude (N) Elevation (m AMSL) | Minimum elevation (degrees) | Rx height above terrain (m) | Uppermost operating frequency URL Geographical characteristics |
| --- | --- | --- | --- | --- |
| IRAM-NOEMA 12×15 m Plateau de Bure, France | 5.9079173°, 44.633889° 2553 | 0 | 15 | 370 GHz https://iram-institute.org/observatories Maximum antenna separation 1.7 km |
| IRAM-30 m, Pico de Veleta, Spain | –3.392778°, 37.06611° 2850 | 0 | 31 | 370 GHz https://iram-institute.org/observatories/ |

ITU-R Region 2

| Observatory name, place, administration | Longitude (E) Latitude (N) Elevation (m AMSL) | Minimum elevation (degrees) | Rx height above terrain (m) | Uppermost operating frequency URL Geographical characteristics |
| --- | --- | --- | --- | --- |
| LMT 50 m Sierra Negra, Puebla, Mexico | –97.313333° 18.985000° 4660 | 7 | 51 | 280 GHz http://lmtgtm.org/ |
| APEX 12 m – Atacama Pathfinder Experiment, Chajnantor, Chile | −67.75888° −23.00583° 4850 | 0 | 13 | 856 GHz http://www.apex-telescope.org/ns/ |
| FYST 6m – Fred Young Submillimeter Telescope Chajnantor, Chile | –67.74028 –22.98583 5600 | 4 | 8 | 1.5 THz https://www.ccatobservatory.org/ |
| ASTE 10 m – Atacama Submillimeter Telescope Experiment, Pampa La Bola , Chile | −67.7033° −22.9714° 4860 | 0 | 13.6 | 940 GHz http://aste.nao.ac.jp/index_e.html |
| ALMA, 54 × 12 m + 12 × 7 m Chajnantor, Chile | –67.754928° –23.022911° 5000 | 0 | 13 | 950 GHz https://www.almascience.org maximum antenna separation 17 km inside a radio quiet zone of radius 35 km |
| NANTEN2 4 m, Pampa La Bola, Chile | –67.7031° –22.970° 4860 | 0 | 7 | 880 GHz https://astro.uni-koeln.de/en/observational-astrophysics/nanten2 |
| Simons Observatory, Chajnantor, Chile | −67.7875° −22.95861° 5200 | 0 | 6 | 300 GHz https://simonsobservatory.org/ |
| ARO SMT 10 m, Mt. Graham, AZ, USA | –109.89201° 32.701303° 3200 | 7 | 11 | 375 GHz https://aro.as.arizona.edu/ |
| JCMT 15 m, Mauna Kea, HI, USA | –155.47694° 19.822833° 4300 | 6 | 17 | 345 GHz https://www.eaobservatory.org |
| SMA 8×6 m Mauna Kea, HI, USA | –155.47500° 19.821667° 4300 | 0 | 8 | 420 GHz https://lweb.cfa.harvard.edu/sma/ |
| South Pole 10 m Telescope, NSF South Pole Research Station, USA | …… −90° 2820 | 0 | 8 | 240 GHz https://pole.uchicago.edu/public/Home.html At the very South Pole |

ITU-R Region 3

| Observatory name, place, administration | Longitude (E) Latitude (N) Elevation (m AMSL) | Minimum elevation (degrees) | Rx height above terrain (m) | Uppermost operating frequency URL Geographical characteristics |
| --- | --- | --- | --- | --- |
| CCOSMA, 3 m, Yangbajing, Tibet China | 90.5258° 30.1033° 4319 | 0 | 4 | 230/345 GHz https://tinyurl.com/yjtcjfnb |
| Korean VLBI Network (KVN) Pyeongchang 21 m radio telescopes, Republic of Korea | 128° 26′ 55.1" 37° 32′ 00.1" 557 | 5 | 15 | 275 GHz https://kvn.kasi.re.kr |
| Korean VLBI Network (KVN) Yonsei 21 m radio telescopes, Republic of Korea | 126° 56′ 27.4" 37° 33′ 54.9" 139 | 5 | 15 | 275 GHz https://kvn.kasi.re.kr |
| SNU 6 m Radio Telescope, Republic of Korea | 126° 57′ 19" 37° 27′ 15" 139 | 5 | 5 | 270 GHz https://astron.snu.ac.kr/en |

Table A8-2

Broadband, background-limited detectors at Parque Astronómico de Atacama, Chajnantor Chile (Report ITU-R RA.2512)

| Telescope or project name, URL | Instrument, detector count | Band centre (GHz) | Fractional bandwidth | Antenna beamwidth (degree) | Directivity gain (dBi) | Instrument field of view (degree) | Primary aperture (m) | Unshielded aperture (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| JHU CLASS Cosmology large angular scale surveyor https://sites.krieger.jhu.edu/class/ | G-band dichroic receiver 1 020 detectors | 216 | 0.17 | 0.27 | 47 | 18.9 | 0.6 | 0.35 |
| Simons Observatory https://simonsobservatory.org/ | POLARBEAR-2 22 764 detectors | 227 275 | 0.2 0.15 | 0.05 0.04 | 70 72 | 4.8 | 2.5 | 0.5 |

Table A8-3

Broadband, background-limited detectors at the South Pole Dark Sector (Report ITU-R RA.2512)

| Telescope or project name URL | Instrument, years of operation detector count | Band centre (GHz) | Fractional bandwidth | Antenna beamwidth (degree) | Directivity gain (dBi) | Instrument field of view (degree) | Primary aperture (m) | Unshielded aperture (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BICEP https://tinyurl.com/bp4tds6z | BICEP Array 2020-current 32 724 detectors | 220 270 | 0.24 0.18 | 0.18 0.15 | 59 61 | 28 | 0.52 | 0.52 |
| South Pole Telescope https://pole.uchicago.edu/public/Home.html | SPT-3G 2016-current 16 260 detectors | 220 | 0.24 | 0.017 | 79 | 1.9 | 10 | 0.685 |

## A8.3	Protection criteria

Tables A8-4 and A8-5 give protection criteria for radio astronomy observations using current values for receiver temperatures above 275 GHz. For an explanation of how the values were calculated, see Recommendation ITU-R RA.769. The entries in Tables 3-1 and 3-2 are identical to those in Tables 1 and 2 of Recommendation ITU-R RA.769 below 275 GHz and should be used in place of Tables 5-6 of Report ITU-R RA.2189 or Tables 9-10 of Report ITU-R SM.2450 at higher frequencies. Criteria from both tables should be satisfied at relevant frequencies.

Table A8-4

Threshold levels of interference detrimental to radio astronomy continuum observations

| frequency f (MHz) | Assumed spectral line channel bandwidth Df (kHz) | Minimum antenna noise temperature TA (K) | Receiver noise temperature TR (K) | System sensitivity(2) (noise fluctuations) |  | Threshold interference levels(1) (2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Temperature DT (mK) | Power spectral density DPS (dB(W/Hz)) | Input power DPH (dBW) | pfd SH Df (dB(W/m2)) | Spectral pfd SH (dB(W/(m2 Hz))) |
| (1) | (2) | (3) | (4) | (5) | (6) | (7) | (8) | (9) |
| 220 000 | 8 000 | 20 | 43 | 0.016 | –276.6 | –187.6 | –119.3 | –218.3 |
| 265 000 | 8 000 | 25 | 50 | 0.019 | –275.9 | –186.8 | –116.9 | –216.0 |
| 345 000 | 8 000 | 24 | 72 | 0.024 | –274.8 | –185.8 | –113.6 | –212.6 |
| 405 000 | 8 000 | 43 | 135 | 0.044 | –272.1 | –183.1 | –109.5 | –208.5 |
| 432 000 | 8 000 | 78 | 135 | 0.053 | –271.3 | –182.3 | –108.1 | –207.2 |
| 500 000 | 8 000 | 115 | 135 | 0.063 | –270.6 | –181.6 | –106.2 | –205.2 |
| 605 000 | 8 000 | 172 | 105 | 0.069 | –270.2 | –181.2 | –104.1 | –203.1 |
| 675 000 | 8 000 | 108 | 105 | 0.053 | –271.3 | –182.3 | –104.3 | –203.3 |
| 710 000 | 8 000 | 154 | 105 | 0.065 | –270.5 | –181.5 | –103.0 | –202.0 |

Table 3-2 A8-5

Threshold levels of interference detrimental to radio astronomy spectral-line observations

| frequency f (MHz) | Assumed spectral line channel bandwidth Df (kHz) | Minimum antenna noise temperature TA (K) | Receiver noise temperature TR (K) | System sensitivity(2) (noise fluctuations) |  | Threshold interference levels(1) (2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Temperature DT (mK) | Power spectral density DPS (dB(W/Hz)) | Input power DPH (dBW) | pfd SH Df (dB(W/m2)) | Spectral pfd SH (dB(W/(m2 Hz))) |
| (1) | (2) | (3) | (4) | (5) | (6) | (7) | (8) | (9) |
| 220 000 | 1 000 | 20 | 43 | 1.41 | –257.1 | –207.1 | –138.8 | –198.8 |
| 265 000 | 1 000 | 25 | 50 | 1.68 | –256.4 | –206.4 | –136.4 | –196.4 |
| 345 000 | 1 000 | 23 | 72 | 2.14 | –255.3 | –205.3 | –133.1 | –193.1 |
| 405 000 | 1 000 | 43 | 135 | 3.98 | –252.6 | –202.6 | –129.0 | –189.0 |
| 432 000 | 1 000 | 78 | 135 | 4.77 | –251.8 | –201.8 | –127.7 | –187.7 |
| 500 000 | 1 000 | 115 | 135 | 5.59 | –251.1 | –201.1 | –125.7 | –185.7 |
| 605 000 | 1 000 | 172 | 105 | 6.20 | –250.7 | –200.7 | –123.6 | –183.6 |
| 675 000 | 1 000 | 108 | 105 | 4.76 | –251.8 | –201.8 | –123.8 | –183.8 |
| 710 000 | 1 000 | 154 | 105 | 5.80 | –251.0 | –201.0 | –122.5 | –182.5 |

## A8.4	Graphical representations

Figure A8-1

Effective zenith system temperature and atmospheric transmission at ALMA

Lower plot: Atmospheric transmission at zenith as a function of frequency at ALMA. The tuning ranges of ALMA receiver bands 5-9 are shown along with the frequency bands identified in RR No. 5.565. The green horizontal lines below 275 GHz show the frequency bands allocated to radio astronomy on a primary basis and the red horizontal lines show the frequency bands subject to RR No. 5.340. Upper plot: Variation of the effective system temperature (Tsky+Trcvr)/Transmission that accounts for attenuation of the incoming signal. Values at frequencies noted in Tables 3-1 and 3-2 are marked on the plot by small open rectangles. ALMA documentation may be found at https://almascience.nrao.edu/proposing/technical-handbook.

## A8.5	Sharing and compatibility analysis

Editor’s Note: Study 1 should be updated with the RLS parameters introduced during the November 2025 meeting of WP 5B

## A8.5.1 	Study 1

This study considers compatibility with a vehicle equipped with X and Y radars as described in Figure 1 of section 4.2.2.

### A8.5.1.1	RAS protection criteria and other frequency-dependent matters

Working Party 7D provided the protection criteria for radio astronomy continuum observations as listed in Table 4.3-1, using the methods and parameters in Recommendation ITU-R RA.769-2 and Report ITU-R RA.2131. Criteria at five frequencies studied now are given Table A8-6: Also given in Table A8-1 are the specific absorption coefficents (dB/km) at 5 km geographic elevation under good conditions at the ALMA site. These can be derived from the empirically measured zenith atmospheric opacities at ALMA shown in Figure 4-2 using a scale height of 4.2 km, or taken from Figure 2 of Report ITU-R RA.2189.

Table A8-6

| Frequency GHz | RAS Threshold dB (W/m2/Hz) | Specific attenuation at 5 km (k_r) dB/km |
| --- | --- | --- |
| 275 | -216 | 0.04 |
| 350 | -213 | 0.10 |
| 405 | -208 | 0.16 |
| 432 | -207 | 0.30 |
| 325 | NA | 1.00 |

### A8.5.1.2	Vehicular radar model

The 8-radar model of Figure 1 of Section 4.2.2 was adopted as schematically illustrated at left in Figure A8-2. Radiation from the vehicle in the horizontal plane in the direction of the RAS site at the band center is plotted as a function of the relative orientation angle fm at right in Figure A8-1. Averaged over the range fm = 0-360° the mean e.i.r.p. density 51.6 dBm/GHz is dominated by the more powerful front- and rear-facing type X radars having a bandwidth of 5 GHz, versus 12.5 GHz for the weaker type Y. Given that the bandwidth associated with the RAS observing is 8 GHz, the protection criteria were adjusted upward (loosened) by 10 log(8/5) = 2.04 dB when calculating the line-of-sight separation distances shown at right in Figure A8-3 and Table A8-2.

Figure A8-2

Geometry and vehicular radar model

### A8.5.1.3	Dynamic analysis methodology

The radiation received at the RAS site was calculated as a function of line-of-sight distance and relative orientation angle fm for the five frequencies noted in Table A.8.1 as

received pfd = EIRP(fm)*exp(-k_r*R)/4pR2

Two overall geometries were considered:

Figure A8-4 at left where the vehicle and the RAS site are at 5 km elevation and the RAS site is in the horizontal plane of the vehicular radar

Figure A8-4 at right where the vehicle is located at 2.5 km elevation and the specific attenuation is scaled with altitude using a 4.2 km atmospheric scale height

### A8.5.1.4	Results of calculations

Figure A8-4

Figure 8-2 plots the pfd received at the RAS site as a function of the line of sight distance in a flat geometry at each frequency. Along each curve the red rectangle is plotted at the pfd value of the protection criterion and the blue circle is plotted at the distance beyond which the maximum 2% exceedance criterion of Rec. ITU-R RA.1513 is met.

The curves labelled 275 GHz are meant to represent the frequency range 252-296 GHz and the protection criteria are not met at any distance up to 300 km. The frequency 325 GHz is in the middle of a strong atmospheric absorption outside the frequency ranges identified for RAS use under RR No. 5.565, where RAS does not observe, and therefore lacks an associated protection threshold or line-of-sight separation.

Line of sight separations meeting the 2% exceedance criterion of ITU-RA.1513 are typically about 1/3 smaller when the radar is at 2.5 km elevation.

Annex 9

# A9 	Common characteristics related to the RLS applications

## A9.1	Antenna Pattern

The following equations provide the antenna radiation pattern that could be used in the analysis of interference:

Gφ,θ= Grefx

Grefx=G0-12x²                       for      0≤x<1.152

Grefx=G0-15-15log(x)      for      1.152≤x

with:

α=arctan⁡tanθsinφ

Ψα=1cosαφ32+sinαθ32

Ψ=arccos⁡(cosφ.cosθ)

x=ΨΨα

where:

G(,):	gain relative to an isotropic antenna (dBi)

G0:	Maximum gain in or near the horizontal plane (dBi)

:	Absolute value of the elevation angle relative to the angle of maximum gain (degrees)

3:	3 dB beamwidth in the vertical plane (degrees)

:	Azimuth angle relative to the angle of maximum gain (degrees)

3:	3 dB beamwidth in the azimuth plane (degrees).

____________________

That is, any building of the ‘traditional’ or ‘thermally efficient’ types.