| U.S. Radiocommunications Sector Fact Sheet |  |
| --- | --- |
| Working Party: ITU-R WP 5C | Document No: USWP5C-04 |
| Ref: Document 5C/271 (Annex 2.3) | Date: March 27, 2026 |
| Document Title: Working document towards draft CPM text on WRC-27 agenda item 1.10 |  |
| Author(s)/Contributors(s): Zahid Islam SpaceX Joe McMichael SpaceX Nader Damavandi SpaceX Jameson Dempsey SpaceX | Email: km.islam@spacex.com Email: Joseph.McMichael@spacex.com Email: Nader.Damavandi@spacex.com Email: jameson.dempsey@spacex.com |
| Purpose/Objective: Add studies to the Working Doc on AI 1.10, i.e. 5C/274 (Annex 2.4) |  |
| Abstract: ITU-R Working Party 5C is tasked with finalizing the draft CPM text on WRC-27 agenda item 1.10 at its upcoming meeting (May 2026). This input contribution will propose regulatory solutions to address Resolution 775 (Rev. WRC-23), including the development of pfd and eirp limits for inclusion in Article 21 of the Radio Regulations for satellite services to protect fixed and mobile services in the frequency bands 71-76 and 81-86 GHz. |  |

Radiocommunication Study Groups. Source: Annex 2.3 to 5C/271; Document TBD. TBD May 2026. English only. United States WORKING DOCUMENT TOWARDS DRAFT CPM TEXT ON WRC-27 AGENDA ITEM 1.10

As the responsible group for WRC-27 agenda item 1.10, Working Party 5C has been conducting the requisite studies called for in Resolution 775 (Rev. WRC-23).  This input contribution updates the draft CPM text to include regulatory considerations based on the results of studies.

Tracked changes, except for modifications to the Radio Regulations, were accepted from 5C/271(Annex 2.3), such that new proposals from the United States could be clearly illustrated.

The intent of this contribution is to aid Working Party 5C in progressing the work on draft CPM text and it presents the United States’ proposals in terms of PFD masks and eirp limits.

# The updates in third draft are highlighted in cyan.

DRAFT CPM TEXT
ON WRC-27 AGENDA ITEM 1.10

CHAPTER 2

Fixed, mobile and radiolocation issues

(Agenda items 1.7, 1.8, 1.9, 1.10)

Agenda item 1.10

(WP 5C* / WP 3J, WP 3M, WP 4A*, WP 4B, WP 4C*, WP 5A*, WP 5B, WP 6A, WP 7C, WP 7D)

1.10	to consider developing power flux density and equivalent isotropically radiated power limits for inclusion in Article 21 of the Radio Regulations for the fixed-satellite, mobile-satellite and broadcasting-satellite services to protect the fixed and mobile services in the frequency bands 71-76 GHz and 81-86 GHz, in accordance with Resolution 775 (Rev.WRC23);

Resolution 775 (Rev.WRC-23) – Power flux-density and equivalent isotropically radiated power limits for inclusion in Article 21 for the fixed-satellite, mobile-satellite and broadcasting-satellite services to protect the fixed and mobile services in the frequency bands 71-76 GHz and 81-86 GHz

# 2/1.10/1	Executive summary

{Executive summary to describe briefly the purpose of the agenda item, summarize the results of the studies carried out and, most importantly, provide a brief description of the method(s) identified that may satisfy the agenda item; should be limited to no more than half a page of text (see also §§ A2.1, A2.3 , A2.5 and A2.6 of Annex 2 to Resolution ITU-R 2-9)}

# 2/1.10/2	Background

.

WRC-2000 adopted co-primary allocations in the 71-76 GHz and 81-86 GHz band for terrestrial (fixed and mobile services) and the fixed-satellite, mobile-satellite and broadcasting-satellite services.  The sharing conditions between the fixed service, mobile service, and satellite services in the frequency bands 71-76 GHz and 81-86 GHz could not be fully defined at WRC-2000 due to limited information on the types of service applications at the time.

In the last two decades, there have been a number of significant technology advances and changes in network requirements in the fixed and mobile services. Regarding fixed use, beyond high capacity backhaul links, advancement of smaller, lighter backhaul antennas to facilitate 5G deployments have resulted in a rise in fixed service use in these bands globally. In addition, new mobile use for point-to-point communications to aircraft and ships has been authorized.

Satellite systems in the 71-76 GHz and 81-86 GHz bands have also seen significant developments since WRC-2000. Advancements have been made in manufacturing and development processes, allowing more satellites to be made at a lower cost, resulting in exponentially more deployments. At the same time, use of the 71-76 GHz band for FSS satellites is growing, supported by a ground network of high-capacity gateway earth stations in the 81-86 GHz band, and thousands of satellites already using this band, with licensed operations in tens of countries.

# 2/1.10/3	Summary and analysis of the results of ITU-R studies

{Summary of the technical and operational studies, including a list of relevant ITU-R Recommendations, and analysis of the results of studies relating to the possible methods of satisfying the agenda item (see §§  A2.3, A2.5 and A2.6 of Annex 2 to Resolution ITU-R 2-9)}

## 2/1.10/3.1	Study to determine PFD limits of satellite systems operating in the 71-76 GHz frequency range

The frequency band 71-76 GHz has co-primary allocations to fixed service, mobile service, fixed-satellite service (space-to-Earth), mobile-satellite service (space-to-Earth), and broadcasting-satellite service.

A number of studies were conducted to determine the maximum permissible PFD limits for each individual service and their respective orbits. A summary of those studies is provided below.

### 2/1.10/3.1.1	Fixed Satellite Service into Fixed Service

[TBD]

#### 2/1.10/3.1.1.1	Geostationary Orbit Satellite

| Study | GSO Spacing | FS Elevation Angle | FS Antenna Pattern | NGSO+GSO Aggregation | Propagation Considered | Calculated PFD Mask |
| --- | --- | --- | --- | --- | --- | --- |
| A | Single | 5, 10 | F.699 |  | Free Space |  |
| M (China) | 1 degree | 0, 5, 10 | F.699, F.1245 |  |  | -142 dB W/MHz/m2 for θ<5° -142 + 0.75 (θ - 5) dB W/MHz/m2 for 5°<θ<25° -105 dB W/MHz/m2 for 25°<θ<90° |
| O (Tonga) | 1.5 Degree | 5 | F.1245 | NGSO+GSO | Atmospheric + Gaseous | -120 dB W/MHz/m2 for θ<5° -120+ 5((θ - 5)dB W/MHz/m2 for 5°<θ<25° -105 dB W/MHz/m2 for 25°<θ<90° |
| USA1 | 2 Degree | 0, 3, 5, 7.5, 10 | F.1245 |  | Free space | -115 dB W/MHz/m2 for θ<10° -115+ 0.66((θ - 10)dB W/MHz/m2 for 10°<θ<25° -105 dB W/MHz/m2 for 25°<θ<90° |

#### 2/1.10/3.1.1.2	Non-Geostationary Orbit Satellite

[TBD]

| Study | NGSO | Nco | FS Elevation Angle | FS Antenna Pattern | NGSO+GSO Aggregation | Propagation Considered | Calculated PFD Mask |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E (Tonga) | D |  | 4 | F.1245 | Three GSOs w/ 1.5 degree spacing | 1All atmospheric attenuations (attenuations due to rain, cloud, gas and scintillation) 2 Rain attenuation only 3 No atmospheric attenuation (clear sky). | NGSO: -115 dB W/MHz/m2 for θ<5° -115 + 0.5 (θ - 5) dB W/MHz/m2 for 5°<θ<25° -105 dB W/MHz/m2 for 25°<θ<90° GSO: -129.85 |
| J (UK) | Systems, I, J, K, L, M and N | 32 | 5 |  | Not Assumed. | Free Space + Atmospheric | -115 dB W/MHz/m2 for θ<5° -115 + 0.5 (θ - 5) dB W/MHz/m2 for 5°<θ<25° -105 dB W/MHz/m2 for 25°<θ<90° |
| K (India) | B, C, D,E, F, G, H, I, J, K, L, M | Not Considered | 0, 2.5, 5, 10 | F.699 | Not Assumed. | Free Space + Atmospheric | [-120] dB W/MHz/m2 for θ<5° [-110] dB W/MHz/m2 for 5°<θ<25° [-100] dB W/MHz/m2 for 25°<θ<90° |
| N (China) | B, L, M, N | 80 | 0, 5, 10 | F.699, F.1245 | Not Assumed. |  | -128 dB W/MHz/m2 for θ<5° -128 + 0.75 (θ - 5) dB W/MHz/m2 for 5°<θ<25° -105 dB W/MHz/m2 for 25°<θ<90° |
| O (Tonga) | B, D | 50 | 5 | F.1245 | NGSO+GSO | Atmospheric + Gaseous | -120 dB W/MHz/m2 for θ<5° -120+ 5((θ - 5)dB W/MHz/m2 for 5°<θ<25° -105 dB W/MHz/m2 for 25°<θ<90° |
| P (USA 3) | B,D,L,M | 90 | 5 | F.1245 ETSI 302 217-4 Class-3 | NGSO+GSO, NGSO only | Free Space + Gaseous (oxygen only) | -125 dB W/MHz/m2 for θ<5°, -125+1.43(θ - 5) dB W/MHz/m2 for 5°<θ<12°, -115+3.33(θ - 12) dB W/MHz/m2 for 12°<θ<15°, -105 dB W/MHz/m2 for 15°<θ<90° |
|  |  |  | 7.5 |  |  |  | -125 dB W/MHz/m2 for θ<12°, -125+6.67(θ - 12) dB W/MHz/m2 for 12°<θ<15°, -105 dB W/MHz/m2 for 15°<θ<90° |
|  |  |  | 10 |  |  |  | -125 dB W/MHz/m2 for θ<5°, -125+(θ - 5) dB W/MHz/m2 for 5°<θ<25°, -105 dB W/MHz/m2 for 25°<θ<90° |

### 2/1.10/3.1.2	Fixed Satellite Service into Mobile Service

[TBD]

### 2/1.10/3.1.3	Mobile Satellite Service into Fixed Service

[TBD] No studies were performed as no characteristics were provided for the mobile satellite service in the same way as it was for the fixed satellite service. It is acknowledged there are MSS filings in the MIFR in these bands. BR has also provided information on characteristics of FSS and MSS networks and systems in Document 5C/148.

### 2/1.10/3.1.4	Mobile Satellite Service into Mobile Service

[TBD] No studies were performed as no characteristics were provided for the mobile satellite service. in the same way as it was for the fixed satellite service. It is acknowledged there are MSS filings in the MIFR in these bands. BR has also provided information on characteristics of FSS and MSS networks and systems in Document 5C/148

### 2/1.10/3.1.5	Broadcasting Satellite Service into Fixed Service

[TBD] No studies were performed as no characteristics were provided for the broadcasting satellite service

### 2/1.10/3.1.6	Broadcasting Satellite Service into Mobile Service

[TBD] No studies were performed as no characteristics were provided for the broadcasting satellite service

## 2/1.10/3.2	Study to determine EIRP limits of satellite systems operating in the 81-86 GHz frequency range

The frequency band 81-86 GHz has co-primary allocations to fixed service, mobile service, fixed-satellite service (Earth-to-space) and mobile-satellite service (Earth-to-space).

### 2/1.10/3.2.1	Fixed Satellite Service into Fixed Service

[TBD]

2/1.10/3.2.1.1	Geostationary Orbit Satellite

Simulation studies were carried out regarding the sharing between the FSS GSO uplink and FS. The studies considered representative characteristics of the FSS GSO earth stations and FS systems for the analysis.  Studies have shown that the use of the limits in No. 21.8 would enable cross-border coordination between FSS GSO earth stations and fixed service operations.

### 2/1.10/3.2.2	Fixed Satellite Service into Mobile Service

[TBD]

### 2/1.10/3.2.3	Mobile Satellite Service into Fixed Service

[TBD] No studies were performed as no characteristics were provided for the mobile satellite service in the same way as it was for the fixed satellite service. It is acknowledged there are MSS filings in the MIFR in these bands. BR has also provided information on characteristics of FSS and MSS networks and systems in Document 5C/148

### 2/1.10/3.2.4	Mobile Satellite Service into Mobile Service

[TBD] No studies were performed as no characteristics were provided for the mobile satellite service in the same way as it was for the fixed satellite service. It is acknowledged there are MSS filings in the MIFR in these bands. BR has also provided information on characteristics of FSS and MSS networks and systems in Document 5C/148

### 2/1.10/3.2.5	Broadcasting Satellite Service into Fixed Service

[TBD] No studies were performed as no characteristics were provided for the broadcasting satellite service

### 2/1.10/3.2.6	Broadcasting Satellite Service into Mobile Service

[TBD] No studies were performed as no characteristics were provided for the broadcasting satellite service

# 2/1.10/4	Methods to satisfy the agenda item

{This section should contain the brief description of the Method or Methods to satisfy the agenda item (see §§  A2.3, A2.4, A2.5 and A2.6 of Annex 2 to Resolution ITU-R 2-9)}

## 2/1.10/4.1 	Method A: Modification of Article 21

- 81-86 GHz, modifications to Table 21-3 to include the 81-86 GHz band for the fixed satellite service. Consequential modifications to Appendix 7 are made to facilitate coordination.

–	71-76 GHz, modifications to Table 21-4 to include a power-flux density for fixed satellite and mobile-satellite geostationary satellite and non-geostationary satellite systems

–	 Resolution 775 (Rev. WRC-23) is suppressed

2/1.10/5	Regulatory and procedural considerations

{Example(s) of regulatory text relating to the Method(s) to satisfy the agenda item (see §§  A2.3, A2.4.6 and A2.5 of Annex 2 to Resolution ITU-R 2-9)}

2/1.10/5.1 	For Method A

ARTICLE 21

Terrestrial and space services sharing frequency bands above 1 GHz

Section III − Power limits for earth stations

21.8	§ 4	1)	The equivalent isotropically radiated power (e.i.r.p.) transmitted in any direction towards the horizon by an earth station shall not exceed the following limits except as provided in No. 21.10 or 21.11:

a)	in frequency bands between 1 GHz and 15 GHz

+40 dBW in any 4 kHz band for θ ≤ 0°

+40 + 3 θ dBW in any 4 kHz band for 0° < θ ≤ 5°; and

b)	in frequency bands above 15 GHz

+64 dBW in any 1 MHz band for θ ≤ 0°

+64 + 3 θ dBW in any 1 MHz band for 0° < θ ≤ 5°,

where θ is the angle of elevation of the horizon viewed from the centre of radiation of the antenna of the earth station and measured in degrees as positive above the horizontal plane and negative below it.

21.9		2)	For angles of elevation of the horizon greater than 5° there shall be no restriction as to the equivalent isotropically radiated power (e.i.r.p.) transmitted by an earth station towards the horizon.

21.10		3)	As an exception to the limits given in No. 21.8, the equivalent isotropically radiated power (e.i.r.p.) towards the horizon for an earth station in the space research service (deep space) shall not exceed +55 dBW in any 4 kHz band in frequency bands between 1 GHz and 15 GHz, or +79 dBW in any 1 MHz band in frequency bands above 15 GHz.

21.11		4)	The limits given in Nos. 21.8 and 21.10, as applicable, may be exceeded by not more than 10 dB. However, when the resulting coordination area extends into the territory of another country, such increase shall be subject to agreement by the administration of that country.

21.12		5)	The limits given in No. 21.8 apply, where applicable, to the services and frequency bands indicated in Table 21-3 below for transmission by earth stations where the frequency bands are shared with equal rights with the fixed or mobile service:

MOD

TABLE  21-3     (Rev.WRC-27)

| Frequency band |  | Services |  |
| --- | --- | --- | --- |
| 2 025-2 110 MHz 5 670-5 725 MHz 5 725-5 755 MHz 6 | (for the countries listed in No. 5.454 with respect to the countries listed in Nos. 5.453 and 5.455) (for Region 1 with respect to the countries listed in Nos. 5.453 and 5.455) | Earth exploration-satellite Fixed-satellite Meteorological-satellite Mobile-satellite Space operation |  |
| 5 755-5 850 MHz 6 | (for Region 1 with respect to the countries listed in Nos. 5.453 and 5.455) | Space research |  |
| 5 850-7 075 MHz |  |  |  |
| 7 190-7 250 MHz |  |  |  |
| 7 900-8 400 MHz |  |  |  |
| 10.7-11.7 GHz 6 | (for Region 1) |  |  |
| 12.5-12.75 GHz 6 | (for Region 1 with respect to the countries listed in No. 5.494) |  |  |
| 12.7-12.75 GHz 6 | (for Region 2) |  |  |
| 12.75-13.25 GHz |  |  |  |
| 14.0-14.25 GHz | (with respect to the countries listed in No. 5.505) |  |  |
| 14.25-14.3 GHz | (with respect to the countries listed in Nos. 5.505 and 5.508) |  |  |
| 14.3-14.4 GHz 6 | (for Regions 1 and 3) |  |
| 14.4-14.8 GHz |  |  |
| 17.7-18.1 GHz |  | Fixed-satellite |
| 22.55-23.15 GHz |  | Earth exploration-satellite |
| 27.0-27.5 GHz 6 | (for Regions 2 and 3) | Mobile-satellite |
| 27.5-29.5 GHz |  | Space research |
| 31.0-31.3 GHz | (for the countries listed in No. 5.545) |  |
| 34.2-35.2 GHz | (for the countries listed in No. 5.550 with respect to the countries listed in No. 5.549) |  |
| 51.4-52.4 GHz 81-84 GHz 84-86 GHz |  | Fixed-satellite Fixed-satellite Mobile-satellite Fixed-satellite |

[USA Note: Regarding the MSS inclusion, no studies have been performed with MSS, which could include multiple types of earth stations.]

Section V − Limits of power flux-density from space stations

21.16	§ 6	1)	The power flux-density at the Earth’s surface produced by emissions from a space station, including emissions from a reflecting satellite, for all conditions and for all methods of modulation, shall not exceed the limit given in Table 21-4. The limit relates to the power flux-density which would be obtained under assumed free-space propagation conditions unless otherwise indicated and applies to emissions by a space station of the service indicated where the frequency bands are shared with equal rights with the fixed or mobile service, unless otherwise stated.

MOD

TABLE  21-4     (Rev.WRC-27)

-

-

-

TABLE  21-4  (end)     (Rev.WRC-27)

| Frequency band | Service* | Limit in dB(W/m2) for angles of arrival (δ) above the horizontal plane |  |  |  |  |  |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° |  |  |  |  |  | 25°-90° |  |
| 40.5-42 GHz | Fixed-satellite (geostationary-satellite orbit) Broadcasting-satellite (geostationary-satellite orbit) | −120 21 | 5°-15° |  |  | 15°-25° |  |  | −105 21 | 1 MHz |
|  |  |  | −120  ( − 5) 21 |  |  | −110  0.5 ( − 15) 21 |  |  |  |  |
| 42-42.5 GHz | Fixed-satellite (non-geostationary-satellite orbit) Broadcasting-satellite (non-geostationary-satellite orbit) | −120 11, 21 | 5°-25° |  |  |  |  |  | −105 11, 21 | 1 MHz |
|  |  |  | −120 + 0.75( − 5) 11, 21 |  |  |  |  |  |  |  |
| 42-42.5 GHz | Fixed-satellite (geostationary-satellite orbit) Broadcasting-satellite (geostationary-satellite orbit) | −127 21 | 5°-20° |  |  | 20°-25° |  |  | −105 21 | 1 MHz |
|  |  |  | −127 + (4/3) ( − 5) 21 |  |  | −107  0.4 ( − 20) 21 |  |  |  |  |
| In Region 1: 47.5-47.9 GHz 48.2-48.54 GHz 49.44-50.2 GHz | Fixed-satellite (geostationary-satellite orbit) | −115 | 5°-25° |  |  |  |  |  | −105 | 1 MHz |
|  |  |  | −115  0.5(δ − 5) |  |  |  |  |  |  |  |
| 71-74 GHz | Fixed-satellite Mobile-satellite (space-to-Earth) (geostationary satellite orbit) | −115 | 5°-25° |  |  |  |  |  | −10527 | 1 MHz |
|  |  |  | −115  0.5(δ − 5) 27 |  |  |  |  |  |  |  |
| 74-76 GHz | Fixed-satellite Broadcasting-satellite (space-to-Earth) (geostationary satellite orbit) | −11527 | 5°-25° |  |  |  |  |  | −10527 | 1 MHz |
|  |  |  | −115  0.5(δ − 5) 27 |  |  |  |  |  |  |  |
| 71-74 GHz | Fixed-satellite Mobile-satellite (space-to-Earth) (non-geostationary satellite orbit) | -12527 | 5°-12° |  | 12°-15° |  |  | 15°-25° | -10527 | 1 MHz |
|  |  |  | -125+1.43(δ-5)27 |  | -115+3.33(δ-12)27 |  |  | -10527 |  |  |
| 74-76 GHz | Fixed-satellite Broadcasting-satellite (space-to-Earth) (non-geostationary satellite orbit) | -12527 | 5°-12° | 12°-15° |  |  | 15°-25° |  | -10527 | 1 MHz |
|  |  |  | -125+1.43(δ-5)27 | -115+3.33(δ-12)27 |  |  | -10527 |  |  |  |

_______________

*	The references to services are those services which have allocations in Article 5.

21.17		2)	The limits given in Table 21-4 may be exceeded on the territory of any country whose administration has so agreed.

{Editor’s Note: In considering WRC-27 agenda item (AI) 1.10. it was recognized that it is limited to calculations or adaption of pfd and e.i.r.p. to protect terrestrial services (fixed and mobile). However, the coordination aspects between terrestrial service and earth station is not included in this Agenda Item, which is an important issue to be addressed, as appropriate. WP 5C considers that this is a consequential action of updating Appendix 7 with information on 81-86 GHz arising from the consideration of WRC-27 AI 1.10, which should be addressed.}

MOD

APPENDIX 7 (REV.WRC-27)

Methods for the determination of the coordination area around an earth
station in frequency bands between 100 MHz and 105 GHz

ANNEX 7

System parameters and predetermined coordination distances for determination of the coordination area around an earth station

MOD

TABLE 7c    (Rev.WRC-27)

Parameters required for the determination of coordination distance for a transmitting earth station

| Transmitting space radiocommunication service designation |  | Fixed- satellite | Fixed- satellite | Fixed- satellite 2 | Fixed- satellite | Space research | Earth exploration-satellite, space research | Fixed-satellite, mobile-satellite, radionavigation-satellite | Fixed- satellite 2 | Fixed-satellite | Fixed-satellite, mobile-satellite |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency bands (GHz) |  | 24.65-25.25 27.0-29.5 | 24.65-25.25 27-27.5 | 28.6-29.1 | 27.5-28.6 29.1-29.53 | 34.2-34.7 | 40.0-40.5 | 42.5-47 47.2-50.2 50.4-51.4 | 47.2-50.2 | 51.4-52.4 | 81-86 GHz |  |
| Receiving terrestrial service designations |  | Fixed (except HAPS), mobile | Fixed (HAPS ground station) | Fixed, mobile | Fixed, mobile | Fixed, mobile, radiolocation | Fixed, mobile | Fixed, mobile, radionavigation | Fixed, mobile | Fixed, mobile | Fixed, Mobile |  |
| Method to be used |  | § 2.1 | § 2.1 | § 2.2 | § 2.2 |  | § 2.1, § 2.2 | § 2.1, § 2.2 | § 2.2 | § 2.1 | § 2.1, § 2.2 |  |
| Modulation at terrestrial station 1 |  | N | N | N | N |  | N | N | N | N | N |  |
| Terrestrial station interference parameters and criteria | p0 (%) | 0.005 | 0.01 | 0.005 | 0.005 |  | 0.005 | 0.005 | 0.001 | 0.00128 | [0.00128] |  |
|  | n | 1 | 1 | 2 | 1 |  | 1 | 1 | 1 | 1 | [1] |  |
|  | p (%) | 0.005 | 0.005 | 0.0025 | 0.005 |  | 0.005 | 0.005 | 0.001 | 0.00128 | [0.00128] |  |
|  | NL (dB) | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | [0] |  |
|  | Ms (dB) | 25 | 10 | 25 | 25 |  | 25 | 25 | 25 | 11 | [11] |  |
|  | W (dB) | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | [0] |  |
| Terrestrial station parameters | Gx (dBi) 4 | 50 | 05 | 50 | 50 |  | 42 | 42 | 46 | 51 | [41.5, 51] |  |
|  | Te (K) | 2 000 | 350 | 2 000 | 2 000 |  | 2 600 | 2 600 | 2 000 | 1 800 | [TBD] |  |
| Reference bandwidth | B (Hz) | 106 | 106 | 106 | 106 |  | 106 | 106 | 106 | 106 | 106 |  |
| Permissible interference power | Pr( p) (dBW) in B | −111 | −134 | −111 | −111 |  | −110 | −110 | –111 | ‒125 | [‒125] |  |
| 1 A: analogue modulation; N: digital modulation. 2 Non-geostationary satellites in the fixed-satellite service. 3 Feeder links to non-geostationary-satellite systems in the mobile-satellite service. 4 Feeder losses are not included. 5 Maximum HAPS ground station antenna gain toward the horizon. |  |  |  |  |  |  |  |  |  |  |  |

____________________

* 	Studies should be carried out in close collaboration between the indicated WPs.

6 	21.12.1	The equality of right to operate when a band of frequencies is allocated in different Regions to different services of the same category is established in No. 4.8. Therefore any limits concerning inter-Regional interference which may appear in ITU-R Recommendations should, as far as practicable, be observed by administrations.

21.16.20  For verifying compliance with the PFD limits in the 71-76 GHz band, the Radiocommunication Bureau shall first apply a reduction to the notified maximum EIRP or EIRP density to account for dry-air gaseous attenuation as listed in the table as a function of angle of arrivals.

| Angle of Arrivals (deg) | Average Dry-air Attenuation (dB) | Angle of Arrivals (deg) | Average Dry-air Attenuation (dB) | Angle of Arrivals (deg) | Average Dry-air Attenuation (dB) | Angle of Arrivals (deg) | Average Dry-air Attenuation (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0.00 | 37.73 | 23.00 | 1.63 | 46.00 | 0.89 | 69.00 | 0.68 |
| 1.00 | 37.73 | 24.00 | 1.57 | 47.00 | 0.87 | 70.00 | 0.68 |
| 2.00 | 18.54 | 25.00 | 1.51 | 48.00 | 0.86 | 71.00 | 0.67 |
| 3.00 | 12.29 | 26.00 | 1.45 | 49.00 | 0.84 | 72.00 | 0.67 |
| 4.00 | 9.20 | 27.00 | 1.40 | 50.00 | 0.83 | 73.00 | 0.67 |
| 5.00 | 7.35 | 28.00 | 1.36 | 51.00 | 0.82 | 74.00 | 0.66 |
| 6.00 | 6.12 | 29.00 | 1.31 | 52.00 | 0.81 | 75.00 | 0.66 |
| 7.00 | 5.25 | 30.00 | 1.27 | 53.00 | 0.80 | 76.00 | 0.66 |
| 8.00 | 4.59 | 31.00 | 1.24 | 54.00 | 0.79 | 77.00 | 0.65 |
| 9.00 | 4.08 | 32.00 | 1.20 | 55.00 | 0.78 | 78.00 | 0.65 |
| 10.00 | 3.68 | 33.00 | 1.17 | 56.00 | 0.77 | 79.00 | 0.65 |
| 11.00 | 3.35 | 34.00 | 1.14 | 57.00 | 0.76 | 80.00 | 0.65 |
| 12.00 | 3.07 | 35.00 | 1.11 | 58.00 | 0.75 | 81.00 | 0.64 |
| 13.00 | 2.84 | 36.00 | 1.08 | 59.00 | 0.74 | 82.00 | 0.64 |
| 14.00 | 2.64 | 37.00 | 1.06 | 60.00 | 0.74 | 83.00 | 0.64 |
| 15.00 | 2.46 | 38.00 | 1.03 | 61.00 | 0.73 | 84.00 | 0.64 |
| 16.00 | 2.31 | 39.00 | 1.01 | 62.00 | 0.72 | 85.00 | 0.64 |
| 17.00 | 2.18 | 40.00 | 0.99 | 63.00 | 0.71 | 86.00 | 0.64 |
| 18.00 | 2.06 | 41.00 | 0.97 | 64.00 | 0.71 | 87.00 | 0.64 |
| 19.00 | 1.96 | 42.00 | 0.95 | 65.00 | 0.70 | 88.00 | 0.64 |
| 20.00 | 1.86 | 43.00 | 0.93 | 66.00 | 0.70 | 89.00 | 0.64 |
| 21.00 | 1.78 | 44.00 | 0.92 | 67.00 | 0.69 | 90.00 | 0.64 |
| 22.00 | 1.70 | 45.00 | 0.90 | 68.00 | 0.69 |  |  |