| U.S. Radiocommunications Sector Fact Sheet |  |
| --- | --- |
| Working Party: ITU-R WP-5B | Document No: USWP5B37-11 |
| Ref: Annex 1.5 to Document 5B/526-E | Date: XX Novenber 2026 |
| Document Title: Working Document towards PRELIMINARY DRAFT NEW RECOMMENDATION ITU-R M.[15.4-15.7_GHz_ARNS] - Characteristics of and protection criteria for radars operating in the aeronautical radionavigation service in the frequency band 15.4-15.7 GHz. |  |
| Author(s)/Contributors(s): Jae Shin Federal Aviation Administration 800 Independence Ave., S.W. Washington, DC 20591 Don Nellis Federal Aviation Administration 800 Independence Ave., S.W. Washington, DC 20591 | Phone: (202) 267-7365 e-mail: jae.w.shin@faa.gov Phone: (202) 267-9779 e-mail: Donald.Nellis@faa.gov |
| Purpose/Objective: The purpose of this contribution is to develop a new recommendation for aeronautical radionavigation systems, including unmanned aircraft systems (UAS) Detect and Avoid (DAA) radar systems, in the 15.4-15.7 GHz band. This contribution will update and/or address comments on the technical parameters of DAA radars in Table A1-1 and landing system in Table A1-2 as applicable. Due to the stability of the technical characteristics data for the airborne and ground based DAA system, this contribution proposes to upgrade the Working Document into Preliminary Draft New Recommendation. This contribution will be an update to the new report found in Annex 1.5 of Document 5B/526-E. |  |
| Abstract: This contribution is a new recommendation for UAS Detect and Avoid (DAA) systems that operate in the 15.4-15.7 GHz Aeronautical Radionavigation Service allocation. This contribution contains characteristics and protection criteria for UAS DAA radar that can be used both on airborne and ground platforms. |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 5B/526 (Annex 1.5) | Document 5B/XX-E |
|  | XX November 2026 |
|  | English only |
| United States of America |  |
| WORKING DOCUMENT TOWARDS A PRELIMINARY DRAFT NEW RECOMMENDATION ITU-R M.[15.4-15.7_GHZ_ARNS] |  |
| Characteristics of and protection criteria for [airborne] detect and avoid radars and aircraft landing systems operating in the aeronautical radionavigation service in the frequency band 15.4-15.7 GHz |  |

Introduction

TBD

Proposal

TBD

______________

Annex

WORKING DOCUMENT TOWARDS A PRELIMINARY DRAFT NEW RECOMMENDATION ITU-R M.[15.4-15.7_GHZ_ARNS]

Characteristics of and protection criteria for [airborne] detect and avoid radars and aircraft landing systems operating in the aeronautical radionavigation service in the frequency band 15.4-15.7 GHz

(202X)

[Editor’s Note: Ongoing discussion on how to address all DAA implementations (airborne and ground)]

Scope

This Recommendation specifies technical and operational characteristics and protection criteria for airborne detect and avoid radars and aircraft landing systems operating in the aeronautical radionavigation service (ARNS) in the frequency band 15.4-15.7 GHz for use in sharing and compatibility studies with systems in other services.

Keywords

15.4-15.7 GHz, aeronautical radionavigation service, radar, detect and avoid, characteristics, protection criterion.

Abbreviations/Glossary

ARNS	Aeronautical radionavigation service

DAA	Detect and avoid

ESA	Electronically scanned array

FMCW	Frequency-modulated continuous wave

RCS	Radar cross-section

RR	Radio Regulations

SNR	Signal-to-noise power ratio

Related ITU Recommendations and Reports

Recommendations

ITU-R M.1372	Efficient use of the radio spectrum by radar stations in the radiodetermination service

ITU-R M.1461	Procedures for determining the potential for interference between radars operating in the radiodetermination service and systems in other services

ITU-R M.1730	Characteristics of and protection criteria for the radiolocation service in the frequency band 15.4-17.3 GHz

ITU-R S.1340	Sharing between feeder links for the mobile-satellite service and the aeronautical radionavigation service in the Earth-to-space direction in the band 15.4-15.7 GHz

Reports

ITU-R M.2170	Compatibility analysis and results for radiolocation systems planned to operate in the 15.4 to 17.3 GHz band and aircraft landing system operating in the 15.4-15.7 GHz band as well as the radio astronomy service operating in the adjacent band 15.35-15.40 GHz, FSS systems and aeronautical radionavigation systems

ITU-R M.2204	Characteristics and spectrum considerations for sense and avoid systems use on Unmanned Aircraft Systems (UAS)

ITU-R M.2547	Various aspects of non-safety aeronautical mobile service systems in the frequency bands 15.4-15.7 GHz and 22-22.21 GHz

The ITU Radiocommunication Assembly,

considering

that the technical and operational characteristics of detect and avoid radars and aircraft landing systems operating in the aeronautical radionavigation service (ARNS) are driven by the performance requirements and depend on the frequency band,

b)	that the standardization of operations of Remotely Piloted Aircraft System (RPAS) in non-segregated airspace is under the purview of the ICAO for international operations, and that the collision avoidance capability needed for such operations relies on airborne Detect and Avoid radars operating in the ARNS;

recognizing

a)	that the frequency band 15.4-15.7 GHz is allocated on a primary basis to aeronautical radionavigation, and radiolocation services;

b)	that the fixed-satellite service (Earth-to-space) is also allocated on a primary basis in the frequency band 15.43-15.63 GHz;

c)	that the aeronautical radionavigation service is a safety service as specified by No. 4.10 of the Radio Regulations (RR);

d)	that some interference suppression techniques between radars are contained in Recommendation ITU-R M.1372, “Efficient use of the radio spectrum by radar stations in the radiodetermination service”;

e)	that No. 5.511A states that the fixed-satellite service (Earth-to-space) operating in the frequency band 15.43-15.63 GHz is limited to feeder links of non-geostationary systems in the mobile-satellite service (MSS) and is subject to coordination under No. 9.11A;

f)	that Recommendation ITU-R S.1340 provides sharing criteria and constraints applicable to coexistence between ARNS systems and MSS feeder links in the Earth-to-space direction in the band 15.4-15.7 GHz;

g)	that the frequency band 15.4-15.7 GHz is allocated worldwide on a primary basis to the aeronautical radionavigation service;

h)	that the frequency band 15.4-15.7 GHz is also allocated worldwide on a primary basis to the radiolocation service and No. 5.511E states that stations operating in the radiolocation service shall not cause harmful interference to, or claim protection from, stations operating in the aeronautical radionavigation service;

i)	that the frequency range 15.4-15.7 GHz is also allocated on a secondary basis to the aeronautical mobile (OR) service by Nos. 5.511G and 5.511H,

recommends

1	that the technical and operational characteristics of the  detect and avoid radars, in particular [onboard] remotely piloted aircraft, and aircraft landing systems operating in the ARNS, described in the annex should be considered in sharing and compatibility studies with systems in other services;

[2	that for sharing and compatibility studies, an interference-to-noise ratio (I/N) of −6 dB should be used as the protection criterion for [airborne] detect and avoid radars and aircraft landing systems operating in the ARNS in the frequency band 15.4-15.7 GHz, and that this represents the aggregate protection level if multiple interferers are present.

Or Option 2: that the criterion of interfering signal power to receiver noise power level, I/N of −6 dB should be considered to protect the detect and avoid radars and landing systems, and that this represents the aggregate protection level if multiple interferers are present.]

Annex 

Technical and operational characteristics of airborne detect and avoid radars and aircraft landing systems operating in the aeronautical radionavigation service in the frequency band 15.4-15.7 GHz

# A1	Introduction

This Annex provides representative technical and operational characteristics of  ARNS radars operating in the 15.4-15.7 GHz frequency band, including

‒	detect and avoid (DAA) systems used for collision avoidance during all phases of flight, and

‒	aircraft landing systems.

These characteristics are intended to support sharing and compatibility studies with systems in other services. They reflect representative airborne system parameters and are not intended to constitute a complete equipment specification.

## A1-1	Characteristics of [airborne] detect and avoid radars for remotely piloted aircraft systems

The safe operation of Remotely Piloted Aircraft System in civil airspace requires addressing the same issues as manned aircraft, namely integration into the air traffic control (ATC) system. Because the pilot is no longer onboard, detect and avoid (DAA) surveillance system enables the capability to see, sense or detect conflicting traffic or other hazards and take appropriate action. DAA radars aim at detecting non-cooperative targets around the aircraft. When installed on-board an aircraft, DAA radar is used in all phases of flight.

The representative technical and operational characteristics  or airborne DAA radars, in particular for remotely piloted aircraft systems, are provided in Table A1-1. Radars with similar transmitter and receiver parameters may also be deployed on the ground, recognizing the need to give priority to airborne detect and avoid radars for remotely piloted aircraft expected to be operated on a worldwide basis.

TABLE A1-1

Representative technical and operational characteristics of radionavigation radar for airborne detect 
and avoid radars for remotely piloted aircraft systems

| Parameter | Units | Radar 1 | Radar 2 | Radar 3 | Radar 4 | Radar 5 | Radar 6 | Radar 7 | Radar 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Platform |  | Airborne (Note 1) | Airborne (Note 1) | Airborne (Note 1) | Airborne | Airborne | Airborne (Note 1) | Airborne | Airborne |
| Maximum operating altitude [Editor’s Note: Mean Sea Level (MSL) or Above Ground Level (AGL) ?] | km | Up to 3 | Up to 3 | Up to 12 | Up to 15 | Up to 15 | Up to 3 | Up to 12 (TBC) | Up to 12 (TBC) |
| Radar type |  | FMCW | FMCW | Pulse-Doppler | Pulse Modulation | Pulse Modulation | FMCW | Pulse Modulation | FMCW |
| Detection range | km | 2.0 | 4.5 | 9 | 5 | 10 | 13 | 13 | 5 |
| Ground relative speed | km/h | 50-100 | 50-100 | Up to 195 | Up to 800 | Up to 800 | 50-100 | 500 | Up to 330 |
| Transmitter |  |  |  |  |  |  |  |  |  |
| Frequency range | GHz | 15.4-15.7 | 15.4-15.7 | 15.4-15.7 | 15.4-15.7 | 15.4-15.7 | 15.4-15.7 | 15.4-15.7 | 15.4-15.7 |
| Channel selection method between radars |  | Pre-programmed and channel selectable | Pre-programmed and channel selectable | SW selectable (Note 5) | SW selectable (Note 5) | SW selectable (Note 5) | Pre-programmed and channel selectable | SW selectable (Note 5) | Pre-programmed and channel selectable |
| Pulse width (1 meter range resolution) | ms | 239 | 239 | 0.25 to 20 (Note 6) | 1 to 20 | 1 to 20 | 239 | 1 to 5 | 58 |
| Pulse rise and fall times | ms | 5/5 | 0.5/0.5 | 0.025/1.5 | < 0.1 | < 0.1 | 0.5/0.5 | <0.05 | − |
| RF emission bandwidth −3 dB −20 dB −40 dB | MHz | 176 184 201 | 152 164 269 | (Band 1-MHz) 25 80 155 | 80 90 110 | 80 90 110 | 152 164 269 | 9 10 11 | 176 184 237 |
| Pulse repetition frequency | Hz | 4 000 | 4 000 | 5 000-200 000 (Note 7) | − | − | 4 000 | 20000 – 60000 | 17241 |
| Pulse repetition interval | ms | 250 | 250 | 14.30-114.03 (Note 7) TBC | 30 to 80 | 30 to 80 | 250 | 17 – 50 | 58 |
| Mean power at transmitter antenna port Editor’s Note: Confirm if Mean power or peak envelope power should be used | W | 2 | 10 | 30 | 15 | 20 | 20 | 120 | 25 |
| Out-of-band emission characteristics | dBc | 60 | 63 | 75 (through 3rd harmonic) | N/A | N/A | 63 | roll off 30 dB/dec | roll off 30 dB/dec |
| Spurious emission characteristics (conducted) | dBc | −72 | −87 | −60 | Rec ITU-R -SM.329 | Rec ITU-R -SM.329 | −87 | Rec ITU-R -SM.329-13 Chapter 4.2 Table 2 | Rec ITU-R -SM.329-13 Chapter 4.2 Table 2 |
| Receiver |  |  |  |  |  |  |  |  |  |
| Receiver IF bandwidth −3 dB −20 dB −60 dB | MHz | 15 32 58 | 15 32 58 | 53 70 114 | 20 40 80 | 20 40 80 | 15 32 58 | 9 10 11 | 5 11 17 |
| Sensitivity (MDS) (at RX input. SNR = 12 dB) | dBm | −147 | −141 | −121 | N/A | N/A | −141 | −148 | −136 |
| Receiver noise figure | dB | 1.5 | 1.5 | 4 | 3 | 3 | 1.5 | 4 | 1.7 |
| Antenna |  |  |  |  |  |  |  |  |  |
| Antenna type |  | AESA (Note 9) | AESA (Note 9) | AESA (Note 9) | MIMO | MIMO | AESA (Note 9) | AESA | MIMO AESA |
| Antenna pattern |  | TBD | N/A | N/A | See section A1-3 | See section A1-3 |  | Pencil beam | Wide Beam |
| Maximum antenna gain | dBi | 14 | 15 | 27 | 9 | 9 | 15 | 32 | 18.5 |
| Antenna pointing |  | N/A | N/A | N/A | Front See section A1-3 | Front See section A1-3 | N/A | Yes | Yes (Tx) |
| First antenna sidelobe | dBi | −17 at 50° | −15 at 50° | 25 at 8° (Vertical) −25 at 14° (Horizontal) | See section A1-3 | See section A1-3 | −15 at 50° | −13 at 8 deg vert / 5.0 deg horz | −19 at 52° |
| Horizontal beamwidth | degrees | 40 | 32 | 4 | 80 (Note 10) | 80 (Note 10) | 32 | 3 (Tx) | 40 (Tx) |
| Vertical beamwidth | degrees | 40 | 28 | 2 | 30 (Note 10) | 30 (Note 10) | 28 | 6 (Tx) | 25 (Tx) |
| Polarization |  | Vertical | Horizontal | Horizontal | Vertical | Vertical | Horizontal | Horizontal | Horizontal |
| Field of regard | degrees | (Note 11) | (Note 11) | +/−65°az +50°/−40° el | +/−110° to +/−180° | +/−110° to +/−180° | Note 11 | +/−110° az +/−25° el | +/−60° az +/−45° el (Note 11) |
| Horizontal antenna scan (from boresight) | degrees | ±60 | ±60 | ±65 | Not applicable | Not applicable | ±60 | ±60 deg | ±60 deg |
| Vertical antenna scan (from boresight) | degrees | ±20 | ±60 | −40, +50 | Not applicable | Not applicable | ±60 | ± 25 deg | ± 45 deg |
| Notes: 1 Radars with similar transmitter and receiver parameters may also be deployed on the ground, recognizing the need to give priority to airborne detect and avoid radars for remotely piloted aircraft expected to be operated on a worldwide basis. 6 Waveform is software-defined on a CPI-by-CPI basis, and optimized for targets, and spectral environments. 7 Based on the PRI, 8,769-69,917 pulses per second for PRR are more accurate. However, these radars are dynamic/cognitive radars and the PRI/PRR can be dynamically changed based on a number of characteristics (terrain, target, etc.) . Editor’s note: Additional explanations on the PRI/PRR adjustment would be helpful, as well as consistence between PRI/PRR figures in the table. 9 For Radars 1, 2, and 6: Each radar has one TX array and one RX array integrated into their design. For radar 3: High T/R ESA RF beamforming on both transmit and receive. 10 For one panel at ‒3 dB. See section A1-3 for the field of regard. 11 These radars do not have multiple panel capability. Each radar has only one TX array and one RX array integrated into their design. Field ofregard (FOR) is programmable for each individual radar. For an individual radar, FOR can be programmed as wide as ±60°. However, four of these radars can be set up orthogonally to each other to achieve a FOR of 360°.] |  |  |  |  |  |  |  |  |  |

Editor’s note: What about vertical pattern?

General notes to Table A1-1

General Note 1 - “TBD” indicates that the value remains to be determined. “TBC” indicates that the value remains to be confirmed. “N/A” indicates that the parameter is not applicable.

General Note 2 - References to Recommendation ITU-R SM.329 indicate the generic unwanted-emission reference applied in the absence of more specific system information.

## A1-2	Characteristics of aircraft landing system

The aircraft landing system is an electronic landing system that provides flight path data to an approaching aircraft as the aircraft flies into range of the landing system. There are two separate surface transmitters, one for azimuth and one for elevation, as well as a receiver installed on the aircraft. The system utilizes a one-way transmission where the angular information is displayed on a cross-point indicator, allowing the aircraft to align itself with the runway.

The technical and operational characteristics are provided in Table A1-2.

TABLE A1-2

Technical and operational characteristics of aircraft landing system

| Parameter | Units | Transmitter | Receiver |
| --- | --- | --- | --- |
| Platform |  | Land/Ship | Aircraft |
| Platform height | km | Land: 0.01 Ship: 0.015-0.024 | Maximum: 2 |
| Relative to ground speed | m/s | Land: 0 Ship: < 14 | 98 |
| Number of aircraft per landing system |  | 1 | 1 |
| Transmitter |  |  |  |
| Frequency tuning range | GHz | 15.4-15.7 | 15.4-15.7 |
| Emission type |  | Pulse | Not applicable |
| Pulse width | ms | 0.3 | Not applicable |
| Pulse rise and fall times | ns | Rise Time: 25-50; Fall Time: 25-200 | Not applicable |
| RF emission bandwidth at −3 dB −20 dB −40 dB | MHz | 4.8 18.5 65 | Not applicable |
| Pulse repetition frequency | pps | 15 000 | Not applicable |
| Out-of-band emission characteristics | dBc | < 43 | Not applicable |
| Spurious emission characteristics | dBc | 65 | Not applicable |
| Average transmitter power | W | Peak: 2 500; Average: 7 | Not applicable |
| Receiver |  |  |  |
| Receiver IF bandwidth −3 dB −20 dB −60 dB | MHz | Not applicable | 12 17 24 [Editor’s note: this may not be an efficient selectivity mask: to confirm] |
| Sensitivity | dBm | Not applicable | −72 |
| Receiver noise figure | dB | Not applicable | 11.5 |
| Calculated conducted Rx noise power | dBW | Not applicable | −121.7 |
| Image rejection | dB | Not applicable | 60 |
| Spurious rejection | dB | Not applicable | SM.329 |
| Antenna |  |  |  |
| Antenna type |  | Slotted waveguide | Horn |
| Antenna placement |  | [Ground/Surface/Ship] | Bottom of aircraft |
| Antenna gain | dBi | Azimuth: 32; Elevation: 26 | 6 |
| Antenna pattern First antenna side-lobe |  | TBD | TBD |
| Antenna pointing |  | TBD | TBD |
| Antenna azimuth beamwidth | degrees | Horizontal: 40; Vertical: 1.3 | 70 |
| Antenna elevation Vertical beamwidth | degrees | Horizontal: 2; Vertical: 6 | 36 |
| Polarization |  | Vertical | Vertical |
| Antenna scan | degrees | Sector Scan | Fixed |

## A1-3	Airborne detect and avoid radars antenna characteristics

Airborne detect and avoid radars Nos. 4 and 5 use antennas with MIMO (Multiple Input Multiple Output) fix panel. Therefore, these characteristics are complemented to obtain the complete radiation pattern for one panel. One panel does not allow to obtain the requested Field of Regard (FoR). The FoR of the radar refers to the area that has to be covered by the radar and is defined by the azimuth and elevation angles in regard to the platform coordinates. To obtain the requested FoR, it is necessary to use several panels. Typical FoR for DAA is ±110°, and is obtained by the use of 3 panels (each one covering 80°).

Figure A1-3

Typical Field of Regard

]

Single antenna pattern in azimuth

Single antenna pattern in elevation

## A1-4	Protection criteria

For the purpose of sharing and compatibility studies involving systems in other services, an interference-to-noise ratio (I/N) of −6 dB should be used as the protection criterion for airborne detect-and-avoid radars and aircraft landing systems operating in the aeronautical radionavigation service (ARNS) in the frequency band 15.4-15.7 GHz.

This protection criterion should also be applied in the assessment of aggregate interference.

An I/N ratio of −6 dB corresponds to approximately a 1 dB increase in receiver noise power in the relevant receiver bandwidth.

In applying this protection criterion, due consideration should be given to the safety-of-life nature of ARNS systems.

__________________

____________________

The criterion of protection does not include an aeronautical safety margin.

This criterion does not include a safety margin.

[Some of these radars may also be used for ground-based radars for detect and avoid operations]

There are two transmit antenna systems one for azimuth and one for elevation.