| U.S. Radiocommunications Sector Fact Sheet |  |
| --- | --- |
| Working Party: ITU-R WP-5B | Document No: USWP5B37-12 |
| Ref: Annex 1.6 to Document 5B/526-E | Date: XX November 2026 |
| Document Title: WORKING DOCUMENT TOWARDS A PRELIMINARY DRAFT NEW RECOMMENDATION ITU-R M.[24.45-24.65_GHZ_ARNS]- Characteristics of and protection criteria for radars operating in the radionavigation service in the frequency band 24.45-24.65 GHz |  |
| Author(s)/Contributors(s): Jae Shin Federal Aviation Administration 800 Independence Ave., S.W. Washington, DC 20591 Don Nellis Federal Aviation Administration 800 Independence Ave., S.W. Washington, DC 20591 | Phone: (202) 267-7365 e-mail: jae.w.shin@faa.gov Phone: (202) 267-9779 e-mail: Donald.Nellis@faa.gov |
| Purpose/Objective: The purpose of this contribution is to develop a new recommendation for aeronautical radionavigation systems, including unmanned aircraft systems (UAS) Detect and Avoid (DAA) radar systems, in the 24.45-24.65 GHz band. This contribution will be an update to the new report found in Annex 1.6 of Document 5B/526-E. |  |
| Abstract: This contribution is a new recommendation for UAS Detect and Avoid (DAA) systems that operate in the 24.45-24.65 GHz Aeronautical Radionavigation Service allocation. This contribution contains characteristics and protection criteria for UAS DAA radar that can be used both on airborne and ground platforms. |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 5B/526 (Annex 1.6) | Document 5B/XXX-E |
|  | XX November 2026 |
|  | English only |
| United States of America. WORKING DOCUMENT TOWARDS A PRELIMINARY DRAFT NEW RECOMMENDATION ITU-R M.[24.45-24.65_GHZ_ARNS]. Characteristics of and protection criteria for radars operating in the radionavigation service in the frequency band 24.45-24.65 GHz. Introduction TBD Proposal TBD |

Annex 1.6 to Working Party 5B Chair’s Report WORKING DOCUMENT TOWARDS A PRELIMINARY DRAFT NEW RECOMMENDATION ITU-R M.[24.[25]24.65_GHZ_ARNS]

Characteristics of and protection criteria for detect and avoid radars used for aeronautical applications in the radionavigation service in the frequency band 24.[25]-24.65 GHz in Regions 2 and 3

(202X)

Scope

This Recommendation specifies the technical [and operational] characteristics and protection criteria of detect and avoid (DAA) radars [used for aeronautical applications] in the radionavigation service (RNS) in the frequency band 24.[25]-24.65 GHz. The characteristics should be used  for sharing and compatibility studies with systems of other services in Regions 2 and 3].]

Keywords

24.[25]-24.65 GHz, radionavigation service, aeronautical radar, detect and avoid, characteristics, protection criterion.

Abbreviations/Glossary

DAA	Detect and avoid

ESA	Electronically scanned array

FMCW	Frequency-modulated continuous wave

RCS	Radar cross-section

RNS	Radionavigation service

RR	Radio Regulations

SNR	Signal-to-noise power ratio

UA	Unmanned aircraft

UAS	Unmanned aircraft system

Related ITU Recommendations and Reports

Recommendations

ITU-R M.1372	Efficient use of the radio spectrum by radar stations in the radiodetermination service

ITU-R M.1461	Procedures for determining the potential for interference between radars operating in the radiodetermination service and systems in other services

[ITU-R M.1851	Mathematical models for radiodetermination radar and aeronautical mobile systems antenna patterns for use in interference analyses]

Reports

ITU-R M.2204	Characteristics and spectrum considerations for sense and avoid systems use on Unmanned Aircraft Systems (UAS)

The ITU Radiocommunication Assembly,

considering

a)	that the technical and operational characteristics of detect and avoid radars used for aeronautical applications in the radionavigation service (RNS) are driven by the performance requirements and depend on the frequency band;

b)	that this frequency band is used for Airport Surface Detection Equipment radars in some countries in Region 3,

recognizing

a)	that the frequency band 24.25-24.45 GHz is allocated on a primary basis to the radionavigation, fixed, and mobile except aeronautical mobile services in ITU-R Region 2;

b)	that the frequency band 24.25-24.45 GHz is allocated on a primary basis to the radionavigation, fixed, and mobile services in ITU-R Region 3;

c)	that the frequency band 24.45-24.65 GHz is allocated on a primary basis to the radionavigation, fixed, mobile except aeronautical mobile, and inter-satellite services in ITU-R Region 2;

d)	that the frequency band 24.45-24.65 GHz is allocated on a primary basis to the radionavigation, inter-satellite, fixed, and mobile services in ITU-R Region 3;

e)	that the frequency band 24.45-24.65 GHz is not allocated to radionavigation service in Region 1;

f)	that the radionavigation service is a safety service for which  No. 4.10 of the Radio Regulations (RR) applies;

g)	that some interference suppression techniques between radars are contained in Recommendation ITU-R M.1372 “Efficient use of the radio spectrum by radar stations in the radiodetermination service”;

h)	that the inter-satellite service operating in the frequency band 24.45-24.65 GHz shall not claim protection from harmful interference from airport surface detection equipment stations of the radionavigation service according to RR No. 5.533,

recommends

1	that the technical [and operational] characteristics of detect and avoid radars [used for aeronautical applications] in the radionavigation service described in the Annex should be considered representative of those operating in the frequency band 24.[25]-24.65 GHz and used in sharing and compatibility studies with systems in other services;

2	that, for sharing and compatibility studies, an interference-to-noise ratio  [(I/N) of −6 dB should be used as the required protection criterion] for detect and avoid radars used for aeronautical applications in the radionavigation service in the frequency band 24.[25]-24.65 GHz, and that this represents the aggregate protection level if multiple interferers are present.

Annex

Technical [and operational] characteristics of detect and avoid radars [used for aeronautical applications] in the radionavigation service in the 
frequency band 24.[25]-24.65 GHz

# A-1	Introduction

Radars used for aeronautical applications operate in the radionavigation service in Regions 2 and 3 on a primary basis in the frequency band 24.[25]-24.65 GHz. This annex presents the technical and operational characteristics of detect and avoid radars used for aeronautical applications in this frequency band.

These radars  may be installed in unmanned aircraft (UA) or on the ground to detect non-cooperative aircraft as a component of an UA Detect and Avoid (DAA) system. Such radars are used for collision avoidance functions on-board UA and can be used as a part of the integration of unmanned aircraft system (UAS). The characteristics provided in this annex should be used in sharing and compatibility studies with stations of other services.

# A-2	Characteristics of radionavigation detect and avoid radars

## A-2-1	System description

…

### A-2-1.4	Multistatic radar (Radar 4)

Overview

Multistatic radar operates using multiple transmitters and receivers. The position of the target can be calculated independently at each transmitter–receiver pair. Alternatively, the distances to the moving target estimated at each transmitter and receiver can be compared with those from other stations, allowing the target position to be estimated using methods such as Time Difference of Arrival (TDOA). Unlike monostatic radar systems, multistatic radar systems can detect moving targets in shadowed regions by deploying multiple transmitters and receivers (see Fig. 2-1.4).

Signal waveform

In the civil aviation field, radar systems typically use pulse signals, which require high transmission power. In contrast, multistatic radar systems employ FMCW signals to reduce the required transmission power and achieve wide-area coverage from the near field to the far field based on the separation of transmitter and receiver.

Configuration

In this radar concept, the radar system is deployed as a ground-based radar system. Multiple ground-based transmitters and receivers are assumed to operate either through network links or in a stand-alone configuration. The integration of information from these stations enables the detection of multiple targets.

FIGURE 2-1.4

Conceptual diagram of multistatic radar

## A-2-2	Technical parameter

Technical [and operational] characteristics of detect and avoid radars used for aeronautical applications operating in the frequency band 24.45-24.65 GHz are provided in Table A-1. These parameters should be used in compatibility and sharing studies with stations of other services, and where necessary, conservative assumptions reflecting the operational characteristics of the radar should be considered.

Editor’s Note: Re-evaluate use of “operational characteristics” at a future meeting to determine if any operational characteristics are provided as inputs or if this can be deleted.

TABLE A-1

Technical [and operational] characteristics of detect and avoid radars

| Parameter | Units | Radar 1 (Note 1) | Radar 2 (Note 1) | Radar 3 (Note 1) | Radar 4 (Note 9) |
| --- | --- | --- | --- | --- | --- |
| Platform |  | Airborne/Ground | Airborne/Ground | Airborne/Ground | Ground |
| Platform height | m | Up to 3 000 / 20 000 for airborne DAA Up to 30 for ground-based DAA | Up to 3 000 / 20 000 for airborne DAA Up to 30 for ground-based DAA | Up to 3 000 / 20 000 for airborne DAA Up to 30 for ground-based DAA | Up to [150] for ground-based DAA |
| Radar type |  | FMCW | FMCW | FMCW | FMCW |
| Detection range | km | 6 | 6 | 3.5 | 3.5 |
| [] |  | [] | [] | [] | [] |
| Ground relative speed [(Note 2)] | m/s | 120 | 120 | 40 (for typical waveform) | N/A |
| Target speeds (max) | m/s | 200 | 200 | 94 (for typical waveform) | 200 |
| Transmitter |  |  |  |  |  |
| Frequency range | GHz | 24.45-24.65 | 24.45-24.65 | 24.45-24.65 | 24.25-24.65 |
| Channel selection method between radars [(Note 4)] |  | Software selectable | Software selectable | Software selectable | single channel |
| RF bandwidth | MHz | 45 | 10 to 50 [(Note 4)] | 20 or 40 | Less than 400 (e.g. 400 or 200 or 100 or 50) |
| Pulse width | ms | 200 | 50 to 200 (Note 5) | 33 (chirp time) | N/A |
| Pulse rise and fall times | ms | 1 | 1 | 0.1 | N/A |
| RF emission bandwidth at −3 dBc −20 dBc −40 dBc | MHz | 47.1 55 75 | 47.1 55 75 | 37 40.75 42.9 | TBD |
| Pulse repetition frequency | Hz | 47 000 | 2 000 to 10 000 (Note 4) | 303 000 | N/A |
| Pulse repetition interval | ms | 5 (Note 6) | 5 (Note 6) | 33 | N/A |
| Mean power at transmitter antenna port Editor’s Note: Confirm if Mean power or peak envelope power should be used | W | [2] TBC | [3.2] TBC | [0.8] TBC | [1.0] TBC |
| Out-of-band emission characteristics [Editor’s note: to harmonize with RF emission bandwidth at −3 dBc] [Editor’s Note: Review this as OOB should be next to desired and not 4th harmonic] | dBc [Editor’s Note: Harmonize the units for this row] | Measured at 84 dBuV/m at 3 m through 4th harmonics | Measured at 84 dBuV/m at 3 m through 4th harmonics | 70 | 60 |
| Spurious emission characteristics (conducted) [Editor’s Note: Review this along with OOB emissions on previous line] | dBc | −65 | −65 | −84 | 60 |
| Receiver |  |  |  |  |  |
| Receiver IF bandwidth −3 dB −20 dB −60 dB | MHz | 9.8 20 60 | 9.8 20 60 | 48.5 65 78 | TBD |
| Sensitivity (Minimum Detectable Signal (at RX input. SNR = 12 dB) | dBm | −131 | −131 | −141 | -103 |
| Receiver noise figure | dB | 6 | 6 | 6.5 | 3 |
| Saturation level | dBm | −40 | −40 | −50 | TBD |
| Antenna |  |  |  |  |  |
| Antenna type |  | ESA (Note 8) | ESA (Note 8) | Phased array (Note 8) | Phased array, sector |
| Antenna pattern |  | Electronically Scanned Face with a ± 60º horizontal and ± 40º vertical scan | Electronically Scanned Face with a ± 60º horizontal and ± 40º vertical scan | Electronically Scanned Face | Front panel |
| Antenna pointing |  | N/A | N/A | N/A | TBD |
| First antenna sidelobe | dBi | 16 | 16 | 5 | TBD |
| Horizontal beamwidth | degrees | 24 | 24 | 28 | N/A |
| Vertical beamwidth | degrees | 24 | 24 | 28 | TBD |
| Polarization |  | Horizontal | Horizontal | Vertical | 17 |
| Field of Regard | degrees | Airborne DAA: −40/40 el −60/60 az Ground based DAA: −40/40 el −60/60 az | Airborne DAA: −40/40 el −60/60 az Ground based DAA: −40/40 el −60/60 az | ±118 Editor’s note: Confirm if it applies in azimuth and if it applies to both airborne and ground based DAA. Specify the elevation field of regards. | 11 |
| Antenna pattern |  | Electronically Scanned Face with a ± 60º horizontal and ± 40º vertical scan | Electronically Scanned Face with a ± 60º horizontal and ± 40º vertical scan | Electronically Scanned Face | Horizontal/Vertical |
| Antenna pointing |  | N/A | N/A | N/A | ±45 |
| Horizontal antenna scan (from boresight) | degrees | ±60 | ±60 | ±45 Editor’s note: Clarify how this value relates to the line above. | ±45 |
| Vertical antenna scan (from boresight) | degrees | ±40 | ±40 | ±45 | N/A |
| 1 The characteristics in Table A-1 may represent airborne or ground-based DAA deployments; however, use of the frequency band 24.45-24.65 GHz for airborne DAA applications is constrained by the non-worldwide allocation of the band 2 Channel selection is purely software-defined and can be changed on-the-fly. Some settings may allow radar to self-configure based on detected spectrum-conflict. 3 The radar may employ software-defined waveforms that vary during operation according to target characteristics and radio-frequency conditions. For sharing and compatibility studies, representative operating configurations should be used. 4 The radar is dynamic/cognitive radar and the PRI/PRR can be changed dynamically based on a number of characteristics (terrain, target, etc.). The PRR/PRI can be higher with future software revisions for specific mission sets. Editor’s note: Additional explanations on the PRI/PRR adjustment would be helpful, as well as consistence between PRI/PRR figures in the table. 5 Compressed bandwidth before processing gain. 6 High T/R ESA RF beamforming on both transmit and receive. 7 This radar system is a multistatic radar employing multiple ground-based transmitters and receivers that operate using continuous radio waves. |  |  |  |  |  |

# A-3	Protection criteria

For the purpose of sharing and compatibility studies involving systems in other services, [an interference-to-noise ratio (I/N)] of −6 dB should be used as the protection criterion for detect and avoid (DAA) radars used for aeronautical applications operating in the radionavigation service (RNS) in the frequency band 24.[25]-24.65 GHz.

This protection criterion should also be applied in the assessment of aggregate interference.

[An I/N ratio of −6 dB corresponds to approximately a 1 dB increase in receiver noise power in the relevant receiver bandwidth.]

In applying this protection criterion, due consideration should be given to the safety-of-life nature of RNS systems.

___________________

____________________

The criterion of protection does not include an aeronautical safety margin.