| U.S. Radiocommunications Sector Fact Sheet |  |
| --- | --- |
| Working Party: ITU-R WP 5C | Document No: USWP5C35-02 |
| Ref: Document 5C/322 (Annex 2.2) | Date: September 14, 2026 |
| Document Title: WORKING DOCUMENT ON SHARING STUDIES UNDER WRC-27 AGENDA ITEM 1.10 |  |
| Author(s)/Contributors(s): Zahid Islam SpaceX Joe McMichael SpaceX Nader Damavandi SpaceX Jameson Dempsey SpaceX | Email: km.islam@spacex.com Email: Joseph.McMichael@spacex.com Email: Nader.Damavandi@spacex.com Email: jameson.dempsey@spacex.com |
| Purpose/Objective: Add studies to the Working Doc on AI 1.10, i.e. 5C/322 (Annex 2.2) |  |
| Abstract: The last WP5C meeting in May 2026 finalized the CPM text for AI 1.10. The US proposed mask made it to the options. The US NGSO mask is equivalent to the one proposed by CEPT. It is methods A3 and A4 of the CPM. Studies showed how the proposed NGSO mask would protect FS victims with an elevation of 10 deg. With this input, the US will perform a very similar study as the one performed by CEPT. Additionally, another study is presented, which uses the “compromise” NGSO mask that is supposed to represent a solution to have a single mask at CPM in April 2027 and that the US can support. |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Received: TBD November 2026 Source: Document 5C/322 (Annex 2.2) Subject: AI 1.10 | Document TBD |
|  | Date: |
|  | English only |
| United States |  |
| WORKING DOCUMENT ON SHARING STUDIES UNDER AGENDA ITEM 1.10 |  |

The last WP5C meeting in May 2026 finalized the CPM text for AI 1.10. The US-proposed NGSO mask is contained method A4 of the CPM text. This mask is equivalent to the one proposed by CEPT and included in method A3. With this input, the US performed a study with this mask, showing protection for FS victims at 10 deg elevation. Additionally, a new study is performed using the so-called compromise mask, i.e. a NGSO mask initially discussed during the 5C meeting in May 2026, which has the potential of meeting the needs of all interested stakeholders. Such a mask could be adopted as a single NGSO mask option already during CPM in April 2027.

# Study #YZ

This study considers FS characteristics as provided in doc 5C/322 (Annex 2.2) and assuming 10 degrees as maximum elevation. With respect to the parameters of non-GSO systems (including minimum elevation, GSO exclusion zone, and max number of co-frequency beams at a specific location), the analysis considered cases with different operational conditions than those communicated by 4A as described below. The various assumptions different from the 4A liaison statement are on purpose all worse than the parameters communicated by 4A. That said, the assumptions remain within the envelope of parameters agreed as high-level guidance in November 2025 by WP5C. Additionally, the methodology considers scenarios with propagation characteristics as per Doc. 5C/74 (guidance provided by WP 3J and 3M).

The statistics of the interference power against time for each power flux density (PFD) mask specified is calculated at each timestep using the method described below:

1)	Collocate an FSS earth station (ES) with the terrestrial FS station.

2)	Based on the parameters of the selected non-GSO systems, determine number of visible satellites to the FS station,  Nv (satellites above an elevation angle of 0°).

3)	Determine the pool of eligible satellites for each of the selected non-GSO systems complying with the system-specific minimum elevation angle, ε, and minimum GSO exclusion angle, α at the ES location.

4)	Select maximum number of non-GSO satellites allowed to transmit with overlapping frequencies towards the same location on the ground, Nco, using random satellite selection strategy. Determine the contribution from the Nco satellites to the FS station.

INco= PFD-10log104πλ2- La+Gα            (eq. 1)

PFD is the assumed power flux density at the Earth’s surface in dB(W/m2/MHz) under free-space condition

λ is the wavelength in meters

La is the atmospheric loss (gaseous attenuation) experienced by non-Nco  satellites of NGSO link when performing dynamic simulation. Such propagation loss should be considered based on ITU-R Recommendations ITU-R P.676 when applicable.

Gα is the gain of the FS receiving antenna at off-axis angle α degrees

5)	For the remaining visible  Nv- Nco satellites (also known as non-Nco satellites), determine contribution of the sidelobes towards the FS station assuming random placement of the beams from the satellite with its field of view.

Inon-Nco= EIRPθ-20log104πdλ- La +Gα            (eq. 21)

EIRPθ is the assumed effective isotropic radiated density from the non-Nco satellite to the FS station direction in dB(W/MHz) at off-axis angle θ degrees. Peak EIRP of the non-Nco satellite is calculated for that satellite to maintain the PFD under free-space condition according to the proposed PFD mask given a randomly selected scan angle within its eligible scan angles.

λ is the wavelength in meters

d is the distance between the non-Nco satellite and FS station in meters

La is the atmospheric loss (gaseous attenuation) experienced by non-Nco  satellites of NGSO link when performing dynamic simulation.

Gα is the gain of the FS receiving antenna at off-axis angle α degrees

6)	Aggregate power levels received at the FS station by combining the received interference power from all Nv satellites of the non-GSO systems.

7)	Aggregate power levels received at a FS station by combining the received interference power from the selected non-GSO systems. The FS station effective system noise temperature is also increased with atmospheric attenuation according to equation (69) in ITU-R P.618-14 and ITU-R P.676-13 Section 4, when atmospheric effect (gaseous attenuation) is considered in simulation.

## Parameters of the Selected non-GSO Systems

The selected parameters of the selected four non-GSO systems are shown in Table 1. According to the WP4A guidance, 1.5° minimum angular separation is used among the three non-GSO systems’ beams serving the same location on Earth.

Table 1

Parameters of the Selected GSO and non-GSO Systems (as per 4A guidance)

| GSO/Non-GSO System | Height (km) | Number of planes | Satellites per plane | Inclination angle (deg) | Number of co-frequency beams, Nco | Min. GSO Exclusion angle, α |
| --- | --- | --- | --- | --- | --- | --- |
| System-B (non-GSO) | 590, 610, 630 | 28, 36, 34 | 28, 36, 34 | 33, 42, 51.9 | 32 | 1° |
| System-D (non-GSO) | 1050 | 12 | 28 | 89 | 8 (assumed) | 1° |
| System-L (non-GSO) | 500, 500, 600, 600, 700, 700, 800, 800, 900, 900, 1000, 1000, 1100, 1100, 1200, 1200, 1300, 1300, 1400, 1400, 8100, 8100, 8100, 8100, 8100, 8100, 8100, 8100, 12000, 12000, 12000, 12000, 12000, 12000, 12000, 12000, 16000, 16000, 16000, 16000, 16000, 16000, 16000, 16000, 20000, 20000, 20000, 20000, 20000, 20000, 20000, 20000, 23222, 23222, | 36, 36, 36, 36, 34, 34, 30, 30, 28, 28, 24, 24, 24, 24, 22, 24, 20, 20, 18, 18, 1, 12, 12, 12, 12, 12, 12, 12, 1, 12, 12, 12, 12, 12, 12, 12, 1, 12, 12, 12, 12, 12, 12, 12, 1, 12, 12, 12, 12, 12, 12, 12, 1, 12 | 36, 36, 32, 32, 32, 32, 32, 32, 30, 30, 24, 24, 24, 24, 24, 24, 24, 24, 20, 20, 96, 10, 10, 10, 10, 10, 10, 10, 96, 10, 10, 10, 10, 10, 10, 10, 96, 10, 10, 10, 10, 10, 10, 10, 96, 10, 10, 10, 10, 10, 10, 10, 96, 9 | 50, 85, 50, 85, 50, 85, 50, 85, 50, 85, 50, 85, 50, 85, 50, 89, 50, 85, 50, 85, 0, 15, 45, 60, 65, 70, 75, 80, 0, 15, 45, 60, 65, 70, 75, 80, 0, 15, 45, 60, 65, 70, 75, 80, 0, 15, 45, 60, 65, 70, 75, 80, 0, 56 | 10 (assumed) | 1° |
| System-M (non-GSO) | 340, 345, 350, 360, 525, 530, 535, 604, 614 | 12, 18, 48, 48, 48, 30, 28, 28, 28 | 110, 110, 110, 120, 120, 120, 120, 12, 18 | 53, 46, 38, 97, 53, 43, 33, 148, 116 | 40 (assumed) | 1° |

This study considers the following operational conditions for the GSO and non-GSO systems:

- There is a total of 18 identical GSO systems along with the three non-GSO systems. The satellites are assumed to be spaced on the GSO arc every 3° between 22.5°W to 28.5°E.

- Nco of System-M is assumed to be 40.

## PFD masks

Table 2 lists the pfd mask used in the Study #YZ for the selected non-GSO systems. This is the identical PFD mask proposed by US in 5C/288 as method for the non-GSO FSS to satisfy the agenda item. All satellites that are in view of the FS station are assumed to be possibly interfering satellites for this study. The Nco satellites transmitting towards the location of FS station are randomly selected from each set of prospective transmitting satellites.

Table 2

PFD masks for the Selected non-GSO systems

| PFD in dB(W/m2) for angles of arrival (δ) above the horizontal plane |  |  |  | Reference Bandwidth |
| --- | --- | --- | --- | --- |
| 0°-5° | 5°-12° | 12°-15° | 15°-90° |  |
| -125 | -125+1.43(δ-5) | -115+3.33(δ-12) | -105 | 1 MHz |

## FS Link Characteristics

FS link characteristics has been used following WP 5C guidance as listed in Table 3.

Table 3

FS Link Characteristics

| Parameters | Specifications |
| --- | --- |
| Frequency (GHz) | 73.5 |
| FS Antenna maximum Receive Gain (dBi) | 41.5 (0.2 m), 51(0.6 m) |
| FS Antenna Pattern | Rec. ITU-R F.1245-3, EN 302 217-4 Class-3 |
| Latitude (degrees) | 24° N, 45° N and 60° N |
| Longitude (degrees) | 3° E |
| Elevation Angles | 10° |
| Receiver Noise Figure (dB) | 8 |
| Antenna height(m) above mean sea level (AMSL) | 30 |
| Above mean sea level (AMSL) (m) | 24° N, 3° E: 528 m 45° N, 3° E: 1014 m 60° N, 3° E: 0 m |

The protection criteria for FS safeguard used in this analysis are:

a	Long-term:   I/N should not exceed –10 dB for more than 20% of the time (derived from Recommendation ITU-R F.758-8)

b	Short-term:  I/N should not exceed +11 dB for more than 0.00128% of the time in any month.

## Results of study # XZ

### ITU-R F.1245-3 FS station antenna pattern

Figure 1-6 shows aggregate I/N from the selected four non-GSO systems for

the ITU-R F.1245-3 FS station antenna pattern,

pointing at two different azimuth directions (results can be provided with 20° intervals upon request)

at a representative frequency (i.e., 73.5 GHz)

Figure 1

Aggregate I/N from Selected Systems at 24°N, Azimuth 0°

Figure 2

Aggregate I/N from Selected Systems at 24°N, Azimuth 180°

Figure  3

Aggregate I/N from Selected Systems at 45°N, Azimuth 0°

Figure 4

Aggregate I/N from Selected Systems at 45°N, Azimuth 180°

Figure  5

Aggregate I/N from Selected Systems at 60°N, Azimuth 0°

Figure 6

Aggregate I/N from Selected Systems at 60°N, Azimuth 180°

### EN 302 217-4 Class-3FS station antenna pattern

Similarly, Figure 7-12 shows aggregate I/N from the same selected four non-GSO systems for

the EN 302 217-4 Class-3 FS station antenna pattern,

pointing at two different azimuth directions (results can be provided with 20° intervals upon request)

at same representative frequency (i.e., 73.5 GHz)

Figure  7

Aggregate I/N from Selected Systems at 24°N, Azimuth 0°

Figure 8

Aggregate I/N from Selected Systems at 24°N, Azimuth 180°

Figure  9

Aggregate I/N from Selected Systems at 45°N, Azimuth 0°

Figure 10

Aggregate I/N from Selected Systems at 45°N, Azimuth 180°

Figure  11

Aggregate I/N from Selected Systems at 60°N, Azimuth 0°

Figure 12

Aggregate I/N from Selected Systems at 60°N, Azimuth 180°

# Study #ZZ

At the May 2026 meeting of WP 5C, an additional non-GSO PFD mask was informally discussed among the interested administrations and industry members. Table 4 presents this mask. Study #ZZ uses NGSO PFD mask while keeping everything else as in Study #YZ; the methodology in steps 1)–7), the selected non-GSO systems and operational conditions (Table 1), the FS link characteristics (Table 3), and the long-term and short-term FS protection criteria are unchanged.

Table 4

Compromise non-GSO PFD mask initially discussed at the May 2026 WP 5C meeting

| PFD in dB(W/m2) for angles of arrival (δ) above the horizontal plane |  |  |  | Reference Bandwidth |
| --- | --- | --- | --- | --- |
| 0°-5° | 5°-10° | 10°-15° | 15°-90° |  |
| -125 | -125+0.8(δ-5) | -121+3.2(δ-10) | -105 | 1 MHz |

## Results of study # ZZ

### ITU-R F.1245-3 FS station antenna pattern

Figure 13-18 shows aggregate I/N from the selected four non-GSO systems for

the ITU-R F.1245-3 FS station antenna pattern,

pointing at two different azimuth directions (results can be provided with 20° intervals upon request)

at a representative frequency (i.e., 73.5 GHz)

Figure 13

Aggregate I/N from Selected Systems at 24°N, Azimuth 0°

Figure 14

Aggregate I/N from Selected Systems at 24°N, Azimuth 180°

Figure 15

Aggregate I/N from Selected Systems at 45°N, Azimuth 0°

Figure 16

Aggregate I/N from Selected Systems at 45°N, Azimuth 180°

Figure 17

Aggregate I/N from Selected Systems at 60°N, Azimuth 0°

Figure 18

Aggregate I/N from Selected Systems at 60°N, Azimuth 180°

### EN 302 217-4 Class-3FS station antenna pattern

Similarly, Figure 19-24 shows aggregate I/N from the same selected four non-GSO systems for

the EN 302 217-4 Class-3 FS station antenna pattern,

pointing at two different azimuth directions (results can be provided with 20° intervals upon request)

at same representative frequency (i.e., 73.5 GHz)

Figure 19

Aggregate I/N from Selected Systems at 24°N, Azimuth 0°

Figure 20

Aggregate I/N from Selected Systems at 24°N, Azimuth 180°

Figure 21

Aggregate I/N from Selected Systems at 45°N, Azimuth 0°

Figure 22

Aggregate I/N from Selected Systems at 45°N, Azimuth 180°

Figure 23

Aggregate I/N from Selected Systems at 60°N, Azimuth 0°

Figure 24

Aggregate I/N from Selected Systems at 60°N, Azimuth 180°

## Conclusion

Both studies #YZ and #ZZ show protection of FS victims at 10 deg is ensured. Moreover, study #ZZ using the “compromise” mask presented informally in May 2026 introduces additional protection with respect to the NSGO mask in methods A3/A4.

____________________

Sidelobe radiation pattern for each of the visible  Nv- Nco satellites of a non-GSO system have been calculated conservatively as per Recommendation ITU-R S.1528 recommends 1.2. In some other working parties’ studies, Recommendation ITU-R S.1528 recommends 1.4 has been considered for sidelobe radiation pattern. The satellite antenna transmit peak gain of  48 dBi, 48 dBi, 51.1 dBi and 52 dBi have been assumed for System B, System D, System L and System M, respectively, with L_n = -20 dB.

For interference analysis, minimum elevation angle 0° have been considered for the non-GSO systems.