| U.S. Radiocommunications Sector Fact Sheet |  |
| --- | --- |
| Working Party: ITU-R WP 5C | Document No: USWP5C-03 |
| Ref: Recommendation F.758 | Date: October 1, 2026 |
| Document Title: WORKING DOCUMENT towards a preliminary draft revision of recommendation F.758 |  |
| Author(s)/Contributors(s): Zahid Islam SpaceX Joe McMichael SpaceX Nader Damavandi SpaceX Rohit Iyer Seshadri SpaceX | Email: km.islam@spacex.com Email: Joseph.McMichael@spacex.com Email: Nader.Damavandi@spacex.com Email: rohit.iyerseshadri@spacex.com |
| Purpose/Objective: Submit studies to justify starting the work towards a new update of Recommendation F.758 |  |
| Abstract: In the recent past the United States started a ground-breaking work at ITU level to take a fresh look at the key Recommendation for most relevant services. The vast majority of these Recommendations have either not been updated since the ‘90s or they were updated but keeping the same (or close to the same) protection criteria from the ‘90s. Recommendation F.758 is certainly part of that party. It is actually one of the very first Recommendations that started explicitly mentioning the “magic” numbers of -6 dB and -10 dB I/N. The goal is to show technically sound evidence demonstrating how conservative are existing criteria in F.758, and also to consider modern Fixed Service links using Adaptive Coding and Modulation (ACM), which in the modern world should represent the vast majority of operated links. For links using ACM, certainly an I/N metric is inappropriate and antiquated, while a more modern, efficient and appropriate metric would be throughput degradation for long-term coupled with an absolute increase in unavailability for short-term. With respect to the links that still in 2026 do not use ACM, I/N can still be considered, but certainly a value of -10 dB I/N for 80% of the time is remarkably conservative, as these studies show, and should be modernized. |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Received: TBD November 2026 Source: Recommendation F.758 Subject: Recommendation F.758 | Document TBD |
|  | Date: |
|  | English only |
| United States |  |
| WORKING DOCUMENT towards a preliminary draft revision of recommendation F.758 |  |

During this study cycle the United States initiative work on several fronts and for several services with a view to modernize key ITU-R Recommendations that either come from the ‘90s or contain protection criteria that have not been updated since the ‘90s. In general, given the tremendous advancements in technology of the past 30 years, it is necessary to take a fresh look at all these key Recommendation. Recommendation F.758 is certainly part of them. It is actually one of the very first Recommendations that started explicitly mentioning the most used criteria of -6 db and -10 dB I/N, which eventually started spreading to almost all services after 1997.

The first draft contained a study based on similar static analysis that showed that the relative degradation framework of ITU-R F.758-8 can make interference effects appear more significant than the actual service impact, particularly for modern links.  The static analysis performed in the first draft on currently registered fixed-service links, equipped with modern radio capabilities, operating in E-band, indicated that the actual service impact to these links, in terms of absolute increase in link unavailability and additionally in terms of percentage degradation in average throughput for links with ACM was comparatively small.  In the second draft, we provided details of the weakest (E-band) links with and without ACM that were examined as part of the analysis in the first draft. In this draft, we repeat the static analysis applied previously to representative fixed-service links operating in Ku and Ka bands.

## E-band FS Link Characteristics

Summarized below are relevant parameters of over 1000, currently registered fixed-service links operating the in E-band, that were considered in the first draft.  Figure 1 plots the CDF of margins of upwards of 1000 links with ACM (left) and without ACM (right).  Figure 2 shows the CDF of path lengths for the same set of links.  Table 1 records statistics of the examined links with ACM. Table 2  records statistics of links without ACM.

Figure 1

Figure 2

Table 1:  ACM

| Parameter | Mean | Median | P90 |
| --- | --- | --- | --- |
| Margin (dB) | 43.4 | 42.3 | 55.1 |
| Path length (km) | 4.2 | 3.3 | 8.8 |
| R001 mm/hr | 34.3 | 26.1 | 53.1 |

Table 2: Non-ACM

| Parameter | Mean | Median | P90 |
| --- | --- | --- | --- |
| Margin (dB) | 41.7 | 38.5 | 58.4 |
| Path length (km) | 12.8 | 11.5 | 21.5 |
| R001 mm/hr | 40.1 | 41.5 | 42.3 |

As an illustrative example, Table 3 documents the parameters of two links with ACM. Path ID  43629 is the link that was displayed in the first draft. Path ID 44875 was found to be worst case among links considered in the first draft.

Table 3

FS Link Parameters (ACM)

| Parameters | Path ID 43629 | Path ID 44875 |
| --- | --- | --- |
|  | First-draft Table 1 ACM example | Worst case ACM link |
| Site 1 | Alfredo Andrews (VI) | Gilbert (IA) |
| Site 2 | Blue Mountain (VI) | Boone (IA) |
| Frequency (GHz) | 83.5 | 83.5 |
| Path length (km) | 2.81 | 24.69 |
| FS antenna maximum receive gain (dBi) | 44.6 | 51.06 |
| Site 1 latitude | 17.736083° N | 42.118639° N |
| Site 1 longitude | 64.781639° W | 93.599333° W |
| Site 2 latitude | 17.754917° N | 42.061778° N |
| Site 2 longitude | 64.799389° W | 93.887806° W |
| Receiver noise figure (dB) | 7.89 | 10 |
| Antenna height AGL Site 1 (m) | 4.15 | 50.29 |
| Antenna height AGL Site 2 (m) | 15.24 | 33.53 |
| Ground AMSL Site 1 (m) | 38.95 | 312.39 |
| Ground AMSL Site 2 (m) | 325.8 | 342.53 |
| Antenna AMSL Site 1 (m) | 43.1 | 362.68 |
| Antenna AMSL Site 2 (m) | 341.0 | 376.06 |
| ACM min / max modulation (listing) | QPSK / 256 QAM | QPSK / 128 QAM |
| Channel / emission (listing) | 250 MHz / 250MD7W | 2000 MHz / 2G00D7W |
| ACM min-mod RSL (listing) (dBm) | −34.10 | −40.06 |
| ACM max-mod RSL (listing) (dBm) | −42.10 | −48.06 |
| Threshold / required RSL highest mode (listing) (dBm) | −55.5 | −50.5 |
| RSS QPSK (Aviat WTM 4800, BER 10⁻⁶) (dBm) | −74.0 | −65.5 |
| Margin QPSK = min-mod RSL − RSS_QPSK (dB) | 39.90 | 25.44 |
| Margin highest mode = max-mod RSL − Threshold (dB) | 13.40 | 2.44 |

Table 4 documents two links without ACM. Path ID 30870 was listed in the first draft and Path ID 6972    was found to be worst case link among the non-ACM links examined in the first draft.

Table 4

FS Link Parameters (No ACM)

| Parameters | Path ID 30870 | Path ID 6972 |
| --- | --- | --- |
| Role | First-draft Table 1 no-ACM example | Worst no-ACM link |
| Site 1 | 755W (NJ) | SWDL (PA) |
| Site 2 | Hilton (NJ) | WLMSPTPA3 (PA) |
| Frequency(GHz) (Receiver at Site2) | 71.5 | 84.125 |
| Path length (km) | 3.18 | 118.57 |
| FS antenna maximum receive gain (dBi) | 54.0 | 50.8 |
| Site 1 latitude | 40.777278° N | 40.199253° N |
| Site 1 longitude | 74.069333° W | 77.430528° W |
| Site 2 latitude | 40.753083° N | 41.233122° N |
| Site 2 longitude | 74.045167° W | 77.079700° W |
| Receiver noise figure (dB) | 8 | 7 |
| Antenna height AGL Site 1 (m) | 19.81 | 36.58 |
| Antenna height AGL Site 2 (m) | 61.87 | 36.58 |
| Ground AMSL Site 1 (m) | 2.01 | 164.68 |
| Ground AMSL Site 2 (m) | 1.62 | 168.6 |
| Antenna AMSL Site 1 (m) | 21.8 | 201.3 |
| Antenna AMSL Site 2 (m) | 63.5 | 205.2 |
| R0.01 (ITU-R P.837) Site 1 (mm/h) | 41.5 | 39.4 |
| R0.01 (ITU-R P.837) Site 2 (mm/h) | 41.4 | 36.4 |
| Selected RSL (listing) (dBm) | −6.89 | −47.71 |
| Threshold / required RSL (listing) (dBm) | −60.0 | −66.0 |
| Margin = Selected RSL − Threshold (dB) | 53.11 | 18.29 |
| Baseline p_ex (rain only) (%) | 0.0035 | 1.45 |
| Equipment / emission | E-LINK LLEAP | E-LINK 1000Q / 1G00G1W |
| Modulation / data rate | QPSK | QPSK / 1250 Mbit/s |

## Ku and Ka-band FS Link Characteristics

Details on registered Ku and Ka band fixed-service links were obtained from https://wireless2.fcc.gov/UlsApp/UlsSearch/searchLicense.jsp.

As an illustrative example, Table 3 documents the parameters of two fixed service links, operating in Ku-band and Ka-band, respectively.

Table 1

FS Link Parameters

| Parameters | Call Sign WQYU458 | Call Sign WQUB840 |
| --- | --- | --- |
|  | Ku-band link | Ka-band link |
| Site 1 | COUGAR MT | CHEYENNE SUB |
| Site 2 | CITY CENTER | CHEYENNE |
| Frequency (GHz) | 11.135 | 18.080 |
| Path length (km) | 10.3409 | 10.421072 |
| FS antenna maximum receive gain (dBi) | 34.7 | 38.9 |
| Site 1 latitude | 47.5415 | 41.113417 |
| Site 1 longitude | -122.1092 | -104.814139 |
| Site 2 latitude | 47.6149 | 41.168139 |
| Site 2 longitude | −122.1936 | -104.713306 |
| Antenna height AGL Site 1 (m) | 45.7 | 15.2 |
| Antenna height AGL Site 2 (m) | 109.7 | 22 |
| Ground AMSL Site 1 (m) | 445.5 | 1867.5 |
| Ground AMSL Site 2 (m) | 50.1 | 1860.1 |
| ACM min / max modulation (listing) | QPSK/256QAM | QPSK/256QAM |
| Channel / emission (listing) | 36M3D1D | 36M3D1D |
| Min-Mod EIRP (dBm) (listing) | 54.2 | 62.9 |
| Max-Mod EIRP (dBm) (listing) | 54.2 | 55.9 |
| RSL min-Mod (dBm) | -44.77 | -36.15 |
| RSL max-MOD (dBm) | -44.77 | -43.15 |
| RSS QPSK (Cambium Networks, LTD Model: PTP11800B/ PTP-18800-B, BER 10⁻⁶) (dBm) | −85.2 | −85.2 |
| RSS 256QAM (Cambium Networks, LTD Model: PTP11800B/ PTP-18800-B, BER 10⁻⁶) (dBm) | −67.1 | −67.1 |
| Margin QPSK = min-mod RSL − RSS_QPSK (dB) | 40.43 | 49.05 |
| Margin highest mode = max-mod RSL − RSS_256QAM (dB) | 22.33 | 23.95 |
| Rain rate R_001 (mm/hr) | 26.23 | 24.29 |

Link-margin

Link-margin at either Site 1 or Site 2 was obtained as follows,

Mm= RSLm- Tm     (2)

where Tm is the receiver threshold (obtained from the equipment datasheet) and RSLm  is the received signal level for the specific modulation and forward error correction code rate supported by the equipment. The RSL is calculated from the EIRPm (listed in ULS), Gain (listed in ULS) and the free-space path loss (FSPL), as follows:

RSLm= EIRPm- FSPL+Gain(receiver)

The margin (dB) at the lowest ACM mode (QPSK) is as follows:

MQPSK= RSLQPSK- TQPSK.

The margin (dB) at the highest ACM mode (256QAM) is:

M256QAM= RSL256QAM- T256QAM.

For ACM radios, a set of margins {Mm} can be defined corresponding to ACM states supported by the radio equipment. Each state typically corresponds to a particular choice of modulation and forward error correction rate. Here, the link-availability is determined by the margin of the lowest ACM state (i.e., ACM min modulation).  For ACM radios, Tm  calculations in this study reference receiver sensitivity levels (RSS) from vendor equipment datasheets where available.

Absolute change in availability

If the available propagation margin is Mm, the baseline rain-induced unavailability percentage pm is obtained from the rain-attenuation curve using the relation

Arainp= Mm

Only rain attenuation is considered in this analysis for Ku and Ka links, such that the total attenuation is

Atotalp= Arainp.

The reduction in margin due to interference is calculated as follows

Mm(IN)= Mm-10∙log101+ 10IN∙0.1

where I/N dB is the interference to noise ratio at the receiver.  As an example, given the rain-rate for this location (26.23 mm/hr), the rain attenuation at p=0.01% (99.99% availability) is 5.93dB, similarly, at p=0.001% (99.999% availability) is 11.95dB and at p=0.0001% (99.9999% availability) is 18.5dB. The margin for a particular mode represents the headroom at the receiver relative to the minimum signal-to-noise ratio required to decode that mode. The corresponding minimum signal level is obtained from vendor data sheets and is labelled as RSS in Table 1. In the absence of external interference, this margin can be translated to a baseline availability using the rain attenuation calculated for the path. For instance, if the margin was at least 18.5dB, in the absence of interference the availability would be 99.9999%. As a comparison, the margin for the highest mode for this link was found to be 22.33dB and for lowest mode was found to be 40.43dB. External interference, modelled conservatively here as an aggregate I/N value at the receiver will reduce the margin (e.g., 1dB for -6dB I/N). For instance, at I/N = -6dB, the margin for highest mode reduces from 22.33dB to 21.33dB, still above the exemplary value of 18.5dB for 99.9999% availability.

Table 2 documents link exceedance (in percentage) and absolute increase in unavailability Ku-band link example of Table 1, where exceedance floor is set to 0.0001% due to the large margins, but the actual availability is close to 100%

Similar analysis is performed for the Ka-band link and results reported in Table 3.

Table 2 Ku-link

| I/N (dB) | Margin (dB) | % exceedance | % Availability | Delta Availability |
| --- | --- | --- | --- | --- |
| -Inf (no interference) | 40.43 | 0.0001 | 99.9999 | 0 |
| -10 | 40.0161 | 0.0001 | 99.9999 | 0 |
| -9 | 39.915 | 0.0001 | 99.9999 | 0 |
| -8 | 39.7911 | 0.0001 | 99.9999 | 0 |
| -7 | 39.6399 | 0.0001 | 99.9999 | 0 |
| -6 | 39.4568 | 0.0001 | 99.9999 | 0 |
| -5 | 39.2367 | 0.0001 | 99.9999 | 0 |
| -4 | 38.9746 | 0.0001 | 99.9999 | 0 |
| -3 | 38.6657 | 0.0001 | 99.9999 | 0 |
| -2 | 38.3056 | 0.0001 | 99.9999 | 0 |
| -1 | 37.891 | 0.0001 | 99.9999 | 0 |
| 0 | 37.4197 | 0.0001 | 99.9999 | 0 |

Table 3: Ka link

| I/N (dB) | Margin (dB) | % exceedance | % Availability | Delta Availability |
| --- | --- | --- | --- | --- |
| -Inf (no interference) | 49.0478 | 0.0001 | 99.9999 | 0 |
| -10 | 48.6339 | 0.0001 | 99.9999 | 0 |
| -9 | 48.5328 | 0.0001 | 99.9999 | 0 |
| -8 | 48.4089 | 0.0001 | 99.9999 | 0 |
| -7 | 48.2577 | 0.0001 | 99.9999 | 0 |
| -6 | 48.0746 | 0.0001 | 99.9999 | 0 |
| -5 | 47.8545 | 0.0001 | 99.9999 | 0 |
| -4 | 47.5924 | 0.0001 | 99.9999 | 0 |
| -3 | 47.2834 | 0.0001 | 99.9999 | 0 |
| -2 | 46.9234 | 0.0001 | 99.9999 | 0 |
| -1 | 46.5088 | 0.0001 | 99.9999 | 0 |
| 0 | 46.0375 | 0.0001 | 99.9999 | 0 |

Average throughput and percentage degradation in throughput

For links with ACM, the possible operational states vary from the most robust (using lowest cardinality modulation and most robust forward error correction code rate supported by the radio equipment) to the highest data rate (using highest cardinality modulation and less robust forward error correction code rate).  The dwell probability at each ACM state can be calculated using the difference in exceedance probabilities of that state and the preceding ACM state. The average throughput degradation as function of I/N is tabulated in   Table 4 using the exemplary Ku-band link from Table 1. Due to very robust margin built into the link, the links spends most of its time at the highest mode (256QAM), a nominal reduction in margin due to I/N (e.g., 1dB or even 3dB) keeps the link at 256QAM. Similar analysis is reported in Table 5 for the Ka-band link.

Table 4: ku link

| I/N (dB) | Average Throughput (Mbps) | % Degradation in Throughput |
| --- | --- | --- |
| −100 | 236.609990 | 0 |
| −10 | 236.609988 | 7.46E−07 |
| −9 | 236.609988 | 9.46E−07 |
| −8 | 236.609987 | 1.20E−06 |
| −7 | 236.609986 | 1.53E−06 |
| −6 | 236.609985 | 1.96E−06 |
| −5 | 236.609984 | 2.52E−06 |
| −4 | 236.609982 | 3.24E−06 |
| −3 | 236.609980 | 4.19E−06 |
| −2 | 236.609977 | 5.45E−06 |
| −1 | 236.609973 | 7.14E−06 |
| 0 | 236.609967 | 9.42E−06 |

Table 5: ka link

| I/N (dB) | Average Throughput (Mbps) | % Degradation in Throughput |
| --- | --- | --- |
| -100 | 236.699191 | 0.00E+00 |
| -10 | 236.699127 | 2.71E-05 |
| -9 | 236.699111 | 3.41E-05 |
| -8 | 236.69909 | 4.27E-05 |
| -7 | 236.699065 | 5.35E-05 |
| -6 | 236.699033 | 6.69E-05 |
| -5 | 236.698993 | 8.36E-05 |
| -4 | 236.698944 | 1.04E-04 |
| -3 | 236.698883 | 1.30E-04 |
| -2 | 236.698808 | 1.62E-04 |
| -1 | 236.698716 | 2.01E-04 |
| 0 | 236.698602 | 2.49E-04 |

Summary

The static analysis method presented for E, Ku and Ka band fixed-service links. These quantify the impact of interference to the absolute change in unavailability and percentage degradation in throughput in modern fixed service links

____________________

https://www.comsearch.com/