| U.S. Radiocommunications Sector Fact Sheet |  |
| --- | --- |
| Working Party: ITU-R WP 5B | Document No: USWP5B37-08 Final Draft |
| Ref: Annex 2.6 to Document 5B/526-E | Date: 28th September 2026 |
| Document Title: DRAFT NEW RECOMMENDATION ITU-R M.[AM(R)S_AMS(R)S_CHAR_5GHZ] - Characteristics and Protection Criteria of Terrestrial and Satellite Unmanned Aircraft System Control and Non-Payload Communications Links operating in the Aeronautical Mobile (R) Service and Aeronautical Mobile Satellite (R) Service in the band 5 030-5 091 MHz. |
| Author(s)/Contributors(s): Name: Don Nellis Org: Federal Aviation Administration Name: Michael Neale Org: ACES Corporation for the FAA Name: Jeff Hougland Org: ACES Corporation for the FAA | Phone: (202) 267-9779 Email: Donald.Nellis@faa.gov Phone: (858) 705-8978 Email: michael.neale@aces-inc.com Phone : (410) 271-3255 Email : Jeff.Hougland@acescorp.net |
| Purpose/Objective: The purpose of this contribution is to update the terrestrial characteristics, based on a recent update to the RTCA MOPS DO-362A that standardizes and defines this terrestrial based CNPC Link. This contribution encapsulates the outcomes of the previous meeting and moves to elevate the preliminary draft new recommendation to a draft new recommendation. |  |
| Abstract: This contribution contains characteristics and protection criteria for terrestrial and satellite-based systems that can be used for remote control of unmanned aircraft. Changes from were made in response to the outcome of previous meetings including the removal of the references to the Japanese High-altitude relay. |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Received : XXX Source: Annex 2.6 to Document 5B/435 Subject: New Recommendation ITU-R M.[AM(R)S_AMS(R)S_CHAR_5GHZ] | Document 5B/XXX-E |
|  | XXX |
|  | English only |
| United Sates of America |  |
| draft new Recommendation Itu-r m.[AM(R)S_AMS(R)S_char_5GHz] |  |
| Characteristics and Protection Criteria of Terrestrial and Satellite Unmanned Aircraft System Control and Non-Payload Communications Links operating in the aeronautical mobile (route) service and aeronautical mobile satellite (R) Service in the band 5 030-5 091 MHz |  |

Introduction

At WRC-2012 it was agreed, under No. 5.443C, that the frequency band 5 030-5 091MHz could be used by the aeronautical mobile (R) service limited to internationally standardized aeronautical systems.  Industry, international standards development organizations and ICAO have been working since then to develop the technology and standards necessary to use that allocation.  Consequently, it is now possible to provide characteristics and protection criteria for such systems for use in any future sharing studies within ITU-R.

Proposal

The United States of America proposes to assist in answering the above need by providing characteristics for such Control and Non-Payload Communications (CNPC) links operating in the AM(R)S allocation under No. 5443C and used in air-ground applications between Unmanned Aircraft (UA) and their Control Station (CS) where the Remote Pilot (RP) is located.

The United States of America proposes that this document should be elevated to a Draft New Recommendation at the November 2026 meeting of WP 5B and if the High Altitude Relay System compatibility study information is still unavailable at that time then the High Altitude Relay System in Section 2.2 is temporarily removed so that the PDNR can be elevated.

Attachment:	1

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document/5B/526-E Subject: New Recommendation ITU-R M.[AM(R)S_AMS(R)S_char_5GHz] | Document 5B/XXX |
|  | Nov 2026 |
|  | English only |
| Annex 2.6 to Working Group 5B Chair’s Report |  |
| DRAFT NEW RECOMMENDATION ITU-R M.[AM(R)S_AMS(R)S_CHAR_5GHZ] |  |
| Characteristics and protection criteria of terrestrial and satellite unmanned aircraft system control and non-payload communications links operating in the aeronautical mobile (route) service in the band 5 030-5 091 MHz and aeronautical mobile satellite (R) service in the band 5 000-5 150 MHz |  |

Editor’s note: How protection is addressed in this recommendation is still to be discussed in next WP 5B meeting. See WG 5B-2 report.

Scope

This Recommendation specifies the characteristics of control and non-payload communications (CNPC) links, carrying command and control (C2) information, operating in the aeronautical mobile (R) service (AM(R)S) in the frequency band 5 030-5 091 MHz and aeronautical mobile-satellite (R) service (AMS(R)S) in the band 5 000-5 150 MHz in order to be used in analysing compatibility between unmanned aircraft systems (UAS) CNPC operating in the AM(R)S and AMS(R)S and other services.

Keywords

Unmanned aircraft systems

Abbreviations/Glossary

AM(R)S	Aeronautical mobile (route) service

AMS(R)S	Aeronautical mobile-satellite (route) service

ARRS	Airborne radio relay system

ARS	Airborne radio system

BGRS	Beyond line-of-sight ground radio system

C2	Command and control

CNPC	Control and non-payload communication

CRC	Cyclic redundancy check

DVB-RCS	Digital video broadcasting - return channel via satellite

FDMA	Frequency division multiple access

GMSK	Gaussian minimum shift keying

GRS	Ground radio system

ICAO	International Civil Aviation Organization

QPSK	Quadrature phase shift keying

RPA	Remotely piloted aircraft

RPAS	Remotely piloted aircraft system

RR	Radio Regulation

TCC	Turbo code comparison

TDD	Time division duplex

UA	Unmanned aircraft

UACS	Unmanned aircraft control station

UAS	Unmanned aircraft system

Related ITU Reports

Reports

ITU-R M.2205	Results of studies of the AM(R)S allocation in the band 960-1 164 MHz and of the AMS(R)S allocation in the band 5 030-5 091 MHz to support control and non-payload communications links for unmanned aircraft systems

ITU-R M.2233	Examples of technical characteristics for unmanned aircraft control and non-payload communications links

ITU-R M.2236	Compatibility study to support the line of sight control and non-payload communication links for unmanned aircraft systems proposed in the frequency bands 5 000-5 010 and 5 010-5 030 MHz

ITU-R M.2171	Characteristics of unmanned aircraft systems and spectrum requirements to support their safe operation in non-segregated airspace

The ITU Radiocommunication Assembly,

considering

a)	that standardization of operations of Remotely Piloted Aircraft System (RPAS) in non-segregated airspace is under purview of the ICAO for international operations;

b)	that AM(R)S in the 5 030-5 091 MHz band and AMS(R)S in the 5 000-5 150 MHz band are allocated worldwide and can provide communications relating to safety and regularity of flights,

recognizing

a)	that the frequency bands 5 030-5 091 MHz and 5 000-5 150 MHz are allocated respectively to the aeronautical mobile (route) service (AM(R)S) and the aeronautical mobile-satellite (route) service (AMS(R)S) which are planned to be used for unmanned aircraft (UA) control and non-payload communication (CNPC) command and control (C2) Links to support the safe operation of UA, among other aeronautical safety-of-life and regularity of flight applications;

b)	that the frequency band 5 000-5 150 MHz is also allocated to the aeronautical radionavigation service;

c)	that some internationally standardized microwave landing systems operate in this band in accordance with Radio Regulations (RR) No. 5.444;

d)	that from RR No. 5.444, in the frequency band 5 030-5 091 MHz, the requirements of microwave landing system have priority over other uses of this frequency band;

e)	that use of the frequency band 5 030-5 091 MHz by the AM(R)S and 5 000-5 150 MHz by the AMS(R)S is limited to internationally standardized aeronautical systems in accordance respectively with RR No. 5.443C and RR No. 5.443D.

recommends

1	that the technical and operational characteristics of the UA CNPC links for C2 operating in the frequency band 5 030-5 091 MHz under AM(R)S and in the frequency band 5 000-5 150 MHz under AMS(R)S, as described in the Annex, should be used in studies of compatibility with systems operating under an allocation to another service;

2	 that an aggregate interfering signal power to receiver noise power level, I/N of -6 dB, should be used as the required protection level for AM(R)S safety-of-life systems operating in the frequency band 5030-5091 MHz and for AMS(R)S safety-of-life systems operating in the frequency band 5000-5150 MHz, and that this I/N represents the net protection level if multiple interferers are present (see Note 1).

Annex

# 1	Introduction and scope

The present ANNEX is scoping the UAS CNPC C2 Links among other aeronautical safety of life and regularity of flight applications.

The characteristics and [protection criteria] of UAS CNPC C2 Links and their spectrum requirements must support the safe operation of unmanned aircraft (UA) in non-segregated airspace. There is a strong and growing demand for the use of UAS (which include remotely piloted aircraft systems (RPAS) within the international civil aviation organization (ICAO)) in civil applications. Some UA flights will share airspace with passenger carrying aircraft, so their operation needs to be managed to safely allow the introduction of this new paradigm in aviation.

As these communications are critical for a safe management of the airspaces, future ICAO standards are obviously mandatory for these kinds of communications.

The CNPC C2 Link between the unmanned aircraft control station (UACS) and the UA support the following two ways of communication:

–	The forward link: Link from the UACS to the unmanned aircraft for flight and navigation equipment control.

–	The return link: Link from the UA to the UACS. It is anticipated that in some flight conditions or in specific airspaces it may be necessary to downlink video streams.

The potential types of C2 information exchanges carried over the CNPC C2 Link system are:

The C2 Link control data – Information to enable the remote pilot and automation in the UACS and UA to manage the operation of the C2 Link system, which includes, inter alia, the C2 Link quality of service delivered reports to the remote pilot and the UA.

The C2 Link user data – Information passing through the C2 Link in support of the remote pilot’s safe and efficient UAS operations.

One aspect of the management of safe UAS operations is the management of the interference received by the receivers that link the UA and the UA CS. To enable this interference analysis to be undertaken the characteristics and [protection criteria/level for protection] are proposed for these links operating in the AM(R)S and AMS(R)S allocations where RR Nos. 5.443C and 5.443D apply. This recommendation contains those characteristics and [protection criteria/level for protection] based on systems which are currently under development and that will have to comply with the international standardization being developed by ICAO.

The CNPC C2 Link consists of air-to-ground links that can be used simultaneously or independently, as required, to provide operational coverage and performance. CNPC C2 Link systems include a terrestrial based component and a satellite-based component.

The terrestrial component uses an airborne radio system (ARS) on the UA to communicate with a ground radio system (GRS) that connects to the UACS. The satellite component uses an airborne earth station on the UA to communicate with a ground earth station (via the geostationary satellite) that connects to the UACS.

[Figure 1

Example command and non-payload communications link system components]

# 2	Characteristics of command and non-payload communication links at 5 GHz

## 2.1	Unmanned aircraft and control station characteristics for terrestrial control 
and non-payload communication link

TABLE 1

Transmission and reception characteristics for the terrestrial control 
and non-payload communication link System 1

| Terrestrial command and non-payload communication System 1 |  |  |  |
| --- | --- | --- | --- |
|  | Units | UA ARS | GRS |
| Frequency of operation | MHz | 5 030 to 5 091 | 5 030 to 5 091 |
| Duplexing |  | Time division duplex (TDD) | TDD |
| Transmit/receive duration | msec | 23 Receive + 1.3 guard time 23 Transmit + 2.7 guard time | 23 Transmit + 1.3 guard time 23 Receive + 2.7 guard time |
| Modulation |  | GMSK or QPSK | GMSK or QPSK |
| Modulation symbol rates | ksps | 3 to 138 including error correction/detection, guard times and synchronization overhead | 34 to 138 including error correction/detection, guard times and synchronization overhead |
| Forward error correction |  | GMSK: Rate 5/8 Turbo Conv. Code QPSK: Rate 5/9 and Rate 1/3 TCC | GMSK: Rate 5/8 Turbo Conv. Code QPSK: Rate 5/9 and Rate 1/3 TCC |
| Error detection |  | 32-bit CRC | 32-bit CRC |
| Baseband Input/Output Signal |  | User Data | User Data |
| User Data Bit Rates | kbps | 20 to 80 Includes TDD duty cycle overhead | 20 to 80 Includes TDD duty cycle overhead |
| Occupied bandwidth, C | kHz | Variable per application with a maximum of 250 | Variable per application with a maximum of 250 |
| Antenna gain | dBi | 2 Omnidirectional | Maximum 20 (see Table 2) |
| Cable loss | dB | 2 | 1 |
| Antenna pattern |  | Constant azimuth Constant elevation | Constant azimuth Tailored in elevation See Table 2 |
| Antenna polarization |  | Vertical with aircraft flying straight and level | Vertical |
| Maximum antenna height | m | 22 860 (mean sea level) Typical 6 000 | 2 to 50 Typical 10 |
| Service range | km | 550 Typical 80 | 550 Typical 80 |
| Transmitter conducted power | dBm | 40 | 40 |
| Transmitter in band emission limits | dBc/ 1 MHz | −66 at 2 MHz offset See Table 3 | −66 at 2 MHz offset See Table 3 |
| Receiver noise figure | dB | 7 | 7 |
| Receiver sensitivity | dBm | 120 | 120 |
| Receiver in band rejection – except the operating channel | dB | One channel separation: 23 Two channel separation: 43 Three channel separation: 57 2 MHz or more separation: 63 | One channel separation: 23 Two channel separation: 43 Three channel separation: 57 2 MHz or more separation: 63 |
| Protection Level (aggregate) I/N | dB | –6 | –6 |

* The protection level does not include the safety margin.

TABLE 2

System 1 GRS antenna characteristics

| Parameter | Units | Intrasystem limit | High gain antenna | Omnidirectional antenna |
| --- | --- | --- | --- | --- |
| Peak gain | dBi | 20 | 19 | 5.2 |
| Elevation Θ3 | Degrees | N/A | 9 | 55 |
| Gain at elevation degrees |  |  |  |  |
| 0.5 | dBi | 19.0 | 18.5 | 5.2 |
| 1.5 | dBi | 19.5 | 18.9 | 4.9 |
| 2.5 | dBi | 20.0 | 19.0 | 4.5 |
| 3.5 | dBi | 19.5 | 18.9 | 1.6 |
| 7 | dBi | 17.0 | 16.0 | −1.4 |
| 11.5 | dBi | 14.0 | 8.2 | −2.8 |
| 16 | dBi | 11.5 | 3.8 | −5.0 |
| 32 | dBi | 6.5 | −12.5 | −8.0 |
| 64 | dBi | 1.5 | −26.4 | −9.5 |
| 75 | dBi | 0.5 | −29.0 | −11.7 |
| 89 | dBi | 0.5 | −31.0 | −16.5 |
| Azimuth Θ3 | Degrees |  | 34 | 360 |
| Steerable/Sectoral/ Omnidirectional |  | N/A | Steerable/ Sectoral | Omnidirectional |

The above table and accompanying graph below contain the characteristics of antennas that will be used to support unmanned aircraft operations using the System 1 whose RF characteristics are described in the previous tables. The information contained in Table 2 includes the GRS elevation and azimuth gains that will support unmanned aircraft (that themselves use omnidirectional antennas) operating up to approximately 65 nautical miles/120 km slant range, at altitudes up to 65 000 ft, down to close in operations including takeoff and landing. Because of the critical nature of the performance of UAS CNPC C2 Links a range of antennas are required to maximize the link margin while maintaining intrasystem compatibility. The maximum gain allowed to ensure intrasystem compatibility is also shown in the table and accompanying graph. The High Gain antenna provides full azimuth coverage by being either steerable in azimuth or by using switching between one of a number of sectorized antennas with the characteristics given in the table. All antennas have the fixed elevation pattern given in the table and graph. In any geographic region at any one time, it is anticipated that there will be a mixture of antennas being used to support the wide variety of unmanned aircraft operations.

TABLE 3

Transmitter out of band emission limits in the 5 030-5 091 MHz frequency band for system 1

| System 1 |  |
| --- | --- |
| Offset from carrier frequency | dBc/kHz |
| Channel width ÷ 2 | −54 |
| 1.5 × channel width | −74 |
| 500 kHz | −90 |
| 2 000 kHz | −96 |

## [

## 2.3	Unmanned aircraft and control station characteristics for control and non-payload communication link via satellite

### 2.3.1	Control and non-payload communication via geostationary orbiting satellite systems

Technical characteristics of control and non-payload communication via GSO systems are contained in Table 4.

Table 4:  Technical characteristics of control and non-payload communication via GSO systems

| Satellite GSO command and non-payload communication |  |  |  |
| --- | --- | --- | --- |
|  | Units | Aircraft receiver | GSO satellite receiver |
| Frequency of operation | MHz | 5 000 to 5 150 | 5 000 to 5 150 |
| Occupied bandwidth, C | kHz | 22.2 per carrier | 139.8 per carrier |
| Antenna gain | dBi | 3 Omnidirectional | 45.1 |
| Cable loss | dB | 2 | 0.5 |
| Antenna pattern |  | Constant azimuth Constant elevation | Rec ITU-R S.672 |
| Antenna polarization |  | RHCP | RHCP |
| Maximum antenna height | m | 22 860 (mean sea level) Typical 6 000 | - |
| Transmitter conducted power | dBm | 43.5 dBm per carrier | 44.1 dBm per carrier |
| Transmitter in band emission limits | dBc/ 1 MHz | ITU-R SM.1541-7 | ITU-R SM.1541-7 |
| Receiver noise figure | dB | 5 | 5 |
| Protection level (aggregate) I/N* | dB | -6 | -6 |

### * The protection level does not include the safety margin

### 2.3.2	Control and non-payload communication via low Earth orbiting non-geostationary satellite orbiting systems

This section describes the links between the satellite and the aircraft.

TABLE 5

Example of aircraft C2 Link earth station characteristics for one NGSO use case

| Parameters | Units | Values |
| --- | --- | --- |
| Altitude | km | 0-20 |
| Frequency range | MHz | 5 000-5 150 |
| Tx necessary bandwidth for each channel* | MHz | Up to 60 |
| Tx e.i.r.p. density | dBW/Hz | Up to −10 |
| Tx e.i.r.p. | dBW | Up to 41.7 |
| Tx loss | dB | 2 |
| Minimum Tx bandwidth per carrier | kHz | 15 |
| OoB emissions | dBm | ITU-R SM.1541-7 |
| Max antenna gain** | dBi | 3 |
| Type antenna gain | / | Omni |
| Rx bandwidth for each channel | MHz | Up to 60 |
| Minimum Rx bandwidth per carrier | kHz | 15 |
| Noise figure | dB | 5 |
| Protection Level (aggregated interference): I/N*** | dB | –6 |
| * The channel may be filled by one or several carriers, and the frequency range may be divided in several channels. ** steerable antenna could be considered later on. *** The protection level does not include the safety margin. |  |  |

TABLE 6

Example of space station characteristics for one NGSO use case

| Parameters | Units | Values |
| --- | --- | --- |
| Altitude | km | 415 - 1 500 |
| Frequency range | MHz | 5 000-5 150 |
| Tx necessary bandwidth of each channel* | MHz | Up to 60 |
| Tx e.i.r.p. density at Nadir | dBW/Hz | -25.2 |
| Tx e.i.r.p at Nadir | dBW | 41.7 |
| Minimum Tx bandwidth per carrier | kHz | 15 |
| OoB emissions | dBm | ITU-R SM.1541-7 |
| Max antenna gain | dBi | 15 |
| Type antenna gain | / | ITU-R S.1528 |
| Rx bandwidth of each channel* | MHz | Up to 60 |
| Minimum Rx bandwidth carrier | kHz | 15 |
| Noise figure | dB | 5 |
| Protection level (aggregated interference): I/N** | dB | −6 |
| * The channel may be filled of carriers, and the frequency range may be divided in several channels. ** The protection level does not include safety margin |  |  |

____________________

1 The protection level does not include the safety margin.