| U.S. Radiocommunication Sector Fact Sheet |  |
| --- | --- |
| Working Party: ITU-R WP-5B | Document No: USWP5B-37-18 |
| Ref: WRC-27 Agenda Item 1.8/Res. 663 (WRC-23), 5B 435/Chapter 1 Annex 1.1 | Date: 17 Aug 2026 |
| Document Title: [WORKING DOCUMENT TOWARDS A] PRELIMINARY DRAFT NEW REPORT ITU-R M.[RLS_231.5-700GHZ]/SUPPORTING MATERIAL FOR WRC-27 AGENDA ITEM 1.8 |  |
| Author(s)/Contributors(s): |  |
| Ryan McDonough (NASA) Joseph Cramer (NASA) Gunnar Juergens (Teradar) Zach Rosenbaum (DLA Piper for Teradar) | Ryan.S.McDonough@nasa.gov joseph.s.cramer@nasa.gov gjuergens@teradar.com Zach.Rosenbaum@us.dlapiper.com |
| Purpose/Objective: To continue development for compatibility and sharing studies performed under WRC-27 Agenda Item 1.8 in accordance with Resolution 663 (WRC-23). |  |
| Abstract: Pursuant to Resolution 663 (Rev.WRC-23), Working Party (WP) 5B is the responsible group for WRC-27 Agenda Item 1.8 request to consider possible additional spectrum allocations to the radiolocation service on a primary basis in the frequency range 231.5-275 GHz and possible new identifications for radiolocation service applications in frequency bands within the frequency range 275-700 GHz for millimetric and sub-millimetric wave imaging systems. This document will provide an additional Study C considering compatibility between terahertz imaging devices and EESS (passive) systems. |  |
| Fact Sheet Preparer: Ryan McDonough, NASA |  |

Attachment:	1

Radiocommunication Study Groups. Source: Document 5B/TEMP/193; Annex 1.1 to Document 5B/526-E. 22 June 2026. English only. Annex 1.1 to Working Party 5B Chair’s Report

PRELIMINARY DRAFT NEW REPORT ITU-R M.[RLS_231.5-700GHZ]

Sharing and compatibility studies in support of possible allocation/identification for the radiolocation service in the frequency range 231.5-700 GHz

Annex 7

A7.4.1.3	STUDY C

A7.4.1.3.1	Terahertz Imaging Device (TID) Characteristics

Terahertz Imaging Devices (TID) are vehicular radiolocation applications performing imaging and ranging functions for advanced driver assistance systems (ADAS) and autonomous driving (AD). TID operate at carrier frequencies in the 306-313 GHz range, which is identified under RR No. 5.564A. TID is characterized as a specific active vehicular radiolocation application within the existing categories of Resolution 663 (Rev.WRC-23), contributing to traffic safety in the ITS context.

The technical characteristics of TID, as described in ETSI TR 104 096, are summarized in the table below.

Table A7-study C-1

Technical parameters for Terahertz Imaging Devices (TID)

| Parameter | Units | TID |
| --- | --- | --- |
| Operating Frequency Range | GHz | 306-313 |
| Modulation Scheme |  | FMCW |
| Modulation Bandwidth | GHz | 1-5 |
| Maximum Peak E.I.R.P. | dBm | 55 |
| Maximum Mean E.I.R.P. (average over signal repetition time) | dBm | 50 |
| Power Spectral Density (mean E.I.R.P.) | dBm/MHz | 7 to 20 (bandwidth dependent) |
| Chirp Time |  | 1 µs - 10 ms |
| Duty Cycle | % | 30 |
| Maximum Antenna Gain | dBi | 28 |
| Antenna Beamwidth (3 dB) | degrees | ≤ 2 |
| Max scan elevation angle | Degrees | 30 |
| Max scan azimuth angle | Degrees | TBD |
| Antenna Polarization |  | Left Hand Circular |
| Typical Operating Range | m | Up to 300 |
| Typical Number of TID per Vehicle |  | 1 |
| Motion |  | Mobile outdoor (vehicular) |
| Note 1: TID characteristics are taken from ETSI TR 104 096. Note 2: The frequency band 306-313 GHz is identified under RR No. 5.564A for active fixed and land mobile service use where no specific conditions are necessary to protect EESS (passive). |  |  |

A74.1.3.2	EESS Compatibility Study for TID in the 306-313 GHz Range

This section presents a sharing and compatibility analysis between TID operating in the 306-313 GHz range and Earth Exploration Satellite Service (EESS) passive sensors. The analysis considers three representative EESS systems operating in adjacent frequency bands: the Ice Cloud Imager (ICI) on Metop-SG-B (830 km SSO altitude, conical scan), the EPS Sterna Radiometer/Arctic Weather Satellite (AWS) (595 km SSO altitude, nadir cross-track scan), and the TWICE instrument (400 km LEO altitude, conical scan). EESS sensor characteristics and protection criteria are taken from Document 5B/526 Annex 1.1 (Table A7-11) and Recommendation ITU-R RS.2017 (Table 2), respectively. The characteristics of TID are taken from ETSI TR 104 096.

The aggregate interference protection criteria for EESS (passive) in the relevant adjacent bands, as given in Recommendation ITU-R RS.2017, are as follows: for the 296-306 GHz band, -160 dBW in a 200 MHz reference bandwidth for nadir or conical scanning modes (0.01% of area), and -194 dBW in a 3 MHz reference bandwidth for limb sounding applications (1% of time); for the 313.5-355.6 GHz band, -158 dBW/200 MHz for nadir/conical modes (0.01% of area) and -194 dBW/3 MHz for limb sounding (1% of time). The analysis in this section addresses the nadir/conical scanning criteria, which represent the more stringent case for ground-based interference sources.

A7.4.1.3.2.1	General Methodology

The compatibility framework follows the analytical approach applied in Report ITU-R SM.2450, consistent with the methodology used in Study B (Section A7.4.1.2) of this Report. TID is assessed as a ground-based, short-range, active vehicular device with predominantly terrestrial interference geometry. SM.2450 finds compatibility of various deployment of fixed and mobile applications in certain frequency ranges, but highlights the potential for interference levels exceeding protection criteria of EESS (passive) under specific parameters including 306-313 GHz. For more information see figures A10-A15 (SM.2450). There are noted operational differences between mobile/fixed and radiolocation service applications.

The compatibility analysis considers two potential coupling mechanisms between TID and EESS (passive) systems. The primary direct coupling path considers interference from TID antenna sidelobes received by the EESS sensor main beam. The received power is calculated as: Received Power (dBm per MHz) = Tx output power per 1 MHz + Gain_Tx (dBi) − Attenuations (dB) + Gain_Rx (dBi), where attenuations account for free space loss, gaseous absorption, sidelobe rejection (15 dB), polarization loss, and packaging loss. For cases where TID does not operate co-frequency with the EESS system, an additional reduction of power in the EESS band is applied.

The secondary indirect coupling path considers reflections from a preceding vehicle, consistent with the methodology described in ECC Report 23 for automotive radar at 24 GHz. At 310 GHz, the wavelength decreases from approximately 12.5 mm (at 24 GHz) to approximately 0.97 mm, causing the same vehicle surface features to represent a much larger fraction of a wavelength. This increases electromagnetic roughness and reduces coherent quasi-specular reflection, redistributing reflected power into diffuse directions. Based on the Rayleigh roughness concept and Gaussian rough-surface Kirchhoff approximation, an additional frequency-dependent rejection of approximately 12 dB is applied at 310 GHz relative to the 24 GHz baseline. Combined with the distance-weighted vehicle scattering gain of 19.8 dB and the hemispherical averaging factor of 4.7 dB from ECC Report 23, the total effective scattering path rejection is conservatively estimated at approximately 20 dB. The margin and permissible density are then calculated using the same framework as the direct path analysis. The validity of extrapolating the methodology in ECC Report 23 from 24 GHz to ~300 GHz remains to be confirmed.

A7.4.1.3.2.2	Interference Coupling Geometry

TID sensors are installed behind the vehicle fascia or a secondary surface, with the main beam directed at the horizon with a maximum scan elevation angle of +30⁰ to support forward-looking vehicular sensing. The EESS (passive) sensors observe from orbital altitudes at off-nadir angles (44.7° for ICI, 54.4° for AWS outer beam, nadir for AWS inner beam). From the perspective of the TID antenna, the satellite direction corresponds to a high elevation angle (37.3° for ICI, 27.2°-90° for AWS), which has the possibility of interacting with the skirts of the  TID main beam, in consideration of ground slope and scattering interactions with the fascia, but not likely with the peak of the TID main beam. At these angles, the TID antenna pattern exhibits some sidelobe rejection relative to the main beam gain of 28 dBi. Based on the antenna pattern simulations from ETSI TR 104 096, and accounting for deviations from the theoretical pattern with a deployed one, a sidelobe rejection of 15 dB is applied for this analysis, yielding an effective maximum TID gain toward the EESS sensor of approximately +13 dBi. Note that ETSI TR 104 096 is a pattern simulation and the behavior of the installed antenna system may substantially differ. Note: a pattern envelope reflecting the coupling of the antenna with its housing would be provided in a later contribution.

Figure A7.4.1.3.2.2-1: Typical THz Transmitter elevation beam pattern at 0deg (simulated) ETSI TR 104 096

Additionally, a packaging loss of 1.25 dB is included in the link budget to account for attenuation through the vehicle fascia material. This is consistent with ECC Report 334 (one-way attenuation <2 dB for radiodetermination systems) and ECC Report 315 (one-way attenuation of 2 dB). A polarization mismatch loss of 3 dB is applied to represent the statistical average coupling between the TID left-hand circular polarization and the linear (H/V) polarization of the EESS sensors, consistent with Recommendation ITU-R S.736 and the approach adopted in ECC Report 334.

A7.4.1.3.2.3	Frequency Selection for Calculations

Following the methodology of Report ITU-R SM.2450, compatibility calculations are performed at the centre frequency of the respective EESS passive bands, on the assumption that key propagation and interference parameters vary only marginally across each band. For the lower adjacent EESS band (296-306 GHz), a reference frequency of 301 GHz is adopted, representative of the TWICE sensor. For the upper adjacent EESS band (beginning at 313 GHz), the frequencies 315.65 GHz (ICI channel 1) and 318.55 GHz (AWS channel 2) are used, as these are the nearest EESS measurement channels to TID operations.

A7.4.1.3.2.4	Apportionment Factor

In the frequency range 300-320 GHz, there are currently no outdoor active radio services with material field deployment. Fixed Service (FS) usage above 275 GHz is expected to consist of point-to-point deployments, resulting in up to 8  link per km² in urban areas (Report ITU-R SM.2450). Radiodetermination systems for industry automation (RDI-S) are characterized as predominantly indoor or enclosed installations (ECC Report 334), contributing negligibly to outdoor aggregate interference. To retain conservative safety margins, the primary compatibility analysis applies a 6 dB apportionment factor.

A7.4.1.3.2.5	Deployment Density Methodology

The compatibility assessment determines the maximum permissible number of simultaneously transmitting TID within the EESS sensor footprint. The margin between the received interference power from a single TID and the EESS protection criterion (adjusted for the apportionment factor) is calculated. This margin is then converted to a maximum permissible density of TID per km² by dividing the linear margin by the EESS footprint area. The relevant footprint areas are: 200 km² for ICI (conical scan), 54 km² for AWS (nadir) and 433 km² for AWS (outer beam), and approximately 50 km² for TWICE. The expected urban deployment density is approximately 23 TID-equipped vehicles per km², based on a 5% long-term penetration rate applied to urban vehicle densities of approximately 453 vehicles/km² are based on Report ITU-R SM.2057-0 (Annex 6, Table 254) and used by WP5B as shown in Draft Report ITU-R M.[RLS_231.5-700GHZ].

A7.4.1.3.2.6a	Results: 1 GHz Bandwidth Configuration – Direct Path

The 1 GHz system operates with a centre frequency of 309.5 GHz and a PSD of 20 dBm/MHz (50 dBm e.i.r.p. over 1 GHz). At this bandwidth, TID operates entirely within the 306-313 GHz band identified under RR No. 5.564A, with no direct spectral overlap with EESS passive bands. A 20 dB reduction of power is applied in the adjacent EESS bands, consistent with the out-of-band emission roll-off characterized in Report ITU-R F.2416-1, Annex 2.

Table A7-study C-2

Compatibility analysis for 1 GHz TID – Direct Path

| Parameter | Units | TWICE (301 GHz) | ICI (315.7 GHz) | AWS nadir (318.6 GHz) | AWS outer (318.6 GHz) |
| --- | --- | --- | --- | --- | --- |
| Parameter | Units | TWICE | ICI | AWS | AWS |
| TID E.I.R.P. PSD | dBm/MHz | 25 | 25 | 25 | 25 |
| Sidelobe rejection | dB | 15 | 15 | 15 | 15 |
| Free space loss | dB | 199 | 204.4 | 198 | 203.6 |
| Gaseous attenuation | dB | 14.1 | 24.4 | 20 | 42.5 |
| Polarization loss | dB | 3 | 3 | 3 | 3 |
| Packaging loss | dB | 1.25 | 1.25 | 1.25 | 1.25 |
| OOB power reduction | dB | 20 | 20 | 20 | 20 |
| EESS footprint | km^2 | 50 | 200 | 54 | 433 |
| EESS antenna gain | dBi | 48 | 55 | 52 | 52 |
| Received power | dBm/MHz | -179.35 | -188.05 | -180.25 | -208.35 |
| EESS threshold | dBm/MHz | -153 | -151 | -151 | -151 |
| Apportionment | dB | 6 | 6 | 6 | 6 |
| Margin | dB | 20.35 | 31.05 | 23.25 | 51.35 |
| Permissible density | vehicles/km² | 2 | 6 | 4 | 315 |

Under these conditions, the minimum margin is 17.35 dB (vs. TWICE at 301 GHz) with a corresponding permissible density of 2 vehicles per km², which is approximately less than the expected urban deployment density of 23 vehicles per km².

A7.4.1.3.2.6b	Results: 1 GHz Bandwidth Configuration – Scattering Path

The following table presents the scattering path analysis for the 1 GHz TID configuration, using the scattering rejection of 37 dB in place of the 40 dB sidelobe rejection applied in the direct path analysis.

Table A7-study C-3

Compatibility analysis for 1 GHz TID – Scattering Path

| Parameter | Units | TWICE (301 GHz) | ICI (315.7 GHz) | AWS nadir (318.6 GHz) | AWS outer (318.6 GHz) |
| --- | --- | --- | --- | --- | --- |
| Parameter | Units | TWICE | ICI | AWS | AWS |
| TID E.I.R.P. PSD | dBm/MHz | 25 | 25 | 25 | 25 |
| Scattering rejection | dB | 20 | 20 | 20 | 20 |
| Free space loss | dB | 199 | 204.4 | 198 | 203.6 |
| Gaseous attenuation | dB | 14.1 | 24.4 | 20 | 42.5 |
| Polarization loss | dB | 3 | 3 | 3 | 3 |
| Packaging loss | dB | 1.25 | 1.25 | 1.25 | 1.25 |
| OOB power reduction | dB | 20 | 20 | 20 | 20 |
| EESS footprint | km^2 | 50 | 200 | 54 | 433 |
| EESS antenna gain | dBi | 48 | 55 | 52 | 52 |
| Received power | dBm/MHz | -190.35 | -199.05 | -191.25 | -219.35 |
| EESS threshold | dBm/MHz | -153 | -151 | -151 | -151 |
| Apportionment | dB | 6 | 6 | 6 | 6 |
| Margin | dB | 31.35 | 42.05 | 34.25 | 62.35 |
| Permissible density | vehicles/km² | 27 | 80 | 49 | 3968 |

Under the scattering path, the minimum margin is 31.35dB (vs. TWICE at 301 GHz) with a corresponding permissible density of 27 vehicles per km², demonstrating that the indirect coupling path may be a significant contributor to observed interference. Further study is required to confirm the technical validity of the scattering rejection value in this frequency regime as well as to investigate a method to combine the direct and indirect path power received.

A7.4.1.3.2.7a	Results: 4 GHz Bandwidth Configuration – Direct Path

The 4 GHz system operates with a centre frequency of 309.5 GHz and a PSD of 14 dBm/MHz (50 dBm e.i.r.p. over 4 GHz). A 20 dB reduction of power is applied in the adjacent EESS bands.

Table A7-study C-4

Compatibility analysis for 4 GHz TID – Direct Path

| Parameter | Units | TWICE (301 GHz) | ICI (315.7 GHz) | AWS nadir (318.6 GHz) | AWS outer (318.6 GHz) |
| --- | --- | --- | --- | --- | --- |
| TID E.I.R.P. PSD | dBm/MHz | 19 | 19 | 19 | 19 |
| Sidelobe rejection | dB | 15 | 15 | 15 | 15 |
| Free space loss | dB | 199 | 204.4 | 198 | 203.6 |
| Gaseous attenuation | dB | 14.1 | 24.4 | 20 | 42.5 |
| Polarization loss | dB | 3 | 3 | 3 | 3 |
| Packaging loss | dB | 1.25 | 1.25 | 1.25 | 1.25 |
| OOB power reduction | dB | 20 | 20 | 20 | 20 |
| EESS footprint | km^2 | 50 | 200 | 54 | 433 |
| EESS antenna gain | dBi | 48 | 55 | 52 | 52 |
| Received power | dBm/MHz | -185.35 | -194.05 | -186.25 | -214.35 |
| EESS threshold | dBm/MHz | -153 | -151 | -151 | -151 |
| Apportionment | dB | 6 | 6 | 6 | 6 |
| Margin | dB | 26.35 | 37.05 | 29.25 | 57.35 |
| Permissible density | vehicles/km² | 9 | 25 | 15 | 57 |

Under these conditions, the minimum margin is 26.4 dB (vs. TWICE at 301 GHz) with a corresponding permissible density of 4 vehicles per km², less than the expected urban deployment density.

A7.4.1.3.2.7b	Results: 4 GHz Bandwidth Configuration – Scattering Path

The following table presents the scattering path analysis for the 4 GHz TID configuration.

Table A7-study C-5

Compatibility analysis for 4 GHz TID – Scattering Path

| Parameter | Units | TWICE (301 GHz) | ICI (315.7 GHz) | AWS nadir (318.6 GHz) | AWS outer (318.6 GHz) |
| --- | --- | --- | --- | --- | --- |
| TID E.I.R.P. PSD | dBm/MHz | 19 | 19 | 19 | 19 |
| Scattering rejection | dB | 15 | 15 | 15 | 15 |
| Free space loss | dB | 199 | 204.4 | 198 | 203.6 |
| Gaseous attenuation | dB | 14.1 | 24.4 | 20 | 42.5 |
| Polarization loss | dB | 3 | 3 | 3 | 3 |
| Packaging loss | dB | 1.25 | 1.25 | 1.25 | 1.25 |
| OOB power reduction | dB | 20 | 20 | 20 | 20 |
| EESS footprint | km^2 | 50 | 200 | 54 | 433 |
| EESS antenna gain | dBi | 48 | 55 | 52 | 52 |
| Received power | dBm/MHz | -190.35 | -199.05 | -191.25 | -219.35 |
| EESS threshold | dBm/MHz | -153 | -151 | -151 | -151 |
| Apportionment | dB | 6 | 6 | 6 | 6 |
| Margin | dB | 31.35 | 42.05 | 33.25 | 62.35 |
| Permissible density | vehicles/km² | 27 | 42 | 49 | 3968 |

Under the scattering path, the minimum margin is 31.35 dB (vs. TWICE at 301 GHz) with a corresponding permissible density of 27 vehicles per km², comparable to the expected urban deployment density of 23 vehicles per km².

.

A7.4.1.3.2.9	Summary of Results

The following table summarizes the compatibility results for both TID configurations across all EESS systems.

Table A7-study C-10

Summary of EESS (passive) compatibility results for TID (306-313 GHz)

| TID Configuration | EESS System | Minimum Margin (dB) | Minimum Permissible Density (vehicles/km²) |  |
| --- | --- | --- | --- | --- |
| 1 GHz Direct | TWICE (301 GHz) | 17.35 | 1 |  |
| 1 GHz Direct | ICI (315.7 GHz) | 28.05 | 3 |  |
| 1 GHz Direct | AWS nadir (318.6 GHz) | 20.25 | 2 |  |
| 4 GHz Direct | TWICE (301 GHz) | 48.35 | 158 |  |
| 4 GHz Direct | ICI (315.7 GHz) | 25.35 | 7 |  |
| 4 GHz Direct | AWS nadir (318.6 GHz) | 36.05 | 20 |  |
| 1 GHz Scattering | TWICE (301 GHz) | 28.25 | 12 |  |
| 1 GHz Scattering | ICI (315.7 GHz) | 56.35 | 997 |  |
| 1 GHz Scattering | AWS (318.6 GHz, nadir) | 23.35 | 4 |  |
| 4 GHz Scattering | TWICE (301 GHz) | 34.05 | 13 |  |
| 4 GHz Scattering | ICI (315.7 GHz) | 26.25 | 8 |  |
| 4 GHz Scattering | AWS (318.6 GHz, nadir) | 54.35 | 629 |  |

A7.4.1.3.3	Summary for TID Compatibility with EESS (passive)

Based on compatibility assessments conducted using parameters for several EESS (passive) systems and the methodology consistent with Report ITU-R SM.2450, it is not entirely clear that the TID bandwidth configurations demonstrate compatibility with EESS (passive) protection criteria under both direct path and scattering path coupling mechanisms. The 1 GHz and 4 GHz configuration,  when operated within the 306-313 GHz band, demonstrate improved potential for compatibility compared with the 1 GHz. This is likely due to the lower PSD level for the wide band mode.  The minimum margin across all configurations, coupling paths, and EESS systems is 23.35 dB (1 GHz direct-path configuration vs. TWICE at 301 GHz), and the corresponding permissible vehicle density of 1 per km² is less than the expected urban deployment density of 23 vehicles per km². Due to uncertainties inherent in this kind of assessment, a dynamic simulation representing the geometric configurations, and extracting statistical information on interference levels is warranted in this case. It is possible that conducting such a study may lead to confirmation of compatibility of TID in 306-313 GHz with EESS (passive) or discover specific EESS protection conditions that can enable mutual compatibility. [Editor’s Note: A study will be provided in the next meeting.]

______________

____________________