| U.S. Radiocommunications Sector Fact Sheet |  |  |
| --- | --- | --- |
| Working Party: ITU-R WP 5B | Document No: USWP5B26-xx-FS4-VDL |  |
| Ref: 5B/225-E Annex 23 | Date: February 01, 2021 |  |
| Document Title: Working Document towards a Preliminary Draft New Recommendation ITU-R M.[AMRS-VDL], “Characteristics and protection criteria for the International Civil Aviation Organization standardized VHF datalink Mode 2 systems operating in the aeronautical mobile (route) service in the frequency band 136-137 MHz” |  |  |
| Author(s)/Contributors(s): Chris Tourigny FAA Spectrum Engineering Services Michael Tran MITRE |  | Phone: 202-267-3071 Email: chris.tourigny@faa.gov Phone: 703-983-1295 Email : mtran@mitre.org |
| Purpose/Objective: The purpose of this contribution is to provide updates to Document 5B/225 Annex 23 to mature the technical characteristics and protection criteria for the VHF datalink (VDL) systems operating in the frequency band 136 – 137 MHz, allocated to the aeronautical mobile (route) service. VDLM2 is internationally standardized by ICAO. |  |  |
| Abstract: This contribution provides updates to document 5B/225 Annex 23 on the technical characteristics and protection criteria for the VHF datalink Mode 2 (VDLM2) systems operating in the 136 – 137 MHz frequency band, allocated to the AM(R)S. |  |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: 5B/225-E Annex 23 Subject: AM(R)S VDLM2 in 136 – 137 MHz | Document 5B/ |
|  | 10 May 2021 |
|  | English only |
| United States of America |  |
| working document towards a preliminary draft neW recommendation itu-r m.[amRS-VDL] Characteristics and protection criteria for the International Civil Aviation Organization standardized VHF datalink Mode 2 systems operating in the aeronautical mobile (route) service in the frequency band 136 – 137 MHz |  |

Introduction

This contribution provides updates to Document 5B/225 Annex 23, on the technical characteristics and protection criteria for ICAO standardized VHF datalink (VDL) Mode 2 (VDLM2) operating in the aeronautical mobile (route) service in the frequency band 136 - 137 MHz.

Attachment:  1

ATTACHMENT

Working document towards a preliminary draft new recommendation ITU-r m.[amrs-vDL]

Characteristics and protection criteria for the International Civil Aviation Organization standardized VHF datalink Mode 2 systems operating in the aeronautical mobile (route) service in the frequency band 136-137 MHz