| US Radiocommunications Sector Fact Sheet |  |
| --- | --- |
| Working Party: WP 5B | Document No: USWP5B26-17 |
| Ref: Recommendation ITU-R M.2135-0 | Date: 02 March 2021 |
| Document Title: Preliminary Draft Revision of Recommendation ITU-R M.2135-0 Technical characteristics of autonomous maritime radio devices operating in the frequency band 156-162.05 MHz |  |
| Author(s)/Contributors(s): Jerry Ulcek US Coast Guard, Washington DC Johnny Schultz Sev1Tech, Inc. Ross Norsworthy REC, Inc. | Phone : (202) 475-3607 E-mail: Jerry.l.Ulcek@uscg.mil Phone : (727) 403-4029 E-mail: johnny.schultz@sev1tech.com Phone : (727) 515-8025 E-mail: Ross_Norsworthy@msn.com |
| Purpose/Objective: The purpose of this document is to provide contributions towards a Preliminary Draft Revision of Recommendation ITU-R M.2135-0 (AMRD). |  |
| Abstract: This document provides a detailed technical update to Recommendation ITU-R M.2135-0. This Recommendation was published to support the work of the WRC-19. This Recommendation lacks sufficient details to define an AMRD Group B. This contribution will provide the technical characteristics required to define an AMRD Group B and how it should operate on channel 2006. |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: ITU-R M.2135-0 Reference: | Document: USWP5B26-17 |
|  | 02 March 2021 |
|  | English only |
| United States of America |  |
| Preliminary Draft Revision of Recommendation ITU-R M.2135-0 |  |

- Introduction

This contribution provides the editorial changes and technical content to Recommendation ITU-R M.2135-0. This Recommendation was published to support the work of the WRC-19.  This Recommendation lacks sufficient details to define an AMRD Group B. This contribution provides the technical characteristics required to define AMRD Group B and how it should operate on channel 2006.

- Summary of changes

Listed below are the proposed changes to Recommendation ITU-R M.2135-0:

- The Recommendation has been updated to reflect the decisions made at the World Radiocommunication Conference 2019.

- Additional transmitter characteristics have been added to Annex 2.

- A channel access scheme has been defined for Annex 2.

- A transmission message structure has been defined for Annex 2.

- Modified Annex 3 transmitter duty cycle to be compatible with the Annex 2 transmitter duty cycle since they could possibly occupy the same channel.

- Attachments

The following attachment contains the proposed changes to Recommendation ITU-R M.2135-0.  The changes proposed are contained in the attached document.