| US Radiocommunication Sector FACT SHEET |  |  |
| --- | --- | --- |
| Study Group: USWP 5B |  | Document No: USWP5B26-22 |
| Reference: WRC-23 5D/398, WRC-19 5D/1192 |  | Date: 9 February 2021 |
| Document Title: Draft Reply Liaison to Working Party 5D concerning WRC-23 Agenda Item 1.2 |  |  |
| Author(s)/Contributors(s): Andrew Meadows Air Force Tan Ly ASMO Kellen Gibson DSO Dominic Nguyen eSimplicity for AFSMO Tito Alvarez MITRE for ASMO Frank Box MITRE for ASMO | Phone: 334-467-4720 E-mail: andrew.meadows.1@us.af.mil Phone: 301-225-3798 E-mail: tan.m.ly.civ@mail.mil Phone: 301-225-3794 E-mail: kellen.k.gibson.civ@mail.mil Phone: 703-606-7396 E-mail: dominic.nguyen@esimplicity.com Phone: 703-983-3839 E-mail: talvarez@mitre.org Phone: 703-983-6283 E-mail: fbox@mitre.org |  |
| Purpose/Objective: To reply to WP 5D with the antenna patterns and percentage of time value in propagation model relating to the required sharing and compatibility studies to be carried out under WRC-23 agenda item 1.2. |  |  |
| Abstract: At the November 2020 meeting, WP 5B sent a reply liaison statement to WP 5D which informs an update on Recommendations ITU-R M.1465 and M.1796. This contribution proposes a further reply from WP 5B to WP 5D on AI 1.2. |  |  |
| Fact Sheet Preparer: Dominic Nguyen |  |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: WRC-19 Document 5D/1192 Subject: WRC-23 agenda item 1.2 | Document 5B/XX |
|  | XX May 2021 |
|  | English only |
|  | SPECTRUM ASPECTS |

United States of America

REPLY LIAISON STATEMENT TO WORKING PARTY 5D

# 1	Introduction

The United States of America proposes a further reply liaison statement from WP 5B to WP 5D on AI 1.2.

Attachment revisions are presented for consideration.

Working Party 5B

REPLY LIAISON STATEMENT to working party 5D

WRC-23 agenda item 1.2 (Sharing and compatibility studies between IMT systems in 3 300-3 400 MHz and radiolocation systems in 3 100-3 400 MHz)

Working Party (WP) 5B thanks WP 5D for the liaison statement (5B/156). In WP 5B’s previously provided liaison statement (5D/398), WP 5B provided updates on Recommendation ITU-R M.1465, “Characteristics of and protection criteria for radars operating in the radiodetermination service in the frequency range 3 100-3 700 MHz.”

WP 5B would like to provide the following guidance on future complementary studies relative to sharing and compatibility between IMT systems and radiolocation systems.

- Recommendation ITU-R M.1851, “Mathematical models for radiodetermination radar systems antenna patterns for use in interference analyses” should be used as an antenna pattern reference for land-based and ship borne radars, as well as airborne radars.

- When applying clutter loss in sharing studies as per Recommendation ITU-R P.2108, WP 5B recommends that clutter loss needs to be applied for IMT sites where their antennas are deployed below rooftop as given below as per Table 3-1 of IMT-2020 characteristics being developed for WRC-23 (Document 5D/TEMP/228 Attachment 4.5 of Chairman’s report “Working Document on Characteristics of Terrestrial Component of IMT For Sharing and Compatibility Studies in Preparation For WRC-23”)

  - None of IMT sites deployed in sub-urban are below rooftop of buildings.

  - 50% of IMT sites deployed in urban are below rooftop of buildings.

- Similar to liaison statement Document 5D/1192 which was sent during the WRC-19 preparation cycle from WP 5B to WP 5D, it is being advised that.

  - Percentage parameter (p) to be used in Recommendation ITU-R P.452 should consider values lower than 10%, e.g., 1%, to ensure a broader range of propagation assumptions are represented. The percentage parameter (p) feeds into Recommendation ITU-R P.452 depends on the incumbent protection criteria.

  - Percentage parameter (p) to be used in Recommendation ITU-R P.528 should consider values lower than 10%, e.g., 1%, to ensure a broader range of propagation assumptions are presented. The percentage parameter (p) feeds into Recommendation ITU-R P.528 depends on the incumbent protection criteria.

A similar liaison statement to WP 5D will be sent later from WP 5B addressing the 10-10.5 GHz band once the IMT characteristics have been finalized.

WP 5B looks forward to continued collaboration with WP 5D on the progress of WRC-23 agenda item 1.2.

| Status: For information and action as appropriate |  |
| --- | --- |
| Contact: | E-mail: |

______________

____________________