| US Radiocommunication Sector FACT SHEET |  |  |  |
| --- | --- | --- | --- |
| Study Group: USWP 5B |  | Document No: USWP5B-26-FS |  |
| Reference: Document 5B/355 Annex 27 |  | Date: 15 September 2021 |  |
| Document Title: Updates to Working document towards a preliminary draft new report ITU-R M.[FOD_EESS_SHARE] |  |  |  |
| Authors | Telephone |  | E-Mail |
| Daniel Bishop, NASA Josh Whitlinger, NASA (ADS) Michael Gasper, NASA | 216-433-5220 216-433-6288 216-433-3881 |  | daniel.w.bishop@nasa.gov joshua.m.whitlinger@nasa.gov michael.r.gasper@nasa.gov |
| Purpose/Objective: Propose updates to Working document towards a preliminary draft new report ITU-R M.[FOD_EESS_SHARE], building upon discussions and proposals at the May 2021 WP 5B meeting. |  |  |  |
| Abstract: This contribution seeks to further this work by updating the studies between FOD detection radars in the 92-100 GHz frequency range, to account for updated characteristics provided at the last meeting of WP 5B, and EESS (passive) in 86-92 GHz and EESS (active) in 94-94.1 GHz. |  |  |  |
| Fact Sheet Preparer: Michael Gasper, NASA |  |  |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Subject: FOD | Document 5BC/XXX-E 2021 English only |
| United States of America |  |
| Updates to Working document towards a preliminary draft new report ITU-R M.[FOD_EESS_SHARE] |  |

Introduction

This contribution proposes to provide updates to Annex 2 of Preliminary Draft New Report ITU-R M.[FOD_EESS_SHARE], which contains compatibility studies between FOD and EESS (passive). Specifically, this contribution will incorporate changes to FOD characteristics and update the analyses accordingly.

Modifications against the existing text in the Chairman’s Report are shown in track changes.

Attachment: 1

[Content prior to section A2-1.2 remains unchanged in this draft.]

[USA Note: Seek comment on dynamic simulations for FOD into EESS (active). Given that this is a system provided by Japan, should a world-wide deployment be considered for a study of EESS (active)?]

[…]

## A2-1.2 	Dynamic analysis based on interference observed with spacecraft orbit simulation

The analysis will be conducted in which the orbit of the EESS (passive) spacecraft under investigation is dynamically simulated, retaining only the data points when the EESS (passive) sensor antenna boresight points within a defined Measurement Area of Interest (MAI), as defined in Recommendation ITU-R RS.2017. Calculations will be performed to determine the potential interference from each of the FOD detection radars into the EESS (passive) sensors under study and will consider the aggregate effect from multiple FOD transmitters. The simulation will propagate the satellite based on its orbital parameters, and the simulation step size is selected to be an irrational number to ensure that the beam dynamics of the passive sensor do not exhibit periodic behavior. At each simulation step, a snapshot of the interference scenario will be generated where the directional vectors from each FOD source to the EESS (passive) sensor will be computed along with the gain of the transmit and receive antennas using their respective antenna patterns.

The interfering signal power level,  Ii,n (dBW/100 MHz), received by a spaceborne radiometer at the nth simulation step from the ith terrestrial source is calculated from:

Ii,n[dBW/100 MHz]= PTX i,n+GTX i,n+ GRX i,n- LFSPL i,n-LMisc i,n#1

where:

PTX i,n:		peak terrestrial source transmitter out-of-band emission power (dBW/100 MHz);

GTX i,n:		terrestrial source antenna gain towards spaceborne sensor (dBi);

GRX i,n:		spaceborne radar antenna gain towards terrestrial source (dBi);

LFSPL i,n: 	Free Space Path Loss (dB);

LMisc i,n:		other losses considered (dB).

The aggregate interference at the nth simulation step, AggIn (dBW/100 Hz), is calculated by the linear summation of the received interference from all i transmitting, terrestrial sources within line of sight of spaceborne radiometer under consideration:

AggIn[dBW/100 MHz]=log10iIi,n[W/100 MHz]#2

I

Using the resulting data containing received interfering power levels, CCDF curves will be generated to assess interference observed over the MAI. the percentage of time for which violations of the limit specified in Recommendation ITU-R RS.2017-0 occurred. From Recommendation ITU-R RS.2017-0, the following limits should hold:

–	For frequency band: 86-92 GHz, reference bandwidth: 100 MHz,

•	Maximum interference level: −169 dBW,

•	Percentage of area or time permissible interference level may be exceeded: 0.01%. The area analyzed should be 2 000 000 km2.

Recommendation ITU-R RS.1861-0 contains the operating characteristics for the 86-92 GHz and the 100-102 GHz band EESS passive sensors.

### A2-1.2.1	Deployment of foreign object debris detection systems

For this initial analysis, the FOD detection systems were assumed to be deployed along the runways of the airports that are classified as large hubs. A large hub is defined as one with greater than 1% of total annual passenger boarding annually. This criterion results in deployment at 1119 runways globally.

This initial analysis considers a deployment over Japan. The FOD detection systems have a down tilt focused on the runways. The FOD detection systems are placed every 200 meters along the runways. In the case where the runway was not a multiple of 200 m, a ceiling function was used to ensure that the entire runway surface would be covered for detection of foreign objects.

### A2-1.2.2	Characteristics of foreign object debris detection systems

The parameters of the FOD detection systems used for this initial analysis are summarized in Table X, with parameters taken from Table 1. The received interference is calculated based on the aggregate power received by all FOD detection systems. The analysis uses the FOD out-of-band power within the 86-82 GHz EESS (passive) band.

[USA Note: Power and spacing taken from liaison statement to 7C, document 7C/204]

Table X

Characteristics of FOD detection system networks

| Parameter | Value |
| --- | --- |
| Maximum out-of-band emission power | -50 dBm/MHz |
| Antenna gain | 44 dBi |
| Antenna Pattern | F.699-8 |
| Antenna Height | 7 m |
| Scan Rate | 15 RPM |
| Antenna Elevation Angle | -1.8° |
| Radiated Rotation Angle (azimuth scan) | +/- 60° |
| Spacing | 200 m |

#### A2-1.2.3	Characteristics of Earth exploration satellite service (passive) system

Relevant information on typical technical and operational characteristics of systems operating in the Earth exploration satellite service (EESS) (passive) systems using allocations between 1.4 and 275 GHz can be found in Recommendation ITU-R RS.1861-0, which is currently under revision in WP 7C [see Document TBD].

Recommendation ITU-R RS.2017-0 contains the performance and interference criteria for satellite passive remote sensing. The following limits should hold

for frequency band 86-92 GHz.

•	Maximum interference level: −169 dBW/100 MHz

•	Percentage of area or time permissible interference level may be exceeded: 0.01%. The area analyzed should be 2 000 000 km2.

#### A2-1.2.3.1	Earth exploration satellite service (passive) sensor area of interest test cases

As described in Section A2-1.2.1, this study will focus on the airports inside of Japan. Figure X shows the measurement area considered along with the FOD deployment.

FIGURE X

FOD deployment in the EESS (passive) MAI

[TBD]

### A2-1.2.4	Simulation parameters and results

#### A2-1.2.4.1	General simulation parameters

The following table gives the relevant aspects of the simulation.

Table X

General simulation parameters

| Parameter | Units | Value |
| --- | --- | --- |
| Duration | days | 20 |
| Time step | s | 0.1*π |
| Atmospheric losses |  | P.676 |
| Polarization losses | dB | 3 |

#### A2-1.2.4.2	Simulation results

# [TBD]

# A2-2	Summary of results

[TBD]

______________

____________________