| US Radiocommunication Sector FACT SHEET |  |
| --- | --- |
| Working Party: ITU-R WP 5B | Document No: USWP5B28-06 |
| Reference: 5B/481 Annex 31, 5B/255 Annex 6 | Date: 23 February 2022 |
| Document Title: Working document towards a draft CPM text for WRC-23 agenda item 1.10 |  |
| Author(s)/Contributor(s): Andrew Meadows Air Force Dominic Nguyen eSimplicity for AFSMO Daniel Bishop, NASA Ryan S. McDonough, NASA | Phone: 334-467-4720 E-mail: andrew.meadows.1@us.af.mil Phone: 703-606-7394 E-mail: dominic.nguyen@esimplicity.com daniel.w.bishop@nasa.gov Ryan.S.McDonough@nasa.gov |
| Purpose/Objective: The purpose of this document is to initiate the WD towards a draft CPM text for WRC-23 AI 1.10. |  |
| Abstract: WRC-19 approved AI 1.10 for the WRC-23 study cycle to consider a possible introduction of new non-safety AMS applications in the 15.4-15.7 GHz band. This contribution initiates the WD towards a draft CPM text for WRC-23 AI 1.10. |  |
| Fact Sheet Preparer: Dominic Nguyen, Ryan McDonough |  |

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 5B/481 Annex 31, 5B/255 Annex 6 Subject: WRC-23 agenda item 1.10 | Document 5B/XX |
|  | XX March 2022 |
|  | English only |
| United States of America. Working document towards a draft CPM text for WRC-23 agenda item 1.10. # 1 Introduction The United States of America would like to initiate the development of a working document towards a draft CPM text for WRC-23 agenda item 1.10. The intent of this contribution is to aid Working Party (WP) 5B as the responsible group in progressing the work on draft CPM text and should not be viewed as the United States of America supporting one method over any other at this time. Attachment: 1 |

ATTACHMENT

CHAPTER 2

Aeronautical and maritime issues

(Agenda items 1.6, 1.7, 1.8, 1.9, 1.10, 1.11)

Agenda item 1.10

(WP 5B* / WP 3K, WP 3M, WP 4A, WP 5A, WP 5C, WP 7C, WP 7D)

1.10	​​to conduct studies on spectrum needs, coexistence with radiocommunication services and regulatory measures for possible new allocations for the aeronautical mobile service for the use of non-safety aeronautical mobile applications, in accordance with Resolution 430 (WRC-19);

Resolution 430 (WRC-19) – Studies on frequency-related matters, including possible additional allocations, for the possible introduction of new non-safety aeronautical mobile applications.

# 2/1.10/1		Executive summary

This agenda item seeks possible new aeronautical mobile service allocations for the use of non-safety aeronautical mobile applications. To address this agenda item, ITU-R has undertaken studies, pursuant to Resolution 430 (WRC-19), on frequency-related matters for the possible introduction of new non-safety aeronautical mobile applications, including consideration of possible additional allocations, (see section 2/1.10/3).

The following methods are considered to answer this agenda item:

–	Method A: NOC

# 2/1.10/2		Background

[Text of the background, not more than half a page of text to provide general information in a concise manner, in order to describe the rationale of the agenda items (or issue(s)). See also § A2.2 of Annex 2 to Resolution ITU-R 2-8]

The frequency range 15.4-15.7 GHz is widely used by the radiolocation and aeronautical radionavigation services for critical applications with a significant investment in airborne radar applications within this band. ITU-R past studies show sharing between RLS and AMS could be difficult, requiring extremely large separation distances. Additionally, the sub-band 15.43-15.63 GHz is allocated to the fixed-satellite service (Earth-to-space) on a primary basis for use by feeder links of non-geostationary systems in the mobile satellite service. The aeronautical radionavigation service in the 15.4-15.7 GHz band is used for landing systems and unmanned aircraft detect and avoid systems. An ITU-R Recommendation is currently being developed to provide characteristics and protection requirements for these aeronautical radionavigation systems (Document 5B/481 Annex 18). The sharing studies for the 15.4-15.7 GHz band should take into account the characteristics and protection requirements being developed and the airborne mobile nature of the aeronautical radionavigation systems that operate in the 15.4-15.7 GHz band.

The 22-22.21 GHz frequency band under consideration is adjacent to the 22.21-22.5 GHz frequency band allocated to the EESS (passive). The 22.21-22.5 GHz frequency band allows for remote sensing observations near an H2O absorption line that is essential not only for measuring atmospheric water vapor, but also for reducing error in other geophysical parameters due to the presence of water vapor. Therefore, adjacent band studies are required to ensure protection of the EESS (passive) in the 22.21-22.5 GHz frequency band.

# 2/1.10/3		Summary and analysis of the results of ITU-R studies

[This section should contain a summary of the technical and operational studies performed within ITU-R, including a list of relevant ITU-R Recommendations. Depending on the agenda item, this section could be divided in two parts, one part dealing with the summary and the other part dealing with the analysis. The results of the ITU-R studies should also be analysed with respect to the possible methods of satisfying the agenda item and presented in a concise manner.]

## 2/1.10/3.1		Relevant ITU-R Recommendations and Reports

The relevant ITU-R Recommendations are:

Characteristics and protection criteria: F.758-7, M.1730-1, M.2089-0, M.2114-0, M.2115-0, M.2116-0, M.2120-0, RA.517-4, RA.769-2, RS.1028-2, RS.1029-2, RS.1861-0, RS.2017-0, S.1340-0, and S.1341-0.

Antenna patterns: F.699-8, F.1245-3, F.1336-5, M.1851-1, RA.1631-0, RS.1813-1, S.465-6, S.580-6, S.732-1, and SA.509-3.

Propagation models and others: F.637-4, M.1461-2, M.1825-0, P.452-17, P.528-5, P.619-4, P.1409-2, RA.1513-2, SA.510-3, SM.337-6, and SM.1541-6.

The relevant sharing studies ITU-R Reports are: M.2170-0, M.2229-0, M.2230-0, and RA.2131-0.

To perform studies required under agenda item 1.10 and Resolution 430 (WRC-19), developed Report ITU-R M.[NON-SAFETY AMS CHARACTERISTICS AND SHARING STUDIES].

## 2/1.10/3.2		Summary of the sharing and compatibility studies

## 2/1.10/3.2.1	Radioastronomy operating in the band 15.35-15.4 GHz

## 2/1.10/3.2.2	Radiolocation operating in the band 15.4-15.7 GHz

Sharing in the 15.4 – 15.7 GHz band should take into account the characteristics and protection requirements for these aeronautical radionavigation systems and the radiolocation systems that operate in the 15.4 – 15.7 GHz band. The current studies C in Doc 5B/481 Annex 31 show that sharing between non-safety AMS and radiolocation systems is not feasible due to a high transmitted EIRP limit from the non-safety AMS resulting in a minimum separation distance of [800] km.

## 2/1.10/3.2.3	Aeronautical Radionavigation operating in the band 15.4-15.7 GHz

## 2/1.10/3.2.4	Fixed-Satellite operating in the band 15.43-15.63 GHz

## 2/1.10/3.2.5	Broadcasting-Satellite operating in the band 21.4-22 GHz

## 2/1.10/3.2.6	Fixed Service operating in the band 22-22.21 GHz

## 2/1.10/3.2.7	Mobile Service operating in the band 22-22.21 GHz

## 2/1.10/3.2.8	Radioastronomy operating in the band 22.21-22.5 GHz

## 2/1.10/3.2.9	Earth Exploration Satellite Service (passive) operating in the band 22.21-22.5 GHz

Sharing in the 22.21-22.5 GHz band should take into account the characteristics and protection requirements for these Earth Exploration Satellite Service (passive) systems that operate in the 22.21-22.5 GHz band. The current studies A in Doc 5B/481 Annex 31 section A2.3.3 show that sharing between non-safety AMS and EESS (passive) systems is not feasible for some proposed AMS scenarios such as in 4.2 and 4.5 due to the anticipated out-of-band emission level from the non-safety AMS operating in 22-22.21 GHz into EESS (passive) sensors operating in 22.21-22.5 GHz. Deployment densities within 22.16-22.21 GHz near to the EESS (passive) band must be maintained by incorporating a guard band designed to limit out-of-band emissions into EESS (passive).

## 2/1.10/3.2.10	Space Research operating in the band 22.21-22.5 GHz

# 2/1.10/4		Methods to satisfy the agenda item

This section should contain the brief description of the Method or Methods to satisfy the agenda item as per Section A2.4 of Annex 2 to Resolution ITU-R 2-8

## 2/1.10/4.1		Method A:

## No changes to the Radio Regulations for the band 15.4-15.7 GHz.

NOC

ARTICLE 5

Frequency allocations

Section IV – Table of Frequency Allocations
(See No. 2.1)

# 2/1.10/5		Regulatory and procedural considerations

2/1.10/5.1		For Method A: No change to Radio Regulations

NOC

Reason: Sharing studies between non-safety AMS and the incumbent services had been performed, the results show that sharing is not feasible with the non-safety AMS. Therefore, no change is proposed for the 15.4 – 15.7 GHz frequency band.

SUP

RESOLUTION 430 (WRC-19)

Studies on frequency-related matters, including possible additional allocations, for the possible introduction of new non-safety aeronautical mobile applications

[Additional sections with example(s) of regulatory text for the other methods to satisfy the agenda item, if any]. [To be populated later]

* Note: See relevant text in CPM23-1 meeting report (Annex 4 to BR Administrative Circular CA/251) on how to facilitate the work related to satellite.